SCHEDULE 2 - ANNEX B - SERVICE REQUIREMENT DOCUMENT

703236450 - BULK FUEL CARRYING VEHICLES,

TESTING, CLEANING, CALIBRATION, CERTIFICATION AND REPAIR (BFCVTCR)

Introduction

1.	The Authority has a legal requirement to carry out testing, and certification at mandated intervals and post repair of all Bulk Fuel Carrying Vehicles (BFCV) listed at Appendix 1 to this SRD.

Requirement

2.	ADR requires Fuel (FL) Vehicles to be subjected to elapsed time Intermediate (Leakproof) and Periodic (Hydraulic) examinations.  These are to be conducted by Examination Bodies accredited by the Competent Body of the EU country in which the vehicle or equipment is operating, assuming that country is a signatory to the regulation.  In addition to the periodic testing it also defines the requirement and safety precautions for gas freeing, tank cleaning, changing of grade/product and FL tank repair.

3.	Compliance with the European Regulation is mandated for the operation of all Ministry of Defence (MOD) FL vehicles.

Providers Technical Requirements

4.	The Contractor shall carry out the requirements whilst meeting the following technical capabilities for the delivery of BFCVTCR in Germany.

a.	Hold Special Tools and Test Equipment certified to complete the Periodic and Intermediate tank tests as detailed within Appendix 1.

b.	Hold the appropriate calibrated Special Tools and Test Equipment to conduct calibration and repair of pumping systems.

c.	Maintain testing facilities with the accredited filtration capture systems for fuel systems up to 20,000 litres.

d.	Accreditation as an Approved Provider by the National Body under ADR Regulations (including International Maritime Dangerous Goods Code (IMDG) in accordance with ADR Regulations.

Providers Legislative Requirements

5.	The Contractor (including any sub-contractor) shall:

a.	Safety, Health, Environmental (SHE).  Ensure full compliance in the performance of the Contract with the requirements of all current and relevant legislation, local standards and regulations for the conduct of this activity in Germany with regard health and safety. This applies to all testing, certification and includes the handling and safe use of petroleum products and when conducting chemical cleaning the safe and environmental approved disposal of waste product.

b.	Staff.  The Service shall be managed and delivered by suitably competent, qualified, experienced and empowered staff; similarly, qualified deputies should be nominated for periods of absence.  All staff that deal directly with the Authority must be able to fluently speak, read and write in the English language.

c.	Quality Assurance.  The Contractor shall have in place Quality Management and Assurance processes that ensure that the Service is delivered in accordance with the Contract. The Contractor shall have ISO 9001:2015 Accreditation or be able to achieve it within 6 months from the date of Contract award for the complete scope of activities pertaining to the Contract.

Authorities Requirements

6.	The Contractor (including sub-contractor(s)) shall meet the following requirements:

a.	Tank Testing.

(1)	Deliver the Periodic (5 year) and Intermediate (2.5 year) tank test and examination in accordance with ADR and IMDG Regulations.

(2)	Carry out repairs and maintenance on the vehicle dispensing/pumping systems listed within Appendix 1.

(3)	Conduct minor Bulk Fuel tank repairs using authorised methods for BFCVs.

(4)	Conduct chemical cleaning for the equipment’s listed within Appendix 1 for entry in to Controlled Humidity Environment (CHE) or as required for shipping in a non-hazardous manner.  Cleaning Certificate, at Appendix 4 to this SRD, to be completed.

b.	Government Furnished Equipment (GFE).  The Contractor shall have systems in place to store and manage the GFE issued by the Authority for the delivery of this contract in accordance with DEFCON 611 and 694.

c.	Security and Storage.  The Contractor shall provide secure storage space, and manoeuvring area, to accommodate up to 5 Authority vehicles listed at Appendix 1 at any one time at all delivery sites being used whether prime or sub-contracted.

d.	Turn Around Times.  The Contractor shall be able to complete:

(1)	Testing and Certification of up to 2 BFCVs per week and, with 5 days’ notice, up to 12 BFCVs within 30 days.

(2)	From the point of being tasked by the Authority, repair of up to 1 BFCV per week if the vehicle is in addition to the requirement listed in 6.d.(1) above.

Note:  These figures are not a commitment on the part of the Authority to actually submit the volume of work to the Contractor.

e.	Locations.

- The Contractor shall deliver the requirement within 50 km of:

Land Training Fleet (Sennelager)

Husaren Strasse 155

33104 Paderborn

Germany

(2)	An ‘Out of Area’ capability is also required for ad-hoc repair and maintenance throughout mainland Europe.  The primary locations may include, but not be limited, to:

(i)	SEF(G)

Ayrshire (Süd) Kaserne

Gladbacher Straße 560

41179 Mönchengladbach

Germany

(ii)	Grafenwöhr Joint Military Training Centre

Tower Kaserne

Lexington Avenue 619

92655 Grafenwöhr

Germany

(iii)	Hohenfels Joint Multinational Readiness Centre

General Patton Road
92366 Hohenfels

Germany

(iv)	Bergen Hohne Training Area

Brucknerstr.3

29303 Bergen

Germany

Contract management

7.	6 monthly contract management meeting at contractors’ premises and 3 monthly reviews of KPI’s per tasking. Meetings to include Engineering Assessor, C2 Head of CRD and CRD desk Officer. Reporting issues back to Army Commercial.

.

703236450 – Schedule 2, Annex B, Appendix 1

BULK FUEL CARRIER WEIGHTS AND DIMENSIONS

FUEL DISPENSING RACK (FDR)

MAN SV 9T UST MM

| MAX LADEN WT MIN UNLADEN WT | 22140 kg (48810 lb) 15715 kg (34650 lb) |
| --- | --- |
| TURNING CIRCLE | 21.2 m (69.5 ft) |
| GROUND PRESSURE | 320 kN/m2 (46.4 lb/in2) |
| BRIDGE CLASS | MLC 23 |

MAN SV 9T UST IMM

| MAX LADEN WT MIN UNLADEN WT | 21800 kg (46690 lb) 15015 kg (32160 lb) |
| --- | --- |
| TURNING CIRCLE | 20.9 m (68.5 ft) |
| GROUND PRESSURE | 320 kN/m2 (46.4 lb/in2) |
| BRIDGE CLASS | MLC 23 |

Trailer – Semi 20000 Litre Fuel Tanker (Close Support Tanker)

| MAX LADEN WT MIN UNLADEN WT | 20000 kg (44092 lb) 9904 kg (21835 lb) |
| --- | --- |
| GROUND PRESSURE | 174 kN/m2 (25.2 lb/ft2) |
| BRIDGE CLASS | UNARMOURED TRACTOR MLC 35 ARMOURED TRACTOR MLC 37 |

Trailer – Semi TAR 15000 Litre Fuel Tanker (Tactical Aircraft Refuelling)

| MAX LADEN WT MIN UNLADEN WT | 23954 kg (52809 lb) 10712 kg (23615 lb) |
| --- | --- |
| GROUND PRESSURE | 152 kN/m2 (21.9 lb/ft2) |
| BRIDGE CLASS | UNARMOURED TRACTOR MLC 33 ARMOURED TRACTOR MLC 35 |

703236450 – Schedule 2 - Annex B - Appendix 2

703236450 – Schedule 2, Annex B, Appendix 3

Cleaning Certificate/Reinigungszertifikat

Nach Army Equipment Support Publication 2320-A-100-522

1.	Dieses Zertifikat bescheinigt, dass die Ausrüstung an dem unten genannten Standort den Bedingungen laut AESP 2320-A-100-522 entspricht und daher als SAUBER eingestuft wird.

Ort:						Zertifikat-Nr.:

Einheit/Depot:				            Datum:

Fahrzeugkennzeichen:				   Uhrzeit:

Tanktyp:________________________        Tankidentifikationsnr.:__________________

2.	Anzahl der Abschnitte:

3.	Produkt	Abschnitt 1 _______________  Abschnitt 2 __________________

Abschnitt 3 _______________  Abschnitt 4 __________________

Abschnitt 5 _______________  Abschnitt 6 __________________

Abschnitt 7 _______________  Abschnitt 8 __________________

4.	Das Pumpensystem und die dazugehörenden Leitungen wurden vor Anfang der Reinigung abgelassen.

5.	Die Koaleszierfilterelemente (falls montiert) wurden entfernt und neue Elemente im Fahrerabteil platziert, die Filterbehältertür wurde im offenen Stand gesichert.

6.	Reinigungsmethode: _______________________________________________

7.	Reinigung wurde durchgeführt am: ____________________________________

8.	Die Pumpenanlage und das Leitungsnetz wurden/wurden nicht* mit Konsovierungsflüssigkeit aufgefüllt.  (nur bei Lagerung)

* Nicht zutreffendes streichen.

9. Art der Konsovierungsflüssigkeit wuerde verwendet / nicht verwendet in die Pumpanlage oder Rohrleitung                                __________________

10.	Der von mir inspizierte o.a. Tank, einschl. Schläuche und Ventile, wurde nach meiner Zufriedenheit gereinigt und ist gasfrei.

Unterschrift ________________________________

In Vertretung _______________________________

(Stempel der Kompanie oder Einheit)

703236450 – Schedule 2, Annex B, Appendix 4

| NATO FORWARD HOLDING BASE (Sennelager) CRD, Athlone Barracks, Sennelager BRITISH FORCES POST OFFICE 16 Tel: 0049 5254 982 2458 Email: Desk Officer geofrey.turtle317@mod.gov.uk E-Mail: Engineering Assessor malcolm.beresford788@mod.gov.uk |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| LTF(S) Job No: | ELECTRICAL CONTINUITY, BONDING, INSULATION & MISCELLANEOUS CHECKSHEET | Registration No: |  |  |  |  |
| A = Satisfactory. B = Marginal. C = Unsatisfactory (FAIL). D = Not Applicable A B C D |  |  |  |  |  |  |
| Check fuses, fuse holders and other protection devices for correct rating, damage and exposed links. |  |  |  |  |  |  |
| Examine Battery, battery cover and protective shrouds (where fitted) |  |  |  |  |  |  |
| Examine junction boxes, conduit joints connectors and all electrical fittings for ingress of water, mechanical (chafing, trapping of cables etc), or heat damage. |  |  |  |  |  |  |
| Ensure that vehicle interlock systems (where fitted) including override devices function correctly. |  |  |  |  |  |  |
| Ensure master switch is clearly visible and the ON/OFF positions clearly marked. |  |  |  |  |  |  |
| Ensure correct operation of master switch including the emergency remote switches (if fitted). |  |  |  |  |  |  |
| Where combustion heater is fitted ensure: - The heater cannot be used when the transfer pump is operated from the PTO during loading and unloading. - The heater can be intentionally switched off from within the driver's cab. - The heater automatically switches off by stopping the engine. The heater may be restarted afterwards manually by the driver. - The heater can only be switched ON by a separate switch in the cab. - There must be a warning plate fitted adjacent to the heater stating “EXPLOSION HAZARD. THE HEATER MUST NOT BE SWITCHED ON AT LOADING AND UNLOADING PREMISES OR DURING LOADING AND UNLOADING PROCESSES”. |  |  |  |  |  |  |
| If the vehicle is electrically wired with an insulated return, ensure there is a notice installed in a prominent place in the cab stating: “CAUTION. THE ELECTRICAL SYSTEM ON THIS VEHICLE IS INSULATED EARTH RETURN. EARTH RETURN COMPONENTS MUST BE USED”. |  |  |  |  |  |  |
| Check operation of the emergency stop switches. |  |  |  |  |  |  |

The following form shall be used for the electrical continuity, bonding, insulation & miscellaneous tasks

The following form shall be used for the electrical continuity, bonding, insulation & miscellaneous tasks.

| LH RH A B C D |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Continuity | Earthing Leads - Rear (Cabinet) Aircraft bonding leads |  |  |  | 10 Ω Max 1 Ω Max |  |  |  |  |  |
| Continuity | Earthing Leads - NS (Cabinet ) |  |  |  | 10 Ω Max |  |  |  |  |  |
| Continuity | Earthing Leads - OS (Cabinet ) |  |  |  | 10 Ω Max |  |  |  |  |  |
| Continuity | Reel hose bonding cable/clip |  |  |  | 1 Ω max |  |  |  |  |  |
| Continuity | Boom nozzle cable/clips |  |  |  | 1 Ω max |  |  |  |  |  |
| Continuity | Grounding Spike - OS - NS |  |  |  | 10 Ω max |  |  |  |  |  |
| Continuity | Earthing Clamp – OS - NS |  |  |  | 10 Ω Max |  |  |  |  |  |
| Continuity | Tractor/Trailer combination. Maximum resistance between the furthest points of the tractor/trailer |  |  |  | 10 Ω Max |  |  |  |  |  |
| Continuity | Trailing Rubber Strips (where fitted) 70 mm ground contact |  |  |  | 1 M Ω at 500v dc |  |  |  |  |  |
| Continuity | Ensure all static and earth bonding leads are connected to the chassis. |  |  |  | 10 Ω Max |  |  |  |  |  |
| Continuity | Check conductive tyres (if fitted) between chassis and an earth spike 6M from equipment |  |  |  | 1 M Ω at 500v dc |  |  |  |  |  |
| Insulation | Boom Hoses |  |  |  | 2.0 M Ω at 500 d c |  |  |  |  |  |
| Insulation | High - Low Pressure reel hose |  |  |  | 2.0 M Ω at 500 d c |  |  |  |  |  |
| Insulation | Insulated return. Check for current leakage to vehicle chassis on both positive and negative circuits (Master Switch On). Note: Disconnect Tractor Unit from Trailer and Reconnect Lighting Cables. (CST/TAR) |  |  |  | Not to Exceed 30mA Note: Vehicles fitted with an earth leakage monitoring system, incorporating filament lamps permanently on at half brilliance, are to have the lamps removed prior to this check being carried out to avoid false readings. |  |  |  |  |  |
| I certify that the above tests have been carried out and any necessary corrective action has been *completed / has not been completed. In my opinion the above tests conform -* do not conform with the requirements AESP 2302-A-100-522 Chapter 7. |  |  |  |  |  |  |  |  |  |  |
| Name: |  | Signature of Examiner: |  |  |  | Date: |  |  |  |  |

Accord European Relatif au Transport International du Merchandis Dangereuses par Route or when translated in to English European Agreement Concerning the International Carriage of Dangerous Goods by Road (ADR)

TüV Nord for Nord Rhein Westfalen.

All Testing and certification staff are to have in date certification from the Accredited Body.

Free from product.

This site is where other vehicle repairs are conducted by the Authority prior to any testing, certification or repair work being carried out by the Contractor.

Cleaning certificate confirms with AESP 2320-A-100-522 Edn:7 Last update 2018/07/25