Supply Chain

HSEQ Requirements

Document Control Sheet

| DOCUMENT TITLE: | Amey Highways - Supply Chain HSEQ Requirements |
| --- | --- |
| DOCUMENT NUMBER: | Amey-Highways-Supplychain-PR-01 |

| Status/Amendment | Prepared | Reviewed | Approved |
| --- | --- | --- | --- |
| Version 001 Date: 2/06/2025 | Name: Amanda Felstead | (Health & Safety) Name: Doug McCron | Name: Joseph Docherty |

Contents

1. Scope	4

2. Purpose	4

3. Definitions and Notes	4

4. Requirements	4

4.1. Zero Code									4

4.2. Drug and Alcohol Testing							5

4.3. Site Security and Access Arrangements						5

4.4. Environmental and Sustainability (E&S)						5

4.5. Supplier’s General Arrangements						7

5. Quality and ITPs	9

5.1. Inspections									9

5.2. Quality control processes							10

6. Safe Systems of Work	10

6.1. Inspections									10

6.2. Underground Services								10

6.3. Work at Height									11

6.4. Delivery process									13

6.5. Plant Pedestrian Segregation							14

6.6 Storage of Materials								16

6.7. De-vegetation									16

6.8. Concrete Works									17

6.9. Excavations									17

6.10. CHS Driven Piles								18

6.11. Masonry										19

6.12. General Site Requirements							19

7. Events (Accidents and incidents)	20

8. Competence and Training	20

9. Tools, plant and equipment	21

10. Lighting and Temporary Electrics	23

11. Personal protective equipment	23

11.1. PPE requirements								23

11.2. CE UKCA Marking								26

12. Scaffolding and edge protection expectations	26

13. Lifting operations including use of Flexible Intermediate Bulk Containers (FIBCs)	27

14. Welfare and First Aid	28

15. Health	28

16. Fatigue	28

17. Activities requiring authorisation.	28

18. Temporary Works	29

19. Appendix A – Build UK Supervisor requirements.	29

20. Appendix B – Safety Critical Plant Requirements	32

21. Appendix C – Works Planning & Delivery of Vegetation Clearance	33

22. Appendix D – Working Hours – Policy and Arrangements	34

24. Appendix E – Amey ESG Strategy	35

# Scope

This standard applies to the procurement of any Supplier resources for Amey highways accounts, for work to be carried out on any Amey highways account.

# Purpose

To enable all contractors arriving at an Amey site to understand the expectations of Amey and include the provision of this service in its tender.

# Definitions and Notes

All references herein to “Supplier” shall, where applicable, be deemed to include all Contractors, Sub-Contractors, Trade or Works Contractors, Suppliers, Designers, Consultants and their employees or others under their control who have access to site.

All references to Principal Contractor or Principal Designer refer to the Principal Contractor/Designer as defined in the Construction (Design & Management) Regulations. The Principal Contractor for each contract will be nominated by the client and may, or may not, be Amey.

# Requirements

## Zero Code

- There are  four key principles of our Zero Code which must be followed to ensure our working conditions are always safe and to prevent any unacceptable standards or situations.

- Ready to Work

- Kitted Out

- Stick to the Plan

- Stay Alert

- Shout Out!

- We must look out for ourselves and those by affected by our work. Zero Code reminds us to reflect on our training, knowledge and instructions and assess equipment, tools and clothing before starting or continuing work. As we work in environments that can change very quickly, we must stay focused and vigilant.

- Everyone who works for Amey (directly or indirectly through our supply chain) has personal responsibility to understand, communicate and live by the Zero Code - with zero exceptions. Not following the Zero Code, means you choose not to work for Amey.

- The current version of the Zero Code video is available through this link  Zero Code Intro

- Let’s work together to ensure everyone who works with and for Amey returns home safe and well every day.

## Drug and Alcohol Testing

Amey recognises the negative impact alcohol or drugs, either recreational, prescription or over the counter, can have on an individual’s performance and as such reserves the right to carry out drug and alcohol testing on all staff and Suppliers, in line with our procedures, at the following times:

Pre-commencement/commencement – prior to commencing work on site.

Post Incident – following accidents and/or incidents.

With Cause Testing – testing undertaken when there is suspicion someone is under the influence of drugs or alcohol.

Random – as part of our commitment to randomly test 20% of our workforce (including supply chain) annually.

A refusal – If an employee refuses to consent or comply with the testing procedure this will be documented, and the employee will be asked to sign a declaration by the Hampton Knight Clinical Technician. The disciplinary policy must be implemented, managers must consult HR for guidance.

Any alleged non-cooperation, will be investigated. Where it is concluded that the employee or contractor did not reasonably cooperate with the arrangements to enable a drugs and alcohol test to occur, a positive result shall be recorded.

## Site Security and Access Arrangements

Amey will provide access/security systems e.g. turnstiles and access registers. The Supplier and their employees will comply at all times with the specific project procedure for access control. Random search of persons/vehicles entering or leaving the site may be instigated at the discretion of the Project Manager.

## Environmental and Sustainability (E&S)

Amey operates an Integrated Management System, certified to ISO14001:2015, the standard for Environmental Management Systems (EMS), in support of Amey's Environmental Policy statement.

Amey manages its supply chain to minimise any environmental impact and conserve natural resources. We require Suppliers to understand their impact on the environment, adhere to relevant legislation and to have in place necessary mitigation. We encourage and support Suppliers to maintain an EMS that is externally certified to the requirements of ISO 14001:2015.

As an Amey supplier, prior to contract commencement, you will be required to:

Develop an Environmental Policy and implement systems that support Amey's Environment Social and Governance (ESG) Strategy, and the relevant policies, procedures and contract specific environmental measures outlined within the Integrated Account Plan (IAP) (or specific Environmental and Social Management Plan), which may be over and above these minimum requirements.

Have access to competent advice on environmental issues, as required.

Comply with Amey and other relevant environmental policies.

Ensure environmental risks are identified, necessary controls communicated and where necessary, implemented.

Comply and where necessary provide evidence of compliance with relevant Permits, Licenses and Consents (PLC), objectives, targets, KPI’s and other compliance obligations.

Undertake required training, awareness, and internal communication activities.

Support communication with others, such as reporting complaints, close-calls and environmental incidents.

Keep noise, nuisance, and lighting to a minimum. Undertake activities in line with relevant Noise Risk Assessments, only undertake noisy works when permitted to do so, in line with relevant Best Practicable Means (BPM)

Reduce emissions to air, dust and odour to As Low As Reasonably Practicable (ALARP). Plant, equipment, and Non-Road Mobile Machinery (NRMM) shall comply with relevant emissions standards. Where necessary, dust and emissions control methods will be employed during works.

Report any potentially hazardous substances, contamination encountered during works, spills, or environmental incidents.

Only work near on, over or under controlled waters, flood plains, defences, coastal waters or navigations, and/or undertake abstraction, dewatering, storage and discharge of water with necessary permissions.

Prevent pollution through appropriate storage of hazardous substances, fuels, oils, construction materials and waste. Small plant, equipment and containers, on plant nappies, or in suitably bunded containers

Only trained/nominated persons shall undertake refuelling, located away from drains and watercourses. Maintain suitable spill kits and undertake regular inspections to identify the potential for pollution or other environmental risks.

Protected natural, designated, historic or protected sites or features near the work.

Protect wildlife and support local biodiversity and relevant Biodiversity Action Plans that are in place on the project, including supporting restoration measures.

Minimise water consumption through use of water efficient processes and water saving technologies and contribute to the project’s Water Reduction Plan

Identify opportunities to reduce carbon, by proposing low carbon solutions when designing, constructing, and maintaining highway infrastructure in accordance with PAS 2080 (Carbon Management in Infrastructure). Solutions will include options, such as:

Substituting high embodied carbon materials such as concrete to low carbon alternatives

Reducing transportation of people, plant, and materials, and reduce the number of road miles travelled for its products by efficient scheduling of deliveries.

Avoiding fossil fuel consumption and use low carbon, renewable and alternative power sources.

Using zero or low emission vehicles on our contracts

Energy efficiency plant, equipment, and solutions

Be members of the Fleet Operator Recognition Scheme (FORS) to improve standards on fuel efficiency, emissions, and Road Safety.

- Materials – Exclusions

Amey complies with UK REACH, which is a regulation that applies to chemical substances that are manufactured in, or imported into Great Britain and stands for Registration, Evaluation, Authorisation and Restriction of Chemicals.

Additionally, Amey has developed a list of materials that cannot be used in any circumstances for the delivery of any contract. Prohibited materials include:

Asbestos

CFCs and HCFCs

Volatile Organic Compounds.

This list will be expanded in line with legislation and validated research.

- Supplier of Timber and Timber Products

Our Suppliers are required to ensure that all timber products they use are legally and sustainably sourced and provide evidence of this to Amey.

Amey requires its Suppliers to only make use of timber from independently verified, legal and sustainable sources which meet the requirements of The Timber and Timber Products Placing on the Market Regulations (UKTR) and UK FLEGT Regulations

Certification from the Forest Stewardship Council or Programme for the Endorsement of Forest Certification may be requested by Amey. Forest management certification confirms that the source forest is being managed in a way that preserves biological diversity and benefits the lives of local people and workers; while ensuring it sustains economic viability.

- Packaging Suppliers

Use of packaging must be minimised and made from materials that can be reused, recycled, or recovered, wherever feasible. Amey requires its Suppliers to:

Minimise and seek to eliminate the use of single-use plastic throughout our supply chain.

Collect their materials and associated packaging for subsequent reuse, recycling, or recovery, wherever possible.

## Supplier’s General Arrangements

The Supplier will ensure that Amey has received the following information:

The Supplier’s Health & Safety, Environmental and Quality policies

The name of the Supplier’s appointed director(s) with overall responsibility for health & safety, environment, and quality matters

The name and CV and contact details of the Supplier’s competent person(s) appointed to assist in complying with health & safety, environment, and quality matters.

The name and CV and contact details of proposed key management personnel agreed.

A pre-commencement meeting will be held with every Supplier at which project specific requirements, including attendances, will be discussed, and agreed. The meeting will also review the detailed arrangements to ensure that the Supplier complies with the requirements of this document.

- CDM regulations

Suppliers will co-operate with Amey in discharging its responsibilities and hence they will:

Always comply with the site rules, emergency plan and procedures in force as set out in the Amey Integrated Account Plan (IAP) or Construction Phase Plan (CPP), or equivalent documents.

Ensure that Amey is provided with any information needed for the Health & Safety file and O&M manuals, in the agreed format

Provide detailed responses relating to hazards and risks highlighted by Amey at tender stage, and particularly those identified in the hazard assessment within the IAP/CPP

Indicate their process for managing RIDDOR reporting including for third parties and their supply chain.

Provide Amey with the following, prior to commencing on site and when changes are made, or additional information is available:

An agreed general method statement with activity risk assessments, which should include the following outputs:

Identify all occupational health and safety hazards, environmental aspects and quality issues associated with its activities. This should be based on both the Supplier’s specialist knowledge and the Hazard Matrix contained in the Amey IAP

Carry out an initial assessment of the above items, to identify those which are of significant risk, based on the likelihood of an incident occurring and its severity, should it occur.

Document formal risk assessments for all significant risks identified and the control measures, including monitoring procedures to be implemented, to minimise the risk of an incident occurring.

Carry out site specific CoSHH assessments (product data sheets alone are not acceptable), maintain records of information, instruction and training to operatives and provide details to Amey prior to using products on site.

Ensure a specific Point of Work Risk Assessment (POWRA) is undertaken for every task, every day or whenever there is a change to the task or environment.

Identify controls required to protect the health and safety of individuals working alone.

A list of activities to be sub-let and details of all proposed sub-tier contractors. The Supplier will be required to demonstrate the competence of those carrying out sub-tier works to Amey.

Completed supervisor’s competence pro-forma

A developed and completed Quality Plan and/or Inspection and Test Plans (ITPs)

A start on site pro-forma, completed for all Supplier’s personnel.

Schedule of weekly (minimum) Toolbox talks

Ensure that where Young Persons (person under the age of 18 years) are employed, either directly, or through Suppliers, you comply with your legal requirements for protecting the health and safety of those Young Persons whilst at work. The Supplier must undertake a specific Young Persons’ risk assessment, provide an appropriate level of supervision, and agree the arrangements with Amey prior to bringing any young person onto the project.

Ensure that where non-English speaking personnel are employed by them or their Supplier, they provide a competent interpreter to ensure that clear instruction can be given, and effective consultation can take place. The interpreter is also required to deliver translation of any pre-start engagement. The interpreter must be able to always provide direct competent communication with the non-English speaking workers. A suitable ratio, agreed at tender stage of not more than 8 non-English speaking personnel to one interpreter, is to be provided.

Provide suitable and sufficient site supervision to Amey's accepted standard, which should be initiated and agreed before commencement on site to control the works, apply, monitor and enforce the site rules with respect to health and safety, quality and environmental standards.

When work is being undertaken by the Supplier, they must ensure a competent person manager/supervisor is on site and available at all times to receive and implement safety instructions from Amey. The manager/supervisor is to attend the informal daily coordination briefing (daily 4Cs meetings (Confirm – Clear – Communicate – Control)) and will also be expected to attend the Amey Supervisors Start of Shift briefings  as requested by the Contract team.

Arrange for a professional safety adviser to conduct a safety inspection at agreed regular intervals (minimum monthly). A copy of the report is to be submitted to Amey immediately following the inspection. When Suppliers are visited by their in-house safety advisor or external safety consultant we require, in line with our safety culture and ‘Don’t Walk By’, that they review safety across the site, rather than restricting focus to the particular areas where their activities are taking place. This will ensure best use of the professional safety advice visiting the project

Have in place suitable arrangements to undertake health surveillance of all personnel at risk, appropriate to their work activities, e.g. Noise, Dust, Hand Arm Vibration Syndrome, Asbestos, Lead and Radiation

Ensure they have established suitable arrangements to monitor and manage the working hours of its personnel.

Be responsible for the removal, and legal and safe disposal, of its waste and surplus materials from the work area and the maintenance of a clean and tidy work area. Should Amey deem it necessary to intervene to maintain clean and tidy work areas, then the costs of this will be borne by the Supplier.

Use designated access routes for vehicles and pedestrians and adhere to the security arrangements imposed by Amey.

Ensure the use of mobile phones on construction sites or in other hazardous environments is restricted to emergencies and essential work-related calls only. Persons must go to a safe refuge, away from work activities or plant/traffic and remain in that location whilst receiving or making calls.

Carry out a fire risk assessment and comply with the Joint Code of Practice – fire prevention on construction sites. This is only applicable where Suppliers are responsible for the provision of temporary accommodation units.

Must comply with the Food Safety and Hygiene (England) Regulations 2013 or country equivalent , where Suppliers employ people to prepare food on the Amey site

Must comply with and adhere to all employer standards applicable to the subcontract works. The Supplier shall also comply with and adhere to all relevant British and European Standards, including quality and environmental standards applicable to the ssubcontract works.

# Quality and ITPs

## Inspections

The Supplier is required to operate a Quality Assurance System that complies with the requirements of Amey Quality Assurance System. This will include the development of ITPs to ensure that the work is compliant with health, safety, environmental and quality requirements.

ITPs must identify:

All the significant items required by the specification, drawings, and main Amey and Supplier’s requirements to make the work compliant.

What is being inspected?

The method of inspection

Acceptance criteria

How often it is to be inspected.

Who needs to inspect?

The record of inspection and where it is retained.

The plan must be a separate document.

Based on the ITP, the Supplier shall prepare check sheet(s) which must include ALL the inspections required by the ITPs. These sheets must be completed on an ongoing basis, as the items to which they relate are inspected, with the person signing off inspections confirming with name or initials. This can include other paperwork such as delivery tickets and other medium. The person responsible for sign-off must be suitably qualified and experienced.

## Quality control processes

- Non-conformance process

Any deficiencies or failures must be recorded, with an agreed course of action and closed out through a Non-conformance Report procedure, either the Supplier’s own or the Amey system.

- Audits

Amey will be given access to audit the works and the test and inspection records as required through the duration of the contracted works.

# Safe Systems of Work

## Inspections

Amey requires that Suppliers carry out and record regular and periodic documented safety inspections of both their workplaces and the plant/equipment used within the workplace. Evidence of these inspections is to be provided to Amey management and a visual record of inspections to be located at the workplace for safety critical plant and equipment (e.g. mobile towers, mobile elevating work platforms (MEWPs), scaffolds, harnesses etc)). Plant and equipment are to be clearly labelled to indicate who is responsible for managing the item in good order.

## Underground Services

Following lessons learned from a particular and all too frequent event occurring on our projects while driving pins into the ground for setting out purposes (e.g. for kerb alignment, road barriers etc.), we have banned this practice, to prevent buried cables being struck by steel pins. Any Supplier working with Amey must comply with our system: steel setting out pins must not be driven into the ground. Suitable alternatives are available, for example GRP pins or other solutions:

| Pinsafe offers a safe and innovative solution, which eliminates the risks involved with the use of existing equipment. It can be used on any degree of ground, e.g. camber of road and unmade ground. | Scrap steel cut and bent to make free standing kerb pins |
| --- | --- |
| Using a reinforced earth wall with Tensar blocks including the use of a short length of fibre glass rod can make an ideal blinding control pin. | Steel pins cast in a concrete haunch, set into excavated ground, and backfilled around. |

The only exception to this is where the pins form a fixed part of a proprietary system (e.g. the Varioguard traffic safety barrier).

Figure 1: Varioguard System with integral pins

Where such products are to be used, it must only be with the written consent of the Amey Account Director (or equivalent) and following a specific risk assessment, detailing how identification and avoidance of underground services will be managed.

This will require more than usual services detection measures e.g. CAT scan etc. The assessment must provide total assurance that no services are in the area; for example, by prior excavation under controlled conditions or raising ground levels by a suitable distance with a ‘clear’ substrate.

Cable avoidance tools must be used in accordance with formal training and manufacturer’s guidance.

Amey mandates the use of gCAT4 with Genny 4 Equipment as a minimum standard and must be capable of storing the information obtained during scan and records should be downloadable. Records may be required for spot checks or as part of incident investigation. The use of any other equipment other than gCAT4 for any persons out with general operatives may be considered on a case-by-case basis (for example a full-time Service Avoidance Technician).

## Work at Height

The Suppliers will identify within their method statements how they will justify and comply with the hierarchy of protective measures. They will also identify and justify the controls they will implement with regard to all working at height. When establishing the safe system of work, Amey work at height equipment selector must be used. Controls are to include a permit to access high level works where appropriate.

- Required controls.

- Where edge protection is installed, the Supplier will be expected to use best practice from the Edge Protection Federation guidance

Where mobile towers are used for access to work at height, there must be a minimum of one PASMA or equivalent trained person as one of the people using the tower. It is not acceptable to have e.g. one PASMA trained person overseeing use of multiple towers in use at the same time. Note: The standard PASMA training covers erection of towers on level ground. Where a tower is required on uneven ground, e.g. stairwell the user must have passed the Towers on Stairs course (1 day)

Manufacturer’s instructions must be available on site. Single width towers only allowed following Amey agreement.

Mobile Elevated Work Platforms – Must be procured in accordance with the process outlined in Appendix B. They should have anti-crush over the controls and “alarms” if obstructions are present.

Ladders/Steps are only to be used for ‘one-man jobs’ of short duration (no more than 10 minutes), with minimum tools and materials. Note: this should be considered as a last resort, covered by the issue of a ladder, authorisation/Tag/Checklist and must be “Class 1” for industrial use and under inspection/checks

Whilst not a principal means of controlling work at height risk, the Suppliers must establish and manage exclusion zones where there is a residual risk of falling objects as a result of their activities on site. Exclusion zones must be suitably robust, fenced, with adequate warning signage, have strictly controlled access (e.g. tally boards) and be fully maintained by the Supplier

When employed in work at height and where personal fall restraint or arrest equipment is used, it is essential that an effective rescue plan has been developed and that personnel receive training in rescue techniques and First Aid?

Where work is carried out within 2 metres of a leading edge then tool tethering must be used.

  - When there is a foreseeable risk of items dropping causing damage to property and persons

  - When working from a MEWP

  - All Amey employees, contractors and subcontractors must:

- Be adequately trained and demonstrate an understanding of drop hazard identiﬁcation, drop prevention techniques and tool tethering systems.

- Shall abide to the contents of this standard at all times.

- Primary Drop Prevention Systems Safety Requirements shall include.

- Tether Points

  - All tether points must be certiﬁed as third-party tested for dynamic load, CE marked and have a unique serial number for traceability. They must be clearly tagged with a safe working load (or max load) which is not to be exceeded.

- Lanyards

  - All tool lanyards must be certiﬁed as third-party tested for dynamic load, CE marked and have a unique serial number for traceability. They must be clearly tagged with a safe working load (or max load) which is not to be exceeded.

- Anchor Points

  - For many small tools, connecting to the worker can be the best option while heavy tools will require a ﬁxed anchor to avoid injury to the worker in the event of a dropped tool. Amey recommends that the below limits are never exceeded for anchor points:

    - Wristbands: Only for tools 1KG or less

    - Tool Belts and Harnesses: Only for tools 3KG or less

    - Fixed Anchor Points (not attached to a person): For all tools over 3KG.

When employed in working at height from MEWP basket, hand tools and other small equipment must be secured by personal tool safety lanyards.

Where a Supplier provides or arranges for the provision of any of the following, it must comply with the Amey standards:

Safety Net Requirements

Edge Protection

Mastclimber Requirements

Tower Cranes

or Hoists as applicable.

Work at height on vehicles should be avoided whenever possible. If it is unavoidable the activities must be planned to prevent or mitigate the risk of falls with appropriate control measures put in place:

Vehicle based edge protection systems

Site based edge protection systems

Fall restraint systems (harness and lanyard with anchor point or running line)

Fall protection systems (airbags and/or mats)

Other local solutions that achieve equivalent risk reduction.

When access is required to lifting points which cannot be achieved from ground level, at least one of the following must be implemented:

Designed lifting points at bottom of load e.g. cabin/container

Lifting frame/spreader beam

Designated offload area with fixed running line and fall restraint harness

Secured ladder point to point (secured top and bottom) or footed ladder.

## Delivery process

Delivery planning must ensure that:

The plan is end to end by design, begins at point of loading and must include the journey route, especially where direction of approach is sensitive, or level crossings are within 200 meters of the access point.

The load is packaged to ensure materials can be offloaded safely and that the shape and size of the package does not increase the risk

Split loads will be secured such that part loads can be removed without risk to the remainder of the load

The off-loading vehicle used is appropriate and is properly equipped for the load

Any site-specific off-loading restrictions are taken into account e.g. overhead lines

Public interfaces are reviewed, and additional control measures taken e.g. traffic management controls and/or the use of a banksman

A safe waiting area is provided

A safe loading/unloading area is provided

Where practicable a Load Angel Site Safe System should be utilised to prevent fall from height.

Suppliers are to include within their method statements the arrangements for the safe unloading or loading of materials from or to delivery vehicles. This must include access and egress arrangements to the vehicle load area if any person(s) must get on, or in, the load area to aid with loading or unloading.

Where there is a risk that materials may fall from the sides of the delivery vehicle or that unloading operations may knock or shift materials from the delivery vehicle, control measures must be adopted:

Vertical restraints such as ‘King Posts’ must be fitted to the delivery vehicle (see above photograph)

Each individual layer of loaded material must be independently secured to the delivery vehicle, so that lower materials are not dislodged whilst upper ones are removed.

When high-sided vehicle containers are to be accessed, at least one of the following must be implemented:

Arrangements to access the vehicle body from ground level

Use of designed lifting points at bottom of cabin/container

Use of automatic sheeting equipment

Provision of secured ladder point to point (secured top and bottom)

Provision of a designated area with fixed base edge protection.

All vehicles must be suitable for the material being transported. This applies to all materials but specifically long materials such as ducting, pipework, timbers, scaffold, and ladders. Flexible materials must be transported flat on the bed of a vehicle as they cannot be suitably secured otherwise due to the nature of the material.

Where long materials are carried in such a way that they project over the cab of delivery vehicles they must:

Have one end within the body of the vehicle (i.e. nothing must project from the rear)

Not project beyond the front H frame by more than 2.0 metres

Be tied to the H Frame at the front and designated lashing points at the rear (i.e. not tow bar eyes etc)

Where multiple lengths of material (e.g. ducts) are being carried, they must also be lashed together

Only be tied on by competent people.

Note: Amey reserves the right to refuse a delivery if the load, or loading/unloading method is deemed unsafe.

## Plant Pedestrian Segregation

The Supplier will comply with Amey's hierarchy, with regard to plant pedestrian segregation:

Barrier all plant and vehicles to prevent pedestrian access

All vehicles are to be fitted with devices to give all around vision or be guided by a suitably qualified person if there is a need to reverse

Banksmen control of a single machine at all times, static and in transit – all pedestrians excluded from work area

Control and use of reversing vehicles must be in accordance with our current policy

Implementation of proximity devices (type as agreed with Amey project team)

Minimum segregation of plant/vehicles and pedestrians by 3 metres (3 metre Rule).

Notwithstanding the above, whatever approach is in place, plant operators and pedestrians must comply with Amey's “Thumbs Up” initiative.

- Reversing Vehicles

To help eliminate risks from reversing incidents, with immediate effect, these MANDATORY CONTROLS must be followed in all Transport Infrastructure controlled sites and activities;

- We MUST design and plan all work activities and sites to eliminate the need for reversing, e.g. one-way systems, drive-through loading and unloading areas.

- Where elimination isn’t possible traffic routes, waste collection routes, loading and unloading areas, work areas and pedestrian routes should be organised and risk assessed to minimise reversing.

- There must be NO reversing of any LGV’s, HGV’s, RCVs or any Vehicle over 2.5t at any time without the use of a reversing assistant (banksman) - (only permitted exception is Temporary Traffic Management activities when collecting longitudinal runs of cones).

- When using a reversing assistant (banksman) at all times (including in depots) they;

  - MUST stand in a safe position, to guide the reversing vehicle without being in its way.

  - MUST be visible to the driver at all times.

  - MUST be wearing full hi-vis PPE

  - MUST use a clear and agreed system of signalling.

- If any Driver of a reversing vehicle loses sight of their reversing assistant (banksman) – they must stop immediately.

- All persons working around moving vehicles, plant and equipment must remain alert to any movements – you must never walk behind a reversing vehicle.

- Should your planned activities or site conditions change – stop - review your plan!

- “Thumbs Up” – Our Approach to Approaching Plant

“Thumbs Up” requires anyone wishing to approach the working area or risk zone of plant to make their intentions clear by attracting the machine controllers’ attention and giving them a “Thumbs Up” gesture (in a highways environment this may mean the banksman). This may be Engineers, Supervisors or Managers entering the area for example, but also includes anyone wishing to work on the machine; for example, to fit or remove lifting equipment.

Pedestrians must not approach further until the machine is in a safe condition and this is confirmed by the operator returning a “Thumbs Up” gesture. Where the item of plant being approached includes a cab arrangement or restricted visibility pedestrians must only approach from the cab/visible side.

“Safe condition” requires the Plant Operator to:

Place moving elements of the plant into a neutral safe position where those approaching simply require entry into the area for example excavator arms must be lowered to the ground

Engage the Deadman handle or isolate the machine from movement by e.g. turning it off

Acknowledge that this action is complete by returning a “Thumbs Up” gesture

Only take plant out of the safe condition once parties have discussed and agreed working arrangements and controls are in place.

Or

Place the machine into a suitable position where those approaching wish to work on the plant, for example to fit or remove lifting equipment

Engage the Deadman handle or isolate the machine from movement e.g. by turning it off

Acknowledge that this action is complete by returning a “Thumbs Up” gesture

Only take plant out of the safe condition once pedestrians have left the risk zone of the machine.

In both situations the operation for pedestrians leaving the working area/risk zone should reverse the process, by letting plant operators know their intention by signalling a “Thumbs Up” gesture and the operator returning the signal.

Where more than one item of plant is required to work or pass by each other in close proximity, their operatives must:

Place their plant in a safe and neutral position and engage with each other to agree the order, sequence, and direction of any movement prior to it taking place

Undertake and confirm the start and finish of actions using agreed hand signalling and the “Thumbs Up” gesture.

## Storage of Materials

Suppliers should ensure that the way in which materials are stored or supported whilst being worked upon, makes it impossible for the material to topple or turn over. This may require custom and practice in the way operations are traditionally undertaken, the training of those involved and the way in which materials are delivered and the storage or logistics arrangements on sites. This should be discussed with the Amey team at the earliest opportunity to ensure that adequate controls are included in proposals.

Examples of such would include large flat materials, such as boards being laid flat on floors or racks, not being leant against walls or other vertical surfaces unless they are adequately strapped back; materials like steel or concrete sections not being laid on their narrow side unless restrained in ‘toast racks’.

Other examples include the fitting of additional restraints to movement (demonstrated in images, above) to prevent their unintended movement. Such support must be available for both materials being worked upon as well temporary storage.

## De-vegetation

Please refer to section 21, Appendix C.

The supply chain partner shall take all reasonably practicable steps to protect passing motorists form projectiles arising from soft estates works including strimming and both ride-on and remotely operated vegetation cutting plant. Where the manufactures’ safety distances cannot be achieved the supply chain partner shall discuss the safest approach with Amey to enable correct hierarchy of control to be followed in partnership with the client.

When soft estates work take place alongside the highways network, it shall be in accordance with the Safety Signs Manual Chapter Eight D3.2. GG115. Requirements for works on the hard shoulder and road side verges on high speed dual carriageways   and Safety at Street Works and Road Works A Code of Practice (Red Book)

## Concrete Works

Working with wet concrete requires careful attention to safety, to prevent accidents and protect yourself from potential health hazards. Suppliers must adhere to the following safety measures:

Wear protective clothing, including long sleeves, long pants, gloves, and waterproof boots

Use a properly fitted N95 respirator or a mask with appropriate filtration when working in areas with poor ventilation or during tasks that generate dust (such as mixing dry concrete)

Use a dust mask to protect against inhaling concrete dust

Wear safety goggles or a face shield to shield your eyes from splashes and dust

Use barrier creams or skin-friendly products and promptly wash off any concrete that comes into contact with your skin. Concrete can be caustic and may cause skin irritation or burns

Ensure good ventilation in enclosed spaces to minimise exposure to airborne dust and fumes

Lift and move concrete carefully to avoid strain and back injuries. Use proper lifting techniques, and if possible, use mechanical aids like wheelbarrows or concrete pumps

Use anti-slip footwear and keep work areas clean to minimise the risk of slips and falls. Wet concrete surfaces can be slippery

Have a first aid kit readily available on-site

Know the location of emergency exits and assembly points

Ensure that all workers are trained in proper concrete handling techniques and are aware of the potential hazards associated with wet concrete

Follow manufacturer guidelines and use proper equipment when mixing concrete

Avoid overloading mixers to prevent spillage and maintain the correct water-to-concrete ratio

Use appropriate formwork to contain and shape the concrete

Be cautious around the edges of forms to prevent collapse or displacement

Follow proper curing procedures to allow the concrete to set and harden correctly. This may include covering the concrete with wet burlap, plastic sheeting, or using curing compounds.

## Excavations

Suppliers must ensure all excavations are risk assessed prior to any works starting, in conjunction with the following requirements:

Permit to dig/Permit to work at height must be obtained

Battered back excavations must be to minimum of a 45 degree angle

Temporary works bracing/trench box/proprietary trench support systems must be used where appropriate

No excavations can be left open, unless risk assessed in the methodology development

All excavations must have safe access/egress

All open excavations must include visible signage alerting nearby passers.

- Hierarchy of Controls to prevent falls in to temporary excavations

To reduce the risk of a person or animal falling into open test pits or trial holes, the following hierarchy should be adopted:

- Task planning - ensure that adequate fencing and signage has been obtained prior to works commencing

Immediate - Backfill any trial holes or test pits when observations are complete

By end of Shift – Backfill of excavation if observations were required later in the shift

If the observations cannot be completed by the end of the shift, the excavation should be filled using bags (e.g. sand bags) filled with the arisings from the excavation

If the excavation cannot be filled with bags, then a fence should be installed a minimum of 45-degree angle from the bottom of the excavation, e.g. the fence should be positioned a minimum of 1 metre away from the top of a 1 metre deep excavation:

A blue rigid barrier must be installed and fitted with signage (Caution Open Excavation)

If on a slope, the Supplier should use a blue netlon fence instead, fixed to 8 wooden stakes with a blue rope at the top and fitted with signage.

## CHS Driven Piles

Following CHS pile installation, the Supplier must supply and ensure exposed piles are left secured via one of the methods detailed below. This shall be included in all Supplier quotations and included as part of their supply, including where piles are free issued.

Uncovered tubular piles can increase the risk of a member of the workforce or a trespasser falling into the pile and injuring themselves

By covering the pile, it reduces the risk of protected species such as Great Crested Newts (GCN) and slow worms from entering the piles

A reusable cover is preferable and more sustainable, which can be removed prior to fitting the OLE structure, and subsequently reused on other piles

If a proprietary system (orange example above) is unavailable, then use a recycled PolyPly material that is cut to size during the construction activity.

## Masonry

Brickwork and blockwork suppliers shall use safety wall ties unless otherwise specified by the designer.

Where Bricklayers line pins are used in newly laid brickwork, a second line pin attached by a short line should be used which acts to restrain the pin should it come out of the wall, taking all of the recoil energy out of the line and greatly reducing the potential for injury.

## General Site Requirements

To support the safe delivery of works in line with our safe systems of work, Suppliers must adhere to the following site requirements:

Where a MEWP is being used, there must be an attendant operative equipped with a dry powder fire extinguisher at ground level, to react in an emergency and execute the rescue plan in line with the Plant Operators Scheme (POS) Plan/RAMs

Oxygen and fuel bottles are to be moved on a proprietary trolley, which will have an appropriate fire extinguisher held on it. All fuel and oxygen bottles are to be fitted with appropriate flashback arrestors. All flexible gas lines are to meet the relevant BS. The use of acetylene is forbidden, unless first obtaining written permission from Amey Group Health Safety and Assurance  through Amey site management. The Supplier will fully comply with the Amey process for management and use of oxy-acetylene on site. Fire extinguishers are to be serviced at not greater than annual intervals and visually checked weekly

Full screening to arc welding is required where controlled access to the welding area is not possible by restricting access

Barriers are required to be placed around pipe threading machines, bench mounted wood and metal working machine areas and band saws

Site hoardings or fences provided by the Supplier around any external works should comply with the Amey corporate image and be provided whenever possible to prevent access by unauthorised persons. This fence is to be at least 2 metres high and be such that it is not easily climbed i.e. close boarded or mesh not exceeding 30mm in size. The design for such fencing must comply with Amey's Temporary Works Procedure and have a design check completed

All temporary protection supplied by the Supplier is to be in accordance with Loss Prevention Standard 1207 i.e. floor protection, window and wall coverings are to be to the appropriate fire protection standard

In areas where there is any risk of a person striking or falling onto reinforcement starter bars or similar, these must be protected with suitable caps or boxing. Where the risk is of a person falling on to the bars the protection must be suitable to withstand the weight/force e.g. reinforced caps or timber boxing

Exposed fixed blade craft knives such as a Stanley knife are not permitted on Amey sites; all tools of that nature must be the self-retracting safety utility knife type

Toolboxes/chests must be safe for use. The uncontrolled closing of lids must be prevented by the use of hydraulic/pneumatic (gas) stays with mechanical lock open capability to prevent injury in the event of a failure.

- Logistics

Specific measures must be in place for delivery drivers to be able to demonstrate at the site that there are all the necessary legal inspection documents (12 months lifting equipment, 6 months for lifting accessories) available and that the driver has the necessary competence certification to operate any on board unloading equipment or devices

All mobile plant and vehicles will be operated in compliance with Amey's Reversing Policy i.e. for all vehicles other than cars or car derived vans all reversing must be controlled by a vehicle marshal, unless it has been specifically risk assessed out and alternative controls put in place

All deliveries will be arranged in compliance with the requirements and restrictions of Amey  Integrated Site Plan.

# Events (Accidents and incidents)

Amey, including their Suppliers will report all accidents, incidents, near misses, environmental, quality incidents and close calls as soon as they are aware of them. In the case of accidents with potential to lost time, wellbeing updates shall be required until the IP is fit for work.

In addition, all reportable accidents or dangerous occurrences as defined in RIDDOR, must be reported to the appropriate enforcing authority (HSE or ORR) by the Contractor Supplier as determined by its employer status.

Incidents must be investigated by supply chain and where GDPR allows supply chain partners must also co-operate with Amey to ensure that investigations are carried out in a timely manner and identify all the possible causes and resultant key learnings.

Where there are specific client requirements and incident  reporting and investigation, the client shall co-operate and collaborate fully to enable Amey and its supply chain to correctly meet Amey’s client requirements.

# Competence and Training

Amey is committed to achieving a fully certified workforce on all its sites. This includes managers, supervisors, plant operators and site operatives. Amey supports the Construction Skills Certification Scheme (CSCS), including its affiliated schemes, in relation to lorry loaders and Hiabs. Amey requires Suppliers who wish to work with us to commit to providing a workforce which is fully and appropriately carded prior to any order being placed.

- Supplier responsibilities

The Supplier shall ensure that all personnel undertaking duties shall provide evidence of competencies to be utilised in undertaking their duties. The Supplier will provide evidence of the training and competence of each of its employees, appropriate to the activities and tasks being undertaken. Failure to provide evidence of appropriate training will result in access to site being denied. Where this training is in-house Amey reserves the right to attend and monitor this training.

It is the Supplier’s responsibility to ensure that all their employees have received suitable training so that they can carry out their work safely, with due regard for the environment and to the desired quality. It must also agree to release employees to attend Amey site training sessions as and when required.

- Underground services

Suppliers whose activities involve disturbing ground are to ensure that the permit receiver is demonstrably competent (e.g. to NVQ Level 3 Gold Skilled Worker CSCS Card for Building, Energy and Utility Skill Registration Scheme (EUSR) card for CNS and the Appropriate CSCS card for Civils).The following groups of personnel must also have attended the Amey training module for underground services:

Supervisor controlling works of disturbing ground

Holders of a “Permit to Dig”

Individuals charged with operating a Cable Avoidance Tool (CAT) and Generator.

- Plant operation

Amey will only allow plant to be operated by a driver holding an appropriate card covered by the Construction Plant Certification Scheme (CPCS) or affiliated scheme, with the exception of HIAB/lorry loaders where ALLMI competence cards will be accepted.

- Supplier commitments

A director of the Supplier will be required to endorse and return to Amey, and Amey Competence of Suppliers Supervisors form for each supervisor employed directly or for second and subsequent tier Suppliers. Amey reserves the right to withhold consent to the deployment of an individual supervisor if their competence to fulfil their managerial or supervisory role is not adequately demonstrated. All such supervisors must have attended the half day Supervisors Working with Amey (SWWC) training course (valid for three years) and meet the Build UK Supervisor training standard outlined in Appendix A of this document.

Amey Training Services is accredited to deliver craft training programmes within the Government’s Modern Apprentice Framework. Suppliers are to commit to providing local work experience opportunities where appropriate as part of the apprentice scheme.

# Tools, plant and equipment

- Statutory requirements

All tools, plant and lifting equipment used by Suppliers shall comply with statutory requirements including Amey Mandatory Arrangements for Safety Critical Plant Doc Ref: AMEY HIGHWAYS-FP-SCP-PR-01 under Appendix B. Suppliers must satisfy themselves that the tools, plant or equipment are fit for purpose, that they are maintained and in satisfactory condition and that Test/Examination Certificates, insurance documentation and registers are available on site.

- Competency of operators’ requirements

All plant and equipment used by the Supplier must only be operated by competent persons with an appropriate CPCS card or approved affiliated equivalent and who have also received familiarisation training for the specific item of plant/equipment being used. Within 3 years of first qualifying and at intervals not exceeding 3 years thereafter, forklift/telehandler operators must attend a one-day refresher training with an approved training provider. All mobile plant operators are required to attend the additional plant operator induction on each Amey site where they are to operate plant. It is a mandatory requirement for all steel erectors and net riggers working on our sites to hold an IPAF PAL+ qualification.

- Process requirements

Specifically for Quick-Hitch attachments, user manuals must be available in the cab of the vehicle and the operators must have received specific training on the Quick-Hitch system being used. This information must be documented and records maintained on site.

The preferred type of quick hitch mechanism to be used on Amey sites is the fully automatic type that locks onto both bucket pins. Alternative types of quick hitch mechanism will be accepted provided their use has been justified and adequate controls are in place to prevent attachments from becoming unintentionally detached. In the case of semi-automatic quick hitches, these must be fitted with a permanently retained or non-removable safety pin such that when it is disengaged it is clearly visible to both the operator of the machine and those working in the vicinity. Manually operated hitch attachments are also acceptable.

The following requirements apply whatever type of quick hitch mechanism is used:

There must be a secondary mechanical interlock to prevent the latch from being accidentally released

Quick hitch activation controls must be designed to prevent unintentional activation e.g. by simultaneous activation of two controls/switches or the activation of a control/switch with a safety feature that requires a continuous positive action to override e.g. a locking safety switch

All quick hitch attachments must be compatible with each other.

The plant operator must also sign the Plant Operator Passport form to confirm they have received training on the specific quick hitch assembly fitted to his machine, commits to complying with the inspections and operational requirements in line with the training received and to stop work should anyone encroach into the swing radius of the machine. The Plant Operator Passport applies to all excavators, with or without quick hitch.

- Specification of tools, plant and equipment

Any mobile plant which when used has the potential to overturn will be fitted with a roll over protection system (ROPS). If such plant has the potential for persons being carried to be thrown from their seat, then seat restraints will be fitted and their use enforced

All plant and equipment brought on site by Suppliers must be such that the emission of noise shall be reduced to the lowest possible levels

All equipment brought on to site by Suppliers where there is a risk from vibration, dust, or noise to those using it, must be accompanied by a suitable and sufficient risk assessment. The assessment must cover as a minimum, control measures to mitigate the risk, health surveillance for employees and what information, instruction and training has been provided

Articulated tipping road vehicles are not to be brought onto site without written agreement from the respective Account Director. Articulated dump trucks are permitted subject to agreement with Amey regarding a safe system of work.

All relevant plant should be supplied with wheel locking nuts

Plant is to have visibility aids such as mirrors, CCTV etc to ensure all blind spots are eliminated. In the case of excavator’s dual mirror upper/lower positions will be needed to ensure visibility when the dipper arm is in the raised or lowered position. This will be mandatory for all plant arriving on site

All plant refuelling must be undertaken in accordance with the sites refuelling operation requirements.

- Mini Excavator restrictions

It is expressly forbidden to use an 1.5t excavator in either an operated or non-operated provision without prior written approval from Amey Senior Management, only excavators which are 3t and upwards are accepted to be used on any Amey contract or site.”

This change to minimum standards will extend to work and equipment used by our Supply Chain. Where, in the opinion of the Account Lead for Operational Needs or an Identified and Assessed Safety Hazard requires the Ad Hoc hire of a 1.5T Digger/ Excavator, a business case must be raised to our  Business Director detailing;

  - Why a 1.5t unit is the only viable option

  - What other options are available and why these are not suitable for use

  - What period will the Ad Hoc Hire be required for

  - What additional local control measures and assurance will be in place for the period of hire/ use

- Use of fibrous blades for cutting steel columns

- All fibrous discs are PROHIBITED for all street lighting column cutting (Horizontal and Vertical)

- Cutting Columns – Approved methods

When cutting Metal (e.g.; Steel, Cast Iron, Aluminium, etc) columns use of a reciprocating saw is  the preferred method

Where a Recip saw is deemed as not suitable for the task, the alternative methods can be adopted subject to the appropriate training and equipment being available;

- Hand Held Band Saw

- Pipe cutter

- Cut off saw with the appropriate, approved diamond tip steel blade

For concrete columns a Cut off saw with the appropriate, approved diamond tip steel blade should be used.

# Lighting and Temporary Electrics

All services installed underground, whether temporary or part of the permanent works must be identified with suitable marker tape above the service along the entire length. Marking must be added during initial backfill operations.

Only competent and authorised persons are to adapt or move the temporary electrical installation and general area safety lighting. Suppliers are responsible for providing and maintaining their own task lighting to a sufficient level that the operation can be carried out safely and to the required standard of workmanship. Suppliers are required to ensure:

Halogen lights are not used, unless properly mounted on a rigid structure away from people and potentially flammable materials

All temporary electrical cabling is routed to avoid creating a trip hazard and where practicable the cables will be suspended at high level

Portable or semi-portable electric tools and temporary lighting are not in excess of 110v, without specific permission of site management. Dual voltage generators are not permitted on site

All electrical tools, leads and equipment have an auditable visual inspection tagging system showing the date of test and when the next test is due. The arrangements for Portable Appliance Testing (PAT) and other regular inspection must be risk assessed and included within the activity method statement.

# Personal protective equipment

## PPE requirements

As a minimum, the following items will be supplied to and worn to control the risks associated with the general working environment:

- Hard hats (with chinstraps when working at height or around deep excavations).

All general issue Safety helmets/hard hats shall comply with EN 397:2012 and any attachments used shall be compatible.

Bump caps complying with standard EN 812:2012 may be worn for specific tasks (see Table 1 Exemptions for use).

- Hand protection

- Hand protection to Standard: EN ISO21420:2020 - General Requirements for Protective Gloves must be worn as a minimum.

- All other hand protection must be in accordance with risk assessment and appropriate standard for type of glove required

- Hand Protection to EN 388:2016. Cut resistance tested to ISO 13997:2023 Grades A to F in addition to the level of protection suitable to the task being undertaken

- The Supplier must note the task specific gloves and grade ratings associated. From January 2024 gloves will be categorised by a real-life grading criterion, A to F, which replaces the cut rating criteria 1 to 5 previously used. A range of grade appropriate task specific gloves for the highways sector are detailed in the ARCO (Highways Preferred Supplier) or Bryson (Rail Preferred Supplier) & SAP PPE catalogue, available on-line at AmeyWorld. For reference to the new grading criterion see below:

  - LEVEL A – Reduced padding with greater dexterity, ideal for general warehouse duties with low-risk environments where scuffs, scrapes and light scratches are a constant threat

  - LEVEL B - Slightly more protection, but not suited to handling sharp objects and edges. Ideal for factory and warehouse work, where exposure to use of small handheld blades is a possibility

  - LEVEL C - Suitable for a wider range of uses, including areas where contact with blades and sharp edges is common. Ideal for task specific activities with trade workers and in industrial settings like railway, building and construction sites

  - LEVEL D/E - Offer a greater amount of flexibility and control for manual handling in building and construction settings. ideal for heavy-duty construction, and industrial settings like railway, building and construction sites

  - LEVEL F – Offer the highest level of protection available and indicates that a garment is safe and suitable for handling seriously sharp objects like knife edges, blades, sheet metal and glass. This maximum rating is reserved for applications where personal injury is a constant threat due to the constant handling of sharp and jagged objects.

- Eye protection

Eye protection to BS EN 166 standard must be worn at all times  and should be , suitable to the task being undertaken. Examples of eye protection include spectacles, goggles, visors, and face shields.

For specialist roles/tasks where requirements necessitate more enhanced eye protection, the protection provided shall comply with the task/Industry related standard(s)

Employees who wear prescription spectacles will be provided with over spectacle eye protection in line with Amey PPE procedures

Anti-scratch prescription safety spectacles must have CE or UKCA compliant frames. Photochromic (e.g., Reactolite) lenses may be worn

Tinted lenses should not be worn for the duration of a task if there is a requirement to distinguish colours accurately.

Photochromatic lenses should be worn with caution; whilst they darken rapidly in bright conditions, they take much longer to clear in dark conditions

- Hearing protection

- Any work where hearing protection is required and there might be moving machinery, vehicles and plant in the vicinity, hear-through should be mandatory and is a reasonably practicable measure in most circumstances.

- Footwear

Footwear requirements include steel toe cap boots with mid-sole protection and ankle support (not rigger boots) to EN ISO 20345:2011 standard. The steel toe cap shall always be covered to prevent the conductivity of electricity.

- Exceptions requiring risk assessment

Tasks that require close inspection/work may require the removal of eye protection, this is only permissible if the task has been risk assessed, recorded in a Task Risk Control document signed off by the responsible manager, PIC advised and recorded in the point of work risk assessment POWRA.

Exemptions specified in the project contract supported by a risk assessment may apply to one or more items of PPE but not to high visibility clothing. The requirements identified in this document are minimum requirements only. Where the ‘Construction (Design and Management) Regulations’ apply to a project then the Principal Contractor may as part of their site rules set additional PPE requirements.

HI Vis clothing :

Hi-Viz orange upper and lower body clothing to class 3 (class 3 garment is required to cover the torso and have long sleeves for forearm protection, with reflective bands or/and full-length trouser legs with reflective bands) EN 471 with upper body sponsor branding

If any works or access to site is required in hours of darkness or where natural lighting is poor, a head torch with a minimum lighting power of 550 lumen (such as the Amey Standard approved Nightsearcher model HT550 or Nightstick head torches) will be supplied and worn in conjunction with a Hard Hat

This is, however, a minimum standard lighting requirement and should not be the sole source of illumination when undertaking a specific task. A suitable and sufficient risk assessment must always be undertaken to ensure the correct mitigation measures are specified and put in place with regards to working in poor lighting conditions.

## CE UKCA Marking

All PPE shall bear a ‘CE’ or ‘UKCA’ mark and comply with the requirements of the Personal Protective Equipment at Work Regulations 2022. This signifies that the PPE satisfies certain basic safety and health requirements and have been tested and certified by an independent body.

| Item | Reason |
| --- | --- |
| Rigger Boots | Lack of ankle support to prevent twisting on uneven ground. |
| Rigger Gloves | Lack of protection due to poor cut and puncture resistance. |
| Shorts or ¾ trousers and short-sleeve tops | Do not mitigate risks from slips, trips and falls, lineside vegetation, sunburn, sparks, chemicals, vermin, and insect bites. |
| Head worn hoodies and non-approved by manufacturer safety helmet headwear | This includes personally purchased garments often worn under PPE as they limit the wearers peripheral vision and ability to hear verbal communications and warning alerts. Non-approved under safety helmet headwear may reduce the effectiveness of the safety helmet therefore invalidate manufacturer certification of acceptance. |
| Hoods on coats/jackets | Must be removed on the infrastructure as they limit the wearers peripheral vision and ability to hear verbal communications and warning alerts. |

Table 1: Prohibited PPE

| Person, Group, or activity | Exception |
| --- | --- |
| Employees of the Sikh religion | The wearing of a safety helmet or hard hat, providing that a Turban is worn. |
| Employees | The wearing of a close-fitting lightweight Hijab or head scarf under a safety helmet or hard hat, but it should not: a, restrict the wearers peripheral vision, b, restrict the ability to hear verbal communications or warning alerts, c, be able to become entangled in machinery, plant and equipment or passing trains. |
| Machine/plant Operators | Are not required to wear a Safety helmet when driving or operating plant and equipment from within an enclosed cab. |
| Telecoms, signals, or electrification & plant employees | Performing close work within switchgear, panels or enclosures. Are not required to wear a Safety helmet whilst undertaking close work but shall wear an Arc Flash helmet when working with HV equipment. |
| Whilst carrying out a work activity/task that requires specific PPE such as chemical overalls that are currently unable to achieve the High visibility requirements in ISO 20471:2013. | High visibility clothing. High visibility upper/lower body clothing. |
| Any person working in water, mud or snow | Normal safety footwear, provided wellington boots with protective toecap and mid-sole protection are worn. |
| Welders or others undertaking hot work | Normal safety footwear, if the alternative boot is to the Standard specification of a Welder/Foundry boot that provides hot metal resistance, ankle support, protective toecaps and mid-sole protection are worn. |

Table 2: Exemptions for use

# Scaffolding and edge protection expectations

Amey maintains robust policies and guidance including scaffolding, edge protection, high rise construction etc. Suppliers are to familiarise themselves with these documents, as they impact the work activities of the Supplier.

Where the Supplier engages the services of a scaffolding contractor, the following Amey conditions apply:

Only one nominated Scaffolding Contractor will be used on a particular Project. The contractor will operate to the Amey Group Standard for scaffolding

Only one nominated Edge Protection Contractor will be used on a particular Project. The Contractor will operate to the Amey Edge Protection Requirements standard.

A proper design brief is to be agreed between parties, so requirements are clearly understood.

A design is produced which meets the design brief and the practical issues are clarified.

The design is checked and approved to suit procedures of both parties.

The design information is passed on to the erection crew together with key issues clarified where reference must be made back to the design.

The erection crew signs off that the work had been completed to the design.

A nominated competent person inspects the completed work and hands over a signed and dated copy, which makes specific reference to any variations from design and limitations imposed.

Where applicable, reference should be made to preferred hook-on points for fall arrest equipment.

Note should be made of any specific requirement in striking the scaffold/edge protection and any limitation on intermediate loading points.

# Lifting operations including use of Flexible Intermediate Bulk Containers (FIBCs)

All lifting operations must be notified to the Amey lifting coordinator, including those carried out as part of piling works. Hired lifting equipment should only be sourced from approved, accredited suppliers. Suppliers must only use cranes supplied by Amey approved suppliers. Supplier owned cranes cannot be used unless the contractor is an approved crane/Hiab supplier, i.e. the Contractor has previously been audited by Amey Managed Plant.

Where deliveries/collections are to off-loaded/load using a vehicle mounted crane, a suitable lift plan must be provided by the special contractor, and subject to the requirements set out below. Amey will provide a suitable area for loading/offloading activities.

- Roles, responsibilities and competency requirements

Lifting Operation plans must be drawn up by a competent person (Appointed Person) and include details of individual items to be lifted and any special lifting arrangements required by the Supplier. This information must be supplied to and reviewed by the Amey lifting coordinator prior to the event to ensure that a safe system of work has been agreed as part of the overall lifting plan.

- FIBCs

FIBCs, (sometimes called bulk bags or One Trip Bags) are commonly used in the construction industry for the transportation of bulk granular materials such as sand or aggregates. They must only be utilised for the single trip as intended and never used to transport steel reinforcement links, construction waste or other materials with sharp edges such as rebar and scaffold tube off cuts that can cut the fabric. The Amey guidance on the use of FIBCs must be complied with at all times. The use of “Easy lift bags” negates the need for access on to the delivery vehicle to attach lifting attachments.

# Welfare and First Aid

Where the Supplier provides welfare facilities, they must meet the Amey standard and be maintained to this high standard. Welfare facility requirements must be agreed with Amey prior to work commencing.

Suppliers are to provide sufficient first aid facilities and the appropriate number of qualified first aiders to comply with the requirements of the Approved Code of Practice and Guidance for The Health and Safety (First Aid) Regulations, having duly assessed their workforce and work activities. Amey is to be kept informed of the current provisions in place.

# Health

The Supplier will be required to demonstrate compliance with Amey's Occupational Health Strategy. Where they employ safety critical workers as defined by Amey, the Supplier will be required to demonstrate that they are fit for work before they commence work on site.

The Supplier will be required to demonstrate that they have in place suitable procedures to ensure the health of their workforce. As part of those procedures, they will need to show that appropriate health surveillance measures are in place to identify any harmful effects at the earliest possible opportunity. These measures should include, but not be restricted to the following:

Noise

Manual Handling

Musculoskeletal disorders

Hand arm vibration syndrome (HAVS)

Full body vibration

COSHH

Respiratory disease prevention

Display Screen Equipment Assessments.

# Fatigue

Please refer to section 22, Appendix D.

# Activities requiring authorisation.

The Supplier will notify Amey and include within its method statements, all permit and authorisation requirements and responsibility for provision and maintenance associated with the works and will comply fully with the requirements.

Amey will appoint a “Permit Coordinator” who will be a senior member of the Project Team. The following list gives examples of authorisations frequently required:

Hot Works Permit

Permit to Disturb Ground

Confined Space, Permit to Enter

Permit to access high level works.

Permit-to-Pump.

Permit to Isolate (electrically/mechanically live area)

Temporary Works, see section 18 below.

Authorisation to work from ladders/step ladders.

In order to obtain authorisation to work from ladders/step ladders, the Supplier must demonstrate that other forms of access have been considered and that ladders/step ladders are the most suitable type of equipment in the particular circumstances, when used under the control measures set out in the authorisation.

The example list is not exhaustive and there may be other “Specific” permits or authorisations that are required as a result of risk assessments.

# Temporary Works

The Supplier shall advise all temporary work requirements to Amey's Temporary Works Coordinator, for entry onto the Amey Temporary Works Register.

Examples of temporary works:

Excavations

Propping and shoring

Tower crane and hoist installations

Foundation underpinning

Fencing, hoardings and signs

Scaffolding

Formwork

False work

Temporary Glazing retention – temporary stitching of curtain wall glazing.

# Appendix A – Build UK Supervisor requirements.

Supervisors must have the ability to demonstrate that they have satisfied the following two key criteria:

That they have sufficient knowledge of health and safety, gained at a construction specific course of at least two days duration. The course content must include the following as a minimum requirement:

The role of the supervisor

Health and safety law and enforcement

Safety systems of work

Hazard identification

Risk assessment

Method statement

Principles of prevention

Special considerations

Health and safety management

Safety policies

Measuring performance

Permit systems

Occupational health

Noise

Vibration

Asbestos

Drugs and alcohol

Dermatitis

Asthma

Health monitoring

Fire prevention, control and other emergencies.

Anatomy of fire

Types of fire

Fire prevention (fire plan)

Raising the alarm and escape

Regulatory Reform (Fire Order) 2005

Fire fighting

Other emergencies

Hazardous substances

Five principles

Hierarchy of COSHH assessment

Control of substances hazardous to health (COSHH)

COSHH symbols

Protecting the work

Harmful substances

Risk assessment and method statements.

Construction (Design Management) Regulations (CDM)

Regulations – part 1 to 5

Notification

Duty holders

Competence

Duties relating to health and safety on construction sites.

Welfare

Working at Height

Exploring Working at heights best practice

Working at heights regulations

Personal Protective equipment (PPE)

Definition of PPE, maintenance, compatibility, suitability

Information, instruction and training

Importance of provision of a demonstrable method of recognising supervisors on site

Manual Handling o Definition of manual handling

Manual handling operations regulations

Manual handling assessment

Information and training

Accidents and First aid.

Definition of accidents

First aid

Data protection

Courses must incorporate a form of testing and training providers must issue a certificate stating whether or not the delegate has passed the examination.

That they have sufficient skills to be an effective supervisor of their workers. They will need to demonstrate having attended training which includes an understanding of behavioural issues, leadership and effective intervention skills. This training must include delivering effective presentations (e.g. Tool Box Talks), role-play, have a form of testing, and successful candidates must be issued with a certificate to demonstrate a suitable standard has been achieved.

# Appendix B – Safety Critical Plant Requirements

# Appendix C – Works Planning & Delivery of Vegetation Clearance

For use on works undertaken on or around the Rail network.

# Appendix D – Working Hours – Policy and Arrangements

# 24. Appendix E – Amey ESG Strategy