FOL15/872: Tree Soil Decompaction Services Framework 2016-20

Specification of Requirements

Table of Contents

Introduction	3

Below-Ground Utilities Surveys	3

Schedule of Services	3

Decompaction Technique Requirements	3

Mulching	4

Storage of Equipment and Vehicles	4

Introduction

The Royal Parks wishes to establish a four-year framework agreement for services to decompact soil beneath mature trees.

Soil decompaction services are required in order to improve the drainage, quality and rooting environment of soil surrounding the base of mature trees.  Decompaction must minimise the damage to arboreal root systems.

Services may be required in the central parks, namely: St James’s Park, The Green Park, Hyde Park, Kensington Gardens, and The Regent’s Park.

Below-Ground Utilities Surveys

Prior to delivery of decompaction services, a survey will be undertaken by separate contractors at TRP’s expense in order to locate and mark any existing below-ground utilities on site.

Schedule of Services

The contractor will have facilities to decompact approximately 20,000m2 annually; with the capacity to complete 2000m2/day.  Services will be required in the months of January or February between the hours of 07:00 until dusk as requested by TRP.  In the event of severe weather (e.g. snow covering survey markings, heavy rain causing water logging etc.), TRP retains the right to suspend the services.

Decompaction Technique Requirements

An appropriate soil decompaction technique will include:

- A probe to penetrate soil vertically to a minimum of 1m depth to inject compressed air to fracture and fissure compacted soil.

- The injection of a treatment medium to enhance soil quality, fill fractures & retain permeability of the soil structure.

- The backfilling of probe holes with a suitable water-retaining material to retain permeability of the soil structure.

- The spacing of no more than 2m between probe sites.

Mulching

Following delivery of your services, TRP may apply mulch to decompacted areas.  Mulching should therefore not form part of your proposals.

Storage of Equipment and Vehicles

Equipment, including vehicles, may be stored on site overnight at the Contractor’s own risk subject to agreement with TRP.  TRP will not be liable for any loss or damage to the Contractor’s equipment whilst stored on TRP’s premises.