| Specification |  |
| --- | --- |

Project / contract information

| Project name | East Cheshire Aquifer Model |
| --- | --- |
| Project reference | ENV0003173C |
| Contract reference | ENV0003173C |
| Date | 14/07/2022 |
| Version number | 1 |
| Author | Adam Chapman |

Revision history

| Revision date | Summary of changes | Version number |
| --- | --- | --- |
|  | First issue |  |

# Overview

## Background

The East Cheshire aquifer is part of the East Cheshire Permo-Triassic Sandstone aquifer system. The area mostly consists of a broad plain which is surrounded by the foothills of the Pennines to the north and east. This high ground is sharply incised by river valleys in places and all of the rivers in the area are tributaries of the River Mersey. Towards the west, the broad level valley of the River Mersey is characterised by the flat peatlands of Carrington Moss and Chat Moss. The area is highly urbanised around the Manchester conurbation but the southern part of the catchment, around East Cheshire, has a few towns and is more rural in nature.

The Sherwood Sandstone in the area is an important source of water for United Utilities Public Water Supply (PWS) abstractions as well as for a number of industrial and private abstractors. This is also the situation in the East Cheshire aquifer system which is in the southern part of this study area. Here the groundwater from the sandstone aquifer provides baseflow to the Dean and Bollin river systems with discharge occurring to the rivers via the overlying superficial deposits.

The East Cheshire aquifer groundwater model was originally completed in 2004 by Environmental Simulations International Limited (ESI Ltd) for the Environment Agency and covers the Dean and Bollin catchments – Southern and Cheadle Blocks, an area of approximately 160 km2 from the River Mersey around Stockport southwards to the area around Alderley Edge and Macclesfield. It was created using MODFLOW96 and Groundwater Vistas modelling software to simulate groundwater flow and groundwater-surface water interaction from 1970 to 2001. The model was set up to assess the groundwater resources in this area and the long-term reliability of water supply from the East Cheshire aquifer. The model is held on the National Groundwater Modelling System (NGMS).

The Environment Agency use the East Cheshire model for assessing the impact of groundwater abstractions, mostly Public Water System for United Utilities (UU), from the East Cheshire aquifer on surface water bodies.

Phase 1 of the 2004 model project developed a series of lumped water balances for the various sub areas, shown in Figure 1, to provide a context for licensing decisions and to feed into the Catchment Abstraction Management Strategies.

Figure 1: various sub areas for lump water balances

Phase 2 of the 2004 model project developed a numerical groundwater model of the Dean and Bollin catchments in the southern part of the study area shown as a grid in Figure 2. The groundwater model was only developed for the southern part of the study area for technical reasons: the groundwater discharge in the southern part of the study area could be constrained by measurements of the baseflow in the Rivers Dean and Bollin thus allowing a total water balance to be developed to support the conceptual model of this area; and the geology and hydrogeology of the northern part of the study area was considered to be too complex to develop a robust groundwater model on the basis of the data currently available.

Figure 2: Study area for numerical model

The scope of this contract is to update the numerical groundwater model developed in Phase 2 of the 2004 model project (MODFLOW96) with meteorological, abstraction and hydrometric data from the last 21 years so that it is up to date, and we can use it confidently to manage the water resources and assess impacts of abstraction.

## Outputs and deliverables

Objective

The specific objective of the project is to update the existing MODFLOW model, and associated recharge model to the end of 2021.

Outcome Specification

The deliverables are the updated MODFLOW model, including 4 model scenarios detailed in task 3, and updated recharge model, as well as a report detailing the work, the results and any issues and recommendations.

The work can be broken down into these tasks:

Task 1:	Data collection and processing including data QA

- Collate and process rainfall and PE data

- Update surface water abstractions and discharges

- Update groundwater abstractions

- Collate and process observed groundwater levels and flows

Task 2: Update Recharge Model

- Update and run recharge model

Task 3: Update and run MODFLOW model including 4 scenarios:

- Calibrated historic

- Naturalised

- Recent Actual

- Fully Licensed

Task 4: Reporting and delivery

- A report detailing the data and information, the modelling process and the results of the modelling is required.  The report should be delivered digitally as a pdf document.

- All model outputs and data produced during the contract should be provided

The existing recharge model and groundwater model will need to be updated to the end of 2021 with up-to-date recharge and groundwater data, which could include abstraction, rainfall, evapotranspiration and river flow data.

## Consultant project management

# In managing the services the Consultant shall:

- Produce a monthly project risk/issues register and update (including Consultant risk budget).

- Attend a start-up meeting within 4 weeks of contract award. This meeting could be face-to face or via video conference.

- Attend progress meetings and draft record minutes, the Client to issue.

- Produce monthly financial updates and forecasts meeting the Client’s project reporting timetable together with progress reports.

- Deliver a monthly progress update giving progress against programme, deliverables received and expected and financial summary against programme.

- Produce a short technical summary from consideration of previous investigations and surveys, explaining how best use will be made of historical data.

- Ensure that all the original data sent to the Consultant (i.e. all model and survey information provided by the Client in an encrypted format (using WinZip 128 bit encryption) according to the Client’s data security policy), which is classed as commercially sensitive, is returned to the Client in an encrypted format using WinZip 128 bit encryption.

- Ensure that project deliverables such as model files, survey data or anything of a personal nature such as questionnaires or address data is returned to the Client in an encrypted format using WinZip 128 bit encryption.

- Deliver a copy of all models, survey data etc, and supporting detailed technical reports. The Consultant will take the risk of the adequacy of existing data quality and quantity.

## Previous studies

The table below contains details of previous studies

| Model | Date | Format | Outcomes of study |
| --- | --- | --- | --- |
| Manchester and East Cheshire Water Resources Study: Phase 2 - Groundwater Model of the Southern Area (Dean and Bollin Catchments | 2004 | Recharge and Groundwater model bas, bcf, csv, dat, drn, err, exe, ghb, gmv, in, mfw, nam, oc, pcg, pdf, rch, riv, wel, xls, asc, imp, out, rmp, txt | MODFLOW96 and Groundwater Vistas modelling to simulate groundwater flow and groundwater-surface water interaction from 1970 to 2001 |
| Manchester and East Cheshire Water Resources Study: Final Report | 2004 | PDF |  |

# Services required

## Model Update

A MODFLOW96 model for the study reach was produced in 2004. A copy of the model will be shared electronically online. The extents of the modelling and assumptions made are within the model report. The Consultant should verify the model with quality and extent checks.

Using available information, the Consultant shall update the 2004 MODFLOW96 model with meteorological, abstraction and hydrometric data from the last 21 years so that it is up to date, and we can use it confidently to manage the water resources and assess impacts of abstraction.

Existing data assumed to be required for the update to the model is included in the table below. The Consultant shall include in their submission any additional data request requirements and any assumptions made within their quotation for the quality/extent of data.

| Information | Format |
| --- | --- |
| Groundwater Level Measurements (AfA075) - data from EA Observation boreholes located within the East Cheshire Permo-Triassic Sandstone Aquifers. | TBC |
| Return data - records of actual surface water and groundwater abstraction (returns) for the current live licences | TBC |
| BGS Geology - Map layers including bedrock geology, superficial geology, artificial, linear and mass movement | TBC |
| BGS Superficial Thickness Model - Mapping layers showing modelled superficial geology thickness | TBC |
| NatMap Vector | TBC |
| Subset of Water Resources GIS: database made up of many GIS layers that are a geographic representation of the Resource Assessment and Management (RAM) ledgers | TBC |
| Monitored river flow from four permanent gauging stations | TBC |
| East Cheshire aquifer groundwater model (Environmental Simulations International Limited) 2004 | TBC |
| Twelve rainfall stations in and around the model area | TBC |
| Land Use data – LCM (all years) | TBC |
| MOSES: EA Hybrid v1 PET (Options3), daily potential evapotranspiration data for the period 01/01/1961 to 31/12/2021 / MORECS Square 106 | TBC |
| 1km gridded rainfall derived from the Environment Agency Daily Rainfall Tool (DRT) for a predefined catchment area | TBC |

### Groundwater & Recharge model

Model deliverables

Provide final report and all supplementary data (models & metadata etc.) in electronic format at the end of the project. All electronic data should be in an agreed format.

Provide a clear technical description of the method used for modelling;

- high level description of the derivation of the run parameters;

- exception reporting (describe what non-standard things have been done to build or run or post process the model). Describe any other criteria used to improve the final results.

- describe what, where and when the model is sensitive to as highlighted by the sensitivity analysis to give an idea of the robustness of the model;

- provide a list of the final design runs, together with where the result files can be found. Ensure that this list acknowledges where specific model runs have been combined to achieve the final products;

- provide a summary of results. These should be selected to represent the purpose of the report.

# Requirements of the programme

## Programme

Start date: 03/10/22

End date:   31/03/23

The Consultant shall provide a detailed project plan showing key milestone dates throughout the model update process.  A baseline plan shall be provided for the project start up meeting and this will be updated as required for monthly progress meetings.

The programme shall cover all the activities to be undertaken by the Consultant and other members of the project team.

# Services and other things provided by the Client

## Data and information  management and intellectual property rights

All of the data listed as being supplied to the Consultant as part of this study remains the IP of the Proprietor as specified in the Prior Rights Schedule.

## Data custodianship

The data custodian for project deliverables from this commission will be the GWCL team.

## Licensing information

Licences for Ordnance Survey mapping, model, survey, hydrometric and historical data will be provided to the Consultant upon award of this commission.

## Data management and metadata

It is a requirement that all information produced by modelling work is appropriately tagged with metadata. The Consultant will supply a register where all relevant metadata can be recorded and handed over on project completion.

## Client’s Advisors

The Client has a number of advisory departments. Instructions will only be deemed enacted from them when they are confirmed by an Instruction from the Client.