# WOKINGHAM TOWN COUNCIL

|  | Town Hall Market Place WOKINGHAM Berkshire RG40 1AS |
| --- | --- |
| Mrs J Nowecki Town Clerk | Tel: (0118) 978 3185 Email: townclerk@wokingham-tc.gov.uk |

INVITATION TO TENDER

SPECIFICATION AND CONDITIONS OF TENDER

- Wokingham Town Council invites separate Tenders for (i) the extension to the car park (c.150sqm) at its Ormonde Road allotments site (RG41 2RA) and (ii) the resurfacing of the existing car park (c.368sqm)

- resurfacing of the existing haulageway at Ormonde Road which extends from the car park to the turning circle (c.710sqm)

A: (i) extension

Foundations shall be constructed using hard, clean, crushed frost-resistant aggregates, laid on geotextile material. The grading of the sub-base material must be such as to provide stability.  The material laid in layers not exceeding 150mm, each layer being compacted before the next is laid.  The sub-base material should be compacted to the requirements of BS 5835-1:1980. Upon completion there should be no detectable movement under the roller.

- All excavated material and arisings to be removed from site

- Unclog the two exting soakaway drain points

- Take out all trees and brambles

- Supply and lay 16m kerbing along boundary railings and extend existing kerbing to join with this

- Supply and lay geotextile material to all of the excavated area

- Supply and lay scalpings (25mm to 40mm) to a depth of 150mm

- Supply and lay 60mm dense macadam

- Line mark car parking spaces with white thermoplastic paint

- Supply and install 9no. 6’ by 5’ dip treated overlap fencing panels with gravel boards and concrete uprights tight to existing metal railings where they abut rear of houses – to afford more privacy to householders

A: (ii) existing car park

- Excavate and remove all loose material and profile to grade and level with extension (i)

- Supply and lay scalpings (25mm to 40mm)

- Line mark car parking spaces with white thermoplastic paint

For both A and B the existing mains drains are to be protected from damage and incorporated into the extended and resurfaced car park. Surface levels must account for the flow of rainwater to the drains.

- A separate tender is required for the relaying of the existing haulageway which leads from the car park by the main entrance gate and runs to the informal turning area by allotment plot OR87

B: existing haulageway

Excavate surface to 250mm and remove spoil from site

Provide a level surface along the length of the haulageway

Supply and lay 410m pre-cast concrete kerb egdings, haunched in concrete

Supply and lay geotextile membrane to all of the excavated area

Supply and lay 150mm crushed concrete

Supply and lay 100mm MOT Type 1 aggregate (40mm to fine dust)

NB: it is expected that the work will be undertaken in the December 2019 to February 2020 period

Tender to be delivered by not later than NOON FRIDAY 4th October 2019

Conditions of Tendering

- The Tender and details of prices must be upon forms furnished by the Council, and must be signed by the Tenderer and the whole of the requirements set forth in the forms must be complied with or the Tender cannot be considered.

- Tenderers are particularly requested to peruse carefully the Supplier’s Contract and Specification, and to make sure that they thoroughly understand them. All necessary explanations may be obtained at the Town Hall but, once a Tender has been accepted, the Council will not entertain any requests for a variation of it on the grounds of any alleged mistake, oversight or misunderstanding.

- The Tender will not be considered if any variation or alteration is made by the Tenderer in the Supplier’s Contract and Specification, as issued by the Council.

- Tenderers are prohibited from directly or indirectly canvassing members or officials of the Council in reference to any Tender.  The Tender of any person who does so canvass will be rejected.

- The Council will reject any late Tender.

- The Council does not bind itself to accept the lowest of any Tender and will

evaluate and consider which Tender is the most economically

advantageous.  The Council also reserves to itself the right at its absolute

discretion to accept a Tender wholly or in  part at the price or prices set out n

the documents accompanying the Tender.  When a Tender is accepted

either wholly or in part the Council will enforce all the terms of the Contract.

- The Council will not be responsible for or pay any expenses or losses that

may be incurred by any person in the preparation of his Tender.

TENDER A

The work as detailed in these documents will be carried out for a total price of:

£ 	in the sum of

……………………(words)

(excluding VAT)

If my tender is successful, the works shall be commenced on:

(date)

(provided that the Council shall have issued to the Contractor a written notice to proceed) and shall be executed in accordance with the Contract for the same.

I agree that this tender and any contract which may result therefrom shall be based on the Council's conditions of tendering, specification and supplier’s contract.

Signed:

In the capacity of:

(BLOCK LETTERS)

Duly authorised to sign tenders for and on behalf of:

Date:

Contractor's name:

Address:

Post Code:

Telephone:

The price quoted in this Tender is valid for ...........days from the Tender Returnable date, subject to that period being not shorter than 60 days. If no validity period is stated, it will be assumed that the offer is firm for 60 days from the Tender Returnable Date or from the date of the last correspondence, whichever is the later.

Tender to be delivered by not later than NOON FRIDAY 28th September 2019

TENDER B

The work as detailed in these documents will be carried out for a total price of:

£ 	in the sum of

……………………(words)

(excluding VAT)

If my tender is successful, the works shall be commenced on:

(date)

(provided that the Council shall have issued to the Contractor a written notice to proceed) and shall be executed in accordance with the Contract for the same.

I agree that this tender and any contract which may result therefrom shall be based on the Council's conditions of tendering, specification and supplier’s contract.

Signed:

In the capacity of:

(BLOCK LETTERS)

Duly authorised to sign tenders for and on behalf of:

Date:

Contractor's name:

Address:

Post Code:

Telephone:

The price quoted in this Tender is valid for ...........days from the Tender Returnable date, subject to that period being not shorter than 60 days. If no validity period is stated, it will be assumed that the offer is firm for 60 days from the Tender Returnable Date or from the date of the last correspondence, whichever is the later.

Tender to be delivered by not later than NOON FRIDAY 28th September 2019