| Contract: | OVERSEAS PRIME CONTRACT (OPC) – CYPRUS CONTRACT NUMBER: 71532400 |
| --- | --- |
| Work Order No: | WC1680036 – Dhekelia Storm Network – Works to prevent further erosion, overland flows, or downstream flooding to the SFA project |
| Employer: | OPC Contractor and Defence Infrastructure Organization (DIO) |
| Main Contractor: | Cyprus Services Provider Joint Venture |
| Site Address: | Dhekelia Station, BFPO58 |
| General Scope: | WC1680036 – Dhekelia Storm Network – Works to prevent further erosion, overland flows, or downstream flooding to the SFA project Demolish and reconstruct (increased in size) open storm drain channel with approximate length of 1.2km. Additionally provide attenuation ponds/basins along storm drain channel route to reduce peak flows - Site preparation, protection works and environmental controls - Demolition and removal of the existing channel structure, including breaking out concrete and removal of all arisings from site to an approved disposal facility. - Excavation to formation level along the existing channel alignment to achieve required levels and gradients for the new channel section. - Placement of sub-base materials, including granular fill and blinding concrete to prepare a level and stable formation. - Installation of reinforcement in accordance with design drawings, including bar fixing, supports, and spacers. - Formwork erection to provide the new orthogonal channel profile, including any temporary works required for stability and access. - Provision and formation of joints, including expansion and contraction joints at locations specified in the design. - Casting and curing of concrete to the specified levels, gradients, and finish, ensuring appropriate curing procedures and quality control. - Completion works, including removal of formwork, backfilling to final levels where required, surface reinstatement, and site clean-up. |
| Project Engineer: | Ioannis Korakis |
| Tender Period: | 6 weeks |
| Tender Validity: | Tender to remain open for acceptance for 20 weeks |
| Tender Return Means: | Tender is to be advertised through the Defence Sourcing Portal (DSP). Tenders should be returned in a sealed envelope delivered in person to Dhekelia Station Cyprus. For tenders submitted via courier, as long as there is proof that the envelope was received by the courier company within the tender period, the tender will be accepted even if it arrives to the tender box after the expiry of the tender period. In this case, you are to provide CSP JV (via email or fax) with a copy of the courier receipt before the tender expiry date. |

With reference to the above works, you are herewith kindly requested to confirm your intention to tender. If you are unable to tender on this occasion, it will not prejudice your inclusion on tender lists for other projects under our direction. Following your agreement to tender, you will receive an invitation to do so.

Please reply by 17/12/25

We hereby confirm our interest in participating in the above tender.

We will not be participating in the above tender.

If you have confirmed your interest in participating in the above Tender, please provide a list of projects executed over the last 5 years with similar scope of work.

| On behalf of: |  | Tel No: |  |
| --- | --- | --- | --- |
| Name: |  | Fax No: |  |
| Signature: |  | E-mail: |  |
| Date: |  |  |  |