Ashbourne Methodist Church

Link Project

Levelling Up Funding Scope

Audio, Video and Performance Lighting Systems

AMCD Link Development Team

July 2025

Issue  - 1.0

Ref : AMC/LNK/ITT/03

Contents

Section 1 – Summary	4

1.1 Introduction and Document Purpose	4

1.2 Summary Scope of Tender	4

1.3 	Overall Project Background – Vision and Aim	4

1.4 	Site address & Buildings	4

1.5 Outline Programme	5

Section 2  Technical Scope	6

2.1 	Design	6

2.2.	Flexible Space Assumptions	6

2.3 	Infrastructure Works – Worship /Performance Space	7

2.4 Audio System – Worship / Performance Space	7

2.5  Video and Streaming System – Worship / Performance Space	8

2.6 Performance Lighting System – Worship / Performance Space	10

2.7 Meeting Room Systems – Lower Link	11

2.8 Billboard Display and other AV – Link Foyer	12

2.9 Training	12

Section 3 – Instructions to Bidders	14

3.1 Definitions	14

3.2 General	14

3.3 Tender Documentation	14

3.4  Confidential Nature of Documents and Bids	14

3.5 Preparation of Tender Response	15

3.6 Proposed Contract Terms and Conditions	16

3.7 Compliance with the Equality Act 2010	16

3.8 Employer Contact	16

3.9 Site Visit	17

3.10 Submission of Tenders	17

3.11 Clarifications, Questions and Answers	18

3.12 Tender Procedure and Timetable	18

3.13 Evaluation of Tenders	18

3.14 Award Process	20

3.15 Bidder Warranties	20

# Section 1 – Summary

### 1.1 Introduction and Document Purpose

This Invitation to Tender is for the provision of Audio, Visual and Performance (Stage) Lighting systems as part of the Ashbourne Methodist Church (AMC) Link Community Hub Project. 
The AV & Lighting scope to be delivered and described in this document, is part funded by a grant from Central Government for Levelling Up Funding (LUF), which is managed by Derbyshire Dales District Council. 
The procurement of these facilities is subject to Government Public Procurement Regulations.

### 1.2 Summary Scope of Tender

1.2.	The Summary Scope of this Tender is

For the Worship / Performance Space

- Design, cabling and hardware infrastructure for the new and upgraded AV systems

- Sound System re-installation and upgrade works

- AV / ‘Proclaim’ Presentation and Streaming System re-installation and upgrade works

- Performance lighting – New Lighting and Control equipment installation

- Tie Lines from the Worship / Performance space to the Foyer and Century Hall

Within the 3 new Meeting Rooms in the Lower Link

- New AV and online meeting facilities.

Within the Foyer

- New ‘Billboard’ display for the community information system.

- Bus departure information unit

Testing, Commissioning, Documentation and Training

1.2.2	Flexible Use of Systems

For the church / performance space the systems must be designed for flexible use of the space.  As the building works have removed the pews and levelled the floor, flexible use of the space will include having the seating and worship/presentation dais in alternative positions.  Alternative camera, presentation, visual display, speaker and main desk positions all have to be allowed for in the design.

1.2.3 	Current Works Status

The  Main Contractor has been on site since November 2024, and is scheduled to hand back completed buildings from late June 2025 onwards, in phases.  The first building handed back is the worship/performance space where most of the AV and lighting systems are to be installed.

The church community are currently worshipping in a local school and the original AV systems and equipment is either in use at the school or has been removed to store.

### 1.3 	Overall Project Background – Vision and Aim

A significant development project has been planned by the Ashbourne Church since 2016.

The Vision and Aim of Project is summarised as follows:

Ashbourne Methodist Church (AMC) is already an active church and community facility, and we have a vision  to develop our activities and facilities to become a 7-days-a-week Church, Community Hub and Performance and Events space.

AMC has large Victorian premises and some land in the centre of town. However, the buildings have severe issues with three buildings on three different levels – with connection between them being outside and via difficult stairs. Whilst AMC has an attractive church space it is inflexible and there is a lack of medium or smaller rooms, there are poor toilets and kitchens, inefficient and challenging maintenance and heating issues. In addition, AMC has difficulties with being secure in what's been called a rabbit warren of buildings.

To address all these issues and provide buildings and facilities which will be suitable for the church and community for decades to come and also make a significant contribution to the revitalisation of the town centre, the LINK PROJECT has been developed: “Linking Buildings, Linking People, Building Community”.

### 1.4 	Site address & Buildings

1.4.1.	The site address is

Ashbourne Methodist Church

36 Church Street,

Ashbourne

DE6 1AE

1.4.2  In summary the existing buildings comprise

- The Church, accessed from Church Street,  seating  ~350 and raised 6 steps above pavement level.

- Gateway accommodation below the Church. Originally built as school rooms this area was previously converted to dormitory accommodation for youth or similar groups.

- Century Hall at church level, and accessed from Station Road

- Waterside Hall, below Century Hall and at the same level as Gateway

- Cornerstone.  Originally built as 3 small shops, this combined space is now a thriving café, with level access from both Church St and Station Road

- Additional buildings on site include Chapel House and the Shop.

The church dates from 1880, and the halls and Cornerstone building from ~1900.  The site is Grade II listed.  Of most importance are the church and Century hall.

1.4.3 The new ‘Link’ building comprises

- A Foyer with level access from Station Road

- A balcony at church level with stairs and lift access

- 3 meeting rooms, toilets and changing places room below the Foyer.  This is level with Gateway and Waterside hall, with stairs and lift connecting to the Foyer

- Level access connection from the foyer to Cornerstone

Overall site plan drawings are included for information

### 1.5 Outline Programme

The target programme for the works is
		Tender Award				Early September 2025
		Site Installation Commencement	November 2025
		Site Installation Complete		December 2025
		Training 				December2025 and January 2026
		Test Events				March 2026

# Section 2  Technical Scope

### 2.1 	Design

2.1.1	The Tenderer is to be the Designer of the installation. 
Outline information is provided in further sections of this Tender document for pricing, but this is only a representation of the final system designs required.  Tender pricing must allow for correcting and developing the outline into a fully workable solution.

All designs must be submitted and presented prior to equipment purchase.  It is expected that 3 iterations of the design will be required before reaching a final system design for implementation. 
Any item to be remain visible within the Grade 2 Listed Worship/Performance space, or fixed to the building fabric requires approval by the Methodist Church.  Detailed drawings at 1:25 scale will be required for wall boxes, equipment wall mounting plates, lighting bars and similar.
All permanent visible equipment, mounting plates etc should be selected or painted to blend in with the location. In most areas this means painted white, with white cables.

### 2.2.	Flexible Space Assumptions

The following layouts and uses are to be considered in the upgraded systems designs, and associated infrastructure cabling

Worship Layouts

2.2.1  Traditional – Seats across the width of the space facing the organ, with worship leaders on a small raised  dais against the organ screen. Worship musicians on a second raised dais  to left or right of worship leaders.  Existing seating (pews) in balcony is available for congregational use

2.2.2.  Modern – Seats across the length of the space facing the East windows, with worship leaders on a small raised dais against the wall under either the centre window or towards the corner near exit to corridor behind the organ. Worship musicians on a second raised dais.  Seating in the balcony is available for use.

2.2.3.  In the round - Worship leaders on a small raised dais near the centre, with seating in horseshoe around.  Worship musicians on a second raised dais.  Seating in the balcony in available for use.

Performance Layouts

2.2.4  Speaker / panel discussion event.  Using a small raised dais and seating as either the Traditional or Modern worship layouts above

2.2.5  Adult Choir – Seating for choir of ~50 on 3 tier dais, arranged against organ screen with conductor in front.  Audience in rows across the space facing the choir, and with the balcony in use.

2.2.6  Adult Choir, Children’s Choir and Musicians.  Adult choir as above, with two side dais for children’s choir and musicians.  (As a ‘Hot House’ concert)

Operator Desk Positions

2.2.7  Infrastructure cabling is required to allow the operators desk position to be either on the performance floor or the balcony above.  Different uses of the space will require the operator’s desk and equipment to be quickly and easily repositioned.

### 2.3 	Infrastructure Works – Worship /Performance Space

2.3.1 Floor and Wall Boxes Cabling
To progress the main contractor’s works, the Client has outlined a scope of infrastructure containment connecting new AV Wall Boxes, Floor Boxes and other similar outlet positions, with a main Equipment / Patch Panel near the operators desk position.

These positions and cabling capacities are to be reviewed, revised as required, and incorporated into the AV Tenderers final Designs.  
Cable containment will be provided by the main contractor. Technical cabling is to be provided by the successful AV Contractor, for pulling in and fixing by the main contractor.

2.3.2  Wall Boxes, main Equipment / Patch Panel and Outlet Plates
The AV contractor is to design, build and install the proposed 6 Wall Boxes, and main Equipment / Patch Panel.  
Floor boxes will be provided by the main contractor with 2 vacant sections for AV cabling.

All technical cabling outlet plates, inserts (eg for XLR, HDMI,5pin DMX, RJ45), and cable terminations are to be provided and installed by the AV Contractor. 
Patch cords are to be provided for the main Equipment / Patch Panel connections between the wall and  floor boxes,  and the main equipment and / or the operator desk.

2.3.3  Main Operator Desk
The design intent is for the operator desk to have a minimum of equipment on it, with the main hardware located remotely in the Equipment /Patch Panel unit wherever possible. 
The desk should then be connected to the Equipment / Patch Panel via an umbilical and a minimum number of plugs (ideally, one for audio, one for vision and camera control, one for Proclaim PC control, one for lighting control).  The sockets in the Main Equipment Panel are to be replicated at the Balcony Wall Box for easy relocation of the Operator Desk position to the balcony.
AV Tenderers should include a Provisional Sum for the Operator Desk ‘woodwork’, which will have 3 positions for Audio, Vision / ‘Proclaim’ and Lighting operators.

### 2.4 Audio System – Worship / Performance Space

2.4.1   The issues with the existing Soundcraft desk include difficulty is separating the local ‘room /speaker’ mix output from the ‘record /Proclaim’ output, while retaining another output for the Hearing loop.  For full control, the piano, and ‘ambient’ mic inputs routing and level controls, need to be different for each of the 3 outputs, which is not currently possible.

An enhanced sound desk is to be provided with sufficient configurable outputs.  For tender pricing a new Soundcraft Signature 16 desk is to be included.  As part of the post contract design work, the desk type and size is to be reviewed to ensure it is functionally suitable. 
The client does not think a fully digital sound desk and infrastructure is a worthwhile investment at this time.  Existing operators have varied levels of experience and the systems for Worship need to be as simple and straightforward as possible.  The proposed cabling infrastructure therefore is based around standard analogue audio cabling.

2.4.2.  Hearing Support
For tender pricing, a new ‘loop’ amplifier and floor tape installation to replace / replicate previous, is to be included.  As part of system design work, post contract, other options for hearing support are to be reviewed as many hearing aid users can no longer connect to loop systems.

2.4.3  Speakers and Amps
2.4.3.1 Where possible, existing equipment is to be re-used.  However the need to avoid feedback with the varied flexible seating arrangements may require additional speakers and equipment, or a complete new system.  For Tender purposes, new speakers for the main floor and, separately the balcony area, are to be included for.  These speakers are for connection to the existing amplifiers.
The proposed cabling infrastructure provides speaker cabling at each wall box and in the roof space, which should be sufficient to allow connections once final speaker positions are agreed.  The AV Contractor is to allow for temporary stands and equipment as necessary to test and demonstrate proposed speaker positions before final connections are made.
Large array speakers will not be accepted by the Methodist Church Conservation Officer, who is responsible for Listed Building consents.

2.4.3.2 Fire Alarm Shutdown
Audio systems are to shut down when the fire alarm is activated.  A fire alarm shut down relay will be provided at the Equipment/ Patch Panel, for the AV  contractor to connect to.

2.4.4.Microphones and Sources
The 3 existing radio mics are to be retained.  The AV contractor is to confirm these are in the correct ‘free to use’ band and advise of any spare channels in the ‘free to use’ band(s) that could be used in future, if required.

The existing microphones used for lecturn, pulpit, piano, congregation / ambience are to be reviewed for continued suitability, and re-used or replaced as appropriate.  Location of fixed mics for congregation / ambient needs to allow for the flexible layouts and avoid speaker pickup and resultant feedback.

The system design is to allow for ‘play-in’ from either a local DVD player or material from a local PC, as well as from Proclaim

2.5  Video and Streaming System – Worship / Performance Space

2.5.1. Proclaim Software and PC. .  Proclaim is the current worship presentation software used, and provides outputs for display in the church and an output for streaming.  The church display and streaming outputs can be configured in Proclaim to have different content.  
This software is currently hosted on a 3 year old laptop, which also hosts the existing OBS streaming software.   Proclaim will remain at the heart of the church’s video presentation system.

For tender pricing, a new PC is to be provided to host Proclaim in the main Equipment / Patch Panel, with remote keyboard video and mouse connections to the operator desk.  A new hardware streaming unit is to be included in the tender pricing to replace the current OBS software streaming solution.  A new ‘Proclaim’ 14 inch monitor is to be provided on the operator desk.

2.5.2 Video inputs, outputs, switching and displays.  
The existing video inputs to the system are from a single fixed camera with pan, tilt and zoom presets and controls, and from Proclaim.  The camera input is normally routed to Proclaim. 
The main Proclaim output is routed to the existing projector for display on a screen in front of the organ.  The HDMI format is used for all existing connections and cables, with a simple 4x 4 matrix used to route and duplicate signals as required. 
This system was installed during Covid and while it has worked well, there are limitations.
  
2.5.3 Cameras – With only a single camera it is not possible to preview a next ‘shot’, and the existing hand held controller (as a TV remote)  makes it unreliable to move to another pre-set or live camera adjustment.  There is also no existing camera ‘preview’ display at the operator position
The new system should provide a second compatible camera, and new hardwired selection and camera control panel.

2.5.4 Camera Control - This is to be upgraded to a cabled connection from a new selection and ‘pan, tilt, zoom’ control unit on the operator desk. The camera video and control system is to be sized for up to 4 cameras for future flexibility.

2.5.5.Video Switcher and Operator display. - A new 4 input video source switcher with ‘preview’ and ‘live’ display is to be provided for the video input to the Proclaim PC.  The two outputs from the Proclaim PC for ‘church displays’ and ‘streaming’ will be routed via a new splitter and the existing matrix.  These will provide at least 6 outputs from the ‘church display’ splitter, and at least 4 outputs of the streaming output.
A quad split screen 14inch monitor at the operator desk is required to show the two Proclaim outputs and possibly the Proclaim input.

2.5.6 Video Format and Convertors - For tender pricing, it is assumed all video switching and manipulation devices, will use HDMI video format.  However, the location of cameras, projector and display screens will exceed the normal HDMI cable route length.  For tender pricing, HDMI/ Cat 6 signal convertors are to be provided for cable lengths beyond the limit of HDMI signals. Convertors shall extend the signal for at least 100m.
Where possible convertors should not need to be used as dedicated pairs.  This will allow, for example, Camera 1 to be plugged into any wallbox (equipped with convertor), and be patched through to the Main Eqpt panel ‘Camera 1’ input  convertor.

2.5.7.Projector and Displays -The existing projector is to be re-installed with a new mount. This will be used for the ‘traditional’ worship layout.
For the ‘modern’ layout the projector will be used for the congregation in the balcony.  
2 new large monitor displays mounted on moveable telescopic stands are to be provided for the congregation on the main floor. The client does not believe display screens mounted on the building walls will be approved by the Methodist Church Conservation Officer.
For pricing, two 75inch UHD screens are to be included.
The mounting height of the screens will exceed standard commercial mobile stands used in offices, as the screens must be fully visible when the congregation is standing. (ie the bottom edge of the display should be ~1.9m from the floor)
The stands are to be telescopically height adjustable, so the screens can be moved through standard doors to alternate rooms for storage.
The existing rear Tv for Presenter feedback is to be retained for use in both traditional and modern layouts.  New mounts are to be provided. 
All cabling to the local AV and power outlet position is to be included for all display equipment.

2.5.8.Camera Positions and Mounts - 
For the traditional and modern layouts, the cameras will need to be mounted in different locations.  To minimise visual impact, a common mount system is to be provided so the cameras can easily be lifted off and move to alternate positions.  An example of a possible mount is included in the drawings and sketches, or alternatively a split batten system could be proposed for approval.

### 2.6 Performance Lighting System – Worship / Performance Space

2.6.1 Overall Outline

The proposed system is to be sized to allow for fixed LED luminaires to provide overall ‘colour washes’ to the various dais positions, and for additional per event rigging of additional hired LED luminaires.  
The fixed luminaires are proposed at high level wall bars, with spare capacity at the bars for additional per event luminaires.  Performance lighting power and lighting control will also be available at the AV wall boxes for hired luminaires  to be rigged on floor stands.

2.6.2 High Level Wall Bars

High level wall bars are a compromise solution, as an originally proposed winched lighting truss was not acceptable either to the structural engineer or Conservation Officer.  
Each bar should be sized for at least 5 luminaires, and be internally wired for power and control.  For tendering, power connections should be 16A ‘Powercon’ and control should be DMX wired via XLR or similar connectors.
Power and control cabling will be pulled in as part of the main contractor’s work to outlet boxes adjacent to the proposed bar positions. The control cabling is for the AV contractor to supply and terminate(see 2.3.1 above).

2.6.3 Lighting Plots

The successful tenderer shall create lighting plots for the dais areas, and for an example play.  This is to ensure the wall bar locations, supported by additional floor stands, can adequately light typical performances and use of the space.  If the plots are found to be heavily compromised, it may be possible to seek approval for one high level bar across the width of the space.

2.6.4 Lighting Control

In additional to hard wired DMX controls, a DMX wireless system shall be provided. The wireless DMX will typically be used for rechargeable battery powered ‘wall washer’ luminaires. For example around the organ.

The infrastructure cabling (see 2.3.1) as currently proposed provides for 5 separate DMX chains for:
High Level Bars – East
High Level Bars – West
Wall Boxes -East and Organ
Wall Boxes – West
House Lighting and Roof Space

The Roof Space connection allows for a possible single bar across the width of the space.

A new DMX control unit is to be provided at the operator’s desk. This is to be tendered as a programmable physical control panel, rather than PC /software solution.

2.6.5  House Lighting

The House LED Lighting will be Dali dimmable in 4 sections with manual dimmer controls provided for day to day (non performance) use.   The Performance Lighting DMX system is to be interfaced to these house lighting circuits so the DMX system can control the house lighting during performances.   The AV contractor is to provide and instal suitable DMX/ Dali interfaces and other equipment as required for this.  This may include a ‘control at’ selector switch to change from the manual controls to the DMX performance lighting controls.

2.6.6 LED Luminaires

For tender pricing,10 multicolour/hex ‘Parcan’ LED luminaires shall be provided for lighting the various dais configurations, with 2 other luminaires for conductor’s lighting and 4 wall wash battery power uplighting luminaires.  The wall wash uplighter units to have DMX over wifi connections.
The wall bar mounting height and distance to the dais, may require luminaires with tighter beam angles than usual.

### 2.7 Meeting Room Systems – Lower Link

2.7.1 Small Meeting Rooms

The two small meeting rooms only require simple ‘ in the room’ displays and wiring. In each room a wall mounted 65inch TV monitor is to be installed with connection looms for PC hook-up in a floor box in the centre of the room, and a second wall plate to the side of the display.  This allows for the presenter to stand to the side of the screen or be seated at the meeting table.

A presented using a wifi connected laptop, would be able to display their Teams call to local colleagues on this display

2.7.2 Large Meeting Room

For ‘in the room’ presentation, the TV screen and 2connections points provided in the small meeting rooms will also be required in the large meeting room.

Additionally, so the room can act as an independent meeting hub, a host PC and software is to be provided, together with simple ‘meet now’ controls.  As a minimum meeting via Teams and Zoom is required.

### 2.8 Billboard Display and other AV – Link Foyer

2.8.1 Billboard

The Foyer acts as the main entrance and gathering point for the building.

A ‘billboard’ portrait 65ich display is to be provided on a commercial mobile stand.  While our Proclaim software may be able to provide suitable billboard screens, Tenderers should include for suitable billboard software, and then evaluate Proclaim’s suitability during the contract.  The software should display a real time clock, and up to 5 rotating screens of information, A section of the each display should always show the local ‘next 3 buses’ details. (For example: destination, due, on time/ minutes delayed).  
At present a ‘bus ticker’ display is proposed by DCC as a separate unit.  However, aesthetically it is preferred to display the bus data on the billboard screen.  If any data conversion is required of the ‘bus ticker’ feed for use on the billboard this will be subject to post contract change.

2.8.2 Other Foyer AV

Audio and video ‘tie’ lines  (4 of each) from the main worship/performance space system are included in the cable schedules.  Thes are to be terminated in 2 deep socket outlet boxes.

New ceiling mounted speakers are to be provided in the Foyer, for connection to exis

ting portable equipment. 
It is not yet agree if any other AV floor boxes or facilities will be provided in the space.

2.9 Training
The following on site training is to be provided

Audio Systems
2 session - Detailed Technical Level training for up to 4 people per session. To cover setting up from scratch, and all desk adjustments. To include re-patching for moving from traditional to modern seating layouts, and for alternative operator desk locations.

2 sessions – Operator Level training for up to 4 people on each occasion. 
To cover basic operation, prevention of feedback, and minor adjustments

Video Systems
2 sessions - Detailed Technical Level training for up to 4 people per session.  To cover setting up from scratch, and all adjustments.  To include re-patching for moving from traditional to modern seating layouts, and for alternative operator desk locations.

2 sessions – Operator Level training for up to 4 people per session. 
To cover basic operation, and minor adjustments.

Lighting Systems
2 sessions - Detailed Technical Level training for up to 4 people per session.  To cover setting up from scratch, all desk programming and adjustments, moving luminaires and re-assigning DMX channels  etc.

Test events
To support 2 separate evening test events, the trainer is required to supervise the operators during both rehearsal and show periods

# Section 3 – Instructions to Bidders

### 3.1 Definitions

References to “the Client”, “the Employer” or “AMCD” shall mean “Ashbourne Methodist Church Development Ltd”.

References to “AMC” shall mean “Ashbourne Methodist Church Trustees”, who have awarded a contract to AMCD for construction of the Link Project.

References to “DDDC” shall mean “Derbyshire Dales District Council”, who are responsible to Government for expenditure of the Levelling Up Funding supporting this project, via a Grant Funding Agreement with AMC.

References to “Tender” shall mean Tender or quotation.

Reference to “Tenderer” shall mean the organisation participating in the Tender or quotation process.

### 3.2 General

These instructions are designed to ensure that all Tenderers are given fair and equal consideration and to ensure compliance with relevant legal requirements.

These instructions shall form the conditions of participating in the procurement process.

Failure to comply with these instructions may result in the rejection of the Tender submission.

### 3.3 Tender Documentation

All materials issued in connection with the Invitation to Tender shall remain the property of the AMCD and shall be used only for the purpose of this procurement exercise.

No unauthorised alteration or additions shall be made to any component of the Tender documentation.

### 3.4  Confidential Nature of Documents and Bids

All information supplied in connection with this tendering process shall be treated as confidential by Tenderers except that information, which may be disclosed so far as it is necessary for the purposes of obtaining sureties, guarantees and quotations required for the preparation and submission of their bid.

Tenderers shall observe all security/confidentiality and data protection obligations laid down by the Employer.  This shall include not divulging to any third party any information or data supplied by or obtained from the Employer in the course of the Tenderer preparing their Tender response, except that which is expressly necessary for the preparation of a bid, or where otherwise approved in writing by the Employer.

Tenderers shall not disclose they have been invited to bid, nor discuss the bid they intend to make nor canvass for its acceptance, other than with professional advisers who need to be consulted.  Bids shall not be canvassed or discussed with any other Tenderer or Employer’s team.

Tenderers shall not at any time release any information concerning the invitation or the Tender documentation to the media or any other person.  Tenderers should note that this project is considered to be subject to the provisions of the FOIA/Freedom of Information Act 2000 (‘the Act’) and the Environmental Information Regulations 2004 (‘the Regulations’).  This means that information may be subject to disclosure to the public unless an exemption applies.  This includes such things as (but not exclusively):

Information in any Tender submitted to AMCD

Correspondence and other papers

In the event that a Tenderer considers that any information supplied by it is either commercially sensitive or confidential in nature, this should be specifically highlighted in Schedule 4 (FOIA Schedule) with the reasons for its sensitivity given and an explanation of the grounds for exempting that information from disclosure.  The Tenderer should note that even where they have indicated that they consider the information to be commercially sensitive or confidential in nature, AMCD may be required to disclose it under the Act or Regulations if a request is received.  Please note that information marked as commercially sensitive or confidential by the Tenderer should not be taken to mean that the Employer accepts any duty of confidentiality by virtue of that marking.

### 3.5 Preparation of Tender Response

Completion of Questionnaires, Schedules and Documents

All questions should to be answered fully in the space provided. If the space for any question is not sufficient please continue on a separate attachment, to be returned with your electronic bid, clearly cross referencing the original question.

For the sake of brevity where appropriate, references to 'your organisation' include partnerships, sole traders and co-operatives, NPO’s etc.

If you are part of a group of companies please answer all the questions specifically as the bidding organisation, not for the group.

Additional documentation may be requested by the Employer where required.

Tenders and all supporting documents must be written in English.

Tendered prices must be in pounds sterling and whole pence to two decimal places, excluding VAT.

Tendered Prices must include all relevant costs required to meet the contract and the needs of the service, as described in this document.

Tenders submitted must be open and valid for acceptance for 180 calendar days from the closing date.

Any subsequent contract which may be entered into shall be subject to and in accordance with the law of England in its formation, interpretation and performance.

It is the responsibility of each Tenderer to obtain for themselves at their own expense any additional information necessary for the preparation of their bid.  The Employer will not be liable for any costs incurred by any Tenderer:

In the preparation and/or submission of their Tender response.

Due to any subsequent requirement to attend meetings, presentations or demonstrations.

### 3.6 Proposed Contract Terms and Conditions

Any appointment arising from this Invitation to Tender and received Tenders shall be governed by the appropriate D&B minor works contract form, based on the JCT standards, and updated to meet the needs of DDDC and/or Public Procurement regulations where applicable.

There is no main contractor support to these works and all site set up, management, access equipment, tools and plant is to be included for in the tender price.  Welfare facilities are available.

Variations During Contract
The pricing schedules to be returned include for labour and materials rates which may be used to inform the valuation of any changes during the contract period.

### 3.7 Compliance with the Equality Act 2010

The Levelling Up Funding will be via DDDC, and so AMCD have to adhere to agreed policies and processes.

As it is a priority of the DDDC’s policy to use whatever opportunities are required or allowed by law, to ensure that contractors and their agents accept a commitment to equalities, AMCD are following this DDDC policy.

- The contractor and any sub contractor employed by the contractor shall adopt policies to comply with all legislation including the Equality Act 2010. The contractor will be required to monitor its performance as an employer and service with regard to equalities as specified by the Employer.

- The contractor is required to ensure that information and services are fully accessible to all sections of the community and that resources are targeted to ensure that best value is provided.

- The contractor is required to make every effort to consult, involve and encourage the participation of local communities.

- In the event of any finding of unlawful discrimination being made against the contractor or any sub contractor employed by the contractor during the contract period, by any court or employment tribunal, or of any adverse finding in any formal investigation by the Equality and Human Rights Commission over the same period, the contractor shall inform the Employer of this finding and shall take appropriate steps to prevent repetition of the unlawful discrimination.

As DDDC will monitor AMC’s performance in this area, AMCD will similarly monitor its contractors and professional team.  Non-compliance will be treated as a serious breach of contract and may result in the contract being terminated and/or the contractor not being permitted to tender for future contracts.

Tenderers shall complete the Equalities information as included in the Qualification Information schedule.

### 3.8 Employer Contact

The Employer’s Project Manager and Quantity Surveyor will manage the Tender process. Their contact details are

Greenwood Projects Ltd
Mr A Bates (PM)	adrian.bates@greenwoodprojects.com
Mr T Birch (QS)		tom.birch@greenwoodprojects.com

The main Employer Contact for applicants will be Mr Richard Barratt of Ashbourne Methodist Church Developments Ltd.  This is the contact to arrange any site visit.  
Mr Ian Marsh will be the secondary contact should the main contact be unavailable.

Contact details are

Mr Richard Barratt 	 	Linkops2@ashbournemethodist.org.uk

07710 725014

Mr Ian Marsh 	 	Linkops1@ashbournemethodist.org.uk

### 3.9 Site Visit

It is expected that all applicants will wish to arrange at least one site visit (by appointment only) prior to submission of tender. Appointments should be arranged with the Employer’s Contact.

### 3.10 Submission of Tenders

The closing date and time for the receipt of Tender submissions, including all supporting documents and additional bid information, is 17:00 hours on Wednesday 27th August 2025.

Tenders are to be submitted via the electronic portal at myTenders.co.uk

It is the Tenderer’s responsibility to ensure that their bid, including all supplementary information, is submitted prior to the stated closing time and date.

Bids submitted after the closing date and time will NOT be accepted.

Tenderers are advised to complete their bid submission early enough to allow for the number and size of documents in their submission.

Tenderers shall complete the following documents and include them as part of their Tender response:

Completed Qualification Information  (Schedules 1A and 1B)

Completed Formal Written Proposal and Quality Evaluation (Schedule 2)

Completed Fee Schedule (Schedule 3 part)

Resource Schedule (Schedule 3 Part)

Signed Form of Tender

CV’s of Project Staff

Evidence of Insurance

DDDC Financial information Schedule (Schedule 1B)

DDDC FOIA Schedule (Schedule 4)

Submission of an offer will be taken as acceptance of the terms and conditions detailed within the Tender documentation.

Failure to provide a complete submission including the documents stated above and the required supporting documentation, will result in your bid being deemed to be non-compliant.

### 3.11 Clarifications, Questions and Answers

Tenderers are required to submit any questions or requests for clarification via the electronic portal at myTenders.co.uk no later than 17:00,  7 days prior to the tender closing date.

All questions will be considered by the Employer, and where appropriate, responses shall be provided by the date detailed in the Tender timetable below. The final decision to answer any question shall be purely at the discretion of the Employer.

All questions and responses which are considered by the Employer to be of a substantive nature will be formally distributed to all Tenderers via the electronic tender portal by the date detailed in the Tender timetable below.  Questions and responses shall be anonymised to protect Tenderers identities

Any questions that are of a sensitive or confidential nature must be clearly identified as such on submission.

### 3.12 Tender Procedure and Timetable

The timetable is as follows:

Invitation to Tender (ITT) advertised: 	Monday 14 July 2025

Site visits (by appointment) 	Wednesday 23rd July to Wednesday 13th August 2025.

Clarification Requests and Questions issued by

17:00 Friday 15th August 2025

Clarification and Question Responses issued by  
  						17:00 Wednesday 20th August 2025

ITT Return Deadline 				Before 17:00 Wednesday 27 August 2025

Evaluation of returned tenders and selection of preferred bidder is targeted to be completed by early September.

AMCD reserves the right to amend the above timetable which is indicative only.

Tenders must be submitted electronically to the mytenders.co.uk portal.

### 3.13 Evaluation of Tenders

Compliance

The Employer will examine submissions for completeness and compliance against the requirements of the Tender documentation, including these instructions for bidders.

The Employer may seek clarification where necessary.

Prior to detailed assessments, the Employer will determine whether a submission substantially fulfils the requirements of the Tender documentation. The Employer reserves the right to reject any submission determined as not substantially fulfilling such requirements.

Assessment

The tenders will be assessed against:

| Qualification Information |  | PASS / FAIL |
| --- | --- | --- |
| Quality Evaluation |  | 30% weighting |
| Pricing Proposal |  | 70% weighting |

Qualification Information

Schedules 1A &1B are to be completed in full and returned in the Tender.  Schedule 1A has ten sections, and Schedule 1B is a DDDC required Financial schedule.  They will be evaluated as Pass / Fail.

Quality Evaluation

Schedule 2 lists the evaluation criteria and response requirements to be included in returned Tenders.  A weighted score will be applied to responses.  The weightings are included in the Schedule.  Before weighting, each element will be scored on a scale of 0-10 as follows:

| Assessment | Detail | Score |
| --- | --- | --- |
| Very Poor | Fails to meet requirements. | 0 |
| Poor | Low standard with significant reservations and unlikely to be acceptable | 1-4 |
| Good | Good standard, acceptable with minor reservations | 5 |
| Very Good | High standard | 6-8 |
| Excellent | Very high standard with no reservations about acceptability | 9-10 |

Price Evaluation

Price will count for 70 marks and will be evaluated on the basis of each applicant’s price.

The lowest overall tendered price will score full marks for price

The marks for price for each other tender will be calculated as follows:

Score = Maximum marks for Price x Price of lowest priced Tender

Price of Tender being evaluated

The costed proposal is to be broken down in table included in Schedule 3.

Clarification Interview

Any clarification interviews or information received post tender return will not be included in the tender scoring.

### 3.14 Award Process

The Invitation to Tender does not constitute an offer and the Employer does not undertake to accept the lowest or part, or all of any bid, even if all the requirements are met. The Employer reserves the right to abandon the Tender process at any time for any reason.

The Employer reserves the right to award a contract to a Tenderer in any or all of the work areas detailed in the specification / scope.

All Tenderers shall be notified in writing whether their bid has been successful or unsuccessful.

As a matter of good practice, the Employer will allow a voluntary standstill period of at least 10 calendar days between notifying Tenderers of the decision to award the contract and executing the formal contract with the successful Bidder.

The acknowledgement of receipt of any Tender submission shall not constitute any actual or implied agreement between the Employer and the Bidder.

Tenderers must not undertake to perform or deliver any services without prior written notification from the Employer that they have been awarded the contract and are required to start performing their obligations under the contract.

### 3.15 Bidder Warranties

In making a Tender submission, the Tenderer warrants, represents and undertakes to the Employer that:

- all information, representations and other matters of fact communicated (whether in writing or otherwise) to the Employer by the Bidder, its staff or agents in connection with or arising out of the Tender are true, complete and accurate in all respects, both as at the date communicated and as at the date of submission of the Tender response;

- they have made their own investigations and research and have satisfied themselves in respect of all matters (whether actual or contingent) relating to the Tender and that they have not submitted their Tender and will not be entering into the contract (if the same be awarded to the Bidder by the Employer) in reliance upon any information, representation or assumption which may have been made by or on behalf of the Employer;

- they have full power and authority to enter into the contract and perform the obligations specified in the contract and will, if requested, produce evidence of such to the Employer.