# Specification

Peter Gooch Room and Kitchen, The Pavilion

| Peter Gooch Room |  |
| --- | --- |
| Fittings | All existing fittings removed and stored in changing rooms. Wall attachments (excluding fire bell, fire exit signs, smoke detector and wifi booster) removed and stored in changing rooms. |
| Ceiling | Whole ceiling recovered with plasterboard and skimmed; or fixed adequately. Painted with minimum 2 coats, brilliant white, good quality paint (Dulux or Crown). |
| Walls | Damage repaired, holes filled. Painted with minimum 2 coats, good quality, durable, washable, matt finish paint (Dulux or Crown) – off white, neutral colour. |
| Window frames & enclosures | Wooden sills and enclosures rubbed down, primed, painted white. Plastic frames thoroughly cleaned. |
| Interior doors | Painted with Ronseal or similar. Door closers fitted where missing. Doors adjusted, fixed as required. Northmost door replaced. |
| Radiators | Rubbed down, primed, painted to match the two new radiators. Pipes painted same colour as wall. |
| Floor | Deep cleaned. Re-grouted where necessary. Door mat replaced. |
| Electrics | Lights replaced to existing number and dimensions. |

| Kitchen (excluding walk-in cupboard) |  |
| --- | --- |
| Fittings | All existing fittings removed and stored in changing rooms. Any wall attachments removed and stored in changing rooms. New industrial cooker (<600mm width) installed including oven and four gas stove tops, to be placed on south wall. Adequate gas and electricity supply. |
| Cabinets (incl sink) | Existing units and sink removed. New kitchen units installed: two overhead 500mm units (north wall); two overhead 500mm units (south wall, to right of central pipe); two 600mm units (south wall); new units and sink/drainer (east wall) to meet units on north wall. Cabinet doors to be smooth with built in handles and easy to clean. Cabinets from Howdens or Benchmarx only. Work surfaces on all floor units. |
| Ceiling | Whole ceiling recovered with plasterboard and skimmed. Painted with minimum 2 coats, brilliant white, good quality paint (Dulux or Crown). |
| Walls | Damage repaired, holes filled. Tiles around sink area removed. New white tiles above all work surfaces to height of bottom of overhead units. Painted with minimum 2 coats, good quality, durable, washable, matt finish paint (Dulux or Crown) – off white, neutral colour. |
| Window frame & enclosure | Wooden sills and enclosures rubbed down, primed, painted white. Plastic frames thoroughly cleaned. |
| Interior doors | Painted with Ronseal or similar. Door closers fitted where missing. |
| Floor | Deep cleaned. Re-grouted where necessary. |
| Electrics | Lights replaced. Extension electrical socket (two sockets) to supply refrigerator. |
| All fixtures and fittings to comply with relevant commercial kitchens regulations. |  |

| Contractor’s recommendations |  |
| --- | --- |
| Any recommendations respecting the above |  |