32T Hookloader Specification

Vehicle Specification

- 32,000kg GVW.

- 8x4 axle configuration.

- Automatic gearbox.

- PTO and hook loader conversion, including sheeting system.

- Day cab with rear window.

- Driver air seat.

- Red seat belts.

- Pneumatic parking brake with audible warning.

- Height indicator in cab.

- No smoking sign in cab.

- Air conditioning and cab heater.

- 360-degree visible amber beacons - roof mounted (light bar or beacon), front, and rear.

- Front and rear facing LED work lights.

- Wheel nut indicators fitted to every wheel.

- Safety equipment FORS Silver compliant and TFL DVS 3 Star Rated:

  - Class V and VI close-proximity mirrors.

  - Side underrun guards (where practicable).

  - Blind spot warning signage (where practicable).

  - Audible left-turn and reversing vehicle alarms.

  - Reverse and nearside blind spot camera systems (min 720p video) with in cab colour display.
Camera system must NOT record or store video and must NOT transmit any data wirelessly.

  - Nearside close-proximity sensors with in-cab audible alarm.

  - Moving off sensors fitted to the front of the vehicle with in-cab audible alarm.

- Side storage locker.

- 2kg fire extinguisher in cab.

Hire Specification

- Hire with maintenance:

  - 36-month term.

  - 2,000km per annum.

- Road Fund Licence.

- Full Maintenance Package:

  - Servicing, maintenance, and repair of vehicle.

  - 12-weekly PMI inspections (Safety inspections / Roadworthiness inspections).

  - Annual testing and Preparation.

  - Servicing, maintenance, and repair of vehicle.

  - 12-monthly LOLER inspections + 6 monthly interim inspection.

  - Tyres.

  - Delivery & Collection service for service, maintenance, and repairs.

  - Maintenance planning and management.

  - Maintenance record keeping.

- Breakdown and Recovery.

Additional Requirements

- Familiarisation training provided to operators on delivery.

- Operating instructions.

- Maintenance instructions.

- Return condition terms and conditions.

- Early termination or break clause terms and conditions.

- Delivery address: Nuclear Restoration Services, Hinkley Point ‘A’ Power Station, Bridgwater, TA5 1YA.