Request for Quotation

To: Whom it may concern

From:

Tel:

Date of Issue: 18/12/2015

Response Deadline: 12:00 on 04/01/2016

Late submissions will not be accepted

Responses to be sent to: Sonia.kirby@cefas.co.uk

Title:  Request for Quotation for Water Sampling Rosette & Conductivity, Temperature, and Depth (CTD) System

Ref:

# Technical Specification (Schedule 1)

# Background to Requirement:

The Centre for Environment, Fisheries and Aquaculture Science (Cefas) has a requirement for a Water Sampling Rosette & Conductivity, Temperature, and Depth (CTD) System for the Research Vessel Cefas Endeavour.

Requirement:

# The System must:

# Work with the current Cefas Endeavour infrastructure (sea-cable). https://www.cefas.co.uk/about-us/facilities/rv-cefas-endeavour/

- Meet the minimum WOCE CTD + DO specification. http://www.go-ship.org/HydroMan.html, http://www.go-ship.org/Manual/IntroCollection.pdf

- Provide integration options for: Licor LI-128, Seapoint STM & SCF & an altimeter - range 0-30m integrated with the CTD data stream.

- Have a water sampler system with greater than >12 bottles, total volume no less than 180 litres.

- Be rated to 2500m minimum.

- Have real-time shipboard telemetry.

- Have on-board and post-cruise data processing software using standard UNESCO / WOCE methods.

- Provide data formats that support Interoperability with international and UK oceanographic data centres.

- Be able to be triggered from the surface.

Optional requirements:

- Integration with Satlantic SeaFET.

# Quantity:

1

Location for goods to be delivered:

Cefas Lowestoft

Delivery Date:

By March 15th 2016

Contract Duration:

5 Year limited Warranty

Quotations will be evaluated on value for money:

Quality	80%

Cost	20%

Information to be provided by the Tenderer:

- CTD and rosette specification.

- Outline of 3rd party integration procedures.

- Maintenance procedures.

# Submitted Pricing (Schedule 2):

| Goods and Services Required | Qty | Unit of Measure | Cost | VAT | Delivery Date |
| --- | --- | --- | --- | --- | --- |
|  |  |  | £ | £ |  |
|  |  |  | £ | £ |  |
|  |  |  | £ | £ |  |
| Total Cost: |  |  | £ | £ |  |

We understand and accept that Cefas’ Standard Terms and Conditions for Goods apply to this project and any subsequent work. These can be found at: Terms and Conditions

| Signed: | For: (Company Name) |
| --- | --- |
| Name: (Block Capitals) | Date: |
| Name of contact to call in the event of a query (if different from above): | Direct telephone number of contact: |