Works required for contract HDG/2022/195

1. SK120 – Shelton New Road / Etruscan Street

- Addition of low level cycle signal heads on two arms of the junction – Shelton New Road, both directions

- Junction to provide an early release to cyclists waiting on Shelton New Road

- Existing Layout shown on drawing SK120-1_Signals.pdf (Drawing does not show cycle signal heads)

Please quote for:

- 2x low level cycle signal heads (poles 5 & 9 on drawing SK120-1_Signals)

- 2x above ground cycle detection (poles 5 & 9 on drawing SK120-1_Signals)

- Re-configuration of existing PTC-1 controller (including producing a new TR2500)

- FAT

- SAT

- Replace one direct buried pole (Pole 1 on drawing SK120-1_Signals)

- Traffic Management

- As Built Drawing

- MOVA dataset and MOVA validation

- Prior to work commencing the site should be checked to ensure sufficient spare cores are available for the additional equipment

2. SK252 – Shelton New Road / Byron Street

Please quote for:

- Supply and install new equipment in accordance with drawing SDD - Near-sided Toucan Crossing - Single - Two Way plus:

  - Tactile cones on each PBU

  - Telent RMU to be installed

  - New poles to be installed in NAL pole sockets

  - All hoods to be primary hoods

  - Traffic Management

- Install poles at dimensions detailed in SSD 05-44 - Tactile Paving for Controlled Pedestrian Crossings