SPECIFICATION FOR THE REPAIR AND REFURBISHMENT OF FIRE SUPPRESSION COMPONENTS UNDER CONTRACT LSBU7/0200

The contents of this specification must not be communicated to a third party or used for any other work than that for which the specification is issued without the written agreement of the Babcock DSG Repair Manager

Airports  |  Communications  |  Defence  |  Education  |  Emergency Services | Energy  |  Mining and Construction  |  Property  |  Rail  |  Training

# Contents

Cover Sheet

Contents	2

List of Tables	2

Amendment Record	3

1.0 Introduction	4

2.0 Publications	5

3.0 Documentation	6

4.0 Repair Policy	7

5.0 Repair Requirement	8

6.0 Performance and Test Acceptance	9

7.0 Preservation & Packing	9

# List of Tables

Table 1 – Equipment Details	4

Table 2 – Support Publications	5

## Amendment Record

| Issue/ Amdt No | Identification of Change Page & Para No | Date Entered | Date Effective | Authorisation |
| --- | --- | --- | --- | --- |

## Introduction

1.1	The equipment covered by this Specification,Statement of Requirements(SoR) belongs to the Protected Mobility Vehicle Programme (PMVP) and provides components that support a fire suppression capability for the Cougar Fleet, Buffalo and Foxhound vehicles; this equipment is listed in Table 1; any previous versions of these items will be repaired and returned as the latest standard unless otherwise instructed.

#### Table 1 – Equipment Details

| Equipment | NATO Stock No (NSN) | Domestic Management Code (DMC) |
| --- | --- | --- |
| VAPORIZING LIQUID,FIRE EXTINGUISHER | 4210-99-851-0612 | 7MF |
| EXTINGUISHER,FIRE,CHARGED | 4210-99-863-1753 | 7BUFF |
| EXTINGUISHER,FIRE,AUTOMATIC TYPE;CARTRIDGE DISCHARGE;POD;C/W REMOTE CHARGE | 4210-99-302-1800 | 7FXH |
| EXTINGUISHER,FIRE,CARTRIDGE DISCHARGE;AUTOMATIC TYPE;POWER PACK;C/W REMOTE CHARGE | 4210-99-152-9563 | 7FXH |

  - This document is intended as an outline specification detailing the engineering requirement to enable a company to apply their expertise to produce a compliant product that meets the in-service user requirements, which shall be acceptable to the Authority and for which a warranty shall be provided.  The performance of completed assemblies shall meet or exceed that of the original equipment manufacturers (OEM) specification and the criteria contained in this specification.  Should any differences exist between the OEM and the Authority’s’ specifications, either in build or test criteria, the Authority’s specification will generally take precedence.  Clarification should be sought from the Babcock DSG Repair Manager.

  - The demanding operational role of Armed Service equipment is significantly different to that of equivalent commercial equipment.  It is essential to ensure that this equipment proves reliable when used and that the end user has the necessary confidence that it will survive the rigours of Service application.

  - It is a requirement of the Authority that contractors hold a current recognised third party Quality Accreditation Certification (UKAS or International equivalent).  For the work requirement of this specification, the contractor shall be registered in accordance with the requirement of ISO 9001:2008 suitably scoped as a minimum.

  - There may be circumstances, such as urgent operational requirements (UOR) where it will be to the Authority’s benefit to accept delivery of products that do not conform to contract requirements, as detailed in Defence Standard 05-61 (Part 1) (Concessions), but there must be a clear and demonstrable benefit to the Authority and approval must be given by the Babcock DSG Repair Manager, (in writing), before this takes place.

  - Any quantities referred to are estimated quantities only.  The Authority may order more or less than those referred to.  Any figures are for guidance only and no guarantee can be given that any specific quantities of repairable items will become available.

## Publications

2.1	Contractors are responsible for obtaining the latest OEM publications, parts lists and supersession lists for the equipment.

  - Publications produced by the Authority for service use are, in general, based upon the commercial publications but the format is specific to the service user. Contractors are responsible for obtaining and maintaining the latest issues of these publications.

  - Publications applicable to the assemblies within this requirement are covered in Table 2 below:

#### Table 2 – Support Publications

| Publication | Title |
| --- | --- |
| JSP 886 | Defence Logistics Support Chain Manual |
| AESP 0200-A-220-013 | Preservation, Identification and Packaging of Assemblies |
| AESP 2320-D-425 Series | Truck Mine Protected Clearance Vehicle MPCV 6x6 Lhd Buffalo |
| AESP 2320-D-424 Series | Rummage Installation for Truck, Mine Protection Clearance Vehicle (MPCV) 6x6 Lhd Buffalo A2 Mk2 |
| AESP 2355-E-100 Series | Foxhound 4x4 Light Protected Patrol Vehicle (LPPV) 1st Edition |
| AESP 2320-D-408 Series | Carrier Personnel Wheeled 6x6 Medium Protected Transport Vehicle - Mastiff |
| AESP 2320-D-419 Series | Carrier Personnel Wheeled 6x6 Medium Protected Troop Carrying Vehicle (TCV) Mastiff 2 |
| AESP 2355-D-100 Series | Carrier, Personnel, Wheeled, 6x6 Medium Protected, Mastiff 3 (All Variants) |
| AESP 2320-D-414 Series | Carrier, Personnel, Wheeled 4x4, Medium Protected Urban Patrol Vehicle, Ridgback All Variants |
| AESP 2320-D-101 Series | Wolfhound Truck Wheeled 6x6 Heavy, Medium Protected Tactical Support Vehicle (TSV) |
| AQAP 2110 | NATO Quality Assurance Requirements for Design, Development and Production |
| AQAP 2105 | NATO Requirements for Deliverable Quality Plans |
| Def Stan 05-61 Pt 1 | Quality Assurance Procedural Requirements - Concessions |
| Def Stan 05-61 Pt 9 | Quality Assurance Procedural Requirements – Independent Inspection Requirements for Safety Critical Items |
| Def Stan 00-56 Pt 1 | Safety Management Requirements for Defence Systems |
| Def Stan 03-32 | Pre-treatment and Painting of Vehicles, Engineering Equipment and Components |
| Def Stan 05-57 | Configuration Management of Defence Materiel |
| Def Stan 05-99 | Managing Government Furnished Equipment in Industry |
| Def Stan 05-135 | Avoidance of Counterfeit Materiel |
| Def Stan 81 Series | Packaging of Defence Material |

## Documentation

3.1	 A draft version of the quality plan (QP) must be set out defined in AQAP 2105 and delivered to the Authority with your Tender submission. The agreed QP shall be supplied within three months of the commencement of the contract.  The QP should identify all risk areas and detail how they will be mitigated and managed throughout the duration of the contract.  The QP shall reference procedures, developed in accordance with the Contractors Quality Registration, which detail how control of the repair relating to the Company Quality Assurance processes will be achieved.  Inspection and test points shall be clearly indicated.  Documentation relating to critical or safety related items and assemblies shall be highlighted.

  - If any equipment provided for repair under this contract cannot be completed in accordance with the repair price menu at Appendix A and A1 to Schedule 2, then the Contractor is to submit a strip survey report (Schedule 14) to the Babcock DSG Repair Manager fully identifying the requirement for all work relating to the assembly, including costs.  No work of this type is to be undertaken by the Contractor until this strip survey report and the associated costs have been sanctioned by the Babcock DSG Repair Manager as ‘fair and reasonable’ , a purchase order has been raised (see schedule 15) and authority is given to proceed.

  - At the commencement of the Contract, and thereafter at reasonable intervals depending upon need arising and priorities, the Babcock DSG Repair Manager and Contractor shall agree a "production plan" for the repair.  The Contractor shall provide a monthly report on the progress of the repair work against the plan to the Babcock DSG Repair Manager.  This report must include expected delivery dates, financial accrual information and any mitigating factors to support repair and/or delivery variations.

  - Records, comprising repair, calibration, inspection, spares and test reports as applicable and defined in this specification, shall be maintained by the Contractor.  Additionally the Contractor is to keep records of all visits/survey reports, approvals and costs incurred in the repair/manufacture of the Contractor deliverables.  Where there is a legislative requirement, documents are to be kept for the period specified in that legal requirement.  All records must be made available to the Authority as required.

## Repair Policy

4.1	 Assemblies submitted for repair will have been removed from service for a multitude of reasons.  The requirement for the remanufacture of these assemblies is to give an expected life of not less than eighty percent of that of a new assembly.  This specification is not to be considered as comprehensive for the work requirement and is not to be used as a reason to limit any work on the assembly.  It is the Contractor's responsibility to produce a comprehensive repair specification for each item and to ensure that the quality of the assembly returned after refurbishment shall meet the requirement of "as new" with the stated life requirement.

4.2	 Fully priced estimates shall be required for any assembly not considered by the repairer as economic to repair.  These must be submitted at the survey stage (before work commences) and not be a result of back stripping or cannibalisation.  The Authority will only agree Beyond Economic Repair (BER) classification where the Contractor has been able to demonstrate that their cost to repair is greater than the eighty percent of new cost as supplied to the Authority.  Once BER has been agreed the Authority will issue disposal instructions for the scrapped carcass accordingly.

4.3 	Any remanufacture of the assembly and its ancillaries shall be to the latest approved OEM specification and modification state, using approved procedures in accordance with the current service/workshop manual for the item.  Completed assemblies, including ancillaries, shall be tested to the OEM test specification.  The performance and quality standard of the assembly shall meet or exceed the requirement of the OEM specification.  Records of performance tests and results as applicable shall be supplied as stated in this specification.  All assemblies and ancillaries shall be covered by a warranty as defined in the terms and conditions of the Contract.

4.4	Procurement of all replacement parts used in the repair shall be the responsibility of the Contractor.  All parts shall meet or exceed the OEM specification and shall be purchased from approved suppliers.  Certificates of conformity (COC) shall be obtained for all parts which have not been sourced through the OEM, and shall be made available to the Babcock DSG Repair Manager or a nominated representative when requested.

  - The following items are to be considered as mandatory 100% replacement components, regardless of their condition:

- All seals, ‘O’ rings and gaskets.

- All throw away locking devices, tab washers, nyloc nuts, split pins, retaining rings and locking wire.

- All flexible hoses.

- All ‘P’ clips.

- Screws, nuts, bolts and spacers.

- Any shelf-lifed items.

  - The Contractor has an obligation towards safety.  Any failures or incidents in relation to the equipment which affects safety shall be reported to the Babcock DSG Repair Manager without delay.  The Babcock DSG Repair Manager shall be entitled to require action to be taken to correct the failure and to prevent reoccurrence.

  - All modifications approved by the OEM & the Authority as defined in the latest technical documentation shall be incorporated as part of the repair. Unauthorised modifications shall not be incorporated.

## 5.0 Repair Requirement

5.1	Assemblies received for repair are to be checked for correct nomenclature and part number and a report produced detailing the modification status (if applicable), serial number, any significant damage and/or missing items.

5.2	Any discrepancies in the items delivered should be reported using MoD Form 445 (Discrepancy Report) (Schedule 13).  These reports shall be completed in accordance with the criteria laid down in JSP 886, Volume 4, Chapter 3 and distributed as required by the Contract with two copies to the issuing depot and one to the Babcock DSG Repair Manager.

5.3	All assemblies and ancillaries must be completely emptied, stripped and thoroughly cleaned and degreased.  A detailed inspection of all components shall be carried out, with a full survey report raised to establish the extent of the work requirements.  The survey report shall be sent to the Babcock DSG Repair Manager for repair approval as per para 3.2.

5.4	The scope of the repairs to be carried out shall be determined from the survey against OEM specification.  At this stage, all those components being replaced are to be disposed of using Contractor’s formal quality control procedures.  All remaining components shall be inspected to establish their suitability for re-use or reclamation.  Those found not suitable are to be disposed of by the Contractor once approval for the repair has been given by the Babcock DSG Repair Manager.

5.5	 Assemblies are to be rebuilt in accordance with the latest OEM specification using reclaimed and new components, incorporating all approved modifications where applicable.

5.6	The Contractor shall permanently fix an identification plate to the assembly indicating that they have been subject to repair.  The plate shall record:

- Re-manufactured for MoD.

- Authority’s Job Number (e.g. PR100012345).

- Date of re-manufacture.

- Assembly Serial Number (if applicable).

- Warranty period (as per the Contract).

## 6.0 Performance and Test Acceptance

6.1		On completion of repair the assembly shall be subjected to suitable static and dynamic testing and acceptance by the Contractor.

6.2	Final testing of all assemblies shall be carried out in accordance with OEM/MoD procedures and standards.  Where discrepancy exists between the OEM and MoD test specification the MoD specification will generally take precedence, but the Contractor shall ultimately seek clarification from the Babcock DSG Repair Manager.  It is the responsibility of the Contractor to ensure that all test equipment is maintained and calibrated.

6.3	Inspection/test records shall be retained for all assemblies for the duration of the contract and will be made available for the Babcock DSG Repair Manager or nominated representatives of the Authority upon request.

## 7.0 Preservation & Packing

7.1     Completed assemblies shall be internally & externally preserved in accordance with DEF STAN 81-62 and DEF STAN 81-41.

7.2     All completed assemblies are to be painted, if applicable, to OEM specification in accordance with the general procedures as laid down in DEF STAN 03-32.

7.3     Completed assemblies are to be packed in accordance with the relevant Service Packaging Instruction Sheet (SPIS) and to the level shown in the contract or order.

7.4     Any replacement wood used in packaging must be ISPM 15 compliant and carry the Forestry Commission, Heat Treated, mark (see below) (DEFCON 129 refers).

Richard Lewis

Repair Manager (PMVP)

DSG

Defence & Security

Bldg B15,

Donnington,

Telford.

Shropshire

TF2 8JT

01952 673989

Richard.Lewis@dsg.MoD.uk

www.babcockinternational.com

© Babcock International Group PLC 2013

Army Equipment Support Publication

For the purposes of DEFCON 611 all contractor deliverables issued under the contract will be issued on as a Contract work Item (formerly Contract Loan) basis.

The contractor is advised that no guarantee can be given or responsibilities accepted by the Authority regarding the completeness or correctness of equipment issued for repair, or give any indications of the level of repair required.

This is the general guide criteria but can be varied depending upon stock levels, new buy time limits and supply urgency, the Babcock DSG Repair Manager will advise in all BER requests.