SPECIFICATION FOR THE REPAIR AND REFURBISHMENT OF OPTICAL LINE REPLACEMENT UNITS UNDER CONTRACT LSBU11/0002

The contents of this specification must not be communicated to a third party or used for any other work than that for which the specification is issued without the written agreement of the Babcock DSG Repair Manager

Airports  |  Communications  |  Defence  |  Education  |  Emergency Services | Energy  |  Mining and Construction  |  Property  |  Rail  |  Training

# Contents

Cover Sheet

Contents	2

List of Tables	2

Amendment Record	3

1.0 Introduction	4

2.0 Publications	5

3.0 Documentation	7

4.0 Repair Policy	8

5.0 Repair Requirement	9

6.0 Performance and Test Acceptance	10

7.0 Preservation & Packing	11

# List of Tables

Table 1 – Equipment Details	4

Table 2 – Support Publications	5

Table 3 - Equipment Modification Equipment.. …. …………………………………………..9

## Amendment Record

| Issue/ Amdt No | Identification of Change Page & Para No | Date Entered | Date Effective | Authorisation |
| --- | --- | --- | --- | --- |

## Introduction

1.1	The equipment covered by this Statement of Work (SoW) belongs to the Armoured Vehicle Protection (AVP) and provide optical units and various equipment for the Challenger, Warrior, Titan and Trojan vehicles; this equipment is listed in Table 1; any previous versions of these items will be repaired and returned as the latest standard unless otherwise instructed.

#### Table 1 – Equipment Details

| NSN | DESCRIPTION | PLATFORM |
| --- | --- | --- |
| 5836-99-975-4063 | Video Switching Unit | T2 |
| 7025-99-351-4124 | Display Unit | T2 |
| 2590-99-301 -8377(pre fab) | Control Unit | T2 |
| 2590-99-813-2337(post fab) | Control Unit | T2 |
| 5998-99-027-0854 | Circuit Card | T2 |
| 1240-99-660-4273 | Controller | 9CR2 |
| 1290-99-383-4248 | Panel Gunners Control | 4CR2 |
| 6670-99-861-9706 | Load Cell | 4CR2 |
| 2540-99-212-7648 | Camera Television | 9CR2 |
| 6710-99-301-8484 | L 3 Camera (T2) low level lighting | T2 |
| 6720-99-386-8233 | Hube Camera | T2 |
| 5836-99-961-3623 | Video Switching Unit | 2 |
| 5855-99-250-1892 | Illuminator | T2 |
| 5855-99-860-0335 | Illuminator | T2 |
| 6610-99-898-0458 | Control Display Unit | T2 |
| 1240-99-341-8137 | Panel | 9CR2 |
| 1240-99-938-2182 | Front Panel | 9CR2 |
| 2540-99-274-4344 (pre fab) | T2ERDSU | T2 |
| 2540-99-376-4132(post fab) | T2ERDSU | T2 |
| 1290-99-902-2395 | Panel | 9CR2 |
| 2510-99-321-1220 | Commander Service Panel (T2) | T2 |
| 2510-99-356-1150(pre fab) | Drivers Control Panel (T2) | T2 |
| 2590-99-919-6411(post fab) | Drivers Control Panel (T2) | T2 |
| 5975-99-852-8942 | Hull Rear Junction Box (T2) | T2 |
| 6110-99-292-4750(pre fab) | Power Dist Panel (T2) | T2 |
| 6110-99-877-4857(post fab) | Power Dist Panel (T2) | T2 |
| 2590-99-784-1891 | Warrior Control Handle | WARRIOR |

  - This document is intended as an outline specification detailing the engineering requirement to enable a company to apply their expertise to produce a compliant product that meets the in-service user requirements, which shall be acceptable to the Authority and for which a warranty shall be provided.  The performance of completed assemblies shall meet or exceed that of the original equipment manufacturers (OEM) specification and the criteria contained in this specification.  Should any differences exist between the OEM and MoD specifications, either in build or test criteria, the MoD will generally take precedence.  Clarification should be sought from the Babcock DSG Repair Manager.

  - The demanding operational role of Armed Service equipment is significantly different to that of equivalent commercial equipment.  It is essential to ensure that this equipment proves reliable when used and that the end user has the necessary confidence that it will survive the rigours of Service application.

  - It is a requirement of the MoD that contractors hold a current recognised third party Quality Accreditation Certification (UKAS or International equivalent).  For the work requirement of this specification, the contractor shall be registered in accordance with the requirement of ISO 9001:2008 suitably scoped as a minimum.

  - There may be circumstances, such as urgent operational requirements (UOR) where it will be to the Authority’s benefit to accept delivery of products that do not conform to contract requirements, as detailed in Defence Standard 05-61 (Part 1) (Concessions), but there must be a clear and demonstrable benefit to the Authority and approval must be given by the Babcock DSG Repair Manager, (in writing), before this takes place.

  - Any quantities referred to are estimated quantities only.  The Authority may order more or less than those referred to.  Any figures are for guidance only and no guarantee can be given that any specific quantities of repairable items will become available.

## Publications

2.1	Contractors are responsible for obtaining the latest OEM publications, parts lists and supersession lists for the equipment.

  - Publications produced by the MoD for service use are, in general, based upon the commercial publications but the format is specific to the service user. Contractors are responsible for obtaining and maintaining the latest issues of these publications.

  - Publications applicable to the assemblies within this requirement are covered in Table 2 below:

#### Table 2 – Support Publications

| Publication | Title |
| --- | --- |
| JSP 886 | Defence Logistics Support Chain Manual |
| AESP 0200-A-220-013 | Preservation, Identification and Packaging of Assemblies |
| DEF STAN: 05-57 | Configuration Management |
| DEF STAN: 05-99 | Government Furnished Equipment |
| DEF STAN: 05-92 | Quality Systems in industry |
| DEF STAN: 05-61 | Deviation/ Production Permits , Waivers / Concessions and QA of Sub Contractor Work |
| DEF STAN:00-56 | Pt 1& 2 Issue 4 (safety Management Requirements for Defence Material) |
| DEF STAN: 05-135 | Avoidance of Counterfeit Material |
| DEF STAN: 81-41 | Packaging of Defence Material |
| AESP 2350-F-Octad | Video Switching Unit |
| AESP 2350-F-Octad | Display Unit |
| AESP 2350-F-Octad | Control Unit (pre- fab) |
| AESP 2350-F-Octad | Control Unit (post fab) |
| AESP 1230-C-Octad | Circuit Card |
| AESP 1230-C-Octad | Controller |
| AESP 1230-C-Octad | Panel Gunners Control |
| AESP 2350-F-Octad | Load Cell |
| AESP 2350-F-Octad | Camera Television |
| AESP 2320-F-Octad | L 3 Camera (T2) low level lighting |
| AESP 2320-F-Octad | Hube Camera |
| AESP 2320-F-Octad | Video Switching Unit |
| AESP 2320-F-Octad | Illuminator |
| AESP 2320-F-Octad | Illuminator |
| AESP 2320-F-Octad | Control Display Unit |
| AESP 1230-C-Octad | Panel |
| AESP 1230-C-Octad | Front Panel |
| AESP 2350-F-Octad | T2ERDSU( pre fab) |
| AESP 2350-F-Octad | T2ERDSU (post fab) |
| AESP 1230-C-Octad | Panel |
| AESP 2350-F-Octad | Commander Service Panel (T2) |
| AESP 2350-F-Octad | Drivers Control Panel (T2) (pre fab) |
| AESP 2350-F-Octad | Drivers Control Panel (T2) (post fab) |
| AESP 2350-F-Octad | Hull Rear Junction Box (T2) |
| AESP 2350-F-Octad | Power Dist Panel (T2) (pre fab) |
| AESP 2350-F-Octad | Power Dist Panel (T2) (post fab) |
| AESP 2350-T-Octad | Warrior Control Handle |
| AESP 2350-T-Octad | Hand Firing Switch |

## Documentation

3.1	 A draft quality plan (QP) will be required at the ITT stage to demonstrate how equipment is to be managed.  The completed QP shall be submitted within three months of the commencement of the contract.  The QP should identify all risk areas and detail how they will be mitigated and managed throughout the duration of the contract.  The QP shall reference procedures, developed in accordance with the Contractors Quality Registration, which detail how control of the repair relating to the Company Quality Assurance processes will be achieved.  Inspection and test points shall be clearly indicated.  Documentation relating to critical or safety related items and assemblies shall be highlighted.

  - If any equipment provided for repair under this contract cannot be completed in accordance with the repair price menu at Appendix A and A1 to Schedule 2, then the Contractor is to submit a strip survey report to the Babcock DSG Repair Manager fully identifying the requirement for all work relating to the assembly, including costs.  No work of this type is to be undertaken by the Contractor until this strip survey report and the associated costs have been sanctioned by the Babcock DSG Repair Manager as ‘fair and reasonable’ and authority is given to proceed.

  - At the commencement of the Contract, and thereafter at reasonable intervals depending upon need arising and priorities, the Babcock DSG Repair Manager and Contractor shall agree a "production plan" for the repair.  The Contractor shall provide a monthly report on the progress of the repair work against the plan to the Babcock DSG Repair Manager.  This report must include expected delivery dates, financial accrual information and any mitigating factors to support repair and/or delivery variations.

  - Records, comprising repair, calibration, inspection, spares and test reports as applicable and defined in this specification, shall be maintained by the Contractor.  Additionally the Contractor is to keep records of all visits/survey reports, approvals and costs incurred in the repair/manufacture of the Contractor deliverables.  Where there is a legislative requirement, documents are to be kept for the period specified in that legal requirement.  All records must be made available to the Authority as required.

## Repair Policy

4.1	 Repair. Assemblies submitted for repair will have been removed from service for a multitude of reasons.  The requirement for the remanufacture of these assemblies is to give an expected life of not less than eighty percent of that of a new assembly.  This specification is not to be considered as comprehensive for the work requirement and is not to be used as a reason to limit any work on the assembly.  It is the Contractor's responsibility to produce a comprehensive repair specification for each item and to ensure that the quality of the assembly returned after refurbishment shall meet the requirement of "as new" with the stated life requirement.

4.2	 Beyond Economical repair (BER). Fully priced estimates shall be required for any assembly not considered by the repairer as economic to repair.  These must be submitted at the survey stage (before work commences) and not be a result of back stripping or cannibalisation.  The Authority will only agree Beyond Economic Repair (BER) classification where the Contractor has been able to demonstrate that their cost to repair is greater than the eighty percent of new cost as supplied to the MoD.  Once BER has been agreed the Authority will issue disposal instructions for the scrapped carcass accordingly.

4.3 	Remanufacture.  The assembly and its ancillaries shall be to the latest MoD approved OEM specification and modification state, using approved procedures in accordance with the current service/workshop manual for the item.  Completed assemblies, including ancillaries, shall be tested to the OEM test specification.  The performance and quality standard of the assembly shall meet or exceed the requirement of the OEM specification.  Records of performance tests and results as applicable shall be supplied as stated in this specification.  All assemblies and ancillaries shall be covered by a warranty as defined in the terms and conditions of the Contract.

4.4	Replacement Parts. Procurement of all replacement parts used in the repair shall be the responsibility of the Contractor.  All parts shall meet or exceed the OEM specification and shall be purchased from approved suppliers.  Certificates of conformity (COC) shall be obtained for all parts which have not been sourced through the OEM, and shall be made available to the Babcock DSG Repair Manager or a nominated representative when requested.

  - The following items are to be considered as mandatory 100% replacement components, regardless of their condition:

- All seals, ‘O’ rings and gaskets.

- All throw away locking devices, tab washers, nylon nuts, split pins, retaining rings and locking wire.

- All flexible hoses.

- All ‘P’ clips.

- Screws, nuts, bolts and spacers.

- Any shelf-life items.

  - Safety. The Contractor has an obligation towards safety.  Any failures or incidents in relation to the equipment which affects safety shall be reported to the Babcock DSG Repair Manager without delay.  The Babcock DSG Repair Manager shall be entitled to require action to be taken to correct the failure and to prevent reoccurrence.

  - Modifications. All modifications approved by the OEM & MoD as defined in the latest technical documentation shall be incorporated as part of the repair. Unauthorised modifications shall not be incorporated. Current known modifications and general instructions are detailed in Table 3.

Table 3 – Equipment Modification

| Publication | Modification | General List |
| --- | --- | --- |
| AESP 2350-F-Octad | 2590-99-301-8377 pre fab 2590-99-813-2337 post fab | Control Unit |
| AESP 2350-F-Octad | 2540-99-274-4344 pre fab 2540-99-376-4132 post fab | ERDSU Camera |
| AESP 2350-F-Octad | 2510-99-356-1150 pre fab 2590-99-919-6411 post fab | Drivers Control Panel |
| AESP 2350-F-Octad | 6110-99-292-4750 pre fab 6110-99-877-4857 post fab | Power Distribution Panel |

## 5.0 Repair Requirement

5.1	Repair Inspection. Assemblies received for repair are to be checked for correct nomenclature and part number and a report produced detailing the modification status (if applicable), serial number, any significant damage and/or missing items.

5.2      MOD Form 445 (Discrepancy Report).  Any discrepancies in the items delivered should be reported using MoD Form 445 (Discrepancy Report).  These reports shall be completed in accordance with the criteria laid down in JSP 886, Volume 4, Chapter 3 and distributed as required by the Contract with two copies to the issuing depot and one to the Babcock DSG Repair Manager.

5.3	Disassembly. All assemblies and ancillaries must be completely emptied, stripped and thoroughly cleaned and degreased.  A detailed inspection of all components shall be carried out, with a full survey report raised to establish the extent of the work requirements.  The survey report shall be sent to the Babcock DSG Repair Manager for repair approval as per para 3.2.

5.4	Repair Requirement. The scope of the repairs to be carried out shall be determined from the survey against OEM specification.  At this stage, all those components being replaced are to be disposed of using Contractor’s formal quality control procedures.  All remaining components shall be inspected to establish their suitability for re-use or reclamation.  Those found not suitable are to be disposed of by the Contractor once approval for the repair has been given by the Babcock DSG Repair Manager.

5.5	 Rebuild. Assemblies are to be rebuilt in accordance with the latest OEM specification using reclaimed and new components, incorporating all approved modifications where applicable.

5.6	Identification and Marking. The Contractor shall permanently fix an identification plate to the assembly indicating that they have been subject to repair.  The plate shall record:

- Re-manufactured for MoD.

- Authority’s Job Number (e.g. PR100012345).

- Date of re-manufacture.

- Assembly Serial Number (if applicable).

- Warranty period (as per the Contract).

## 6.0 Performance and Test Acceptance

6.1		On completion of repair the assembly shall be subjected to suitable static and dynamic testing and acceptance by the Contractor.

6.2	Final testing of all assemblies shall be carried out in accordance with OEM/MoD procedures and standards.  Where discrepancy exists between the OEM and MoD test specification the MoD specification will generally take precedence, but the Contractor shall ultimately seek clarification from the Babcock DSG Repair Manager.  It is the responsibility of the Contractor to ensure that all test equipment is maintained and calibrated.

6.3	Inspection/test records shall be retained for all assemblies for a period of four years in accordance with DEFCON 609 and made available for the Babcock DSG Repair Manager or nominated representatives of the Authority upon request.

## 7.0 Preservation & Packing

7.1     Completed assemblies shall be internally & externally preserved in accordance with DEF STAN 81-62 and DEF STAN 81-41.

7.2     All completed assemblies are to be painted, if applicable, to OEM specification in Light Stone Chemical Agent Resistant Coating (CARC) to DEFSTAN 80-208 and in accordance with the general procedures as laid down in DEF STAN 03-32.

7.3     Completed assemblies are to be packed in accordance with the relevant Service Packaging Instruction Sheet (SPIS) and to the level shown in the contract or order.

7.4     Any replacement wood used in packaging must be ISPM 15 compliant and carry the Forestry Commission, Heat Treated, mark (see below) (DEFCON 129 refers).

Steven Thomas

Repair Manager (AVP)

DSG

Defence & Security

Bldg. B15,

Donnington,

Telford.

Shropshire

TF2 8JT

01952 673997

Steven.thomas@dsg.MoD.uk

www.babcockinternational.com

© Babcock International Group PLC 2013

Army Equipment Support Publication

For the purposes of DEFCON 611 all contractor deliverables issued under the contract will be issued on as a Contract work Item (formerly Contract Loan) basis.

The contractor is advised that no guarantee can be given or responsibilities accepted by the Authority regarding the completeness or correctness of equipment issued for repair, or give any indications of the level of repair required.

This is the general guide criteria but can be varied depending upon stock levels, new buy time limits and supply urgency, the Babcock DSG Repair Manager will advise in all BER requests.