Appendix 4b

Henry Dickens Estate - Fencing Specification -

| 1.00 | Undertake an onsite inspection of MUGA Fencing on the Henry Dickens Court Estate. |
| --- | --- |
| 2.00 | Breakout approx. 87.5 + 27.5 linear meters of MUGA mesh fencing and posts including disposal. |
| 3.00 | Supply approx. 87.5 linear meters of 4M high panels on 5M high posts. Panels to be manufactured with 50mm x 200mm welded mesh twin wire fencing. Posts may be either 60mm x 40mm OR 80mm x 40mm Rectangular Hollow Sections. Posts must be fixed within the ground. Colour to be standard Green RAL 6005 |
| 4.00 | Supply approx. 27.5 linear meters of 5M high panels on 5M high posts Panels to be manufactured with 50mm x 200mm welded mesh twin wire fencing. Posts may be either 60mm x 40mm OR 80mm x 40mm Rectangular Hollow Sections. Posts must be fixed within the ground and each spaced at 2525mm centres. Colour to be standard Green RAL 6005 |
| 5.00 | Supply 1x Double Leaf gate 4M high by 2.8M wide (3M with lintel panel) Panels to be manufactured with 50mm x 200mm welded mesh twin wire fencing. Posts may be either 60mm x 40mm OR 80mm x 40mm Rectangular Hollow Sections. Posts must be fixed within the ground and each spaced at 2525mm centres. Colour to be standard Green RAL 6005 |
| 6.00 | Supply 1x Double Leaf gate 5M high by 2.5M wide (3M with lintel panel) Panels to be manufactured with 50mm x 200mm welded mesh twin wire fencing. Posts may be either 60mm x 40mm OR 80mm x 40mm Rectangular Hollow Sections. Posts must be fixed within the ground and each spaced at 2525mm centres. Colour to be standard Green RAL 6005 |
| 7.00 | Panels to be manufactured ideally for usage in areas covered by CCTV, be a welded mesh twin wire fencing system which is rigid, robust and strong. The horizontal wires are to be installed on the Attack face of the fence. Panels galvanised and polyester powder coated to pass 1008 hours salt spray testing. Posts to be galvanised to EN ISO 1416 standard and polyester powder coated to pass 1008 hours salt spray testing. |
| 8.00 | Include the installation of ASSA Locks with 10x keys for 1no Double Leaf gate 4m High and for 1no Double Leaf gate 5m. |