Little Thetford Parish Council

Chairman: Cllr G James

Clerk: Mrs A Jacklin, c/o Little Thetford Village Hall, The Wyches, Little Thetford CB6 3HG

07828 402 093   parish.clerk@littlethetford.org.uk

Little Thetford Parish Council are looking for a contractor to complete a Children’s playground upgrade in public space. This would include removal and disposal of old equipment, supplying and installing new playground equipment and re-surfacing ground area.

We are interested in the following equipment.

- 1.8m Log swing with cradle seats and anti-wrap chains.

- 2.4m Log swing with basket seat.

- A large play tower with a bumpy slide attached.

- Sunken mini trampoline (1.5mx1.5m) including frame and rubber tiles.

- Modify existing earth mound to include a 6m long crawl through and 2x Embankment log ramp rope climbers.

- 2.4m Log swing with flat seats and anti-wrap chains.

- 1.8m bench with back

For surfacing we are looking for

- 21m2 of raised 40mm GREEN wetpour (over existing wetpour, inc chase)

- 32m2 of raised 50mm GREEN wetpour (over existing wetpour)(Inc 2m2 of 100mm groundworks, additional PCC edging and chase out.

- 26m2 of raised 20mm GREEN wetpour skim (over existing wetpour, inc chase)

- 53m2 of raised 50mm RED wetpour (over existing wetpour)(inc 28m2 of 100mm groundworks, additional PCC edging and chase out)

- 20mx300mm wide RED wetpour (inc 100mm groundworks and PCC edging)

- 17m2 of recessed 40mm RED wetpour (inc 100mm groundworks and PCC edging)

- 24m2 of recessed 60mm GREEN wetpour (inc 3m2 of 100mm groundworks, 30% groundworks top up and additional PCC edging)

Removal of

- Cradle swing seats, climbing frame, seesaw, embankment slide, metal trail equipment, flat seat swings, 2x benches, 7mx3m of 60mm wetpour.

- Removal of any spoil or waste material from site.

All work would need to include a post independent inspection

Please contact the Parish Clerk- Amy Jacklin on the details at the top of the page if you have any queries or questions.

Job to be on contact finder until 31st March 2021