Appendix D


[image: image1.emf]
SPECIFICATION

FOR

PERIODIC SERVICING AND MAINTENANCE OF 

AIR CONDITIONING EQUIPMENT

AND ASSOCIATED PLANT

IN

CITY COLLEGE PLYMOUTH
2018 - 2021
TECHNICAL SPECIFICATION

Contract

This specification details the work required for the programmed maintenance, servicing and cleaning of Air conditioning Plant. The tender figure must include all costs of layout, plant cleaning and servicing, consumables, tools & equipment, travelling, insurance and all overheads required to fulfill the contract as detailed in the following paragraphs. The cost must also include for one full filter change per annum on all plant where applicable, shown as a separate cost on the tender return (Appendix A).
Work Required

The work required under this contract has, for convenience only been divided into categories. In preparing the tender figure the Contractor must allow for all the work involved at each property to be included irrespective of whether all of each category of work is applicable. The tender figure must allow for the entire system associated with the air conditioning equipment to be cleaned and serviced.

‘System’ shall include, external covers, insulation, protective mesh and fixings; the electrical equipment deemed part of the ‘system’ shall be the motor electrical supply, overload devices, contactors, switches, isolators, actuators, detectors, and indicator lamps.

Service Visits Related To Schedule
(Ref. Appendix B)

The Maintenance Contract will commence 1 August 2018 when all items listed on the Schedule shall be serviced and be complete by the end of 31July 2019, for the first year of contract.
Servicing

The Contractor is to service the equipment in the manner described in the following clauses at the properties listed within the main document. The service intervals to be as detailed in the programme dates within the main RFQ document.
Labels
Each item of equipment services shall be fitted with an adhesive label indicating TESTED-DATE-INITIALS. Each label shall be correctly filled in using a permanent ink pen. Outdoor plant should have the label fixed inside the access door of the plant where possible.
Service Operations

A guide to the minimum service requirements are outlined below. In all cases servicing must meet the manufacturer’s recommendations.

Minimum Service Requirements (6 Monthly)

a) Clean air filters and suction grilles and air outlet on ceiling cassette units.

b) Check cassette units and associated room controller for damage.
c) Clean condensate collection tray and check operation and effectiveness of pump and drain.
d) Check the flow, level or pressure of refrigerant in the system. A low charge will indicate a leak. Report to Estates and carry out leak checking of system as required.
e) Clean condenser coils and comb fins as necessary.
f) Check for damage and corrosion to outdoor condenser unit. Treat any corrosion as necessary.
g) Check for damage to exposed insulation on refrigerant pipework and repair as required.
h) Check operation of de-ice controls, HP, LP safety controls & compressor contactor, where required.
i) Check suction & discharge operating pressure.

j) Check all electrical connections, controls and safety functions.

k) Run units and check both indoor and outdoor units for excessive noise and vibration.
l) Check that units cool and heat as required, and check for correct operation of room controls.
m) Update refrigerant log where replacement refrigerant charge is used.
2
6/8/2018



