Amey – DIO

PVCu Windows & Doors

Specification (Rev O-1)

Design

- The Services Provider shall provide a design service as part of this contract. The Services Provider is to attend site prior to commencing work to obtain the exact dimensions and design requirements of each window and door. The Services Provider shall undertake surveys in accordance with BS 8213-4:2016 - Windows, doors, and roof lights. Code of practice for the survey and installation of windows and external door sets.

- The Services Provider shall include the door and window design including section details for the proposed frames for all elements and product literature for all ironmongery etc. within the tender submission

- The outer surface of the frame of each new window and door must be in the same position in relation to the outside of the building as the original. This is to ensure they will fit in with the general appearance of the other windows in the building. Exceptions to this will be notified by Amey, for example in circumstances where the installation of External Wall Insulation is planned.

- The appearance of the external cill must remain the same and it must not be adversely affected by the installation of the new window. The contractor is to alert Amey at tender if it appears that the existing cill may need replacing or over cladding.

- The brickwork/wall finish surrounding windows, cills and doors must be left in a condition that will prevent the ingress of moisture. Its appearance must not be affected.

- All habitable rooms shall have at least one outward opening window light and one top hung fan light. At 1st floor level all habitable rooms shall have an opening light that allows emergency egress; at least one room on the ground floor shall allow emergency egress.

- Where secondary glazing exists, the new PVCu windows are to be designed to match or exceed the sound attenuation qualities that is currently achieved with both units in place (the existing windows and secondary glazing).

- All first-floor windows must have the hinge capacity to allow for the cleaning of the glazing from inside the property.

Regulations and Standards

The Services Provider is to ensure all relevant standards and regulations are met with regard to the works including the following,

British Standards

The Services Provider is to ensure that the following British Standards are met with regard to the works as well as those mentioned in other clauses:

                     BS 8213-1:2004 - Windows doors and roof lights. Design for safety in use and during cleaning of windows, including door-height windows and roof windows. Code of practice

- BS 7412:2007 Specification for windows and doorsets made from unplasticised polyvinyl chloride (PVC-U) extruded hollow profiles.

- BS 7412:2007 Specification for windows and doorsets made from unplasticised polyvinyl chloride (PVC-U) extruded hollow profiles.

- BS 6375-1:2015 (+A1:2016) - Performance of windows and doors. Classification for weather-tightness and guidance on selection and specification

                     BS 6375-2:2009 - Performance of windows and doors. Classification for operation and strength characteristics and guidance on selection and specification

                     BS 6213:2000(+A1:2010) - Selection of construction sealants.

                     BS EN 12209:2016 - Building hardware. Locks and latches. Mechanically operated locks, latches and locking plates. Requirements and test methods.

                     BS EN 1906:2012 - Building hardware. Lever handles and knob furniture. Requirements and test methods.

                     BS EN 13724:2013 (Incorporating Jan 2016 Amendment) - Postal services. Apertures of private letter boxes and letter plates. Requirements and test methods.

                     BS EN 1670:2007 - Building hardware. Corrosion resistance. Requirements and test methods.

                     BS EN 12051:2000 - Building hardware. Door and window bolts. Requirements and test methods.

                     BS EN 1303:2015 - Building hardware. Cylinders for locks. Requirements and test methods.

                     BS EN 1279 Insulating Glass Units

                    BS 12150 Safety Glass

                     BS EN 356:2000 Glass in building. Security glazing. Testing and classification of resistance against manual attack

Other Standards

The Services Provider is to ensure that the following Standards and Codes of Practice are met where relevant with regards to the works as well as those mentioned in other clauses:

                     The Services Provider is to be a FENSA approved installer and provide copies of their registration certification to Amey with their tender document return.

                     The Glass and Glazing Federation (GGF) codes of practice.

                     BES 6001 Responsible Sourcing of Construction Products

                     PAS 24: 2022 Enhanced security performance requirements for doorsets and windows

MOD Standards

The Services Provider is to ensure the manufactured end product is compliant with Secured by Design (SBD) criteria PAS 24 Requirements. The Services Provider is to provide evidence of ‘Secured by Design’ license of the manufacturer for the windows and doors. These should also be validated on the ‘Secured by Design’ Portal.

Planning Considerations

The Services Provider must alert Amey at tender stage if they believe planning permission from the relevant Local Authority will be required or that the properties fall within a conservation area.

Building Regulations

The Services Provider shall ensure that the new window design does not compromise compliance with the Building Regulations Section J Combustion appliances and fuel storage systems.

Safety Glazing - Toughened glass in accordance with Part K of the Building Regulations

Means of Fire Escape - Part B

Ventilation - Part F

Thermal Efficiency and Performance - Part L

Toughened Safety Glass - Part N

Security - Part Q

To ensure compliance with current Building Regulations the installer will be required to issue certification of compliance and details of the address and installation details to FENSA Ltd within 20 days of completion. The contractor should inform FENSA that all certificates should be issued with Amey detailed as the consumer and sent to Amey, Swales Pavilion, RAF Wyton, Cambridgeshire, PE28 2EA

Standard Specification – double glazed windows

PVC Windows

- 70mm white PVC frames

- White locking handles as standard

- All frames are inside security glazed

- Trickle vents are included in opening sashes, where shown

- All frames are standard reinforced

- All units are clear 28mm argon filled, warm edged, double-glazed units incorporating 4mm toughened glass where necessary

- Windows will achieve an overall U-value of 1.3w/m2k

- Obscure stippolyte in W/C and bathroom windows

- Standard espag locks and friction type hinges

- 1 no. bedroom window made suitable for fire escape purposes with non-locking handle and egress hinges – Frame 10

- White external cill to all windows

PVC Doors

- 70mm white PVC frame

- Option for gold/chrome/dull silver, black or white locking handle

- All frames are security glazed inside

- All units are clear 28mm argon filled warm edged double-glazed units incorporating 4mm toughened glass both sides

- Front and back doors glazed with white R97 or P97 panel below midrail standard ply reinforced

- Standard multi point locking system

- Standard 40/10/45 silver locking cylinders

- Single doors opening inwards, French doors opening outwards

- Trickle vents are not included on doors

- 3 no. white flag hinges to each door leaf

- Standard 60mm PVC threshold on all doors

- Front door ONLY includes chrome letterbox in midrail

- Front door includes chrome security chain

Enhanced Specification – triple Glazed windows

PVC Windows

- 70mm white PVC frames

- White locking handles as standard

- All frames are inside security glazed

- Trickle vents are included in opening sashes, where shown

- Grd floor frames are full steel reinforced. First floor frames are standard reinforced

- All units are clear 40mm argon filled, warm edged, double-glazed units incorporating 4mm toughened glass where necessary

- Windows will achieve an overall U-value of 0.9w/m2k

- Obscure stippolyte in W/C and bathroom windows

- All grd floor windows include enhanced TSL espag locks and casement sash protectors for ‘Secure by Design’ stds

- All first-floor windows include standard espag locks and friction type hinges

- Standard espag locks and friction type hinges

- 1 no. bedroom window made suitable for fire escape purposes with non-locking handle and egress hinges – Frame 10

- White external cill to all windows

PVC Doors

- 70mm white PVC frame

- Option for gold/chrome/dull silver, black or white locking handle

- All frames are security glazed inside

- All units are clear 40mm argon filled warm edged double-glazed units incorporating 4mm toughened glass to all panes

- Front and back doors glazed with white R97 or P97 panel below midrail double ply reinforced

- High security multi-point locking system with hook locks included

- 3 star cylinders for ‘Secure by Design’ stds

- Single doors opening inwards, French doors opening outwards

- Trickle vents are not included on doors

- 3 no. white flag hinges to each door leaf

- Standard 60mm PVC threshold on all doors

- Front door ONLY includes chrome letterbox in midrail

- Front door includes chrome security chain