Pavilion refurbishment project specification 2020.

· Supply and fit secure fencing and appropriate signage to site. It is the contractors responsibility to ensure no unauthorised entry to site.

· Protection to grass where required unless otherwise agreed with Martin Lewer

· Demolition aspects to include.

· Removal of all non load bearing walls (assumed to be all sue to roof design although no visual inspection of roof area carried out) to be checked by contractor before commencement of works.
· Remove showers and sanitary ware. 

· Remove all defunct pipe work or plumbing scrap to be removed from site immediately.

· Remove existing lighting and wiring.

· Remove windows and doors as required.

· Remove guttering.

· All waste to be managed and where possible recycled. All waste to be removed from site. Strictly no fires.

· Exterior works to include.

· Supply and fit of 70mm celotex mechanically fixed to existing exterior.

· Vapour barrier to be provided.

· 50x25 horizontal batten gauged at 400ctrs 

· Insect prevention mesh along possible entry points.

· 100mm cedar vertical cladding shiplap style. Cedar or similar to be used for low ongoing maintenance. Cladding to be fixed in accordance with manufacturers best practice.

· Supply new guttering to be fitted in to existing soakaway pipes with a gutter brush fitted.

· Fit outside lights on PIR

· Interior works to include.

· New loft access and ladder.

· Ceiling void filled with 2 layers earthwool or similar insulation.

· 70mm celotex or similar RIB on top of existing floor with 22mm chipboard floating floor on top all joints to be glued and left free of damage to receive floor finish by separate contractor.

· New stud wall to create unisex toilet and store area. Wall to be sound insulated with heavy duty insulation batts.
· Install cubicle for new toilet.

· Ceilings and walls to have gypsum fireboard (pink) and skim finished or joint and tape contractors preference left with smooth finish. Allowance to be made for possible battening out.
· Fit 2 no internal doors fire rated including ironmongery and 5 lever mortice lock for store.

· MDF skirting and architrave throughout bull nose or chamfered style.

· Supply and fit 11 new windows in anthracite grey. Sizes to be confirmed by contractor.

· Supply and fit one number front double door with key fob entry or similar keyless entry. Anthracite grey.
· Supply and fit one number back door with fire exit bar. Anthracite grey.
· Supply and fit kitchenette to include: 4 600mm cupboards and one 400mm tall cupboard. 1 sink and associated formica worktops. Colour tbc for pricing assume light grey.
· Plumbing to include.
· Supply and fit 1 disability friendly toilet and sink with separate electric hot water tap.

· Supply and fit new kitchen tap with separate electric hot water tap if possible usable for hot drinks.

· Electrical to include:

· New consumer unit.

· 20 double sockets with usb. Heights to be disability friendly.

· 8 led strip lights 1500mm (or similar) long

· Supply and fit bell only alarm. Not connected to police or security.

· Supply and fit cctv 3 cameras to hard drive.

· Supply and fit 5no 1.5kw radiators.

· Supply and fit 1no output for large water heater in kitchenette.
· Toilet alarm

· Supply and fit hand dryer

· Supply and fit smoke and heat detection as required.

All the above to be checked on site for suitability by contractor visits to be conducted with Cllr present.

