Passenger Lift Refurbishment and Recommissioning,

Rivers House, Bridgwater

Pre-Construction Information

December 2020

Job number: EAFMREV20-092

Contents

1	Description of the project	4

1.1	Project description	4

1.2	Project location	4

1.3	Application of the Workplace (Health, Safety and Welfare) Regulations 1992	4

1.4	Programme details	5

1.5	Project team	6

1.6	Extent and location of existing records and plans	9

2	Client’s considerations and management requirements	9

2.1	Planning for and managing the construction work	9

2.2	Health and safety goals	9

2.3	Proposed communication/liaison process	10

2.4	Site security	10

2.5	Welfare provision	11

2.6	Clients requirements relating to health and safety	11

3	Environmental restrictions and existing on-site risks	15

3.1	Safety hazards	15

3.2	Health hazards	19

4	Significant design and construction hazards	21

4.1	Significant design assumptions and suggested work methods.	21

4.2	Information on significant risks identified during design.	21

4.3	Materials requiring particular precautions.	21

4.4	Arrangements for co-ordination of on-going design.	21

4.5	Information on significant environmental risks identified.	21

5	The Health and Safety File	22

5.1	Clients requirements	22

5.2	File arrangement	22

Appendix 1 – Extent and location of existing records and plans	23

Appendix 2 – Project Organisation	25

Appendix 3 – Construction Phase Plan Assessment Form	26

Appendix 4 – Designers Buildability Statement	30

Appendix 5 – Designer Hazard Inventory	31

Appendix 6 – Identified Hazardous Substances	32

Document control sheet

| JOB NUMBER: IMAN |  |  | PCI Main Body |  |  |
| --- | --- | --- | --- | --- | --- |
| Revision | Purpose Description | Originated | Reviewed | Authorised | Date |

Note to authors: As the preparation, review and authorisation of the document is a joint integrated team effort, please see the "Originated/reviewed/authorised" sections are completed by the "designer, Principal Designer and Client" respectively

# Description of the project

## Project description

The project consists of Refurbishment and Recommissioning of two passenger lifts in the lift lobby area at Rivers House, Bridgwater.  These lifts are approximately 32 years old, originally fitted within the building during its construction 1988/89.

These two passenger lifts move from ground floor, through first and stop at second floor of the building and usually have a manufacturer’s weight limit of 8 persons or 630kg.  Over the past four years, difficulties and entrapment incidents have occurred making the lifts unreliable.

This is obviously not an ideal scenario and we require the lifts to work to the correct capacity and to give customers reassurance that the lifts will work reliably.

There are five government agencies hosted within this one building, Animal and Plant Health Agency, Natural England, Environment Agency, Rural Payments Agency and Defra, with an approximate usage of the building (under normal business circumstances) of anywhere from 180 – 280 staff working from this office on a day to day.  This number can increase dependent on visits by members of the public, visitors, contractor and staff from other areas using the site as there base for the day.

## Project location

The project is located at:

Rivers House

East Quay

Bridgwater

TA6 4YS

NGR : ST3006537793

Map references :

X (Easting): 330065, Y (Northing): 137793

Latitude: 51.135121,    Longitude: -3.0009232

## Application of the Workplace (Health, Safety and Welfare) Regulations 1992

The completed structure(s) is envisaged to be classified as a workplace in accordance with the definitions contained in Workplace Health Safety and Welfare Regulations 1992.

All project Designers for the structure(s) shall ensure that they take suitable account of the Workplace Health Safety and Welfare Regulations 1992 in their design particularly in relation to the health and safety for the future use and maintenance of the structure. It must be remembered that even though the structure may not be a workplace for normal use consideration must be given towards maintenance activities and the safe access and egress to and from the structure by and with adequate segregation of vehicular and pedestrian movements.

## Programme details

The current programme key milestone dates can be seen below:

| Outline design period | December 2020 |
| --- | --- |
| Estimated date for tender | 14 December 2020 |
| Estimated PAR submission date | {Insert mm/yy} |
| Appointment of principal contractor and contract award | {Insert mm/yy} |
| Planning, detailed design and contract finalisation | {Insert mm/yy to mm/yy} |
| Time allowed for principal contractor to mobilise | 2-3 weeks |
| Start date on site | {Insert dd/mm/yy} |
| Construction phase duration (weeks) | {Insert no. weeks} |
| Completion date on site | {Insert mm/yy } |

## Project team

The project team is as follows:

### Client

| Name of organisation: | Department of Environment, Food and Rural Affairs |
| --- | --- |
| Address: | Rivers House, East Quay, Bridgwater, Somerset, TA64YS |
| Tel: | 020 847 47324 |
| Mobile: | 07766 505358 |
| Contact person | Anna Howe |
| Email address | Anna.howe@environment-agency.gov.uk |

### Designers

| Name of organisation: | Avison Young |
| --- | --- |
| Address: | 3 Brindley place Birmingham, B1 2JB |
| Tel: | 0121 609 8519 |
| Mobile: | 07903 295 313 |
| Contact person | Mark Chapman |
| Email address | mark.chapman@avisonyoung.com |

### Principal Designer

| Name of organisation: | Halcrow/Jacobs |
| --- | --- |
| Address: |  |
| Tel: |  |
| Mobile: |  |
| Contact person | TBC |
| E-mail address: |  |

### Principal contractor

| Name of organisation: | To be confirmed |
| --- | --- |
| Address: |  |
| Tel: |  |
| Mobile: |  |
| Contact person |  |
| Email address |  |

### ECI contractor

| Name of organisation: | To be confirmed |
| --- | --- |
| Address: |  |
| Tel: |  |
| Mobile: |  |
| Contact person |  |
| Email address |  |

### Others

| Name of organisation: | Defra Electrical AP |
| --- | --- |
| Address: | Trentside Office, Nottingham, Nottinghamshire NG2 5FA |
| Tel: |  |
| Mobile: | 07825 996884 |
| Contact person | Jason Twell |
| Email address | Jason.Twell@environment-agency.gov.uk |

### Principal landowners

The principal landowners on and adjacent to the site are:

Landlord – Real Estate Income Trust Ltd Suite S04, The Atkins Building, Lower Bond Street, Hinckley, Leicestershire, LE10 1QU

East Quay Medical Centre, East Quay, Bridgwater TA6 4GP - Duty Manager – Rachel Stark Tel: 01278 444666

Monmouth Scientific, Units 5 & 6, Kilnside, East Quay, Bridgwater TA6 4DB - Nigel Hamlin Tel: 01278 555145 or 01278 458090

Mayflower Hydraulics, Symons Way, Castlefields Trading Estate, Bridgwater TA6 4DR Tel: 01278 450226

- Delta Plumbing, Unit 10, Robins Dr, Bridgwater TA6 4 Tel: 01278 456642

Southgate Plastics, Unit 9-10 Robins Drive, Bridgwater TA6 4DL Tel: 01278 455326

## Extent and location of existing records and plans

Any existing asset drawings and related documents available for the scheme are detailed on the table in appendix 1 and within the Premises Section Records – Lift Folder – at Rivers House, Bridgwater.

# Client’s considerations and management requirements

## Planning for and managing the construction work

### Organisation structure

The Project Team Organogram is located in Appendix 2.

### Construction phase plan

The Principal Contractor or Contractor shall prepare a suitable Construction Phase Plan (CPP) which is to be submitted to the Principal Designer or client CDM Advisor as early as practical but at least a minimum of 10 working days prior to the planned start of any construction phase (including advanced works, site investigation or other construction activities).

The CPP will be reviewed according to the content identified in Appendix 3. However, depending upon the scope of the project this may be amended to suit specifics of the works.

No works will be allowed to commence until the CPP is deemed suitable by the Principal Designer or client CDM Advisor on behalf of the client and an instruction is given by the client for commencement.

The Principal Contractor or Contractor must plan, manage and co-ordinate work during the construction phase taking account of the information contained in the PCI provided by the Principal Designer or client CDM Advisor on behalf of the client, and any other information provided by contractors. The CPP must also comply with the specific requirements detailed in Safety is Paramount-Constructing a better environment.

## Health and safety goals

The Environment Agency’s Core Health and Safety values are:

All of us have the right to remain healthy and injury free at work. We are all vital to improving health and safety and we will:

- prevent all injuries and occupational illnesses;

- all be responsible for health and safety;

- always check and learn from what we are doing;

- challenge and respond to challenge in a positive way.

Remember health and safety is a way of life both at home and at work.

The generic Health and Safety goals for all ncpms projects can be found in NCF2 contract, Schedule 12 Appendix A, Safety is Paramount-Constructing a better environment. However in addition to these generic goals the following project specific goals will be implemented. In all instances the designers, ECI contractor, principal contractor and contractor’s performance will be expected to meet or exceed the minimum standards required under the appropriate legislation.

### Client focussed health and safety goals

Health and Safety of our staff, visitors and contractors is at the heart of everything we 	do, together we can ensure the Environment Agency premises are a safe place to work, 	where we feel healthy and fulfilled. We all have a part to play and we all have the right 	to go home, as safe and healthy and as well as when we came to work.

### Design focussed health and safety goals

LIT 16559 - Safety, Health, Environment and Wellbeing (SHEW) Code of Practice

LIT 12520 - Designers’ safety, health and environmental Red Amber Green (RAG) list

Principal contractor and contractor’s focussed health and safety goals

SHEW handbook compliance checklist – supporting document
SHEW Cope of Practice – Supporting document

## Proposed communication/liaison process

The client’s representative will be resident on site. A communication/liaison process will be agreed with the contractor to comply with the mutual QA procedures.

The Health and Safety Timeline in Appendix D of Safety is Paramount Constructing a better environment diagrammatically shows the CDM process to be followed and details who needs to be involved at what stage.

It is mandatory that the Principal Designer is informed of the following:

- Appointment of any designers by anyone (temporary works, specialists, etc.).

- Design changes where safety considerations are required for all permanent and temporary works.

- Proof that safety is considered for all design changes and variations

## Site security

Site security and preventing unauthorised persons entering the site area is of paramount importance to achieve this.  The client expects the minimum standard of security to include the following:

- Access to the main site is via main driveway from East Quay, Bridgwater.

- Main compound fencing will be adjacent to the rear entrance and cold store in the rear car park at Rivers House, East Quay, Bridgwater.

## Welfare provision

On site welfare facilities, including hand basins (with hot and cold running water), toilets, showers, drinks making facilities and a rest area are available for use on site.

Alternatively the contractor can provide full welfare provision for the site using portable cabins etc.  The Client, PC and PD will agree how best to provide these facilities, in accordance with Regulation 13(7),  22 (1) (c) and Schedule 2 of the CDM regulations plus additional provisions as stated in Safety is Paramount Constructing a better environment section 6.3. Proposals shall be fully detailed in the Construction Phase Plan submitted to the client for review under Regulation 16, this shall include a schematic showing the compound layout and welfare facilities.

Proposed main compound and welfare location has been identified and is shown on drawing:

## Clients requirements relating to health and safety

### Competency and training

The Client has very specific competence and training requirements as detailed in 	section 4.1 Safety is Paramount Constructing a better environment.  The PC must 	ensure that these are adhered to.

### Site hoarding/fencing requirements

No other security hoarding or fencing requirements

### Site transport/vehicle movement restrictions

A traffic management 	plan will be required from the contractor and should be included 	as part of the CPP information.  Deliveries to site can be accommodated from 	08:30 – 17:00 Monday to Friday, when a contractor’s representative should be on 	site to take these deliveries, unless prior authorisation has been given by the Site 	Responsible Officer, Anna Howe – Facilities Team Leader – for the Facilities 	Management Team to accept a delivery on 	behalf of the contractor.

If larger vehicles (larger vehicles is defined in this case as a vehicle over 16 tonne 	vehicles) are attempting to make these deliveries, the contractor will be expected to 	provide a banksman to ensure safety on site, during the course of the delivery.

### Client permit to work system

The client has permit to work systems specific to this project.

| Ref | Activity | Permit reference | Permit owner |
| --- | --- | --- | --- |
| 1 | Hot works | TBC | Site Responsible Officer (SRO) Anna Howe |
| 2 | Excavations | Not applicable |  |
| 3 | Confined space entry | TBC | Contractor own system |
| 4 | Isolations (mech/elec) | TBC | Jason Twell – CASHEW Technical Team |
| 5 | Electrical working | TBC | Jason Twell – CASHEW Technical Team |
| 6 | Asbestos working | TBC | SRO Anna Howe |
| 7 | Lifting operations | TBC | Contractor own system |
| 8 | Fire Alarm & Intruder Alarm Systems (interruption of services) | TBC | SRO Anna Howe |

The above list of client specific permits does not remove the contractors own requirements for permit operations where required through their own quality assurance system.

### Fire precautions

The client has existing site fire risk assessments, plans and procedures.

The principal contractor shall carry out suitable fire risk assessments and arrange his own procedures and fire plan. Details must be included in the construction phase plan prior to commencement of work on site. It is expected that site specific plans are produced covering:

- Main compound area

- Lift Lobby

- Lift Shaft

- Plant Room

- Other areas as assessed by the principal contractor

### Emergency procedures/means of escape

The client has existing site emergency plans and procedures.

The nearest Accident and Emergency facility is located at:

Musgrove Park Hospital

Parkfield Drive

Taunton

TA1 5DA

Tel: 01823 333444

Opening hours: 24 hours a day, 7 days a week.

The nearest Minor Injuries facility is located at:

Bridgwater Community Hospital

Bower Lane

Bridgwater

TA6 4GU

Tel: 01278 436555

Opening hours: 08:00 – 21:00 every day.

The principal contractor shall carry out suitable assessment for emergency planning and arrange his own procedures. Details must be included in the construction phase plan prior to commencement of work on site. It is expected that site specific emergency plans are produced covering:

- Confined space incidents

- Working at heights and harness recovery

- Recovery in the event of contact with electricity/electrocution

- Recovery in the event of mechanical failure / crush injury

The Principal Contractor should ensure that the RIDDOR reporting procedure detailed in Appendix A of Safety is Paramount Constructing a better environment is followed.

### Details of “no-go” or authorisation requirements

The client has deemed the following as no-go areas:

| Ref | Location | Location owner/contact |
| --- | --- | --- |
| 1 | Hydrometry & Telemetry Compound | Hyrometry & Telemetry Team Leader or Technical Specialist. |
| 2 |  |  |

The client has deemed the following areas require authorisation for access:

| Ref | Location | Authorisation reference | Location owner/contact |
| --- | --- | --- | --- |
| 1 | Plant Room | TBC | Rivers House, Bridgwater |
| 2 | Roof Space/Boiler Room | TBC | Rivers House, Bridgwater |
| 3 | ISDX Room | TBC | Rivers House, Bridgwater |

### Designated confined spaces

The Client has deemed the following areas as confined spaces and is subject to entry permit requirements:

| Ref | Location | Permit Reference | Permit Owner |
| --- | --- | --- | --- |
| 1 | Lift Pit & Shaft | Contractors own system ref | Contractors own appointed person |
| 2 |  |  |  |

### Smoking restrictions

Smoking on the project is not permitted.  This is a no smoking site, including in vehicles parked in the car parking areas to the front and rear of the building.

### Parking

All contractor personnel should park within the designated car park areas.

The principal contractor shall ensure that a traffic plan is produced. Details must be included in the construction phase plan prior to commencement of work on site. It is expected that traffic management plans are produced detailing the following as a minimum:

- Access to site

- Vehicular traffic routes around site

- Pedestrian walkways around the site

- Parking areas

- Storage areas

# Environmental restrictions and existing on-site risks

## Safety hazards

### Access to site

- Site access is from the main road on East Quay, there is only one access point onto and off site.

- The site driveway must not be blocked by vehicles, parked or waiting, at any time (double yellow lines indicate this around the site).

- The width of the driveway is: 6 meters.  This measurement is taken at the narrowest point between the barrier control box and barrier arm receiver.

- The car park barrier is open (in the raised position) between 07:00 – 19:00, Monday to Friday.  The barrier then closes (in the lowered position) on a timer between 19:00 – 07:00, Monday to Friday.   During Saturdays and Sundays the barrier is down (in the lowered position) from 19:00 on Friday evening until 07:00 on Monday morning. Anyone requiring vehicular access to site when the barrier is closed with need prior authorisation from the Site Responsible Officer, Anna Howe.

- There are currently no height restrictions in place on site.

- Large vehicles will need the aid of a banksman.  Larger vehicle will need to turn right into Assembly Point C and then reverse back down rear driveway (with the aid of the banksman) before they can then drive forward to leave the site.

- East Quay is a busy road but it is mainly an industrial area, there no schools or residential area restrictions here.  Site Map below (not to scale).

### Boundaries

Site perimeter is captured in plan at 3.2.2.

### Restrictions on deliveries and waste requirements

Deliveries to site should only be made between the hours of 08:30-16:00

Monday – Friday, whilst a contractor’s representative is available to receive it.

Deliveries at weekends will only be accepted by prior arrangement with the Site Responsible Officer, Anna Howe.

### Adjacent land use and known construction activity

- There are currently no known construction activity at adjacent sites or nearby, that  may affect works

### Existing services

Underground and overhead services information has been provided for the following:

- Gas

- Water

- Electric

- Telecommunications - BT

- Telemetry

- Foul Sewer and Surface Water Drains

Site Services Plan can be found at Appendix 7

** hard copies can be provided upon request**

Detailed utility search including ground penetration radar has not been carried out due to the nature of the scheme and location.

A pipelines search using www.linesearch.org has been carried out and the results show that pipelines have not been identified in the area of the proposed works.

Results from this search were returned by Western Power Distribution and Wales and West Utilities.  Maps below contain the search results.  Hard copies can be provided upon request.

www.linesearch.org searches results (conducted 16 June 2020).

This does not remove the principal contractor’s requirement to carry out safe working practices in relation to overhead and underground services.

### Ground conditions

No information has been collated or instructed on ground conditions.

### Unexploded ordnance

An unexploded ordnance search has not been carried out. The Site Responsible Officer has never found any UXO.  This is a working office and we have no reason to believe that we would find such a device.  To the best of our knowledge works will not require any breaking ground and so, no UXO search has been carried out. If any above ground UXO devices would have been dealt with immediately.

### Existing structures

There are no existing structures that may be effected by these works.  Works are contained within Rivers House, Bridgwater.

## Health hazards

### Asbestos, including results of surveys

The client is not aware of asbestos on site that will impact these works.

A demolition and refurbishment asbestos survey has been carried out, please see link below:

### Existing storage of hazardous materials

Below: Rivers House, Bridgwater Hazard Map:

### Contaminated land, including results of surveys

The client is not aware of any contaminated land.

### Health risks from clients activities

- APHA colleagues deal with veterinary procedures, so have Sharps waste and Clinical waste.  These items are specifically identified by containment buckets and bags and are securely stored in their workshop.  This workshop is located on the ground floor, next to the Laboratory and is not in the immediate work areas of this project.

- Rentokil provide pest control for vermin on this site.  Information can be provided, on request, from their hard copy file on the type of bait used and number of occurrences of vermin being found on site.

# Significant design and construction hazards

## Significant design assumptions and suggested work methods.

The designer has produced a buildability statement (including details of all design and methods of working assumptions made by the designers during the design of the structures.

This is located in Appendix 4.

## Information on significant risks identified during design.

The designer has produced a design hazard inventory identifying residual risks and assumed control measures based upon the design.

This is located in Appendix 5

## Materials requiring particular precautions.

The designers have not identified any substances which will require health precautions in their design (i.e. COSHH products).

A table identifying these is located in Appendix 6

## Arrangements for co-ordination of on-going design.

The client’s representative will be resident on site and will be responsible, in conjunction with the designer for co-ordination. A communication/liaison process will be agreed with the contractor to comply with the mutual QA procedures.

It is essential that the Principal Designer is informed of the following:

- Appointment of any designers (piling, temporary works, etc.)

- Any design work still to be carried out

- Design changes where safety considerations are required for all permanent and temporary works.

- Proof that safety is considered for all design variations (however minor).

## Information on significant environmental risks identified.

An EAP must be in place for all projects before any works commence on site (including site set-up).

The specific environmental risks are detailed in the Environmental Action Plan (EAP).

# The Health and Safety File

## Clients requirements

A Health and Safety File is to be prepared for the project and available for use by all those who will have a responsibility for the on-going maintenance of the structure or scheme.

The principal contractor, contractors, designers and client, shall, where applicable, provide the Principal Designer with all relevant information for inclusion in the Health and Safety File. The Health and Safety File format and content is to be issued under as a separate document.

The Health and Safety File shall be substantially complete and available at project handover and to facilitate this all parties shall cooperate fully with the Principal Designer in his duty to prepare this and meet agreed deadlines for preparing and submitting the requisite information requested by the Principal Designer.

## File arrangement

Principal contractor, contractors, designers and client shall ensure that information is collated during the project and issued at the completion of each {agreed section/stage/agreed point}.

## Appendix 1 – Extent and location of existing records and plans

Any existing asset drawings and related documents available for the scheme are detailed below.

| Ref | Document Title | Location (Asite Link, etc) | Date of Issue |
| --- | --- | --- | --- |
| 1 | Welfare Provision – Site Map (not to scale) | Produced Locally. Embeded in Section 2.5 of this document – pg 11. | 27 NOV 2020 |
| 2 | Asbestos Refurbishment and Demolision Survey. Ref: 151910_292308 Rivers House Bridgwater Full Report V1 | Embedded in Section 3.2.1 of this document - pg19. Survey provided by Socotec Ltd. | 29 JUL 2020 |
| 3 | Wales and West Utilities, Job ref: 19130743 | Section 3.1.5 of this document – pg 18. Search provide by: www.linesearchbeforeudig.co.uk | 16 JUN 2020 |
| 4 | Western Power Distribution Job Ref: 19130743_WPD | Section 3.1.5 of this document – pg 18. Search provided by: www.linesearchbeforedig.co.uk | 16 JUN 2020 |
| 5 | Bridgwater Hazard Map | Produced Locally. Embedded in Section 3.2.2 of this document – pg 20 | 27 NOV 2020 |
| 6 | Site Surveys of Underground and Overhead Services. Rivers House, East Quay, Bridgwater. | Provided by Southern Electric Contracting. Appendix 7 – pg 33 of this document. | 18 DEC 2002 |

Other standard documents which are relevant are as follows:

| Ref | Document Title | Location (Asite Link, etc) | Date of Issue |
| --- | --- | --- | --- |
| 1 | Constructing a Better Environment Safety, Health, Environment and Wellbeing (SHEW) Code of Practice | Ref : LIT 16559, embedded in Section 2.2.2 of this document – pg 10. | May 2018 |
| 2 | Designers’ safety, health and environmental Red Amber Green (RAG) list | Ref: LIT 12520, embedded in Section 2.2.2 of this document – pg 10. | Aug 2015 |
| 3 |  |  |  |

## Appendix 2 – Project Organisation

## Appendix 3 – Construction Phase Plan Assessment Form

| ACOP Recommendation | Comment | Location in Plan |
| --- | --- | --- |
| 1. Description of Project |  |  |
| - project description and programme details; | - Project Description: - Programme: - Schedule of method statements / RA: |  |
| - details of client, CDM coordinator, designers, principal contractor, specialist contractors and other consultants | - Client: - PD: - Designers: - PC: - Contractors: - Specialists: |  |
| - extent and location of existing records and plans that are relevant to health and safety on site, including information about existing structures when appropriate |  |  |
| 2. Communication and management of the work |  |  |
| - management structure and responsibilities; | - Organisation chart: - Management Responsibilities: |  |
| - health and safety goals for the project and arrangements for monitoring and review of health and safety performance; | - Project safety goals: - Monitoring arrangements: |  |
| - arrangements for: |  |  |
| - regular liaison between parties on site including any key dates; |  |  |
| - consultation with the workforce; |  |  |
| - the exchange of design information between the client, designers, CDM coordinator and contractors on site; |  |  |
| - arrangements for management of design, including temporary works; |  |  |
| - handling design changes during the project; |  |  |
| - the selection and control of contractors; |  |  |
| - the exchange of health and safety information between contractors; |  |  |
| - site security |  |  |
| - site induction | - Safety is Paramount included |  |
| - on site training |  |  |
| - first aid arrangements; |  |  |
| - Welfare Arrangements | - Compound Layout Plan - Sanitary conveniences - Female facilities - Washing facilities - showers - Drinking water - Changing rooms and lockers - Facilities for rest |  |
| - the reporting and investigating of accidents and incidents including near misses; | - Include EA reporting requirements |  |
| - the production and approval of risk assessments and written system of work; | - Briefback included |  |
| - site rules |  |  |
| - drugs & alcohol policy |  |  |
| - emergency procedures. | - Fire & evacuation: - Water rescue: - Confined spaces: - Harness rescue: - UXO: - Other: |  |
| 3. Arrangements for controlling significant site risks |  |  |
| a) safety risks, including; |  |  |
| - delivery and removal of materials (including waste) and work equipment taking account of any risks to the public; |  |  |
| - dealing with services - water, electricity and gas, including overhead powerlines and temporary electrical installations; | - Underground services: - Overhead services: - Temporary electrics & services: |  |
| - accommodating adjacent land use; |  |  |
| - stability of structures whilst carrying out construction work, including temporary structures and existing unstable structures; |  |  |
| - preventing falls; |  |  |
| - work with or near fragile materials; |  |  |
| - control of lifting operations; |  |  |
| - the maintenance of plant and equipment; |  |  |
| - work on excavations and work where there are poor ground conditions; |  |  |
| - work on wells, underground earthworks and tunnels; |  |  |
| - work on or near water where there is a risk of drowning; |  |  |
| - work involving diving; |  |  |
| - work in a caisson or compressed air working; |  |  |
| - Work involving explosives and/or UXO risks |  |  |
| - traffic routes and segregation of vehicles and pedestrians; | - Traffic management plan |  |
| - storage of materials(particularly hazardous materials) and work equipment; |  |  |
| - other significant safety risks. |  |  |
| b) health risks, including: |  |  |
| - removal of asbestos; |  |  |
| - dealing with lead paints & products |  |  |
| - dealing with contaminated land; |  |  |
| - manual handling; |  |  |
| - use of hazardous substances, particularly where there is a need for health monitoring; |  |  |
| - reducing noise; |  |  |
| - reducing vibration; |  |  |
| - work with ionising radiation |  |  |
| - exposure to UV radiation |  |  |
| - other significant health risks. |  |  |
| 4. The Health and Safety File |  |  |
| - layout and format; - arrangements for the collection and gathering of information; - storage of information. |  |  |
| 6. Initial Method Statements |  |  |
| - Inset subject |  |  |
| 6. Additional Comments |  |  |
| - : |  |  |

## Appendix 4 – Designers Buildability Statement

Insert Design Buildability Statement

## Appendix 5 – Designer Hazard Inventory

Insert into this table or replace with designers actual table

| DRA ref. | Activity | Residual hazards | Control |
| --- | --- | --- | --- |

## Appendix 6 – Identified Hazardous Substances

The most important significant issues identified by the designers are summarised on the following table:

Insert into this table or replace with designers actual table

| DRA ref. | Activity | Material name and hazard | Control |
| --- | --- | --- | --- |

APPENDIX 7 – Site Services Map