Where we have been asked questions regarding this procurement, the answers to which may be relevant to all bidders, these questions have been reproduced below along with the associated answers.

Question 1

The macadam on the footpaths is shown on the Highway Construction Details 25264_02_080_02.1 as 20mm of AC 6mm and 100mm of AC20 but the bill states 25mm of AC 6mm and 55mm of AC 20 in a number of places.

Are the distribution roads to be the same specification as the carpark section on the specification (80mm of AC 20mm and 30mm of AC 6mm) or as the carriageway cross section (150mm road base, 60mm binder, 40mm surface) – the carriageway specification seems very high for a distribution road that will be used by light vehicles.

Answer 1

There are three (3) surface treatments proposed:

- Highway – (150mm road base, 60mm binder, 40mm surface)

- Footpath (adjacent to Carriageway) – 20mm of AC 6mm and 100mm of AC20

- Cemetery Paths (around the cemetery and proposed to be coloured gravel to look aesthetically better) – 25mm of AC 6mm and 55mm of AC 20

Question 2

As part of this scheme, the car park to the East of the site has been omitted, as the car park is not shown on the new Rev B Hard Landscaping drawing 6 of 7 (embedded below).

Answer 2

The car park is to be delivered as per 4938–006F Localised Site Entrance Plan

Question 3

Is the soft landscaping and planting price to the cemetery area to be included in the “Optional Extras” under Open Space Corridor Soft Landscaping after substantial completion, as there is no item in the pricing schedule for soft landscaping in cemetery?

Answer 3

Correct. Please cost all soft landscaping under items 1 & 2 of the “Optional Extras” under Open Space Corridor Soft Landscaping.

Question 3

Could we be provided with an unlocked version of the pricing document (for use with estimating software)?

Answer 3

Yes, please find below, which has now also been uploaded to Contracts Finder.

Question 4

The Pricing Schedule SoWR – Site Entrance Alterations Items 3&4 refer to tree felling and removal of tree Nr T19,24,25. However the RammSanderson Tree Schedule (RSE_3173_TS_V1 states that tree T19 is to be maintained by repeat pruning on a 3–5 year cycle. Tree T24 & T25 – No works required. Both to remain.

The schedule does however have a list of trees to be removed, T7, T3, T4, T35,T62, T64, T88, T169, T173, T215, T219, T220, T221, T223, T233, T235, T237, T240, T242, T250–T61.

Please can you clarify what is required?

Answer 4

Tree T19 is to be removed as it is restricting operation of the CCTV camera. T24 & T25 are to be replaced as part of the planning permission.

Question 5

Are there any floor plans/elevation drawings available for the Existing Toilet Block to be refurbed, identified in the Pricing Schedule SoWR?

Answer 5

Unfortunately, the Council do not have as–built information of the existing building. Bidders are encouraged to visit site to satisfy themselves of requirements.

Question 6

Please confirm if the Refurbishment of existing Storage Area to form welfare area, identified in the Pricing Schedule OE, is the storage area beside the Stonemasons storage area.

Answer 6

Yes, this is correct.

Question 7

Are there any floor plans/elevation drawings available of existing the Storage Area to form welfare area, identified in the Pricing Schedule OE?

Answer 7

Unfortunately, the Council do not have as–built information of the existing building. Bidders are encouraged to visit site to satisfy themselves of requirements.

Question 8

Can the response deadline for this procurement be extended?

Answer 8

No, due to external pressures, the Council is unable to offer any extension of time to the response deadline, on this occasion.

Question 9

Page 5 – Strip Out: Could you confirm the location(s) of item 2 – Carefully remove existing goal wall fencing and break out posts as shown on tender specification drawings for future use by Council – 120m?

Answer 9

The goal wall is located at the western edge of current cemetery. It is strongly recommended that Bidders familiarise themselves with the site before returning their Tenders Responses.

Question 10

Page 5 – Site Entrance alterations: Site entrance, item 7 – could you confirm the route of proposed ducting – 400m (e.g. with ext verge and/or footway)?

Answer 10

It will run in verge adjacent to the existing path and then around perimeter of the new car park extension.

Question Group 11

Page 7 & 8 – Car Park extension:

- Item 4, 5, 6 – is the excavation item for these works included within item 1?

- Could you confirm the specification (sizes) / detail of timber edging?

- Item 7 – confirm location of these works. In addition, there appears to be a scaling issue with drg 006 F (car parking bays measure 2.8m long approx. when printed on A0 at 1:200).

- Could you advise of the item for kerbing to the car park?

- Item 10 – Drainage to carpark – please could you provide details?

- We understand the assumed CBR value is 2–5% as per SI report. Could you confirm if a capping layer is required and advise of the proposed depth? In addition if required, should this element be included with the carpark / highway measurement item?

- Confirm colour of PPC block required.

- Item 13 – advisory signs – please provide details.

Answer Group 11

- No

- 150mm x 19mm tanalised timber with 40mm x 40mm treated stakes.

- This is the new access into car park off existing cemetery turning head

- Please assume 125x225 HB2 Half Batter Kerbs.

- Item 3 includes item to review car park design and provide a setting out drawing for car park and associated drainage to agree with existing levels.

- Please assume a 300mm capping layer.

- Please assume a Brindle colour.

- Specification for signs should be included in tender specification – Annex 8.

Question 12

Page 8 – Gas main diversion: Are there any construction details (depths etc) of the proposed gas main?

Answer 12

Please allow a 1.2m deep trench 450mm wide

Question Group 13

Page 9 & 10 – Cemetery Extension:

  - Item 6 – is this item to include 150mm of capping (eg cstn depth of 450mm less excavation depth of 600mm)?

  - Item 7 – is the excavation element to this item (footways) is included within item 2?

  - Item 8 – Car parking bays as detailed on drawing N0681(90)001 B are specified as Marshalls Drive setts which is a 50mm deep block (three pack size), please confirm colour required. In addition, MEC standard details advise of an 80mm block is required to car parking area, please advise.

  - Item 8 – (as per item 6) will capping material be required?

  - Item 14 – Could you confirm the depth of the land drain to headstones?

  - Item 19 – can you forward construction details for headstone strips?

  - Item 20 – details of advisory signs required.

  - Item 22 – please forward construction details for Golpla Reinforced Pre–grown grass.

Answer Group 13

- Please assume a 300mm capping layer.

- Correct.

- Please assume a 50mm Brindle colour block.

- Capping material will not be required.

- Please assume a minimum depth of 650mm.

- No construction is required – just a sett at each end of the strip so Council can cut the strip as required.

- Specification for signs should be included in tender specification – Annex 8.

- Bidders have freedom to propose a suitable solution and comply with manufacturers installation requirements.

Question Group 14

Page 12 – Welfare facility:

  - Item 1 – to include 150mm of capping (eg cstn depth of 450mm less excavation depth of 600mm)?

  - Item 1 – do we need to include PCC kerbing within this item?

  - Item 3 – To confirm, trenching works for the incoming electric supply are to be completed by ourselves?

Answer Group 14

- Correct.

- Yes.

- Correct.

Question Group 15

Page 13 – Preparatory Works:

  - Item 4 – Could you provide cross sections for further details of the works required?

  - Item 5 – Please confirm there is no requirement for handling of tipped material at Abington Park (to be completed by others).

Answer Group 15

- The Council does not have a cross section for these works.

- Contractor is required to allow for removal off-site excavated material, transport, grade and install in 200mm layers to alter levels.

Question 16

Page 13 – Site entrance alteration: Item 1 – Confirmation of material for 50mm wearing course (55/10 HRA?)

Answer 16

55/10 HRA is acceptable.

Question 17

Page 13 – 14 – Preparatory Works: Item 1 & 2 – Please confirm location of works area (in addition to link road?) spec as per link road.

Answer 17

This item is a temporary access along the route of the sports ground link road if budget prevents the permanent road being constructed.

Question 18

Page 15 – Strip out & removal: Item 2 – Please confirm location of the works.

Answer 18

Office and toilet block adjacent to site entrance.

Question 19

Page 15 – Welfare facility: Item 2 – Please confirm specification details for Flygt Micro pump station.

Answer 19

Bidder should work with supplier to ensure proposed station will be suitable.

Question 20

Page 17 – Open space corridor: Item 4 – Please confirm specification details for bird boxes.

Answer 20

Bidders have freedom to propose a suitable specification.

Question 21

Page 17 – Landscaping: We note drawing N0681 (96) 003 B details a 5–year maintenance period, including the replacement of any failed trees, shrubs & seeding works. Could you confirm the details of the maintenance required – e.g. monthly watering visits, etc.

Answer 21

Please allow for replacement of failed trees, shrubs and seeding work and maintenance for 1 year. Routine maintenance after year 1 will be undertaken by others.

Question 22

In addition to the above could you advise, unless the item coverage specifically states, material to be stored on–site ALL excavated material is to be disposed of off–site, including drainage arisings?

Answer 22

Correct.

Question 23

Could you confirm proposal for car parking bay markings – is the proposal to mark using a contrasting block – noting the block paving is being laid on a 45 degree herringbone pattern, or a thermoplastic marking (with associated primer for concrete blocks)?

Answer 23

Please assume a thermoplastic marking.