ITT for Kresen Kernow Design, Supply and Installation of Archive Shelving Equipment

Shelving requirements for document processing rooms

The three small document processing rooms for which we require shelving are the quarantine, document production and document reception rooms. They will all be used to shelve records in all formats and of various sizes and weights.

General description

- Dimensions and other information about the rooms can be found in Appendix 3: Plans and descriptions of rooms

- Imposed loading limit is 5kN/m². Please confirm this is acceptable.

Requirements for shelving applicable to all rooms

- Static

- Up to the full height of room

- Minimum height of the lowest shelf to be 150mm

- Secured to walls

- Able to cope with large point loads

- Solid ends

- Solid shelves

- Unperforated top covers

- Adjustable

- Shelves each approximately 300mm high

Quarantine room (Number 0.26 on plans)

This room is shown on GA drawings:

- 235110 250 Proposed Archive Ground Floor Plan

- 235110 256 Proposed Sections 1

- 235110 266 Proposed Sections 6

Area: 8m2

Floor to ceiling height: 2570mm

Requirements:

- Shelving 500mm deep, secured to west wall, running the full length of the room

- Work surface 1500mm x 1000mm secured to north wall, at standing height, with shelving underneath with a gap for legs in the middle.

Document reception room (Number 0.17 on plans)

This room is shown on GA drawings:

- 235110 250 Proposed Archive Ground Floor Plan

- 235110 266 Proposed Sections 6

Area: 32m2

Floor to ceiling height: 2570mm

Room split lengthwise in half with east half effectively a corridor

Requirements:

- Space to be left in west half to accommodate adjustable preservation table in centre measuring 2000mm x 2000mm.

- Shelving 500mm deep on north, west and south walls (these form L shapes with the table in the middle)

Document production room (Number 1.05 on plans)

This room is shown on GA drawings:

- 235110 258 Proposed Brewhouse First Floor Plan

- 235110 266 Proposed Sections 6

Area: 13m2

Floor to ceiling height: 2570mm

Western third of room empty with doors off

Requirements:

- Shelving on east wall, 400mm deep, running the full length of the wall