TCA 3/7/991

PROVISION OF AN EMERGENCY TOWING VESSEL (ETV)

- How the tender evaluation will be conducted? –

The tender who offers the overall best value for money to the MCA will be awarded the contract. Best value will be assessed by the cost to be paid (Price) and meeting the requirement (Quality).

This tender had the following weightings

| Price | 40% |
| --- | --- |
| Quality | 60% |

- How the tender will be scored:

Your tender will be assessed against the criteria and sub criterion identified in sections 4 & 5 of this document. Each sub criterion will be allocated a score from 0-5 depending on the information that is provided in the tender submission. Below details what each score represents –

5 - Proposal fully meets the requirement

4 - Proposal fulfils the requirement but with minor omissions

3 - Proposal covers only the basic requirement

2 - Proposal falls short of fulfilling the requirement

1 - Proposal inadequate

0 - Requirements not addressed

Your total score achieved for each Criteria weighting i.e. Primary Capability, Secondary function etc. will be turned into a percentage against the total score achieved for each section.

- Price

Price score will be calculated in the following way -

The lowest priced tender is given 100 points.  All other higher priced tenders are then base lined against the best priced tender i.e.

Lowest Bid Cost

Cost Score = --------------------------   x   Weightings

Each Bidder’s Cost

95% of the cost weighting is allocated to the day rate of the vessel. 5% is allocated to the livery of the vessel.

- Quality

The quality aspect of the tender will be weighted as follows

Weightings

Primary Capability 	55

Secondary Functions 	20

Environmental Particulars 	20

Contract Management 	 5

- Quality Criteria and Sub Criterion

| Detailed below is the sub criterion that we are looking to assess against each criteria weighting. Further details of each of these sub criterion can be found within the Statement of User Requirement. Criteria - Primary Capability - 55%. Sub Criterion; Reference on Statement of User Requirement & Information required; Maximum potential score. Livery; 5 & Annex B; At contract start, the vessel will be required to be liveried in such a way that it is readily identifiable as an official resource of the charterer. The design, arrangements for and execution of this livery change, and subsequent re-instatement of original livery on contract termination is at Annex B.; 5. Vessels Age; 6; In order to achieve greater efficiency and to take advantage of more recent advances in technology, strong preference will be given to tendered vessels which are less than 15 years old. Vessels older than this will be considered, and their comparative merits across the entirety of this SUR will be evaluated using the criteria at Annex C; 5. Flag, Class & ISM Compliance; 7 & 8; It will be operated, maintained and manned in accordance with the requirements of its Flag State and be ISM compliant. The Contractor will be required to ensure that the ETV is classed and remains in survey with an IACS classification society that is recognised by the MCA as at the commencement date and throughout the Contract term. It is a condition of delivery that the vessel must achieve a satisfactory inspection in accordance with the Common Marine Inspection Document (CMID) as published by the International Marine Contractors Association (IMCA).; 5. Bollard Pull (BPT); 10.1; The Contractor will be required to ensure that the ETV has a bollard pull of not less than 115 tonnes. Certificates attesting the BP of the vessel must not be any older than 5 years at the contract commencement date.; 5. Design; 10.1; Vessels tendered will have seakeeping and overall efficiency and effectiveness in mind, it is anticipated that the LOA will be in the region of 60m-70m, breadth, 15-16m and draft around 7m.; 5. Free Running Speed; 10.4; Continuously cruise (when free running) at a speed of no less than 14 knots.; 5. All Weather Capability; 10.5; Be safely operated and manoeuvred to connect and effect towage in the sea and weather conditions that may reasonably be foreseen in UK waters, eg. Wind conditions of Force 9-10 and Sea States of 6-7 are common in the designated operation area;; 5. Redundancy - Propulsion; 10.3; Have 100% redundancy in both the propulsion and towing systems.; 5. Redundancy - Towing; 10.3; Have 100% redundancy in both the propulsion and towing systems.; 5. Deployment Sustainability; 10.6; Be capable of carrying sufficient water, stores and fuel to be able to operate continuously at sea for no fewer than 10 days at the maximum rate of consumption.; 5. Crane Available; 10.7; Deploy a deck mounted crane capable of a load of at least 12 tonnes at 12 metres.; 5. Work boat Available; 10.8; At least one suitable work boat with propulsion should be carried onboard for transferring personnel and/or equipment from the tug to another vessel , amongst other roles.; 5. Towing Equipment Capability; 12.1.1-12.1.3; Deliver emergency towage as directed using suitable towing equipment to be provided by the vessel operator. As a minimum tendered vessels will have: 12.1.1 a certified towing winch of capacity matching the bollard pull, remote operated from the wheel house and with towline load monitoring; 12.1.2 a main towing line of not less than 1000 metres; 12.1.3 one spare towline of minimum length and strength as required for the main towing line shall be onboard the towing vessel, preferably on a separate drum on the towing winch or on a storage drum.; 5. Reliability - 24/7/-365- 98% availability - 30mins; 14,15 & 16; The Contractor will be required to provide ETV services with the operational capability to respond, as directed by the MCA, to any incident within the designated operations area on a 24 hours per day/ 7 days per week basis. The vessel will be maintained in a fully serviceable condition and operated to achieve at least 98% availability for mobilisation within 30 minutes of being tasked. The tender will clearly demonstrate contingency arrangements to provide continuous station cover for emergency replacement, planned docking and maintenance periods.; 5. Manning - Officer and Crew; 17; ETVs will be manned in accordance with the requirements laid down by her flag state, and commensurate with the tasking of an Emergency Towing Vessel, but not less than 10 crew including the Master.; 5. Spare Accommodation; 18; The provision of additional accommodation to support at least 6 personnel is expected, with appropriate SOLAS III life-saving appliances and arrangements; 5. Crew Training and Competencies; 19,20,21 & 22; The charterer would strongly prefer that the vessel‘s officers be European Economic Area (EEA) nationals and that as a minimum, the Master(s) and senior officers be proficient in English. The operator must produce evidence of the professional competency and experience of the vessels’ crew to the charterer upon request. Crew competency is to be maintained at the highest operational standard. Tenderers must confirm that they have the operational capability to respond, as directed by the MCA, to any incident within the area. Master(s) and crew members directly involved in towing operations should have proven emergency rescue towing and ocean towage experience. Documentary evidence of crew capability and experience is to be provided by the tenderer. The contractor will be expected to work with the charterer in developing and implementing a training programme designed to maintain crew skills. Vessel capability and crew competence will be audited annually.; 5. Total Score available for this section; 85. Criteria - Secondary Functions - 20%. Sub Criterion; Reference on Statement of User Requirement & Information required; Maximum potential score. Dynamic Positioning; 11.1; Ability to manoeuvre to a minimum of DP1 standard.; 5. Fast Rescue Craft; 11.3; A SOLAS approved Fast Rescue Craft (FRC).; 5. Salvage Support; 13.1; Basic salvage support, through provision of patching equipment, emergency portable power and pumping capability, including submersible pumps - (50 m3- 200 m3/hr).; 5. SAR/ On Scene Coordinator; 11.2 & 13.3; SAR Incident Support, to provide support as required by Her Majesty’s Coastguard (HMCG); Capability to act as On Scene Coordinator (OSC) during SAR operations, on request of HMCG. This would require a minimum onboard communication capability of Marine VHF, a GMDSS suite and MF/HF; 5. Fire Fighting; 13.2; Fire Fighting to a minimum industry standard FiFi Class 2 is preferred, though vessels fitted to FiFi 1 will be considered;; 5. CP Support; 13.4; Act in support of counter pollution response, to be able to carry/store nominated pollution response equipment and operate as a deployment platform as directed by the MCA.; 5. Training platform; 13.5; Operate as a training platform for helicopter winch operations as directed by the MCA.; 5. Commercial Support; 13.6; Provision of platform which has the potential to be utilised for other commercial activities unrelated to emergency towing (eg. civil hydrograph, academic research, anchor handling and buoy laying operations). The vessel should have the ability to embark and temporarily store (on deck if appropriate), limited quantities of specialist equipment, to be operated by a specialist embarked team.; 5. Total Score available for this section; 40. Criteria - Environmental Particulars - 20%. Sub Criterion; Reference on Statement of User Requirement & Information required; Maximum potential score. Environmental Statement; 9; Tenders will be required to provide an environmental policy statement, detailed provision of fuel consumption and emissions will be declared, fuel efficiency, and emissions controls measures will be key considerations in the tender assessment.; 5. Fuel Consumption; 9; Tenders will be required to provide an environmental policy statement, detailed provision of fuel consumption and emissions will be declared, fuel efficiency, and emissions controls measures will be key considerations in the tender assessment.; 5. Efficiency; 9; Tenders will be required to provide an environmental policy statement, detailed provision of fuel consumption and emissions will be declared, fuel efficiency, and emissions controls measures will be key considerations in the tender assessment.; 5. Emissions Declaration; 9; Tenders will be required to provide an environmental policy statement, detailed provision of fuel consumption and emissions will be declared, fuel efficiency, and emissions controls measures will be key considerations in the tender assessment.; 5. Total Score available for this section; 20. Criteria - Contract Management – 5%. Sub Criterion; Reference on Statement of User Requirement & Information required; Maximum potential score. Contract Review meetings; 23; The contractor will be required to attend contract review meetings as required. These will take place on a quarterly basis dates to be agreed with the contractor, The MCA reserves the right to call a meeting at any time during normal office hours to resolve a service delivery issue.; 5. Administrative management; 24; The contractor should identify their administrative management structure within their tender proposal. This should also include escalation procedures for service delivery management.; 5. SLA / KPI; 25; Tenderers should identify Service Level Agreements (SLA's) for all areas of service delivery. This will be monitored against agreed KPI’s and reported to the MCA on a monthly basis. The tenderer should identify opportunities which could realise cost savings through alternative service delivery models.; 5. Total Score available for this section; 15. Please note that tenders that do not score a 5 for the sub criterion detailed in the table below will be excluded from this tender process. Bollard Pull; The Contractor will be required to ensure that the ETV has a bollard pull of not less than 115 tonnes. Certificates attesting the BP of the vessel must not be any older than 5 years at the contract commencement date.. Free Running Speed; Continuously cruise (when free running) at a speed of no less than 14 knots.. Towing Equipment Capability; As a minimum tendered vessels will have: A certified towing winch of capacity matching the bollard pull, remote operated from the wheel house and with towline load monitoring. A main towing line of not less than 1000 metres. One spare towline of minimum length and strength as required for the main towing line shall be onboard the towing vessel, preferably on a separate drum on the towing winch or on a storage drum.. Crew Training and Competencies; As a minimum crew will be expected to have Ocean towing & Emergency Rescue Towing experience.. |  |  |
| --- | --- | --- |