Introduction

Strode College has undertaken some pre-market engagement to determine if we can replace our existing analogue phone system with an IP based system at a cost below our current budget for calls, line rental and maintenance. In addition to reducing costs the College is also looking to increase functionality, and simplify administration.

Our pre-marked engagement has determined that our objectives are achievable and the College is now seeking proposals to buy an IP telephony system / service. As guidance to suppliers solutions, which need to include all hardware, software, maintenance, network access and calls, need to be priced below c£85k over 5 years to have a realistic chance of being successful.

Background / current system

Strode College is a mid-sized tertiary college located on a single campus in Street, Somerset. Over recent years, the College has invested significantly in the ICT infrastructure, which has included a migration, in part, to IPT.

The current provision is based on an Alcatel 4000 series PBX, with 167 telephone extensions.  These extensions link to a wide range of handsets including analogue, digital and ip.  Additional service features include voice-mail and auto-attendant with night-service, together with standard features including hunt groups and call forwarding.  An ISDN30 connection allowing for 10 concurrent external connections is in place, and this is supplemented by a number of independent direct lines that support alarm services, and broadband connections.

The College is currently connected to the internet through the JANET academic network via a 400MB link (on a 1GB bearer).  The College expects that this connection provides sufficient capacity for voice traffic, in addition to its current data use.

General requirement

Based on our pre-market engagement the College is particularly interested in proposals for an on premise solution but hosted, or hybrid solutions will be considered if they can meet the price ceiling.

Solutions need to include all hardware, software, maintenance, network access and calls. Currently call costs are c£500 per month to a mix of UK fixed and mobile numbers. The solution should contain sufficient call allowances to cover this level of usage.

Any proposal, in addition to being able to demonstrate both a cost saving compared to the current provision, must also provide a level of resilience / redundancy that minimises the risk of the phone system being unavailable for more than reasonable amount of time (probably no more than 4 hours but bidders are invited to provide a solution that mitigates the risk of unavailability to an acceptable level). The proposed solution should also have a comprehensive, easy to use management / administration system.

It is believed that the current cabling infrastructure would support the introduction of IP based telephony and we will replace a few network switches to ensure there is adequate POE. Bidders should provide details of any further network improvements they feel are necessary to ensure the implementation is successful.

There are limited numbers of network ports across the campus and as a consequence we either need IP phones that have dual gigabit Ethernet ports or an alternative solution for sharing Ethernet ports.

Specific requirements of the system are included at Appendix A.

Additional requirement

As part of this project the College is keen to improve circuit resilience and it would therefore like to obtain costed options for a secondary internet connection that could be used for a variety of purposes including failover for voice and limited data, and for a public internet service for visitors to its campus.  As a guide to bidders solutions should be no more than £2,500+ vat per annum.

Instructions to bidders and general terms

The College is not obligated to accept any proposal and will not cover the costs of any bidders in responding to this request for proposal.

Proposals will be assessed on the basis of both quality and price with the following weighting:

Price: 				70%

Quality Submission		30%

The quality of the submission will be assessed against matching the core and specific requirements (50%) the implementation programme and on-going support (25%) and the interview (25%).

If bidders cannot meet the implementation window there proposals may not be evaluated.

Bidder’s responses should take the form of a costed technical proposal which should, as a minimum, contain the following:

- Detailed cost breakdown identifying hardware, software, licences, call charges, and associated costs by item to be supplied over 5 years.

- If available a lease / monthly cost over 5 years plus renewal / secondary term costs after 5 years

- List of reference sites.  (Ideally organisations with similar requirements)

- Implantation plan, based on a proposed implementation date of early August 2017

- Costed options for technical / customer support

The proposals should be capable of acceptance by the College by placing an order up to 90 days after the date of submission.

The timetable below will be used for this procurement. It is essential that bidders can meet the implementation timetable.

| Request For Proposal issued | 5th May 2017 |
| --- | --- |
| Request For Proposal return | 12 noon 26th May 2017 |
| Interview Day Clarification | Week Commencing 5th June 2017 |
| Order Placement | 9th June 2017 |
| Contract Start | 12th June 2017 |
| Implementation window | 1st August 2017 to 8th August 2017 |

As this procurement is not subject to the Public Contract Regulations and due to the condensed timescales there will not be a standstill period prior to order placement.

Tenders should be returned by email to Janet Rhind-Tutt, Procurement Officer - jrhind-tutt@strode-college.ac.uk by 12 noon on 26th May 2017.

For further information, please contact either:

Janet Rhind-Tutt

Procurement Officer - jrhind-tutt@strode-college.ac.uk.

Tim Blake

Head of IT Services    tblake@strode-college.ac.uk 01458 844 425

Or

Matt Seymour

Senior Engineer    mseymour@strode-college.ac.uk 01458 844 400

Appendix A – Specific Requirements

Please note that this is not an exclusive list and should additional features be available, the College would be keen to explore additional functionality)

| Feature | Additional information | Criteria |
| --- | --- | --- |
| 162 standard (simple) desktop handsets (Fewer handsets may be procured depending on the final solution and the pricing provided must enable the College to adjust the volumes easily) | (additional 4 handsets to current service accommodate growth/handset failure) Please also include pricing for cordless phones. | Essential |
| 6 advanced handsets for Personal Assistants | Exec Office 3 units DB, LH, LB | Essential |
| 3 Switchboard handsets | Two in main reception, one in separate building for evening service | Essential |
| Softphone option (laptop / desktop) |  | Desirable |
| Ability to make and receive external / Internal calls | Control of extensions / blocking international / premium rate, number masking etc | Essential |
| Resilience / failover / rerouting of calls | Should main circuit or power fail | Desirable |
| Ability to place calls on hold | Ability select ‘on hold music / message’ | Essential |
| Ability to Mute conversations | Privacy | Essential |
| Ability for Speaker (headset free) calls and conference calls | Conference call and shared handset/speaker | Essential at least on some phones |
| Call redirect and transfer | Ability to withhold Senior team phone numbers eg secretary manager function – divert to secretary but secretary able to transfer through divert. | Essential |
| Call transfer | Ability to Transfer calls to internal extensions including the ability to talk to the colleague prior to transferring the call | Essential |
| Caller number display (Internal) |  | Desirable |
| Caller number display (External) |  | Desirable |
| Call forward (lost call prevention) | Ability to redirect to alternative internal extensions should call “bounce back” to reception | Desirable |
| Voice-mailbox (Easy switch of line to corporate mailbox) | Capability to switch to voicemail at end of day and when college is closed | Essential |
| Call termination | Ability to end calls without having to press an additional “release” button | Desirable |
| Speed-dial | Ability to store regular numbers for 1 touch dialling | Desirable |
| Call identification (type) | Ability to be able to differentiate between internal and external calls (eg different ring tone) | Desirable |
| Auto-attendant | Requirement to route incoming calls on voice or tone activated signal to department | Essential |
| Voicemail | Individual mailbox with secure passcode | Essential |
| Voicemail (unified) | Ability to retrieve voicemail through email | Desirable |
| Hunt Group | Easy setup / administration | Essential |
| Group pick up | Easy setup / administration | Essential |
| Programmable keys | Speed dial function | Desirable |
| Other extension monitoring | To view status of other extensions | Desirable |
| Follow-me phone feature for mobile workers. (mostly campus based). |  | Desirable |
| Voice recording feature | Not at this point deemed essential, but capability to add to service should be identified | Desirable |
| Headset flexibility for roaming user / Hands-free option for handsets | A number of staff ‘roam’ within an office and Bluetooth or similar connectivity would be required. | Essential |