L20 545 SLIDING STACKING PANEL PARTITIONS

- Project: Judith’s Field Pavilion, Godmanchester.

- Location: Meeting Room 1 / 2

- Quantity: 1no.

- Drawing References: Refer to drawing no. x

- Manufacturer & product reference: Dorma Variflex 88 Moveable Wall.

- Supplier: Style Midlands, Unit 3 Phoenix Park, Telford Way, Stephenson Industrial Estate, Coalville, Leicestershire, LE67 3HB.  Tel: 01530 831144, Fax: 01530 831184.

E-mail: sam.ingley@style-partitions.co.uk   Web: www.style-partitions.co.uk

- Method of operation: Manually operated moveable wall system. Each element incorporates spring loaded top and bottom mechanical seals. Seals are operated by means of a removable handle and engaged by a winding scissor mechanism (not cam) to ensure optimum contact pressure of 1500N minimum. Seals are to be constructed of aluminium with flexible rubber inter locking contact points.

- Telescopic element must maintain all the same quality and acoustic criteria as the standard element elements, and should be operated by a single mechanism for both top, bottom and lateral seals. Lateral seal extension to be 120mm.

- Characteristics: Moveable wall comprising individual top hung full height elements of equal size, made to measure to suit site dimensions.  When wall is closed between fixed abutments it gives the appearance of a solid flush wall with concealed interlocking concave and convex extruded aluminium edge profiles which incorporate integral magnetic strips.

- Construction: Tri-block elements to be constructed of a non-deflecting steel tube frame complete with independent aluminium edge profiles, element thickness 88mm. The frame must be clad either side with 10mm MDF elements hung in acoustic isolation from the frame and allowed to vibrate independently of the frame. The facing boards must be easily removable without the need to remove the elements from the track. Track extruded aluminium, track junctions incorporating drop forged roller bearings. Track sections should be easily removable without disruption to ceiling/bulkhead. Carriers fully adjustable and secured to elements by horizontal ball bearings engaging with shock absorbing roller pins.

- Opening size: 2700mm wide x 2300mm high.

- Arrangement: Double point suspension moveable wall comprising of 3no. elements total; 2no. standard elements (VE) and closure by means of a telescopic element (TE).

- Acoustic insulation:  A minimum of Rw 53dB tested to DIN EN ISO 10140-2010 & DIN EN ISO 717-1:2013.   An independent acoustic test certificate bearing the manufacturer’s name must be provided prior to procurement.

- Sustainability: Product must be supplied with a valid ISO14025 Environmental Product Declaration.

- Element weight: Not to exceed 32kg per M²

- Element & wall post finishes: Standard Laminate from Dorma’s Classic Collection.

- Track finish: Polyester powder coated to RAL 9010.

- Element edges: K type - concealed edge profiles satin anodised aluminium.  Incorporating integral magnetic strips.

- Parking position: PL2 double point.

- Fixing void/structure:  Track fixed to structural steelbeam/concrete slab, void depth (min.150mm), subject to approval from the structural engineer.

- Other Requirements: Refer supporting main Architectural specification for additional requirements.

- [BY OTHERS] Ceiling void acoustic baffle comprising of an acoustic slab insulation with plasterboard encasement with all perimeters and penetrations foil taped, designed to give similar acoustic performance to the moveable wall. See Style drawing STY-SD-03 for further information. The design, supply and installation of any acoustic breaks to adjacent walls / floors and baffle system to the floor void (where applicable) would be undertaken by others, usually as part of the floor works / drywall package.