Part 2 – ITQ – Specification 

PART TWO
SPECIFICATION

REMOVAL, SUPPLY AND INSTALLATION OF NEW BUS SHELTERs at eskdaill street, kettering
Kettering Borough Council require the removal and replacement of an existing approximately 40m long four bay bus shelter on Eskdaill Street and undertake the installation of 3 no 5.7m long cantilever bus shelters.  
The Contractor shall remove to a depth of 150mm below current finished level and dispose of shelter and any arisings to licensed recycling facility, install 50mm ducts for electric supply to 3 no. new bus shelters, litter pick and reinstate street furniture and surface finishes to match immediate surrounding surface finishes. 
They should supply and install 3 No. 5.7m x 1.5m cantilever shelters with ½ end panels and a style that prevents spray from passing vehicles on Eskdaill Street, drainage, perched bench seating, LED lighting and compatible for real time passenger information display installed by others.  Provision should be made for an A1 Advertising board in each shelter 
Shelters and installation must complement existing bus shelters and street furniture and comply with Kettering Borough Council’s Public Realm Strategy
They must ensure that existing street furniture is retained and not damaged during work.  If any street furniture is temporarily removed it must be re-installed in the same condition as when it was removed. The Contractor will be responsible for making good any damage incurred as a result of the removal, supply and installation of the new bus shelters.  Alternative temporary street furniture and shelters must be provided from the date the existing bus shelter is removed to the date the new bus shelters are operational. 
The Contractor shall:

· comply with Project Managers reasonable instructions and the requirements of street works licences and needs of bus operators to provide alternative temporary bus stops.
· be suitably qualified for and ensure that the area is adequately fenced and or that the area is barriered off and provide appropriate signage in accordance with Chapter 8 of Traffic Signs Manual to keep members of the public outside of working area. 

· Connect and commission electrical supply to isolated feeder pillar provided by others

· Provide indicative programme for completion of works by 21st October 2020.

1

