KWL CODE: DOR10

STORM DOOR & FRAME WITH FANLIGHT, PLYWOOD OR GLAZED FANLIGHT, PRIMED, SOFTWOOD, GRADE 5TH

Frame size up to 3000mm Height x 1200mm Width

Specification of Materials

Frame

Mortice & Tenoned 45mm x 95mm PSE softwood timber with loose stop lath 22mm x 45mm, finished with water based primer

Door

22mm T&G Match Boarding

22mm x 120mm Rails & Braces

Glue and screw both sides

Use Knotting Solution were required

Fill & sand screw holes

I coat white primer