KWL CODE: DOR12

FRAMED, LEDGED & BRACED DOOR & FRAME, SOFTWOOD, GRADE 5TH

TOPLIGHT WITH PLYWOOD INFIL

PRIMED AND ASSEMBLED

Size up to 3000mm Height x 1000mm Width

Specification of Materials

Frame

Mortice & Tenoned 45mm x 95mm PSE softwood timber with loose stop lath 22mm x 45mm, finished with water based primer

Door

45mm x 95mm stiles and top rail

22mm x 95mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer