KWL CODE: DOR15

PAIR OF GARAGE DOORS, APPROXIMATELY 2180 WIDE X 2165 HIGH,

ALL PRIMED, SOFTWOOD, GRADE 5TH

Specification of materials

45mm x 145mm stiles and top rail

22mm x 145mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer