KWL CODE: DOR16

PAIR OF GARAGE DOORS AND FRAME 2180 WIDE 2165 HIGH OVERALL SIZE ALL PRIMED

Specification of materials

Frame

Mortice and Tenoned 3”x4” PSE Timber

1”x2” PSE Timber stop laths

Doors

45mm x 145mm stiles and top rail

22mm x 145mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer