KWL CODE: DOR18

BIN AND METER CUPBOARD DOOR AND FRAME, ANY SIZE, ASSEMBLED AND PRIMED, SOFTWOOD, GRADE 5TH

Specification of Materials

Frame

Morticed and tenoned

45mm x 95mm Square or bevelled timber

45mm x 12mm Rebates

Doors

44mm Solid Flush doors cut from standard blank and edged in PSE timber

Use knotting solution were required

1 coat white primer

Meter door approx. 900mm high x 600mm wide

Dustbin door approx. 1100mm high x 600mm wide