KWL CODE: DOR22

EXTERNAL PREMDOR, FD30 DOOR & FRAME, WITH OR WITHOUT SILL, PRIMED, SOFTWOOD, GRADE 5TH

Frame to be altered to fit standard door. INTUMESCENT STRIPS REBATED IN TO FRAME, FIRE RETARDANT BOARD FOR INFIL TO FANLIGHT.

Door to be fitted on 3no CE fire approved 100mm hinges in frame.

(Excluding Letterbox, handle & lock)

Door size UPTO 2400mm Height x 850mm Width

Specification of Materials

Frame

Morticed and tenoned

Hardwood sill if required

Door frame to be square or bevelled as required.

Normal section size 45mm x 95mm pse but up to 125mm

Use knotting solution were required

1 coat of white primer

Door

1 External PREMDOR FD30 Fire Door, 44mm

Frame to be rebated for 10mm/15mm smoke seals

Frame to be morticed and tenoned

Glue screw and sand

Use knotting solution were required

1 coat white primer