Septic tanks

Annual costs

Site No 	3		
Site Name: 5 The Crescent, Crazies Hill
Properties served: 1

Descriptions in this listing are abbreviated forms of the full description in paragraph 10 of Schedule 1 of the Conditions of Contract – The Services; the numbering also relates to that section.

| No | Description | Unit | Quant’y | Rate | £ | P |
| --- | --- | --- | --- | --- | --- | --- |
| 1) | Safety: Follow safety requirements specified and all other good working practices at all times. |  |  |  |  |  |
| 4 (I) (c) | Annually: Septic Tank Remove covers: (i) Check appearance of internals and ensure that all retaining fittings are present and secure. (ii) Clear any debris from the inlet and outlet pipes and other areas of the mechanism where they may cause malfunction. (ii) Check the condition of any media mats, remove and clean as necessary. (iii) Reseed the plant as necessary (seeding media to be provided at the Council’s cost) (iii) Replace covers and make secure. | Per visit | 1 |  |  |  |
| 4) (II) (c) (i) 4) (II) (c) (ii) 4) (II) (c) (iii) | Desludging: Wastewater treatment work tanks are to be emptied by tanker at the frequencies indicated in the site datasheets. Annual volume shown. Emptying includes assessing general condition of tanks as far as is practicable from the surface through the following actions: (a) Carrying out all necessary safety precautions. (b) Checking condition of covers and frames. (c) Reporting any defects found to the Contract Administrator. (d) Break up surface crust, cleaning walls, weirs and channels. (e) Cleaning up any spillages and ensuring covers are replaced and any locking devices are in place before leaving site. Following emptying Sewage Treatment Plant the plant is to be jetted, washed down and partially refilled in accordance with manufacturer’s recommendations. Tanks are to be left with sufficient media to allow natural reseeding of the plant when put back into use. | Per litre Per inspection Per clean | 3200 1 1 |  |  |  |
| 4) (II) (d) | Sampling (i) The Contractor shall undertake a programme of sampling and testing of final effluent discharged from wastewater treatment works (no sampling shall be undertaken within 48 hours of tank emptying). (ii) Samples shall be submitted for analysis to a laboratory accredited by the United Kingdom Accreditation Service (UKAS). (iii) Copies of test results shall be forwarded to the Contract Administrator within 3 working days of receipt by the Contractor. (iv) Attention should be drawn to any adverse test results or other results that may indicate a potential problem | Per lab sample Per lab sample Per lab sample As required | 1 1 1 |  |  |  |