Schedule 1 Specification

Corfe Castle West Street Specification

General Description: (Existing)

The toilet block currently consists of  ladies, gents and disabled toilet facilities with a central store room containing all cisterns and access to the foul drains. The work consists of a refurbishment of the current facilities to the following specification. Alternative options may be considered but pricing of the tender must be based on the following specification. (See ITT 2.9)

- Preparation

  - Remove all existing cubicle doors, sanitary ware, Wallgate wash units, lights and all other unnecessary material and dispose of.

  - Strip out and isolate water supplies, remove all existing water cisterns.

  - All interior walls (excluding Store Room) to be dry lined using 12.5mm plasterboard sheets

- Flooring

2.1	Install 300 x 300mm non slip dark grey floor tiles to all interior areas (excluding the store room), overlaid over the existing floor.

3.  	Walls

3.1	Affix pearl marble white gloss shower walls to all interior walls and inside   cubicles (or equivalent with agreement), excluding store room.

4. 	Ceiling

4.1	To affix white plastic wood effect boards to the ceilings with edge trims in the    ladies, gents and disabled toilets.

5. 	Doors

5.1	Replace the 5 x interior doors to cubicles, locks and interior D handles and coat  hooks. (type and finish to be agreed)

5.2 	Replace the 3 x exterior doors with suitable weather resistant exterior doors. (type and finish to be agreed)

6. 	Plumbing

6.1 	All connections are to be made to existing water feeds and waste points

6.1	Strip out and isolate water supplies

6.2	Supply and install all 1st fix plumbing as needed including isolation valves, copper pipe and all necessary materials to connect to existing drainage.

7. 	2nd Fix plumbing

Supply and install :-

7.1	6 x back to wall ceramic toilets including cisterns, sensor flush (to mains electric) and soft close seats. Cisterns to be located in the rear store room.

7.2 	5 x sensor operated taps (to mains electric) (2 X Gents, 2 X Ladies, 1X Disabled)

7.3	3 x ceramic square urinals (Gents) with sensor flush (to mains electric)

8. 	Electrical

8.1	Isolate all redundant electrical items and lights and dispose of.

Supply and install:

8.2 	10 x 15W LED down light fittings to be switched by PIR motion sensors (location to be agreed)

8.3	3  X bulkhead lamps outside gents, ladies, disabled toilets switched by PIR motion sensors.

8.4 	3 x low energy blow down hand dryers (1 per gents, ladies & disabled toilets).

9. 	Vanity units

Supply and Install :-

9.2	2 x solid surface vanity tops incorporating 2 solid surface basins in each. 1 unit located in ladies, 1 in the gents.

9.3	1 x ceramic basin in the disabled toilet

9.4	3 x baby change units. (1 per Gents, Ladies, Disabled)

10. 	Sundries

Supply and install:

10.1	3 x individual safety backed mirrors 60cm x 60cm  (1 per Gents, Ladies, Disabled)

10.2	5 x stainless steel liquid soap dispenser (1 per basin)

10.3	5 x ‘Not drinking Water’ signs above each basin

10.4	Replace existing external Gentleman, Ladies and Disabled Toilets signs.

11. 	Paint & Decorate

11.1	Undercoat and paint all interior woodwork in each block

11.2	Inspect and replace where necessary all exterior fascia boards.

11.3	Undercoat and paint all exterior woodwork, fascias etc.

11.4	Clean (or replace where necessary) all guttering and downpipes.

12.	Commissioning:

12.1	Test and commission all electrical and plumbing work to ensure all is in working order.

12.2	Clean down and wipe all windows and other interior surfaces leaving in clean condition.