Call-Off Schedule 20 (Call-Off Specification)

# PURPOSE

## The purpose of this requirement is to provide the Department for Transport (DfT) with a maintenance service for its network of approximately 350 permanent automatic traffic counters, which are located on roads of all types across the full extent of Great Britain.

# SCOPE OF REQUIREMENT

## The Potential Provider will be responsible for maintaining the DfT’s network of ATC sites to the Authority’s specification, in order that they continuously produce good quality data, and that the safety of the sites is maintained.

## The maintenance service required includes the following elements:

## Regular inspection and maintenance of the sites and all associated equipment in full working order

## Central collation and management of incoming data from the network, up to the point it is transferred to DfT.

## Prompt identification and rectification of faults or damage affecting the network or the data

## Installation, re-installation or removal of sites and equipment as required

## Management and storage of a suitable stock of equipment and spares on behalf of DfT, for use in maintaining the network

## Expert advice and support to DfT on all the above issues, including responsibility for all appropriate Health & Safety, working procedures and permissions for operations on the road network

## This Contract will commence from 26th September 2022 to provide overlap with the current maintenance Service Provider until 1 November 2022, when the appointed Service Provider will be solely responsible for managing the network. The Potential Provider must be fully operational and be able to carry out all works by 1 November 2022 and must have a maintenance recording and reporting management system in place.

## The Contract duration is three years initially with the option to extend for two further years (3+1+1) subject to satisfactory performance and continuing need. The Authority will review both the contract performance and ongoing needs for the contract midway through the third year. The Authority reserves the right to extend the contract at their discretion at the end of the third or fourth years.

# THE REQUIREMENT

## MANAGEMENT OF WORKS

## The Service Provider will be responsible for all health and safety matters and must ensure that all employees working on this contract fully comply with all schemes related to working safely on the road.

## The Potential Provider will be responsible for obtaining all relevant permissions, licences, traffic management, electrical/civil permissions required for conducting all works, and for taking all reasonable steps to ensure that any third party costs are minimised.

## The Service Provider will provide a central management function for all the services provided under this contract, and a single point of contact for the Authority.

## The remainder of this Section describes the main types of activity required under the contract.

## ROUTINE SITE INSPECTIONS (RSIs)

## It is expected that the supplier will meet their responsibilities for maintaining the network (see 5.2.1) mainly through a programme of Routine Site Inspections (RSIs), that is scheduled visits conducted at each site by a competent engineer to perform regular checks and maintenance.

## The purpose of the inspections is to ensure that sites continue to perform optimally, providing good quality data. They also provide important opportunities to identify any potential hazards, to carry out preventive repairs to any emerging issues, to improve environmental conditions at the sites and to report any changes to the local road layout that may be relevant to interpreting the data collected.

## As a minimum, the supplier must:

## Carry out inspections twice per year at every site (including those with live faults or those whose data is not being used), following a schedule to be approved by the DfT. The current schedule is shown in the site list at Annex H.

## Include in inspections the activities listed in Annex I.

## Ensure that the attending engineer produces a contemporaneous report of the RSI in a standardised format, and that this report and all supporting documentation is submitted to the Authority within one working day of completing the inspection. Annex I shows an example of the RSI report used in the current contract.

## Ensure that the attending engineer carries out any preventive works or minor repairs to the off-road equipment, if discovered during the inspection, where it is safe to do so. Common examples include repairs to the cabinet (locks and hinges), attaching signage, changing batteries or updating settings/parameters in the unit.

## The supplier may propose changes or improvements to the schedule or content of inspections, but DfT’s approval is required before these may be implemented.

## RSIs must be conducted during normal working hours and in “usual” traffic conditions, so as to give an insight into site operation in these conditions. Before conducting an inspection, the Service Provider must endeavour to identify events or road works which could disrupt traffic at a particular site, and avoid carrying out the RSI at such times.

## DATA MANAGEMENT

## The sites in the network are configured to transmit data via GPRS every 5 minutes to a central server (see Annex A for further description of the network).

## The supplier is responsible for receiving and collating this data in real time, and securely storing it on the central server, which they will provide, until the Authority has taken delivery of it (see 5.2.2).

## The supplier must provide for the Authority to be able to access the collated data on the suppliers’ server via file transfer protocol (FTP) at will (but normally at least daily, with the majority of data being transferred at 1am and 6am) and download it to the Authority’s own servers.

## The Authority requires there to be minimal change to its own data acquisition processes at the outset of the contract, therefore the existing file formats and data structures should be retained until otherwise agreed. If required, a copy of the Q-Free Communications Instation software, currently used to manage the receipt of data from the classifiers, will be provided.

## In the slightly longer term, the Authority is keen to move to a model in which the Supplier provides the data directly to a cloud hosting platform, such as Google Cloud Platform, utilising an object store, such as Cloud Storage buckets. This would allow the Authority can share a real-time view of the data with the Supplier, audit the receipt and storage of data in real-time and access the data for processing. The Authority would expect to take the lead in developing its requirements for this new model, while the Supplier would be responsible for engaging positively with the Authority and any of its partners to facilitate the successful adoption of the new model.

## FAULTS

## Any faults which arise in the network must be identified as early as possible, and resolved to get sites in good working order at the earliest instance (see para 5.2.3). Targets for responding to/resolving faults are included in the project KPIs and pricing model discussed in later sections of this document.

## Faults may be identified in various ways including:

## By the Service Provider, using monitoring software for real-time data analysis, during planned site visits/inspections or when visiting a site for another reason.

## By the Authority, carrying out checks on incoming or historic data or conducting random site visits.

## By Third parties reporting damage to sites.

## Issues that can be found during the data checks which may indicate a fault include:

## Vehicle mis-classification.

## In-road equipment producing fault indicators (known as flags).

## Reduced/low battery voltage level readings.

## Unusual changes in the size of data files being sent to the server.

## Sites failing to communicate with the server and therefore failing to send data to the server.

## Sites communicating but still failing to send data files to the server.

## When a new issue is identified in the network, the Authority must be notified of the details immediately and the Service Provider must inform the Authority of any recommendations. If an engineer is already on site, for example because the fault was identified at an RSI, the opportunity should be taken to provide more complete evidence (e.g. photographs, other contextual information). The Authority will review the details prior to any works being started, and it is up to the Authority to decide whether they wish to initiate a Job to resolve the issue. The process for initiating a Job is described below.

## AMENDMENTS TO THE NETWORK

## The Authority may require amendments to be made to the network (see para 5.2.4). Typical works in this category are described further in Annex L. For administrative purposes, any on-site works not triggered by a suspected fault are likely to be treated as an ‘amendment’, even if in a few cases (such as trials and testing) they do not result in a permanent change to the site.

## The work plan and costs relating to such works must be agreed with the Authority before they are carried out. Pricing schedule rates will apply where relevant. Any additional works falling outside of the pricing schedule must be covered by a quotation.

## Works will be managed under Job numbers raised as described below.

## INITIATING AND CARRYING OUT JOBS

## If the Authority decides to initiate a Job they will instruct the Service Provider to assign the issue a unique Job Number. This Job Number must be allocated to this fault or amendment only, from the point of initiation and for each subsequent visit until the Job has been closed down by the Authority.

## The Job initiation process and an example of the form used by the Authority to start the process is shown in Annex J.

## The works carried out during a job visit are dependent on the job type. The main types of jobs that may arise are explained in more detail in Annex K, including the timescales within which the Authority expects each kind of job to be resolved.

## A Job Visit Report must be generated at the end of each visit, including details of the issue(s), any recommendations and the actions taken to resolve it. It must include all activities, test results, observations, relevant photographs etc. In general, such reports may use the same pro-forma as the RSI report shown in Annex I, with only the sections relevant to the job visit completed. The report and all supporting documentation must be submitted to the Authority within one working day of completing the inspection.

## Occasionally a fault is suspected but during the initial site visit no issue is found. The Authority will pay for all completed visits which have been properly initiated and implemented.

## The Authority may at any time instruct the Service Provider not to attend a particular site(s) or may require a job to be cancelled.

## At the start of the contract, the new Service Provider must conduct an initial visit (‘administrative’ visit) to all open, unresolved jobs carried over from the previous contract. This is to allow the new Service Provider to assess the outstanding works required to resolve the issue and to continue the works to rectify the fault.

## SPARES AND STORAGE FACILITIES

## The Service Provider is responsible for managing and maintaining the stock of equipment and spares owned by the Authority for use in the management of the ATC sites (see para 5.2.5).

## The Service Provider must ensure that there are sufficient spares available to fully meet the maintenance requirement and must inform the Authority if stock needs replenishing. A list of the kinds of materials replenished by the Service Provider in the current contract can be seen in Annex Q. Equipment will be ordered by the Service Provider from reputable suppliers at a competitive price and charged onto the Authority at cost price plus a contract mark-up which will be stated as a part of your tender (See Attachment 4 – Pricing Schedule, Table F). The Service Provider must submit a quote and the equipment must be approved before the order is placed. The Authority have the right to refuse before purchase. When billing the Authority proof of the cost price must be supplied with the relevant invoice.

## A quantity of the Authority’s stock will be made available at the start of the contract, which must be collected from the Authority’s current storage facility. Annex O shows the quantity of stock in the Authority’s storage facility at the time of the last stocktake in summer 2022. The Service Provider must provide a secure and weatherproof indoor storage facility to hold the Authority’s stock.

## A stock management system must be put in place containing an itemised list of all equipment held for the Authority, and should include the items description, location, serial number and type. This includes all equipment in the stockroom, at the sites or being carried as spares by the engineers. All equipment must be made available to the Authority on request. In the current contract, the Authority’s stock is barcoded to produce an electronic stocklist, which will be made available at the start of the contract and supplied in Microsoft Office Excel format.

## The Authority will conduct an audit of the stock once a year to review all stocklists and the Service Provider must provide support on this.

## The Service Provider must have facilities in place to store rechargeable 6V and 12V batteries. Batteries must be clearly labelled with the number of times they have been fully recharged. They must be recharged in a dedicated, well ventilated area which is compliant with relevant health and safety procedures and battery manufacturer’s recommendations. When a battery can no longer be used, it must be disposed of using the correct methods.

## Any equipment no longer required at a site must be removed carefully and returned to the Authority’s storage facility to enable it to be reused in the future. Items returned as faulty or with a suspected fault, will be labelled and stored separately from other spare items. Defective units will be removed from the site and returned to the original manufacturer for repairs. All items must be repaired, replaced or with the Authority’s approval disposed of using the correct methods. The equipment must be checked to ensure that it is working well before it is re-installed. The re-use and repair of equipment should be managed so as to obtain best value for money for the Authority.

## TRAFFIC MANAGEMENT

## The Service Provider is responsible for arranging all traffic management that may be needed in the course of works required under this contract, ensuring that it complies with the requirements of all current and relevant legislation.

## The Service Provider must ensure that sufficient traffic management time is booked to complete the works. There may be occasions when in-road work cannot be completed at a location in one day. On these occasions, traffic management should be arranged on consecutive days as one session, to minimise delays in bringing the site back into working order.

## The Authority will pay traffic management fees at cost price. The invoice must be submitted to the Authority for approval together with documentation and details of traffic management costs. The administrative costs to the contractor of arranging for traffic management should be included within their fixed prices for site installations (see Attachment 4 - Pricing Schedule, Table B), but for any other work involving requiring traffic management they may include a fixed arrangement fee in their quotation (see Attachment 4 - Pricing Schedule, Table C).

## For a limited range of circumstances compensation may be available for traffic management bookings which have to be abandoned or rescheduled for reasons outside the Service Provider’s control. For all claims, the Supplier must provide documentary evidence upon which the Authority can base any decisions regarding any compensation. However, the number of authorised compensation payments during the contract is expected to be low. The Authority will not consider compensation if the circumstances could have been reasonably anticipated.

## COSTS FOR REPAIRING DAMAGE BY THIRD PARTIES

## If accidental damage has been caused to the network by a third party – the most common example being a highway authority contractor resurfacing the road - the Service Provider must take reasonable steps to claim the cost of rectifying the damage from the third party. If the Service Provider has successfully claimed the full costs or partial costs from the third party, full documentation of the payment agreement must be provided to the Authority for audit purposes. However, if unsuccessful, the Authority must be satisfied that the reasonable steps have been taken to reclaim the costs, after which the Service Provider may bill the Authority as per the pricing schedule.

## ELECTRICAL SAFETY TESTING

## The Service Provider must ensure that electrical safety inspections are carried out at all mains powered sites by a qualified, competent electrician, registered with the National Association of Professional Inspectors and Testers (NAPIT). This must be carried out every 5 years to ensure compliance with the relevant health and safety legislation. The Service Provider is responsible for arranging and completing the next batch of tests due in 2026. A plan for carrying out the inspection and a quotation must be agreed with the Authority prior to the works being carried out. The Service Provider is also responsible for ensuring the electrical testing plan remains compliant with any future changes in relevant standards. Note that only around 30 sites are currently mains connected.

# KEY MILESTONES AND DELIVERABLES

## The following Contract milestones/deliverables shall apply:

| ### Milestone/ Deliverable | ### Description | ### Timeframe or Delivery Date |
| --- | --- | --- |
| ### 1 - Traffic data | ### Continuous delivery of the traffic data on a vehicle by vehicle level including vehicle classifications, speeds, axle configurations, cycle data etc. ### Exact details of data transfers between site, contractor and DfT to be agreed with DfT on award of the contract, but all potential providers must be able to receive, store and process data from the sites on a secure, regularly backed up server, and provide DfT with access to the data from this server. ### Further info: 6.10-6.14 | Within 4 weeks of contract award. ### Good quality data must be delivered to the data server every 5 minutes from all operational sites in the network. |
| ### 2 - RSI Report | ### A report generated with every inspection. It will contain an accurate record of the activities completed and results generated during the inspection. ### Format: PDF unless otherwise agreed. ### Further info: 6.5-6.9, Annexes I & J: | Within 3 weeks of contract award, then with every inspection. Must be provided to DfT with all supporting documentation within 1 working day of completing the inspection. |
| ### 3 – Job Visit Report | ### A report generated with every job visit. It will contain an accurate record of the activities carried out during the visit. ### Format: PDF unless otherwise agreed. ### Further info: 6.25 | Within 3 weeks of contract award, then with every job visit. Must be provided to DfT with all supporting documentation within 1 working day of completing the visit. |
| ### 4a - Weekly Job Status Report | ### A progress summary of all jobs completed or remaining open in the week. ### Format: PDF unless otherwise agreed. ### Further info: Annex N | Within 3 weeks of contract award, then weekly. ### Must be provided to DfT each Friday by 5pm. |
| ### 4b - Cumulative Job Status Report | ### A machine-readable cumulative summary of all jobs, job visits and RSIs raised, from the beginning of the contract to the end of the current week. ### Format: Microsoft Excel unless otherwise agreed. ### Further info: Annex N | Within 3 weeks of contract award, then weekly. ### Must be provided to DfT each Friday by 5pm. |
| ### 4c – Cumulative RSI Status Report | ### A machine-readable cumulative summary of all RSI visits, from the beginning of the contract to the end of the current week. ### Format: Microsoft Excel unless otherwise agreed. ### Further info: Annex N | Within 3 weeks of contract award, then weekly. Must be provided to DfT each Friday by 5pm. |
| ### 5 – Monthly Report | ### A cumulative summary of network condition and maintenance activity from the start of the contract until the end of the month reported on. ### Format: PDF unless otherwise agreed. ### Further info: Annex N | Within 4 weeks of contract award, then monthly. ### Must be provided within 3 working days of the end of the month reported on. |

# MANAGEMENT INFORMATION/REPORTING

## Documentation relating to maintenance of the network will be agreed at the start of the contract by the Authority. All engineering and maintenance works must be carried out in accordance with these procedures. Any deviation from these procedures must be discussed and agreed with the Authority before being implemented.

## The Service Provider must keep all documentation in up-to-date condition and in good order throughout the contract as the Authority may require access to it at any time, and it must be returned to the Authority at the end of the contract.

## The regular reports that the Authority requires are summarised in the Deliverable table in Section 7 above, with further details in Annex N. These set out minimum requirements for reporting. The Authority is willing to discuss the specification of any management or reporting tools which may provide enhanced reporting. A requirement for any enhanced processes is that they preserve the Authority’s ability to readily access information meeting at least the minimum reporting requirements independently of any software or management tools used, both during and after the contract.

# VOLUMES

## For convenience, this section summarises some of the key characteristics of the requirement in terms of volumes and resources required. All of these points are discussed in more detail elsewhere in this document.

## The contract requires the supplier to maintain approximately 350 sites at 300 locations throughout Great Britain.

## They must carry out routine inspections twice a year at every site, using engineers suitably trained for the task and familiar with the equipment and DfT requirements.

## In addition, engineers must attend and where possible repair any faults within 2-3 working days. The number and type of faults likely to arise during this contract is not readily predictable, but as a guide, between 1 Nov 2017 and 22 June 2022, 1,075 jobs relating to faults or suspected faults were raised.  In addition, a further 300 jobs were raised in connection with site amendments, of which approximately 70 were for new site installations. Further details are given in Annex M.

## The supplier must be able to provide a specialist installation / commissioning / decommissioning service tailored to the specific equipment used in the network, anywhere in Great Britain. Such services should be available for scheduling at short notice and without undue delay.

## The supplier must maintain and store a stock of equipment and spares in order to maintain the network in good condition without delays due to lack of parts, or their delivery to where needed.

# CONTINUOUS IMPROVEMENT

## The Supplier will be expected to continually improve the way in which the required Services are to be delivered throughout the Contract duration.

## The Supplier should present new ways of working to the Authority during the monthly contract review meetings.

## Changes to the way in which the Services are to be delivered must be brought to the Authority’s attention and agreed prior to any changes being implemented.

## The Service Provider must support the Authority in carrying out development and improvement works in the network. The Authority’s development priorities currently include:

## To move to a Cloud-based model for data transfer (see para 6.14).

## To carry out trials to assess new and improved classification algorithms for the ATC classifiers on the network.

## To carry out field trials of alternative traffic counting methodologies, including comparisons with the existing ATC equipment. These alternative methods may be intended for either permanent or temporary deployment.

## To improve the ability of the network to capture data on pedal cycles and similar vehicles.

# SUSTAINABILITY

## The Service Provider and any subcontractors should have regard to sustainability in planning all aspects of their work, for example when planning travel, purchasing equipment and carrying out installation or decommissioning work.

# QUALITY

## A minimum requirement is that the Service Provider shall be quality assured to National Highway Sector Scheme 8 for the types of work required in this contract, for the duration of this contract.

## The Service Provider, subcontractors or any other contractors used shall comply with the Highways Electrical Registration Scheme (HERS). In particular the staff carrying out works on site shall be registered to and must successfully complete mandatory training under the relevant areas of the scheme. Staff must carry valid registration cards as this provides evidence that the person has obtained the required training and skills required the types of works they are carrying out. The Service Providers must supply evidence of compliance with the scheme and provide evidence of passed the HERS audit.

## EQUIPMENT MANAGEMENT

## The Service Provider is responsible for ensuring their engineers are equipped with the equipment, tools and materials necessary for completing works. The equipment should not just relate to the works that they are attending, but the engineer must be prepared for any situation they may face such as maintenance, measurements and setting up sites. Any fault found unrepaired as a result of the Service Provider failing to carry sufficient equipment will be resolved by the Service Provider at no cost to the Authority. Any additional costs relating to damage or theft of any equipment are to be met by the Contractor.

## The engineers must also carry equipment to the sites that enables them to:

- Connect to the classifier

- Communicate with the Contractor remotely

## The Authority will supply the Service Provider with copies of any software required to connect to the Authority’s classifiers to check data quality at the sites.

## All equipment must be maintained and kept in good working order by the Service Provider. Tools used to take measurements directly from the site must undergo regular calibration under current standards and must be organised and paid for by the Service Provider. The Authority reserves the right to request documentation from the calibration tests at any time during the contract. Annex O shows an example of the specialist tools the engineers carry in the current contract.

## The Service Provider will ensure that engineers working on the contract have access to vehicles that full comply with the relevant requirements and markings set out in the Traffic Signs Manual, Chapter 8 (part 2, section 5).

# STAFF AND CUSTOMER SERVICE

## The Supplier shall provide a sufficient level of resource throughout the duration of the Contract in order to consistently deliver a quality service.

## The Supplier’s staff assigned to the Contract shall have the relevant qualifications, experience and competence to deliver the Contract to the required standard. Detail of on-going training procedures and performance management for personnel used on this contract should be made available to the Authority.

## The Supplier shall ensure that staff understand the Authority’s vision and objectives and will provide excellent customer service to the Authority throughout the duration of the Contract. They must ensure they are well informed on the current status of the network at all times and be able to advise the Authority in a timely and responsive manner.

## The appointed Service Provider will work very closely with the Authority’s Traffic Surveys team and is likely to be in day to day contact. The Service Provider must be available to respond promptly and effectively to any issues and enquiries from the DfT’s Traffic Surveys team on Monday-Friday from 9am-5pm. The primary method of communication with the Authority will be telephone, e-mail and virtual meetings.

## The Supplier must be contactable out of usual office working hours so that any faults or damage considered to be potentially dangerous or high risk can be attended by the Supplier as soon as possible and at most within 8 hours of when the Supplier is made aware of the issue to make the site safe. If necessary, the Supplier must remain at the site and notify the relevant authority to make the site safe.

## To achieve successful delivery of this contract, the engineering team must be flexible and able to provide an effective service out of hours and in emergencies. Due to the nature of this work, access to some road space can only be granted at nights or weekends. There will be no separate charge for working unsociable hours, however this should be factored into the pricing schedule and engineer’s rates. Night works are likely to occur when resolving faults and installing new sites.

## The Service Provider must support the DfT Traffic Surveys Team to ensure that their staff can, if required, maintain compliance with HERS by providing assisting with advice, training and training records.

## At the start of the contract, a training programme may be agreed with the Authority to facilitate the handover of work from the current contract. The Service Provider will cover the costs of their staff to attend training.

## TUPE - The Potential Provider should consider the Transfer of Undertakings Protection of Employment (TUPE). The employees who would fall under this category are detailed in Attachment 6 of this bid pack. Any further information that is required regarding TUPE can be provided by way of clarification questions.

# SERVICE LEVELS AND PERFORMANCE

## The Service Provider’s performance will be monitored by the Authority’s contract management team in various ways, which include but are not limited to:

### Key performance indicators (KPIs) specified by the Authority. These are described in Annex R. The Service Provider is responsible for collating and analysing the performance data for these.

### Monthly reports – see Section 7 & Annex N. These will include information on all activities from the previous month, as well as the latest KPIs.

### Monthly contract meeting - see Section 18.

### Ad-hoc site visits or shadowing engineers - the Authority may conduct site visit to check performance, quality of work or health and safety related matters.

### Reviewing reports and other documentation - all reports submitted will be reviewed by the Authority to ensure sufficient information has been provided, which is of an acceptable standard.

## The Service Provider will have the opportunity to provide further evidence of performance at monthly contract meetings should they not agree with the contract management team’s assessment.

## Poor performance or non-performance will be regularly reviewed by the Authority’s contract management team and could lead to an improvement plan being required to urgently address such poor performance. Should such an improvement plan not result in satisfactory improvement then the Authority could seek to exercise its rights to terminate the contract as set out in Framework Core terms Section 10, ‘Ending a Contract or any Sub-contract’.

## Any fault attended more than two days from the time when it was initiated by the Authority, or any Job which takes more than the expected number of visits to resolve (see annex K), may only be billed at two thirds of the normal rate.

## The performance arrangements and scoring will be further discussed in the initial contract meeting at the start of the contract.

# SECURITY AND CONFIDENTIALITY REQUIREMENTS

## Special attention should be given to any security issues regarding engineer’s safety whilst working at the site with valuable equipment. The contractor must have the relevant lone working procedures in place to ensure staff are safe. Any documentation relating to security must be provided to DfT as part of the tender along with evidence to demonstrate that staff are up-to-date with the latest security procedures.

## The traffic data collected by the counter network do not currently include any personal information and are not sensitive. However, in order to comply with the statistics code of practice principles of orderly release of data, the supplier must not supply any data to any third party without the prior authority of the DfT. Similarly, requests for information about the operation or specification of the network should be referred to the DfT, except for straightforward operational matters involving the highway authority or the maintenance of public safety.

## The traffic data collated from the counter network must be securely stored, with appropriate backup and business continuity plans in operation.

## The DfT’s stock of equipment must be stored securely in a suitable weatherproof environment.

# PAYMENT AND INVOICING

## Invoices shall be submitted following an invoice schedule agreed. The Service Provider will invoice all scheduled works (RSIs) monthly in arrears. Other works will normally be invoiced individually after completion of the work and its being accepted by the Authority. All invoices must be submitted for payment, no later than two weeks after the period to which the payment relates or the works being cleared by the Authority.

## Payment can only be made following satisfactory delivery of pre-agreed certified products and deliverables.

## Before payment can be considered, each invoice must include a detailed elemental breakdown of work completed and the associated costs. Draft invoices should be sent to and agreed with the Authority prior to the submission of actual invoices. All supporting documentation must accompany the invoice. This should include, but is not limited to, any cost price invoices related to the fault/work. The Authority will provide the Service Provider with the appropriate contact email address at the start of the Contract.

## The Authority will check the invoice against the pricing schedule agreed at the start of the contract and/or quotations previously agreed with the Authority, therefore the invoice must be itemised and contain all relevant information. Additional data checks will also be carried out on any works that could directly affect the site(s) operation to confirm good data quality is being produced.

## The Authority will inform the Service Provider when they are content for the invoice to be paid. At this point, invoices should be submitted to DfT’s Shared Services Centre:

## Department for Transport,

## Accounts Payable Team,

## 5 Sandringham Park,

## Swansea Vale,

## SA7 0EA

# CONTRACT MANAGEMENT

## An initial meeting will be held following the award of the contract at the Authority’s offices. The purpose of the meeting will be to discuss matters such as the working arrangements, the period of overlap and to discuss the contract more broadly.

## During the lifetime of the contract, it is expected that key supplier staff will attend a monthly formal contract meeting with DfT. The purpose of this meeting will to be to review KPIs and progress in priority areas, to identify and discuss  emerging issues and to build and maintain understanding between the supplier and DfT. It is expected that some of these meetings will be held virtually and some face-to-face.

## The Authority may wish to be represented at any event/briefing/training sessions, in particular to assist with issues relating to data collection activities, maintenance of the network and also to provide background information on the contract. The Service Provider must be able to supply support on this.

## At the end of the contract, the Service Provider will provide to the Authority:

## A final report summarising all work carried out during the contract, details of any outstanding works to be carried over to the new contract and details of lessons learned.

## Any equipment and stock which is to be returned to the Authority,

## Copies of any software with all necessary licences required for keeping the network working,

## Working versions/source code of any applications developed during the period of the contract,

## Copies of any training records or documentation relating to the Authority’s staff

## At the end of this contract there may be a period of overlap with the newly appointed Service Provider. The Service Provider is to ensure that there is a smooth transition between Service Providers to minimise disruption to the network.

## Attendance at the above meetings shall be at the Supplier’s own expense.

# LOCATION

## The Services will be carried out within the UK.

## Details of current ATC network locations are given at Annexes D and H.

## Annexes

## ANNEX A 	Description of DfT ATC network & sites

## ANNEX B 	Map of DfT ATC sites

## ANNEX C 	Photographs of typical ATC sites

## ANNEX D 	Summary of the ATC network

## ANNEX E 	Summary of the equipment in the network

## ANNEX F 	Technical drawings of sensor configurations

## ANNEX G	Vehicle categories for the ‘national’ core census

## ANNEX H 	Site list

## ANNEX I 	Routine Site Inspection (RSI)

## ANNEX J 	Example of an RSI report

## ANNEX K	Initiating a job

## ANNEX L 	Job types: faults and amendments

## ANNEX M 	Summary of jobs raised during 2017-2022 contract

## ANNEX N 	Minimum weekly & monthly reporting requirements

## ANNEX O 	Example of engineer’s equipment

## ANNEX P 	Example of DfT’s stock held at the DfT’s storage facility

## ANNEX Q 	Example of DfT’s spares replenished by the Contractor

## ANNEX R 	Initial KPIs and Performance Measures

## ANNEX A 	Description of DfT ATC network & sites

## Sites are located on all road types across Great Britain. The distribution can be seen in the site map in Annex B, with more details in Annexes D, E and H.

## The Authority owns all of the equipment currently installed or in stock.

## The data collected at each site contains characteristics of individual passing vehicles and can be classified into various types, to at least 20 categories shown in Annex G.

## All sites have GPRS enabled SIM cards installed to allow 2-way data transfer. Sites connect to the internet via a modem using General Packet Radio Service (GPRS) and upload the collected data and diagnostic information to a central data server every five minutes. (The central data server is to be provided by the supplier (see Section 6, paras 6.10-6.14).)

## Each ATC site typically consists of the following:

## Features of an ATC site

### Roadside cabinet - To store the classifier and other equipment associated with the site.

## Classifier - Connected to the in-road equipment used to collect and process data. Our current classifiers are manufactured by Q-Free and the network is almost entirely standardised on the HI-TRAC® EMU3 classifier, supplemented with a handful of HI-TRAC® 100+, HI-TRAC® CMU and HI-TRAC® UTC-WIM. ## Technical specifications for these units can be viewed in the link https://www.q-free.com/products-solutions/. A summary of the numbers of units deployed in the network can be seen in annex E.

## GSM/GPRS modem and antenna - To enable GPRS connection for data communication to a data server. The sites transmit data

## Inductive loops and piezoelectric sensors - One inductive loop per lane, which is wound in two halves, shaped similar to two “d’s” back to back. This double “d” loop configuration is installed together with two piezoelectric sensors which are installed 1m apart. This configuration is unique to the Authority. ## Currently there are four sites which have a different configuration consisting of one inductive loop per lane, wound in a square shape positioned between two piezoelectric sensors, installed 1m either side of the loop (piezo-loop-piezo configuration). ## Technical drawings of both configurations are included at Annex F.

## Piezoelectric cycle sensors - Located at most non-motorway sites, may be fitted in one or both of the near side lanes and/or on a designated cycle path outside of the road and/or on a footpath (where cycling may be illegal but still occur). See Annexes E and H for more detail on sites with cycle sensors.

## Power source: Almost all sites are now solar powered, using one or more solar panels fixed to the cabinet, connected to one or more batteries (usually 6V, some 12V). ## At around 30 sites the former mains power supply remains in place, although it is not being used to power the classifier at all of them. Where such sites are on motorways, the supply will be fed from the intermediate 609 cabinets in or on the fence line.

### Haldo type pillars - Used as a point of supply/isolation point for the electrical supply at mains powered sites. At some solar powered sites the charge regulators are also fitted within the pillar. Since the majority of current solar powered sites were formerly mains powered, many will still have Haldo pillars, even though the supply from the pillar to the cabinet has been removed.

## An earth spike and associated chamber - A safety feature in the cabinet.

## A heater and thermostat - Located at mains powered sites only, to reduce moisture in the cabinet.

## Cable looms/leads - These interconnect between the in-road equipment and the classifier. Also connected between the classifier and other equipment in the cabinet (e.g. battery, modem).

### Ducting, ducting chambers and cabling - These connect between the in-road piezoelectric sensors/inductive loops and the roadside cabinet. At motorway sites there is also ducting and cabling between the roadside cabinet and the 609 cabinets.

### Dual carriageway sites – at many dual carriageway locations two separate sites are installed, each covering one carriageway. These can be identified in the site lists by their 4-digit site codes, the first 3 digits being a common ‘location’ code, the last digit being either 1,2,3 or 4 for northbound, eastbound, southbound or westbound carriageways.

## Trial/Test Sites – at four locations standard ATC sites are co-located with test sites. The test sites are similar to the standard sites and are required to be maintained in the same way. They may cover only one lane or have different equipment installed. They can be identified in the site list by their 5-digit Site Numbers, with a ‘4’ or ‘40’ prefixing the number of the site they are co-located with.

## ANNEX B 	Map of DfT ATC sites

## ANNEX C 	Photographs of typical ATC sites

Site general view (double d loops)	Roadside cabinet

In-road configuration (double d)	In-road configuration (piezo-loop-piezo)

## ANNEX D 	Summary of the ATC network

Note: The information contained in these table is correct as of June 2022. The network is subject to change.

| Road Type | England | Scotland | Wales | Grand Total |
| --- | --- | --- | --- | --- |
| Motorway | 56 | 3 | 2 | 61 |
| Trunk A | 41 | 12 | 2 | 55 |
| Principal A | 111 | 7 | 2 | 120 |
| Minor | 102 | 6 | 6 | 114 |
| B | 43 | 3 | 1 | 47 |
| C | 27 | - | - | 27 |
| U | 32 | 3 | 5 | 40 |
| Grand Total | 310 | 28 | 12 | 350 |

| Site Location by Country, Region and Local Authority | Road Type |  |  |  |  | Grand Total |
| --- | --- | --- | --- | --- | --- | --- |
|  | M | A | B | C | U |  |
| England | 56 | 152 | 43 | 27 | 32 | 310 |
| East Midlands | 6 | 17 | 2 | 3 | 2 | 30 |
| City of Derby | - | - | - | 1 | - | 2 |
| City of Nottingham | - | 1 | - | - | - | 1 |
| Derbyshire | 2 | 3 | - | - | - | 6 |
| Leicestershire | 2 | 2 | - | 1 | - | 6 |
| Lincolnshire | - | 2 | - | 1 | - | 3 |
| Northamptonshire | - | 1 | - | - | 2 | 3 |
| Nottinghamshire | 2 | 2 | 1 | - | 1 | 9 |
| East of England | 4 | 17 | 4 | 3 | 3 | 31 |
| Bedford | - | - | - | 1 | - | 1 |
| Cambridgeshire | 2 | 5 | 2 | - | 1 | 10 |
| Essex | 2 | 2 | - | 1 | - | 5 |
| Hertfordshire | - | 1 | 1 | - | 1 | 3 |
| Norfolk | - | 4 | - | - | 1 | 5 |
| Southend-on-Sea | - | 2 | - | - | - | 2 |
| Suffolk | - | 3 | - | 1 | - | 4 |
| Thurrock | - | - | 1 | - | - | 1 |
| London | 2 | 27 | 16 | 9 | 6 | 60 |
| Bromley | - | 2 | 1 | - | 2 | 5 |
| City of Westminster | - | 2 | - | - | - | 2 |
| Croydon | - | - | 1 | - | - | 1 |
| Ealing | - | 1 | 3 | - | - | 4 |
| Enfield | - | 2 | 1 | - | - | 3 |
| Greenwich | - | - | 1 | - | - | 1 |
| Haringey | - | 1 | - | 2 | - | 3 |
| Harrow | - | 1 | - | - | - | 1 |
| Havering | 2 | 2 | 1 | - | - | 5 |
| Hillingdon | - | 2 | 2 | 1 | 1 | 6 |
| Hounslow | - | 1 | - | - | - | 1 |
| Islington | - | 2 | - | 1 | - | 3 |
| Kensington and Chelsea | - | - | 1 | 1 | - | 2 |
| Kingston upon Thames | - | 1 | - | - | - | 1 |
| Lambeth | - | 1 | - | - | - | 1 |
| Lewisham | - | 1 | - | 1 | 1 | 3 |
| Merton | - | - | - | 2 | - | 2 |
| Newham | - | 1 | 1 | - | - | 2 |
| Richmond upon Thames | - | 2 | - | - | - | 2 |
| Southwark | - | 1 | 1 | - | - | 2 |
| Sutton | - | 2 | 1 | - | 1 | 4 |
| Tower Hamlets | - | - | - | 1 | 1 | 2 |
| Waltham Forest | - | 1 | - | - | - | 1 |
| Wandsworth | - | 2 | 1 | - | - | 3 |
| North East | 2 | 6 | 1 | 1 | 2 | 12 |
| Durham | 2 | 2 | - | - | - | 4 |
| Gateshead | - | 1 | - | - | - | 1 |
| Middlesbrough | - | - | - | 1 | - | 1 |
| Newcastle upon Tyne | - | - | - | - | 1 | 1 |
| Northumberland | - | 1 | - | - | 1 | 2 |
| Stockton-on-Tees | - | 2 | - | - | - | 2 |
| Sunderland | - | - | 1 | - | - | 1 |
| North West | 10 | 20 | 4 | 2 | 4 | 40 |
| Blackburn with Darwen | - | 1 | - | - | - | 1 |
| Bury | - | 1 | - | - | - | 1 |
| Cheshire East | 1 | - | - | - | - | 1 |
| Cheshire West and Chester | 3 | 2 | - | - | - | 5 |
| Cumbria | 2 | 5 | - | 1 | - | 8 |
| Lancashire | 2 | 3 | 2 | 1 | 1 | 9 |
| Manchester | - | - | - | - | 1 | 1 |
| Rochdale | - | - | 1 | - | - | 1 |
| Sefton | - | 3 | - | - | - | 3 |
| St. Helens | 2 | - | - | - | 1 | 3 |
| Stockport | - | 1 | - | - | - | 1 |
| Tameside | - | 1 | - | - | - | 1 |
| Wigan | - | - | 1 | - | - | 1 |
| Wirral | - | 1 | - | - | - | 1 |
| Warrington | - | 2 | - | - | - | 2 |
| South East | 15 | 19 | 9 | 2 | 3 | 48 |
| Buckinghamshire | 2 | 1 | - | - | - | 3 |
| City of Southampton | 1 | - | - | - | - | 1 |
| East Sussex | - | 3 | 1 | - | - | 4 |
| Hampshire | 3 | 3 | 2 | 1 | - | 9 |
| Isle of Wight | - | 1 | - | - | - | 1 |
| Kent County | 2 | 1 | 1 | 1 | - | 5 |
| Milton Keynes | - | 1 | 1 | - | 2 | 4 |
| Oxfordshire | 2 | 2 | 1 | - | - | 5 |
| Reading | - | 1 | - | - | - | 1 |
| Surrey | 2 | 3 | 1 | - | 1 | 7 |
| West Berkshire | 3 | - | 1 | - | - | 4 |
| West Sussex | - | - | 1 | - | - | 1 |
| Windsor and Maidenhead | - | 1 | - | - | - | 1 |
| Wokingham | - | 1 | - | - | - | 1 |
| Worthing | - | 1 | - | - | - | 1 |
| South West | 4 | 17 | 3 | 4 | 5 | 33 |
| Bath and North East Somerset | - | 1 | - | - | - | 1 |
| City of Plymouth | - | 2 | - | 1 | - | 3 |
| Cornwall | - | 2 | - | 2 | - | 4 |
| Devon | - | 2 | 1 | - | 1 | 4 |
| Dorset | - | 2 | 1 | - | - | 3 |
| Gloucestershire | 4 | - | 1 | - | - | 5 |
| North Somerset | - | - | - | - | 3 | 3 |
| Poole | - | 1 | - | - | - | 1 |
| Somerset | - | 4 | - | - | 1 | 5 |
| South Gloucestershire | - | 1 | - | - | - | 1 |
| Wiltshire | - | 2 | - | 1 | - | 3 |
| West Midlands | 6 | 15 | 2 | 2 | 4 | 29 |
| Birmingham | - | 1 | - | - | 1 | 2 |
| Herefordshire | - | 2 | - | - | - | 2 |
| Coventry | 2 | - | - | - | - | 2 |
| Dudley | - | 2 | - | - | - | 2 |
| Sandwell | - | - | 1 | - | 2 | 3 |
| Shropshire | - | 2 | - | 1 | - | 3 |
| Solihull | - | 1 | - | - | 1 | 2 |
| Staffordshire | - | 3 | 1 | 1 | - | 5 |
| Warwickshire | 2 | 3 | - | - | - | 5 |
| Worcestershire | 2 | 1 | - | - | - | 3 |
| Yorkshire and The Humber | 7 | 14 | 2 | 1 | 3 | 27 |
| Barnsley | - | 1 | - | - | - | 1 |
| Bradford | - | 2 | 1 | - | - | 3 |
| Doncaster | 1 | - | - | - | 1 | 2 |
| East Riding of Yorkshire | - | 1 | - | - | - | 1 |
| Hull | - | 1 | - | - | - | 1 |
| Kirklees | - | 1 | - | - | - | 1 |
| Leeds | - | 3 | 1 | - | - | 4 |
| North Lincolnshire | 2 | - | - | - | - | 2 |
| North Yorkshire | - | 4 | - | 1 | 2 | 7 |
| Rotherham | - | 1 | - | - | - | 1 |
| Wakefield | 2 | 1 | - | - | - | 3 |
| Scotland | 3 | 19 | 3 | - | 3 | 28 |
| Aberdeenshire | - | 1 | - | - | 2 | 3 |
| Dumfries and Galloway | - | 2 | 1 | - | - | 3 |
| Dundee City | - | 1 | - | - | - | 1 |
| East Ayrshire | - | 1 | - | - | - | 1 |
| East Lothian | - | - | 1 | - | - | 1 |
| Falkirk | - | - | 1 | - | - | 1 |
| Fife | 1 | 2 | - | - | - | 3 |
| Glasgow City | - | 1 | - | - | - | 1 |
| Highland | - | 4 | - | - | - | 4 |
| Midlothian | - | 1 | - | - | - | 1 |
| Moray | - | 2 | - | - | - | 2 |
| North Lanarkshire | - | 1 | - | - | - | 1 |
| Renfrewshire | - | 1 | - | - | - | 1 |
| Scottish Borders | - | 1 | - | - | - | 1 |
| South Lanarkshire | - | - | - | - | 1 | 1 |
| Stirling | 1 | 1 | - | - | - | 2 |
| West Lothian | 1 | - | - | - | - | 1 |
| Wales | 2 | 4 | 1 | - | 5 | 12 |
| Abertawe - Swansea | - | - | 1 | - | - | 1 |
| Caerffili - Caerphilly | - | - | - | - | 1 | 1 |
| Gwynedd - Gwynedd | - | - | - | - | 1 | 1 |
| Merthyr Tudful - Merthyr Tydfil | - | 1 | - | - | - | 1 |
| Pen-y-bont ar Ogwr - Bridgend | 2 | - | - | - | 1 | 3 |
| Powys - Powys | - | 1 | - | - | - | 1 |
| Sir Benfro - Pembrokeshire | - | 1 | - | - | - | 1 |
| Sir Ceredigion - Ceredigion | - | 1 | - | - | - | 1 |
| Sir Gaerfyrddin - Carmarthenshire | - | - | - | - | 1 | 1 |
| Sir Ynys Mon - Isle of Anglesey | - | - | - | - | 1 | 1 |
| Grand Total | 61 | 175 | 47 | 27 | 40 | 350 |

## ANNEX E 	Summary of the equipment in the network

Note: The information contained in these table is correct as of June 2022. The network is subject to change.

| Cycle Lane(s) Installed at the ATC Sites | England | Scotland | Wales | Grand Total |
| --- | --- | --- | --- | --- |
| Yes | 165 | 6 | 5 | 176 |
| Cycle sensors only | 1 | - | - | 1 |
| No | 144 | 22 | 7 | 173 |
| Grand Total | 310 | 28 | 12 | 350 |

| Power Source at the ATC Sites | England | Scotland | Wales | Grand Total |
| --- | --- | --- | --- | --- |
| 12V Battery - Wind/Solar | 30 | 4 | 1 | 35 |
| 6V Battery - Solar | 259 | 24 | 7 | 290 |
| Mains Powered | 21 | - | 4 | 25 |
| Grand Total | 310 | 28 | 12 | 350 |

| Classifiers installed at the ATC Sites | England | Scotland | Wales | Grand Total |
| --- | --- | --- | --- | --- |
| HI-TRAC® EMU3 | 300 | 27 | 12 | 339 |
| HI-TRAC® 100+ | 4 | - | - | 4 |
| UTC-WIM | - | 1 | - | 1 |
| HI-TRAC® CMU | 1 | - | - | 1 |
| No unit present | 5 | - | - | 5 |
| Grand Total | 310 | 28 | 12 | 350 |

## ANNEX F 	Technical drawings of sensor configurations

## Drawing 2: Piezo - loop – piezo array (2 lanes), also showing slot cross-sections

## ANNEX G	Vehicle categories for the ‘national’ core census

## ANNEX H 	Site list

Note: The information contained in these table is correct as of June 2022. The network is subject to change.

| Site Number | Road Name | Road Number | LHA Name | Region | OSGRX | OSGRY | Vehicles & Cycles | RSI month 1 | RSI month 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 272 | M9 | M9 | Stirling | Scotland | 281771 | 688062 | Vehicles Only | January | July |
| 273 | M8 | M8 | West Lothian | Scotland | 293634 | 665447 | Vehicles Only | January | July |
| 300 | Ringwood Road | A31 | Dorset | South West | 409477 | 101881 | Vehicles Only | June | December |
| 301 | Warminster Road | A36 | Bath and North East Somerset | South West | 377861 | 165871 | Vehicles & Cycles | June | December |
| 302 | Kingsway | A38 | Derby | East Midlands | 433089 | 336700 | Vehicles Only | January | July |
| 303 | Manchester Road | A628 | Derbyshire | East Midlands | 401521 | 397224 | Vehicles & Cycles | December | June |
| 304 | Hospital Lane, Chelsea | B302 | Kensington and Chelsea | London | 527918 | 178132 | Vehicles & Cycles | December | June |
| 305 | Coulsdon Road | B2030 | Croydon | London | 530983 | 158249 | Vehicles & Cycles | December | June |
| 306 | Ruislip Road | B455 | Ealing | London | 513381 | 182720 | Vehicles & Cycles | December | June |
| 307 | Mains Lane | A585 | Lancashire | North West | 335856 | 440506 | Vehicles & Cycles | October | April |
| 308 | Hensigham Bypass | A595 | Cumbria | North West | 298450 | 516360 | Vehicles & Cycles | December | June |
| 309 | Breakspeare Road | Unclassified | Hillingdon | London | 507549 | 188483 | Vehicles & Cycles | December | June |
| 310 | Wrythe Lane | B278 | Sutton | London | 526384 | 166289 | Vehicles & Cycles | December | June |
| 311 | Bedford Hill | B242 | Wandsworth | London | 529325 | 172285 | Vehicles & Cycles | October | April |
| 312 | Katherine Road | B109 | Newham | London | 541918 | 183994 | Vehicles & Cycles | October | April |
| 313 | A19 Peterlee | A19 | Durham | North East | 440982 | 539046 | Vehicles Only | January | July |
| 314 | Brielle Way | A249 | Kent | South East | 591746 | 172413 | Vehicles & Cycles | October | April |
| 315 | East Ferry Road | Unclassified | Tower Hamlets | London | 537937 | 179040 | Vehicles & Cycles | January | July |
| 316 | A66 Thornaby on-Tees | A66 | Stockton-on-Tees | North East | 445502 | 518118 | Vehicles Only | June | December |
| 317 | Market Road | C | Islington | London | 530178 | 184646 | Vehicles & Cycles | January | July |
| 318 | White Hart Lane | C | Haringey | London | 532032 | 191351 | Vehicles & Cycles | January | July |
| 319 | Watling Street | A5 | Staffordshire | West Midlands | 397385 | 308851 | Vehicles & Cycles | December | June |
| 320 | Chelsfield Lane | Unclassified | Bromley | London | 547779 | 166449 | Vehicles & Cycles | June | December |
| 321 | Crockenhill Roaf | B258 | Bromley | London | 547731 | 167800 | Vehicles & Cycles | June | December |
| 322 | Mead Way | Unclassified | Bromley | London | 540043 | 167386 | Vehicles & Cycles | June | December |
| 323 | New Road | A26 | East Sussex | South East | 544709 | 102385 | Vehicles & Cycles | August | February |
| 330 | A4174 | A4174 | South Gloucestershire | South West | 364572 | 170860 | Vehicles Only | December | June |
| 331 | Tulse Hill | A204 | Lambeth | London | 531255 | 173642 | Vehicles & Cycles | October | April |
| 332 | A556 | A556 | Cheshire West and Chester | North West | 367202 | 372085 | Vehicles & Cycles | October | April |
| 333 | A629 | A629 | Bradford | Yorkshire and The Humber | 404770 | 443928 | Vehicles Only | January | July |
| 334 | A64 | A64 | North Yorkshire | Yorkshire and the Humber | 493075 | 476821 | Vehicles Only | December | June |
| 335 | B454 | B454 | Ealing | London | 515042 | 178956 | Vehicles Only | March | September |
| 336 | B4166 | B4166 | Sandwell | West Midlands | 397756 | 292193 | Vehicles & Cycles | April | October |
| 337 | B1421 | B1421 | Havering | London | 555956 | 185340 | Vehicles Only | March | September |
| 338 | A5 | A5 | Northamptonshire | East Midlands | 456330 | 271790 | Vehicles Only | April | October |
| 339 | A259 | A259 | East Sussex | South East | 573017 | 108078 | Vehicles Only | January | July |
| 340 | Arundel Road | A27 | Worthing | South East | 512133 | 105777 | Vehicles Only | January | July |
| 3412 | Bryan Clough Way | A52 | Nottinghamshire | East Midlands | 451578 | 338076 | Vehicles & Cycles | February | August |
| 3414 | Bryan Clough Way | A52 | Nottinghamshire | East Midlands | 451578 | 338076 | Vehicles & Cycles | February | August |
| 342 | Coxmoor Road | B6139 | Nottinghamshire | East Midlands | 451250 | 358950 | Vehicles & Cycles | January | July |
| 343 | Northfields Avenue | B452 | Ealing | London | 516710 | 179923 | Vehicles & Cycles | March | September |
| 344 | A19 Peterlee | A1 | Northumberland | North East | 417221 | 621678 | Vehicles Only | February | August |
| 345 | Longdike Lane | B6137 | Leeds | Yorkshire and The Humber | 442311 | 4298570 | Vehicles Only | December | June |
| 346 | Addington Road | A2022 | Bromley | London | 538483 | 164881 | Vehicles & Cycles | March | September |
| 347 | Brookhill | A409 | Harrow | London | 514947 | 192097 | Vehicles & Cycles | March | September |
| 348 | Station Road | B5030 | Staffordshire | West Midlands | 410633 | 339504 | Vehicles Only | January | July |
| 349 | Southwark Park Road | A2206 | Southwark | London | 534243 | 178830 | Vehicles & Cycles | March | September |
| 3501 | Soke Parkway | A47 | Cambridgeshire | East of England | 516516 | 300264 | Vehicles Only | March | September |
| 3503 | Soke Parkway | A47 | Cambridgeshire | East of England | 516585 | 300257 | Vehicles Only | March | September |
| 351 | Hedon Road | A1033 | Hull | Yorkshire and the Humber | 512786 | 429296 | Vehicles & Cycles | March | September |
| 352 | Church Street | B154 | Enfield | London | 533901 | 193739 | Vehicles & Cycles | March | September |
| 353 | Garnet Street, Tower Hamlets | C | Tower Hamlets | London | 535077 | 180625 | Vehicles & Cycles | March | September |
| 355 | Greenford Road | A4127 | Ealing | London | 514233 | 180980 | Vehicles & Cycles | March | September |
| 356 | Putney Bridge Road | A3209 | Wandsworth | London | 524800 | 175168 | Vehicles & Cycles | March | September |
| 357 | Charlton Park Road | B210 | Greenwich | London | 541902 | 177876 | Vehicles Only | March | September |
| 358 | A42 Ashby De-La-Zouch | A42 | Leicestershire | East Midlands | 436263 | 315608 | Vehicles Only | January | July |
| 359 | Keighley Road | B6143 | Bradford | Yorkshire and The Humber | 404270 | 439278 | Vehicles & Cycles | March | September |
| 360 | West Drayton Road | B465 | Hillingdon | London | 507523 | 181238 | Vehicles Only | March | September |
| 361 | Middleton Road | C | Sutton | London | 526747 | 166828 | Vehicles & Cycles | March | September |
| 362 | Bromley Road | A21 | Lewisham | London | 537979 | 172159 | Vehicles & Cycles | March | September |
| 363 | Farnborough Way | A21 | Bromley | London | 544945 | 164470 | Vehicles & Cycles | March | September |
| 364 | Sea Road South | A35 | Dorset | South West | 346855 | 92223 | Vehicles & Cycles | December | June |
| 365 | Belgrave Road | A3213 | Westminster | London | 529490 | 178366 | Vehicles & Cycles | February | August |
| 366 | Royal Dock Road | A1020 | Newham | London | 543917 | 181884 | Vehicles Only | January | July |
| 367 | Shepherds Hill | C | Haringey | London | 528983 | 188175 | Vehicles & Cycles | December | June |
| 369 | Brighton Road | A23 | Surrey | South East | 528979 | 157333 | Vehicles Only | March | September |
| 370 | Birmingham New Road | A4123 | Dudley | West Midlands | 394061 | 293485 | Vehicles & Cycles | April | October |
| 3712 | Dunnings Bridge Road | A5036 | Sefton | North West | 335603 | 398581 | Vehicles & Cycles | April | October |
| 3714 | Dunnings Bridge Road | A5036 | Sefton | North West | 335763 | 398650 | Vehicles & Cycles | April | October |
| 372 | Oxford Road | A418 | Buckinghamshire | South East | 480036 | 212376 | Vehicles & Cycles | January | July |
| 373 | Bounds Green Lane | A109 | Haringey | London | TBD | TBD | Vehicles Only | TBD | TBD |
| 501 | Hospital Lane | Unclassified | Newcastle upon Tyne | North East | 418251 | 565623 | Vehicles & Cycles | December | June |
| 502 | Withycombe Village Road | Unclassified | Devon | South West | 300662 | 81710 | Vehicles & Cycles | December | June |
| 503 | Holyhead Road | Unclassified | Sandwell | West Midlands | 398186 | 295073 | Vehicles & Cycles | April | October |
| 504 | Harbour Road | Unclassified | North Somerset | South West | 347204 | 176582 | Vehicles & Cycles | April | October |
| 505 | Longwater Lane | Unclassified | Norfolk | East of England | 616972 | 311362 | Vehicles & Cycles | November | May |
| 506 | Queensway | Unclassified | Northamptonshire | East Midlands | 487538 | 268136 | Vehicles & Cycles | February | August |
| 507 | Kirkmanshulme Lane | Unclassified | Manchester | North West | 387618 | 396065 | Vehicles & Cycles | December | June |
| 508 | Pannal Ash Road | Unclassified | North Yorkshire | Yorkshire and The Humber | 429342 | 453486 | Vehicles & Cycles | November | May |
| 509 | Hatch Lane | A3044 | Hillingdon | London | 505904 | 177315 | Vehicles & Cycles | November | May |
| 510 | Newington Green Road | A105 | Islington | London | 532745 | 184962 | Vehicles & Cycles | April | October |
| 690 | Royal Artillery Way | A1159 | Southend-on-Sea | East of England | 590891 | 186708 | Cycles Only | May | November |
| 700 |  | Unclassified | Gwynedd - Gwynedd | Wales | 260623 | 305842 | Vehicles & Cycles | March | September |
| 701 | Ingarsby Road | C | Leicestershire | East Midlands | 467923 | 306260 | Vehicles Only | May | November |
| 702 | Hollow Lane | Unclassified | Cambridgeshire | East of England | 529661 | 284733 | Vehicles & Cycles | February | August |
| 703 | Main Street | B1377 | East Lothian | Scotland | 345183 | 676629 | Vehicles & Cycles | January | July |
| 704 | Dunmail Avenue | Unclassified | St. Helens | North West | 352088 | 398159 | Vehicles & Cycles | January | July |
| 705 | Chilwell Lane | Unclassified | Nottinghamshire | East Midlands | 450945 | 337113 | Vehicles & Cycles | May | November |
| 706 | Crediton Road | B3215 | Devon | South West | 259747 | 95807 | Vehicles & Cycles | November | May |
| 707 | Ladymead | A25 | Surrey | South East | 499654 | 150828 | Vehicles & Cycles | January | July |
| 708 | Church Lane | C | Bedford | East of England | 500918 | 253264 | Vehicles & Cycles | January | July |
| 709 | Jossey Lane | Unclassified | Doncaster | Yorkshire and The Humber | 455867 | 406119 | Vehicles & Cycles | January | July |
| 710 | Warsop Lane | B6020 | Nottinghamshire | East Midlands | 459019 | 357948 | Vehicles & Cycles | April | October |
| 711 | M90 | M90 | Fife | Scotland | 313244 | 694558 | Vehicles Only | April | October |
| 712 | Eversley Road | A327 | Wokingham | South East | 476299 | 165956 | Vehicles Only | February | August |
| 714 | Wick Road | Unclassified | Somerset | South West | 367219 | 119371 | Vehicles & Cycles | March | September |
| 715 | Priory Road | A389 | Cornwall | South West | 207498 | 66931 | Vehicles & Cycles | January | July |
| 716 | School Road | C | Cornwall | South West | 228956 | 70131 | Vehicles Only | April | October |
| 718 | Unclassified | C | Cumbria | North West | 360844 | 524160 | Vehicles & Cycles | March | September |
| 719 | Black Lane | C | Wiltshire | South West | 420924 | 119864 | Vehicles & Cycles | February | August |
| 720 | Fairlee Road | A3054 | Isle of Wight | South East | 450914 | 90450 | Vehicles & Cycles | August | February |
| 721 | Ambleside Road | Unclassified | Surrey | South East | 492956 | 162001 | Vehicles & Cycles | February | August |
| 722 |  | B7020 | Dumfries and Galloway | Scotland | 309914 | 576548 | Vehicles Only | December | June |
| 724 |  | A91 | Fife | Scotland | 316476 | 708028 | Vehicles Only | April | October |
| 726 | Betws Road | Unclassified | Pen-y-bont ar Ogwr - Bridgend | Wales | 290158 | 186265 | Vehicles & Cycles | April | October |
| 729 | Kings Head Hill | A110 | Waltham Forest | London | 537927 | 194614 | Vehicles & Cycles | July | January |
| 730 | St Martin's Way | A5268 | Cheshire West and Chester | North West | 340204 | 366614 | Vehicles Only | March | September |
| 731 | Hilton lane | A6044 | Bury | North West | 381032 | 402510 | Vehicles Only | March | September |
| 732 | Dereham Road | A1074 | Norfolk | East of England | 620956 | 309325 | Vehicles & Cycles | February | August |
| 733 | Oxford Road | A651 | Kirklees | Yorkshire and The Humber | 420610 | 426762 | Vehicles Only | January | July |
| 734 | Quick Road | A6050 | Tameside | North West | 397567 | 403567 | Vehicles Only | March | September |
| 735 | London Road | A4 | Reading | South East | 472278 | 172993 | Vehicles & Cycles | March | September |
| 736 | Whitehall Road | A58 | Leeds | Yorkshire and The Humber | 428157 | 432429 | Vehicles Only | April | October |
| 737 | High Street | A610 | Derbyshire | East Midlands | 441872 | 350113 | Vehicles Only | April | October |
| 738 | Great North Road | A638 | Nottinghamshire | East Midlands | 466722 | 384278 | Vehicles Only | July | January |
| 739 | Spalding Road | A151 | Lincolnshire | East Midlands | 533665 | 324730 | Vehicles & Cycles | January | July |
| 740 | Cole and Broughton Road | A56 | North Yorkshire | Yorkshire and The Humber | 391717 | 449250 | Vehicles Only | February | August |
| 741 | Stamford Road | A4300 | Northamptonshire | East Midlands | 489195 | 283093 | Vehicles Only | February | August |
| 742 | Embankment Road | A374 | Plymouth | South West | 250381 | 55383 | Vehicles Only | April | October |
| 743 | Stirling Road | A73 | North Lanarkshire | Scotland | 276599 | 667732 | Vehicles Only | February | August |
| 744 | A96 | A96 | Moray | Scotland | 298390 | 857149 | Vehicles & Cycles | May | November |
| 745 | A30 | A30 | Devon | South West | 315436 | 100846 | Vehicles Only | May | November |
| 746 | Holmer Road | A49 | Herefordshire | West Midlands | 350614 | 241875 | Vehicles & Cycles | February | August |
| 747 | Garrison Lane | A154 | Suffolk | East of England | 630019 | 235387 | Vehicles Only | May | November |
| 748 | Beccles Road | A143 | Norfolk | East of England | 651279 | 304489 | Vehicles Only | May | November |
| 749 | Park Road | A590 | Cumbria | North West | 319186 | 471212 | Vehicles & Cycles | May | November |
| 750 | Western Drive | Unclassified | Caerffili - Caerphilly | Wales | 314343 | 198896 | Vehicles Only | July | January |
| 751 | Garthwaite Crescent | Unclassified | Milton Keynes | South East | 482902 | 235190 | Vehicles & Cycles | April | October |
| 761 | St Dunstan's Hill | A217 | Sutton | London | 524406 | 164232 | Vehicles & Cycles | April | October |
| 762 | Harlington Road | A437 | Hillingdon | London | 507970 | 181278 | Vehicles & Cycles | April | October |
| 763 | Great Cambridge Road | A10 | Enfield | London | 534236 | 195435 | Vehicles Only | May | November |
| 764 | Twickenham Road | A316 | Richmond upon Thames | London | 517330 | 175000 | Vehicles & Cycles | June | December |
| 765 | Woodmansterne Lane | A2022 | Sutton | London | 528336 | 161370 | Vehicles & Cycles | June | December |
| 766 | Harrow Road | A404 | Westminster | London | 524000 | 182440 | Vehicles & Cycles | May | November |
| 767 | High Street | A1004 | Enfield | London | 529688 | 193964 | Vehicles & Cycles | May | November |
| 768 | Aylmer Road | A1 | Haringey | London | 527491 | 188611 | Vehicles & Cycles | May | November |
| 769 | Brunel Road | B205 | Southwark | London | 535258 | 179722 | Vehicles & Cycles | June | December |
| 770 | Iver Lane | B470 | Hillingdon | London | 505072 | 182285 | Vehicles & Cycles | April | October |
| 771 | Roehampton Lane | A306 | Wandsworth | London | 522100 | 175000 | Vehicles & Cycles | June | December |
| 772 | Twickenham Road | A310 | Hounslow | London | 515968 | 174992 | Vehicles & Cycles | June | December |
| 773 | Brecknock Road | A5200 | Islington | London | 529597 | 185350 | Vehicles & Cycles | May | November |
| 775 | Traps Lane | B283 | Kingston upon Thames | London | 521133 | 169552 | Vehicles & Cycles | May | November |
| 776 | Harmondsworth Road | C | Hillingdon | London | 506364 | 178339 | Vehicles & Cycles | April | October |
| 777 | St Ann's Road | C | Kensington and Chelsea | London | 523865 | 180588 | Vehicles & Cycles | April | October |
| 778 | Hampton Court Road | A308 | Richmond upon Thames | London | 514803 | 169135 | Vehicles & Cycles | April | October |
| 780 | Commonside East | C | Merton | London | 528858 | 168415 | Vehicles & Cycles | May | November |
| 786 | Lilleshall Road | C | Merton | London | 526585 | 167391 | Vehicles & Cycles | May | November |
| 788 | Lambs Croft Avenue | Unclassified | Lewisham | London | 541720 | 172262 | Vehicles & Cycles | May | November |
| 789 | Hare and Billet Road | C | Lewisham | London | 538997 | 176370 | Vehicles & Cycles | May | November |
| 790 | Royal Artillery Way | A1159 | Southend-on-Sea | East of England | 590891 | 186708 | Vehicles Only | May | November |
| 791 | Wollaton Road | A609 | Nottingham | East Midlands | 453518 | 340044 | Vehicles & Cycles | April | October |
| 792 | Gravel Hill | A349 | Poole | South West | 401414 | 95250 | Vehicles & Cycles | April | October |
| 793 | York Road | A64 | Leeds | Yorkshire and The Humber | 435751 | 435129 | Vehicles & Cycles | June | December |
| 794 | Formby Bypass | A565 | Sefton | North West | 330639 | 409764 | Vehicles & Cycles | May | November |
| 795 | A1027 | A1027 | Stockton-on-Tees | North East | 443999 | 522071 | Vehicles & Cycles | June | December |
| 796 | Witherley Road | A5 | Warwickshire | West Midlands | 432592 | 296986 | Vehicles Only | June | December |
| 797 | Horley Road | A23 | Surrey | South East | 527617 | 149098 | Vehicles & Cycles | June | December |
| 798 | Lichfield Road | A5127 | Birmingham | West Midlands | 411916 | 297379 | Vehicles & Cycles | April | October |
| 799 | Mudford Road | A359 | Somerset | South West | 356282 | 117897 | Vehicles & Cycles | June | December |
| 800 | Main Road | A35 | Hampshire | South East | 435486 | 112142 | Vehicles Only | December | June |
| 801 | Lytton Way | A602 | Hertfordshire | East of England | 523274 | 225005 | Vehicles Only | April | October |
| 802 | Duncan Edwards Way | A461 | Dudley | West Midlands | 393548 | 289344 | Vehicles Only | April | October |
| 804 | Offerton Road | A627 | Stockport | North West | 393385 | 387633 | Vehicles & Cycles | June | December |
| 805 | Burniston Road | A165 | North Yorkshire | Yorkshire and The Humber | 502604 | 491143 | Vehicles & Cycles | June | December |
| 806 | Broadway | A587 | Lancashire | North West | 331751 | 444663 | Vehicles Only | April | October |
| 807 | Ellesmere Road | A528 | Shropshire | West Midlands | 349722 | 316003 | Vehicles & Cycles | February | August |
| 808 | Winwick Road | A49 | Warrington | North West | 360348 | 391547 | Vehicles Only | April | October |
| 809 | Higher Lane | A56 | Warrington | North West | 369244 | 386480 | Vehicles & Cycles | April | October |
| 810 | Whitmore Road | A53 | Staffordshire | West Midlands | 383924 | 344557 | Vehicles Only | January | July |
| 811 | Newark Road | A607 | Leicestershire | East Midlands | 461328 | 309412 | Vehicles Only | April | October |
| 821 | Unclassified | C | North Yorkshire | Yorkshire and The Humber | 421073 | 472047 | Vehicles & Cycles | #N/A | #N/A |
| 822 | Main Street | C | Lincolnshire | East Midlands | 498246 | 381627 | Vehicles Only | June | December |
| 823 | Unclassified | C | Cornwall | South West | 227145 | 94670 | Vehicles Only | February | August |
| 826 | Chestnut Avenue | C | Essex | East of England | 551094 | 238004 | Vehicles & Cycles | March | September |
| 827 | Sandling Road | C | Kent | South East | 614826 | 136230 | Vehicles Only | January | July |
| 830 | Rushford Road | C | Suffolk | East of England | 592417 | 281020 | Vehicles & Cycles | February | August |
| 834 | Unclassified | Unclassified | Aberdeenshire | Scotland | 391560 | 797850 | Vehicles Only | May | November |
| 838 | Beggarwood Lane | C | Hampshire | South East | 459957 | 148235 | Vehicles & Cycles | May | November |
| 839 | Birch Green Road | C | Lancashire | North West | 349127 | 406612 | Vehicles & Cycles | March | September |
| 840 | Bampton Road | C | Plymouth | South West | 251239 | 57965 | Vehicles Only | April | October |
| 841 | Bank Dike Hill | Unclassified | North Yorkshire | Yorkshire and The Humber | 415210 | 457023 | Vehicles & Cycles | June | December |
| 846 | Unclassified | Unclassified | Sir Ynys Mon - Isle of Anglesey | Wales | 262069 | 380017 | Vehicles & Cycles | July | January |
| 847 | Unclassified | Unclassified | Aberdeenshire | Scotland | 347240 | 805826 | Vehicles Only | May | November |
| 850 | Barnhorn Road | A259 | East Sussex | South East | 569682 | 107835 | Vehicles & Cycles | July | January |
| 851 | A75 | A75 | Dumfries and Galloway | Scotland | 313078 | 570049 | Vehicles Only | June | December |
| 852 | A39 | A39 | Devon | South West | 240391 | 124284 | Vehicles & Cycles | February | August |
| 853 | A303 | A303 | Somerset | South West | 364352 | 126502 | Vehicles Only | March | September |
| 854 | Quantock Road | A39 | Somerset | South West | 328172 | 137094 | Vehicles Only | May | November |
| 856 | Oxford Road | B430 | Oxfordshire | South East | 453454 | 223126 | Vehicles Only | February | August |
| 857 | A49 | A49 | Shropshire | West Midlands | 355714 | 328633 | Vehicles Only | February | August |
| 858 | Stonebridge Road | A446 | Warwickshire | West Midlands | 420148 | 285424 | Vehicles & Cycles | March | September |
| 859 | Unstone Dronfield Bypass | A61 | Derbyshire | East Midlands | 435032 | 377591 | Vehicles Only | July | January |
| 860 | London Road | A16 | Lincolnshire | East Midlands | 538736 | 376589 | Vehicles Only | February | August |
| 861 | A30 | A30 | Cornwall | South West | 177186 | 48680 | Vehicles & Cycles | July | January |
| 862 | Alcester Road | A46 | Warwickshire | West Midlands | 412303 | 256539 | Vehicles & Cycles | April | October |
| 863 | Hull Road | A1174 | East Riding of Yorkshire | Yorkshire and The Humber | 505856 | 437643 | Vehicles & Cycles | March | September |
| 864 | A66 | A66 | Cumbria | North West | 364412 | 524598 | Vehicles Only | March | September |
| 865 | A596 | A596 | Cumbria | North West | 319875 | 546212 | Vehicles & Cycles | April | October |
| 868 | Head of the Valleys Road | A465 | Merthyr Tudful - Merthyr Tydfil | Wales | 300659 | 207366 | Vehicles & Cycles | January | July |
| 869 | A483 | A483 | Powys - Powys | Wales | 324352 | 309209 | Vehicles Only | February | August |
| 870 | A75 | A75 | Dumfries and Galloway | Scotland | 279575 | 567965 | Vehicles Only | June | December |
| 871 | A90 | A90 | Aberdeenshire | Scotland | 410050 | 839181 | Vehicles Only | May | November |
| 872 | A77 | A77 | East Ayrshire | Scotland | 241370 | 633874 | Vehicles Only | June | December |
| 874 | Ipswich Road | A12 | Suffolk | East of England | 607198 | 236571 | Vehicles Only | April | October |
| 876 | A9 | A9 | Highland | Scotland | 278635 | 832310 | Vehicles & Cycles | May | November |
| 878 | Roman Road | A419 | Wiltshire | South West | 412472 | 192151 | Vehicles Only | February | August |
| 879 | Yarmouth Road | A47 | Suffolk | East of England | 653887 | 295233 | Vehicles Only | May | November |
| 880 | A629 | A629 | North Yorkshire | Yorkshire and The Humber | 397995 | 450526 | Vehicles & Cycles | May | November |
| 881 | A836 | A836 | Highland | Scotland | 321934 | 970935 | Vehicles Only | May | November |
| 882 | Riverside Drive | A85 | Dundee City | Scotland | 337413 | 729607 | Vehicles Only | April | October |
| 883 | Barrhead Road | A736 | Glasgow City | Scotland | 251864 | 661229 | Vehicles Only | February | August |
| 884 | A338 | A338 | Wiltshire | South West | 423591 | 156864 | Vehicles Only | May | November |
| 885 | Bridge Street | A509 | Milton Keynes | South East | 488827 | 250924 | Vehicles & Cycles | January | July |
| 886 | Banbury Road | A361 | Oxfordshire | South East | 439633 | 232187 | Vehicles Only | February | August |
| 887 | A689 | A689 | Durham | North East | 434081 | 528470 | Vehicles & Cycles | April | October |
| 888 | Broadway | A6120 | Leeds | Yorkshire and The Humber | 422657 | 437264 | Vehicles Only | April | October |
| 889 | A4103 | A4103 | Herefordshire | West Midlands | 371580 | 247453 | Vehicles Only | March | September |
| 890 | Longton Road | A520 | Staffordshire | West Midlands | 390381 | 334395 | Vehicles Only | January | July |
| 891 | A607 | A607 | Leicestershire | East Midlands | 478719 | 322088 | Vehicles & Cycles | May | November |
| 892 | Wayford Road | A149 | Norfolk | East of England | 636517 | 325292 | Vehicles & Cycles | February | August |
| 893 | Fender Lane | A553 | Wirral | North West | 328334 | 390572 | Vehicles & Cycles | February | August |
| 894 | Watling Street | C | Staffordshire | West Midlands | 415996 | 303296 | Vehicles Only | March | September |
| 895 | A686 | A686 | Cumbria | North West | 359641 | 536045 | Vehicles Only | March | September |
| 896 | A985 | A985 | Fife | Scotland | 299367 | 687032 | Vehicles Only | April | October |
| 897 | A482 | A482 | Sir Ceredigion - Ceredigion | Wales | 247598 | 259858 | Vehicles Only | January | July |
| 902 | Bridgford Street | A6097 | Nottinghamshire | East Midlands | 469107 | 342569 | Vehicles Only | February | August |
| 903 | A82 | A82 | Stirling | Scotland | 235906 | 727787 | Vehicles Only | March | September |
| 904 | A737 | A737 | Renfrewshire | Scotland | 238668 | 659862 | Vehicles & Cycles | June | December |
| 905 | A95 | A95 | Moray | Scotland | 317257 | 836234 | Vehicles Only | May | November |
| 906 | Temeside Way | A4440 | Worcestershire | West Midlands | 384238 | 251798 | Vehicles Only | May | November |
| 907 | Main Street | A68 | Midlothian | Scotland | 339776 | 663828 | Vehicles Only | January | July |
| 908 | Andover Road | A343 | Hampshire | South East | 445124 | 163127 | Vehicles Only | May | November |
| 909 | A339 | A339 | Hampshire | South East | 451746 | 160548 | Vehicles Only | May | November |
| 910 | A4130 | A4130 | Oxfordshire | South East | 449804 | 191380 | Vehicles & Cycles | May | November |
| 911 | Isle of Ely Way | A141 | Cambridgeshire | East of England | 540011 | 289155 | Vehicles Only | April | October |
| 912 | Unknown (formerly A477) | Unclassified | Sir Gaerfyrddin - Carmarthenshire | Wales | 225510 | 213976 | Vehicles Only | January | July |
| 913 | A148 | A148 | Norfolk | East of England | 588201 | 330846 | Vehicles Only | April | October |
| 914 | Brownhill Drive | A6119 | Blackburn with Darwen | North West | 368873 | 430853 | Vehicles & Cycles | March | September |
| 915 | Roman Road | A167 | Gateshead | North East | 427146 | 556634 | Vehicles & Cycles | March | September |
| 917 | Furze Plant Road | A308 | Windsor and Maidenhead | South East | 487244 | 182674 | Vehicles & Cycles | July | January |
| 918 | Doncaster Road | A638 | Wakefield | Yorkshire and The Humber | 436333 | 418885 | Vehicles Only | June | December |
| 919 | Clitheroe Bypass | A59 | Lancashire | North West | 374793 | 439508 | Vehicles & Cycles | May | November |
| 920 | Kelso Road | A698 | Scottish Borders | Scotland | 383106 | 639328 | Vehicles Only | January | July |
| 921 | A890 | A890 | Highland | Scotland | 191844 | 838217 | Vehicles Only | March | September |
| 923 | Mansfield Road | A617 | Derbyshire | East Midlands | 449586 | 365126 | Vehicles & Cycles | March | September |
| 924 | Solihull Bypass | A41 | Solihull | West Midlands | 416524 | 279341 | Vehicles Only | April | October |
| 926 | Upper Wortley Road | A629 | Rotherham | Yorkshire and The Humber | 439212 | 394390 | Vehicles & Cycles | February | August |
| 927 | Otley Road | A658 | Bradford | Yorkshire and The Humber | 417045 | 433978 | Vehicles & Cycles | March | September |
| 933 | Narberth Road | A478 | Sir Benfro - Pembrokeshire | Wales | 213028 | 201081 | Vehicles & Cycles | July | January |
| 940 | Blandford Road | B3082 | Dorset | South West | 393301 | 104581 | Vehicles & Cycles | May | November |
| 941 | London Road | B245 | Kent | South East | 558273 | 147582 | Vehicles & Cycles | February | August |
| 942 | High Street | B4216 | Gloucestershire | South West | 372042 | 226191 | Vehicles & Cycles | March | September |
| 943 | West Common | B2028 | West Sussex | South East | 534106 | 125229 | Vehicles & Cycles | February | August |
| 944 | Black Fan Road | B195 | Hertfordshire | East of England | 525249 | 213110 | Vehicles & Cycles | July | January |
| 945 | Bashley Road | B3058 | Hampshire | South East | 424207 | 97020 | Vehicles & Cycles | August | February |
| 946 | Stifford Hill Road | B186 | Thurrock | East of England | 559288 | 181003 | Vehicles & Cycles | June | December |
| 947 | llangfelach Road | B4489 | Abertawe - Swansea | Wales | 264964 | 198105 | Vehicles Only | July | January |
| 948 | Station Road | B810 | Falkirk | Scotland | 293199 | 678298 | Vehicles Only | October | April |
| 950 | Oremsby Road | C | Middlesbrough | North East | 451813 | 518463 | Vehicles & Cycles | February | August |
| 951 | North Road | B1284 | Sunderland | North East | 435280 | 546952 | Vehicles & Cycles | April | October |
| 952 | Lambs Road | B4512 | Lancashire | North West | 334910 | 441782 | Vehicles & Cycles | May | November |
| 953 | Buckingham Road | B34034 | Milton Keynes | South East | 486225 | 233377 | Vehicles & Cycles | May | November |
| 954 | Ashley Road | B365 | Surrey | South East | 510191 | 165955 | Vehicles & Cycles | July | January |
| 955 | Rowner Road | B3334 | Hampshire | South East | 457568 | 102445 | Vehicles Only | February | August |
| 956 | Station Road | B2100 | East Sussex | South East | 555813 | 129843 | Vehicles & Cycles | July | January |
| 957 | Schofield Lane | B5235 | Wigan | North West | 365912 | 403761 | Vehicles & Cycles | February | August |
| 960 | Cateran Way | Unclassified | Northumberland | North East | 427020 | 576083 | Vehicles Only | February | August |
| 961 | Ladybank Road | C | Derby | East Midlands | 430473 | 335443 | Vehicles & Cycles | April | October |
| 962 | Chorley Old Road | Unclassified | Lancashire | North West | 358182 | 423072 | Vehicles & Cycles | April | October |
| 963 | Crossdale Road | Unclassified | Wigan | North West | 362988 | 404065 | Vehicles & Cycles | February | August |
| 964 | Frankholmes Drive | Unclassified | Solihull | West Midlands | 414317 | 276627 | Vehicles & Cycles | May | November |
| 965 | Longdales Road | Unclassified | Birmingham | West Midlands | 403874 | 277640 | Vehicles & Cycles | October | April |
| 966 | Sandy Lodge Way | Unclassified | Hertfordshire | East of England | 508984 | 192380 | Vehicles & Cycles | January | July |
| 967 | Old Mixon Road | Unclassified | North Somerset | South West | 334072 | 158611 | Vehicles & Cycles | March | September |
| 968 | Pattison Lane | Unclassified | Milton Keynes | South East | 487298 | 238613 | Vehicles & Cycles | May | November |
| 969 | Copthorne Park | C | Shropshire | West Midlands | 347565 | 312753 | Vehicles & Cycles | February | August |
| 970 | High Common Road | Unclassified | South Lanarkshire | Scotland | 265090 | 654017 | Vehicles Only | August | February |
| 971 | Stanhope Road | Unclassified | Sandwell | West Midlands | 402001 | 287315 | Vehicles & Cycles | February | August |
| 976 | Fell Road | B6478 | Lancashire | North West | 372198 | 445708 | Vehicles & Cycles | April | October |
| 977 | B4494 | B4494 | West Berkshire | South East | 444024 | 177608 | Vehicles & Cycles | March | September |
| 978 | Prickwillow Road | B1382 | Cambridgeshire | East of England | 557842 | 281964 | Vehicles & Cycles | February | August |
| 979 | Wildhouse Lane | B6225 | Rochdale | North West | 392860 | 413942 | Vehicles Only | July | January |
| 982 | A87 | A87 | Highland | Scotland | 164574 | 823415 | Vehicles & Cycles | March | September |
| 2521 | M1 | M1 | Nottinghamshire | East Midlands | 450502 | 342840 | Vehicles Only | March | September |
| 2523 | M1 | M1 | Nottinghamshire | East Midlands | 450560 | 342789 | Vehicles Only | March | September |
| 2531 | M1 | M1 | Barnsley | Yorkshire and The Humber | 434184 | 401049 | Vehicles Only | February | August |
| 2533 | M1 | M1 | Barnsley | Yorkshire and The Humber | 433396 | 403944 | Vehicles Only | February | August |
| 2542 | M4 | M4 | Buckinghamshire | South East | 491641 | 179721 | Vehicles Only | February | August |
| 2544 | M4 | M4 | Buckinghamshire | South East | 491678 | 179702 | Vehicles Only | February | August |
| 2552 | M4 | M4 | West Berkshire | South East | 434690 | 173647 | Vehicles Only | June | December |
| 2554 | M4 | M4 | West Berkshire | South East | 434679 | 173590 | Vehicles Only | June | December |
| 2571 | M5 | M5 | Gloucestershire | South West | 372086 | 197470 | Vehicles Only | February | August |
| 2573 | M5 | M5 | Gloucestershire | South West | 372147 | 197432 | Vehicles Only | February | August |
| 2581 | M5 | M5 | Worcestershire | West Midlands | 392798 | 267600 | Vehicles Only | March | September |
| 2583 | M5 | M5 | Worcestershire | West Midlands | 392820 | 267533 | Vehicles Only | March | September |
| 2591 | M6 | M6 | Cumbria | North West | 350229 | 530193 | Vehicles Only | May | November |
| 2593 | M6 | M6 | Cumbria | North West | 350301 | 530081 | Vehicles Only | May | November |
| 2611 | M6 | M6 | St. Helens | North West | 359180 | 396303 | Vehicles Only | July | January |
| 2613 | M6 | M6 | St. Helens | North West | 358986 | 396447 | Vehicles Only | July | January |
| 2632 | M6 | M6 | Coventry | West Midlands | 437700 | 282869 | Vehicles Only | April | October |
| 2634 | M6 | M6 | Coventry | West Midlands | 437652 | 282845 | Vehicles Only | April | October |
| 2662 | M3 | M3 | Surrey | South East | 489294 | 160416 | Vehicles Only | April | October |
| 2664 | M3 | M3 | Surrey | South East | 489214 | 160289 | Vehicles Only | April | October |
| 2671 | M11 | M11 | Essex | East of England | 546259 | 198323 | Vehicles Only | February | August |
| 2673 | M11 | M11 | Essex | East of England | 546320 | 198359 | Vehicles Only | February | August |
| 2682 | M50 | M50 | Gloucestershire | South West | 377851 | 233033 | Vehicles Only | April | October |
| 2684 | M50 | M50 | Gloucestershire | South West | 377828 | 233017 | Vehicles Only | April | October |
| 2691 | A1(M) | A1(M) | Durham | North East | 431305 | 538624 | Vehicles Only | June | December |
| 2693 | A1(M) | A1(M) | Durham | North East | 431318 | 538650 | Vehicles Only | June | December |
| 2742 | M56 | M56 | Cheshire West and Chester | North West | 351954 | 378614 | Vehicles Only | August | February |
| 2744 | M56 | M56 | Cheshire West and Chester | North West | 351988 | 378578 | Vehicles Only | August | February |
| 2752 | M40 | M40 | Oxfordshire | South East | 470079 | 200329 | Vehicles Only | May | November |
| 2754 | M40 | M40 | Oxfordshire | South East | 470218 | 200093 | Vehicles Only | May | November |
| 2791 | M18 | M18 | Doncaster | Yorkshire and The Humber | 463424 | 402599 | Vehicles Only | July | January |
| 2812 | M55 | M55 | Lancashire | North West | 349750 | 433879 | Vehicles Only | May | November |
| 2814 | M55 | M55 | Lancashire | North West | 349725 | 433838 | Vehicles Only | May | November |
| 2822 | M20 | M20 | Kent | South East | 606954 | 139221 | Vehicles Only | May | November |
| 2824 | M20 | M20 | Kent | South East | 606891 | 139191 | Vehicles Only | May | November |
| 2831 | M3 | M3 | Hampshire | South East | 453428 | 139959 | Vehicles Only | May | November |
| 2833 | M3 | M3 | Hampshire | South East | 453499 | 139959 | Vehicles Only | May | November |
| 2852 | M180 | M180 | North Lincolnshire | Yorkshire and The Humber | 501793 | 409816 | Vehicles Only | July | January |
| 2854 | M180 | M180 | North Lincolnshire | Yorkshire and The Humber | 501784 | 409773 | Vehicles Only | July | January |
| 2871 | M25 | M25 | Havering | London | 558339 | 185004 | Vehicles Only | June | December |
| 2873 | M25 | M25 | Havering | London | 558391 | 185055 | Vehicles Only | June | December |
| 2891 | M1 | M1 | Leicestershire | East Midlands | 451544 | 306342 | Vehicles Only | July | January |
| 2893 | M1 | M1 | Leicestershire | East Midlands | 451593 | 306353 | Vehicles Only | July | January |
| 2902 | M40 | M40 | Warwickshire | West Midlands | 423003 | 265336 | Vehicles Only | March | September |
| 2904 | M40 | M40 | Warwickshire | West Midlands | 422985 | 265234 | Vehicles Only | March | September |
| 2911 | A1(M) | A1(M) | Cambridgeshire | East of England | 518301 | 279398 | Vehicles Only | February | August |
| 2913 | A1(M) | A1(M) | Cambridgeshire | East of England | 518344 | 279398 | Vehicles Only | February | August |
| 2922 | M4 | M4 | Pen-y-bont ar Ogwr - Bridgend | Wales | 294227 | 181949 | Vehicles Only | March | September |
| 2924 | M4 | M4 | Pen-y-bont ar Ogwr - Bridgend | Wales | 294154 | 181959 | Vehicles Only | March | September |
| 2931 | M6 | M6 | Cheshire West and Chester | North West | 374036 | 369060 | Vehicles Only | May | November |
| 2933 | M6 | M6 | Cheshire East | North West | 374084 | 369070 | Vehicles Only | May | November |
| 2941 | A1(M) | A1(M) | Wakefield | Yorkshire and The Humber | 446968 | 425549 | Vehicles Only | February | August |
| 2943 | A1(M) | A1(M) | Wakefield | Yorkshire and The Humber | 447017 | 425555 | Vehicles Only | February | August |
| 7272 | M27 | M27 | Southampton | South East | 441933 | 117231 | Vehicles Only | March | September |
| 7274 | M27 | M27 | Hampshire | South East | 442028 | 117124 | Vehicles Only | March | September |
| 7281 | M1 | M1 | Derbyshire | East Midlands | 444613 | 371292 | Vehicles Only | May | November |
| 7283 | M1 | M1 | Derbyshire | East Midlands | 444652 | 371270 | Vehicles Only | May | November |
| 7602 | Eastern Avenue | A12 | Havering | London | 552594 | 190736 | Vehicles & Cycles | April | October |
| 7604 | Eastern Avenue | A12 | Havering | London | 552932 | 190596 | Vehicles & Cycles | April | October |
| 8492 | Southend Arterial Road | A127 | Essex | East of England | 565997 | 189982 | Vehicles Only | May | November |
| 8494 | Southend Arterial Road | A127 | Essex | East of England | 566009 | 189947 | Vehicles Only | May | November |
| 8552 | A14 | A14 | Cambridgeshire | East of England | 565063 | 266096 | Vehicles Only | July | January |
| 8554 | A14 | A14 | Cambridgeshire | East of England | 565075 | 266053 | Vehicles Only | July | January |
| 40742 | Embankment Road | A374 | Plymouth | South West | 250381 | 55383 | Vehicles Only | April | October |
| 40967 | Old Mixon Road | Unclassified | North Somerset | South West | 334072 | 158611 | Vehicles Only | February | August |
| 42554 | M4 | M4 | West Berkshire | South East | 434679 | 173590 | Vehicles Only | June | December |

## ANNEX I 	Routine Site Inspection (RSI)

- Visual inspection - The engineer must inspect the road condition at the site, paying attention to the in-road equipment, checking for any physical damage or degradation. The inductive loops and piezoelectric sensors must also be secured in the road. Efforts must be made to prevent further damage where safe to do so.

- Site inspections - The engineer must inspect areas surrounding the site for any factors that may influence the data, including changes to the speed limit, traffic calming measures, bus stops and schools. The engineer must complete a detailed checklist of the local area and record if there has been changes since the last site visit.

Site access routes (pathways, steps, walkways etc.) must be inspected. Efforts should be made to ensure they are kept clean, tidy, and vegetation must be maintained. The engineer must endeavour to make it a safe working environment for any persons accessing the site and the general public.

- Equipment inspection - The engineer must check all off-road equipment installed to ensure it is present and secured. The equipment must be kept clean and free from debris. The sites communication signal strength must also be checked and recorded. The types of equipment to be inspected include:

- Cabinet (hinges, door, locks, solar panels)

- Cabinets contents (modem, cables, classifier)

- Duct and seals

- Electrical safety inspection (applicable to mains powered sites) - The engineer shall conduct electrical safety tests to ensure that the site is operating safely, as equipment can deteriorate with age and use. The engineer is responsible for preventing accidents occurring to any persons who may come in contact with the equipment and the Contractor will be held liable for any errors or damages.

- Battery level checks - The engineer must test and record the voltage levels and the condition of the battery, including any back up batteries. This is to ensure that they have sufficient charge to power the site. The batteries shall be replaced if they are damaged, degraded, or if the levels are too low.

- Site performance analysis - The engineer must download and analyse data stored by the classifier using software supplied by DfT, to assess the site’s performance. The data assessed includes average lengths, vehicle speeds, traffic distribution, proportion of error codes and classification fault indicators. Any values which fall outside of the tolerances provided at the start of the contract, could indicate a potential problem at the site and may require a fault to be initiated.

- In-road equipment tests - The engineer must test all in-road equipment installed. The inductive loops and piezoelectric sensors shall be tested by disconnecting them from the unit and conducting tests from the BNC connectors. The engineer must use the appropriate tests to ensure that the in-road equipment is not damaged and has not deteriorated over time.

The inductive loops and piezoelectric sensors must be reconnected to the unit correctly and the tested in the HI-COMM software. The following tests will be recorded in each lane:

- Waveform graphs shall be generated, showing the response from the in-road equipment when a vehicle passes over the site. The engineer should make efforts to collect waveforms from a variety of vehicle types such as light (cars/motorcycles) and heavy (HGVs) were possible. The waveform graphs should be recorded and assessed. Any necessary amendments should be made to optimise the response of the in-road equipment. For any installed cycle lanes, waveforms can be sourced from vehicles if pedal cycles are not seen during the recording period.

- A sensor test must be carried out to record the total number of vehicles passing over the site for a duration of 10 minutes. The performance of the inductive loop and each individual piezoelectric sensor shall be assessed for any imbalances. The results produced from each lane shall be reported.

- Data checks - The engineer must check the sites operation, after all of the amendments have been made to the site. Traffic shall be observed and comparisons made against the real-time data produced by the in the HI-COMM software. All erroneous observations should be noted at a vehicle-by-vehicle level and the site shall be amended to ensure it’s working correctly. The duration of the data checks must be appropriate, to ensure a sufficient number of vehicles have been observed, to have a level of confidence that the site is working correctly.

- Photographs - The engineer must submit photographs showing the following;

- The in-road equipment and road conditions

- Evidence of specific problems or defects in the road

- Each direction of traffic approaching the site

- The cabinet and its contents

- The quality of the path from the Contractor’s vehicle to the cabinet

- Fault information and traffic management details (if applicable) - Details of the suspected issues must be logged, including any recommendations. Details must be provided of traffic management requirements and the lane(s) that need to be assessed.

- General comments and any minor works undertaken - Details of any works completed and any observations made during the inspection must be recorded by the engineer. A final check must completed to ensure that all of the equipment has been reconnected, correctly configured and is communicating with the server.

## ANNEX J 	Example RSI / Job Visit report form (uses same pro-forma)

R.S.I. REPORT SHEET ATDC CONTRACT

| SITE NUMBER: |  | Location: |  |
| --- | --- | --- | --- |
| VISIT DATE: |  |  |  |
| JOB NUMBER: |  | DATE JOB RAISED: |  |
| JOB VISIT NUMBER: |  | JOB RAISED BY: |  |
| JOB TYPE |  |  |  |
| DETAILS OF REPORTED JOB: |  |  |  |

| SITE VISIT INSPECTION LIST | COMMENTS |
| --- | --- |
| In-Road Equipment (Sensors, Loops and Feeders) |  |
| Road Surface Condition (Rutting, Holes, Cracks) |  |
| Site Access routes (Paths, Steps and Walkways) |  |

| ELECTRICAL SAFETY INSPECTIONS (If applicable) |  |  |
| --- | --- | --- |
| Check undertaken | Visual Examination Comments |  |
| Adequacy and condition of Earthing/Bonding |  |  |
| Condition of wiring |  |  |
| Test Undertaken | Test Result and Comment |  |
| Power Supply Phase Check |  |  |
| Heater Operation / Thermostat setting |  |  |
| Backup Battery Voltage |  |  |
| RCD Functionality Tests |  |  |
| Test Type | 0 Phase | 180 Phase |
| Half test |  |  |
| Full test |  |  |
| Fast test |  |  |
| RCD test comments |  |  |
| NA |  |  |

| ALTERNATIVE POWERED SITES TESTS |  |  |
| --- | --- | --- |
| Item | Voltage | Comments, including weather conditions |
| Solar Panel Output (Volts) |  |  |
| Regulator Output (Volts) (combined output of solar panel and wind turbine if applicable) |  |  |
| Battery Voltage (Regulator output switched off) |  |  |
| Voltage and Load Test |  |  |

| PAT Testing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Visual Check | Earth Continuity <0.1 Ohm | Insulation >1.0MOhm | Polarity | Fuse |
| Power Lead (Lead) |  |  |  |  |  |
| Surge Device (Lead) |  |  |  |  |  |
| HI-Trac + Lead (Class 1) |  |  |  |  |  |
| Comments |  |  |  |  |  |  |

| EQUIPMENT INSPECTION LIST | COMMENTS | WORK UNDERTAKEN/REQUIRED |
| --- | --- | --- |
| Cabinet and fittings (Hinges Door Locks and Solar Panel if fitted) |  |  |
| Inroad equipment Termination Checks |  |  |
| Duct and seals |  |  |

Equipment Serial Numbers

| ELECTRONIC EQUIPMENT | SERIAL NUMBER | REPLACEMENT SERIAL NUMBER | RETURNS NUMBER | REASON FOR REPLACEMENT |
| --- | --- | --- | --- | --- |
| Hi-Trac |  |  |  |  |
| Power Supply |  |  |  |  |
| Primary |  |  |  |  |
| Secondary |  |  |  |  |
| Motherboard |  |  |  |  |
| Auxiliary board |  |  |  |  |
| Loop Profiler |  |  |  |  |
| Backplane |  |  |  |  |
| Front Panel |  |  |  |  |
| LCD Board |  |  |  |  |
| Loop Oscillator L1,2 |  |  |  |  |
| Loop Oscillator L3,4 |  |  |  |  |
| Piezo Board Slot 1 |  |  |  |  |
| Piezo Board Slot 2 |  |  |  |  |
| Cycle Board |  |  |  |  |
| Modem |  |  |  |  |
| Laptop USB Comms |  |  |  |  |
| Modem Comms |  |  |  |  |
| Comms PCB |  |  |  |  |
| Opto I/O PCB |  |  |  |  |
| EMS Amplifier |  |  |  |  |
| Surge Protector |  |  |  |  |
| IEC Lead |  |  |  |  |
| SIM Card (phone number) |  |  |  |  |
| IMEI Number |  |  |  |  |
| Bluetooth Module |  |  |  |  |
| Battery 6V |  |  |  |  |
| Battery 12V |  |  |  |  |
| Battery 12V Large |  |  |  |  |
| Battery 12V Large |  |  |  |  |
| Battery 12V Large |  |  |  |  |
| Other |  |  |  |  |
| Other |  |  |  |  |

Performance Analysis from Stored Data

| Data Description | Lane 1 Results | Lane 2 Results | Lane 3 Results | Lane 4 Results |
| --- | --- | --- | --- | --- |
| Average car length |  |  |  |  |
| Average car spacing |  |  |  |  |
| Average car speed |  |  |  |  |
| Average car overhang |  |  |  |  |
| Lane Distribution |  |  |  |  |
| % Validity Code Error |  |  |  |  |
| Two Wheeled Motor Vehicle Percentage |  |  |  |  |
| Rigid Vehicle Data |  |  |  |  |
| Percentage N Classification |  |  |  |  |
| Light Goods Vehicle/Car Ratios |  |  |  |  |
| Check Class Index counts for consistency |  |  |  |  |
| Chassis Code Between 125 and 135 |  |  |  |  |
| Comments |  |  |  |  |

Loop Test Results

| LOOP | Impedance To earth | Series Resistance | Inductance | Cable Type |
| --- | --- | --- | --- | --- |
| Loop 1 |  |  |  |  |
| Loop 2 |  |  |  |  |
| Loop 3 |  |  |  |  |
| Loop 4 |  |  |  |  |
| Loop 5 |  |  |  |  |
| Loop 6 |  |  |  |  |
| Loop 7 |  |  |  |  |
| Loop 8 |  |  |  |  |
| Comments |  |  |  |  |

Sensor Tests Prior to Installation (if applicable)

| Location | Sensor Serial Number | Date of Manufacture | Impedance Core/Screen | Cap/nF |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |
| 11 |  |  |  |  |
| 12 |  |  |  |  |

Sensor Test Results

| Axle Sensor | Impedance |  | Cap nF | Raw Noise mV | Raw Peak Signal |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sig/Scr | Scr/Earth |  |  | +ve | -ve |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 (cycle) |  |  |  |  |  |  |
| 10 (cycle) |  |  |  |  |  |  |
| 11 (cycle) |  |  |  |  |  |  |
| 12 (cycle) |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |

| Axle Sensor Lengths |  |
| --- | --- |
| Lane | Length/m |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |

Sensor Waveforms for Vehicle Lanes

| Waveform Number | Lane 1 |  | Lane 2 |  |
| --- | --- | --- | --- | --- |
|  | Light Vehicle Description | Heavy Vehicle Description | Light Vehicle Description | Heavy Vehicle Description |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

| Waveform Number | Lane 3 |  | Lane 4 |  |
| --- | --- | --- | --- | --- |
|  | Light Vehicle Description | Heavy Vehicle Description | Light Vehicle Description | Heavy Vehicle Description |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Sensor Waveforms for Cycle Lanes

| Waveform Number | Lane 5 |  | Lane 6 |  |
| --- | --- | --- | --- | --- |
|  | Light Vehicle Description | Heavy Vehicle Description | Light Vehicle Description | Heavy Vehicle Description |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Sensor Test Results. (For 10 Minute interval Test)

| Lane Number | Sensor 1 Count | Sensor 2 Count | Loop Count | Vehicle Count |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

| Details (start and finish times) of operational checks undertaken: ½ hour for RSI, 1 hour for motorways and 2 hours for other roads. | Start Time |  | First Vehicle Time |  |
| --- | --- | --- | --- | --- |
|  | End Time |  | First Vehicle Number |  |
|  | Duration |  | Last Vehicle Time |  |
|  |  |  | Last Vehicle Number |  |
| Comments on Mis-Classified Vehicles |  |  |  |  |  |

| Possible sources of influence on Traffic Data | Comments |
| --- | --- |
| Speed limit at classifier? |  |
| Is there a different speed limit sign in sight? |  |
| If yes, record limit and direction from classifier |  |
| Is car parking likely? |  |
| Is a bus stop nearby? |  |
| Is a school nearby? |  |
| Is a works entrance nearby? |  |
| Are there shops nearby? |  |
| Is there a pedestrian crossing nearby? |  |
| Are there traffic lights nearby? |  |
| Loading/Un-loading nearby? |  |
| Traffic calming |  |
| Other factors that may influence site data collection. |  |
| Grid reference |  |
| Any changes since previous site visit. |  |

Details of all minor work Undertaken During R.S.I.

Details of all Outstanding Fault works and recommendations

| Details of Traffic Management Requirements and Lanes that need to be Accessed |  |
| --- | --- |
| Marker Post Number | N/A |

General Comments

Details of Any Public Engagements or Interactions During Visit

Stock Usage

| Serial No. or Batch | Quantity | Description |
| --- | --- | --- |

Chargeable Works Undertaken

| Type | Quantity |
| --- | --- |

| Pre-Site Departure Activities | Comments |
| --- | --- |

| Engineers Details |  |
| --- | --- |
| Name of Engineer Attending R.S.I. |  |

## ANNEX K	Initiating a job

All jobs must be formally initiated by DfT only, whether the issues have been found by the Contractor or by DfT.

DfT will contact the Contractor and obtain next job identification (Job ID) number from the job management system maintained by the Contractor.

DfT will then issue to the Contractor a job initiation form (example below from the current contract) this will be submitted to the Contractor. The job initiation form will include the following information: -

- Job ID number

- site number

- site name

- road type

- date and time that job is raised

- job type

- job details

- whether the job is data affected fault (DAF) or data not affected fault (DNA)

If it is a Data Affected Fault (DAF), this information will include: -

- The lane or lanes affected

- The problem being seen

- The duration the problem has been identified for or if the problem occurs only during particular hours of the day the hours effected

If it is a Data Not Affected (DNA) fault i.e. a failure to download data, an explanation of the problem being experienced at the site shall be provided.

- The job initiation form will then be issued to the engineer attending the site by the Contractor.

- The job initiation form will be named in accordance to a file naming structure agreed between DfT and the Contractor at the start of the contract.

All subsequent job visits made to the site will then be referenced to that Job ID number, with the agreed naming structure.

EXAMPLE FORM USED TO INITIATE A JOB

| JOB NUMBER: |  |  |
| --- | --- | --- |
| SITE NUMBER: |  |  |
| SITE NAME: |  |  |
| ROAD TYPE: |  |  |
| DATE AND TIME ISSUE REPORTED TO SUPPLIER: |  |  |
| JOB TYPE: | Loop fault Sensor fault No contact Road repair Site installation Site decommissioned Other |  |
| JOB DETAILS: |  |  |

Data Affecting Fault (DAF)	            		Data Not Affected (DNA)

| Data Affecting fault Location of Problem |  |  |  |
| --- | --- | --- | --- |
| Lane Details | Loop | Sensor | Sensor |
| Lane 1 Loop and Sensors 1/2 |  |  |  |
| Lane 2 Loop and Sensors 3 /4 |  |  |  |
| Lane 3 Loop and Sensors 5/6 |  |  |  |
| Lane 4 Loop and Sensors 7/8 |  |  |  |
| Lane 5 Cycle Sensors 9/10 |  |  |  |
| Lane 6 Cycle Sensors 11/12 |  |  |  |
| All Data Affected |  |  |  |
| Duration/Periods Fault Occurs |  |  |  |

## ANNEX L 	Job types

L1  Faults

- Health and safety - This is a priority for DfT and any concerns must be raised and dealt with with the highest priority. Any issues found with the access routes to the site, the cabinet location or overgrown vegetation must be resolved. The number of fault visits cannot be estimated as it is dependent on the health and safety concern raised. It is expected this type of fault will take one visit to resolve.

- Administrative visit - Required for all open faults at the start of the contract to ensure effective handover. It is expected that only one visit per open fault will be required.

- No contact - A site fails to communicate and stops sending data files to the server for a period of time, causing the site to fall behind the rest of the network. If it’s found that this is caused by factors outside of the Contractor’s control, like network coverage issues, then a fault visit may not be necessary. It is expected this type of fault will take one visit to resolve.

- Battery or mains power issues - At alternative powered sites, the battery may have discharged to an insufficient level to power the classifier. At mains powered sites, the power source may have been cut off to the site, causing the power from the back up battery to be used. It is expected this type of fault will take one visit to resolve.

- Road repairs - When the road condition at the site deteriorates, the damage can cause potholes, cracks and other problems in the road surface. The inductive loops and piezoelectric sensors may become unsecured in the road. This could also cause further damage if left unresolved and cause potential problems to the in-road equipment. It is expected this type of fault may take two visits to resolve.

- Replacing the inductive loops and/or piezoelectric sensors - The in-road equipment can deteriorate over its lifetime which can reduce the data quality. The expected number of visits required to resolve the issue is dependent on the road type. It is expected to take five visits for a motorway site, four visits for a major road site, and three visits for a minor road site. There may be access restrictions with some local authorities or agencies, which may affect the time allocated to resolving the fault.

- Defective classifying unit and/or its associated components - A classifier and/or its components may can become faulty. On some occasions the cables/leads may become loose or disconnected. These factors can affect the data quality or produce no data output. It is expected this type of fault will take one visit to resolve.

- Repair to damage - There may be occasions where accidental damage may occur at the site caused by Third Party works. This can include a road being resurfaced or accidental damage to the in-road and/or off-road equipment. The Contractor shall discuss with DfT the actions to be taken to rectify the fault and this must be agreed before any works are carried out. The number of fault visits cannot be estimated as it is dependent on the extent of the damage and the resolution agreed with DfT.

L2  Amendments to the network

- Site surveys - DfT may propose potential new locations for new sites to be installed. The Contractor will be instructed to conduct a desktop assessment (using maps and street views) to ascertain which locations look suitable and should be investigated further.

DfT may then require physical site inspections to be completed at likely locations. The Contractor must attend the location during “usual” traffic conditions and report details including;

- Site observations

- Grid reference

- Road name and address

- Details of the road (speed limits, road condition and number of lanes)

- Proposed roadside cabinet location

- Direction of traffic on the roadside cabinet side

- Traffic management requirements

- Parking for service vehicles

- Communication signal strength

- Traffic behaviour

- Distance to nearest junction/traffic lights/crossings

- Technical drawings and photographs of the proposed location

A site survey report must be submitted to DfT to be reviewed. It is expected that a site survey will take one visit to complete.

- Site installation - The Contractor is responsible for installing a new site, once authorised by DfT. This involves installing all in-road equipment, the roadside cabinet and its contents. Equipment from DfT storage facility will be used for the installation and can consist of re-used equipment that is in good working order or brand new equipment purchased using DfT approved methods. The Contractor is also responsible for obtaining all permissions from the relevant local authority or highways authority to install the site at the proposed location.

After permission has been granted by the authority, the Suppliers must obtain documentation from the Land Registry for the plot of land where it is intended the roadside cabinet will be installed, to ensure that it is publicly owned.

All evidence must be submitted to DfT and authorised prior to the installation. The Contractor must arrange all traffic management. The site shall be installed using the relevant civils, mechanical and electrical works to complete the install. The in-road equipment must be tested before and after being installed and all equipment serials numbers must be recorded. The expected number of visits required to install a new site is dependent on the road type. It is expected to take five visits for a motorway or other major road site, and three visits for a minor road site, including one visit to commission the site, as described in the following paragraph. There may be access restrictions with some local authorities or agencies, which may affect the time allocated to the installation.

- Site commissioning - Following a site installation, relocation, sensor or loop replacement or upgrade, the site must be setup to send good quality data. Checks are carried out to confirm that the installation has been carried out in accordance with procedures. Also, equipment checks must be conducted check they are connected correctly, adjusting if necessary.

The site will also undergo its first official operational check similar to an RSI, excluding the site performance analysis as there is no historical data available prior to the setup. These checks must be carried out during “usual” traffic conditions. DfT will carry out extensive data checks before approving the installation works. Sites should be commissioned (or re-commissioned) following the works, where appropriate or by the end of the next working day, and should take no more than one visit to complete.

- Site decommissioning (removal) - This is the complete removal of a site so that it is no longer operational. The roadside cabinet and relevant equipment must be removed using the relevant civils, mechanical and electrical works. An effort must be made to salvage usable equipment and this must be returned to DfT’s storage facility. Any faulty or damaged equipment must be disposed of using the appropriate methods. The in-road equipment will be left in situ and checked to ensure it is secured in the road. The Contractor is responsible for ensuring the site is left in a safe condition and doesn’t pose a risk to the general public. It is expected to take one visit to decommission a site.

- Site mothballing - There may be occasions where a site may need to be kept in working order, but the sites operation is suspended and its data are not used. This site will be left dormant, but can be restored if required at a later date. This will include removal of all equipment located in the roadside cabinet and the temporary removal of the power source. The roadside cabinet and the in-road equipment will remain in place. It is expected to take one visit to mothball a site.

- Site relocation - This is the complete removal of a site from its existing location and then re-installing the site at a new location, or at a location greater than 100 metres from its original location. This may be due to the site being unsuitable, poor road conditions or the road being resurfaced at the existing location. The new location must be agreed prior to the installation and must follow the procedures set out in the site installation section. A site survey may be requested by DfT, prior to the installation. The expected number of visits required to relocate a site is dependent on the road type. It is expected to take five visits for a motorway and a major road site, and three visits for a minor road site. There may be access restrictions with some local authorities or agencies, which may affect the time allocated to the installation.

- Site upgrades - Occasionally DfT may require upgrades to sites – for example the installation of a new type of classifier, new or additional sensor arrays, or changes to the power supply. In all cases, the engineer must check the operation to after the upgrade to ensure it. The expected number of visits will depend on the type of upgrade.

- Trials and testing - The DfT may also occasionally require assistance with trials of equipment, new technologies or maintenance facilities. Timetables, plans of action and costs for any such work will be agreed in advance between both parties. The work is likely to draw on the Suppliers’ technical expertise and experience of managing and carrying out roadside projects.

## ANNEX M 	Summary of Jobs in the ATC network, 1 Nov 2017 – 24 June 2022

Table M1: Jobs raised and job visits take to clear, by job type

| Job type | Number of Jobs raised | Total Job Visits to complete 1 |
| --- | --- | --- |
| All faults | 1,075 | 1,761 |
| Battery issue | 210 | 228 |
| Cabinet issue | 22 | 30 |
| Counter issue | 51 | 56 |
| Data issue | 16 | 27 |
| Environment | 12 | 12 |
| No contact/ comms issue | 152 | 191 |
| Power/ mains issue | 18 | 19 |
| Repair to damage/ road repair 2 | 75 | 95 |
| Replace sensors/loops 3 | 454 | 1041 |
| Sensor issue 4 | 65 | 62 |
| All site amendments | 300 | 440 |
| New site | 70 | 206 |
| Site upgrade 5 | 225 | 229 |
| Reconfiguration | 1 | 1 |
| Decommissioning 6 | 4 | 4 |
| All Jobs 7 | 1,375 | 2,201 |
| Notes: 1. A total of 274 out of these 1375 Jobs were initiated as a result of Routine Site Inspections (RSIs). In this table the RSI visit has not been counted towards the total job visits. Over the period covered by this table, a total of 3,028 RSIs were carried out, including the 274 leading to Jobs. 2. Generally repairs to in-road equipment, such as resealing sensor or repairs to the adjacent road surface. Damage to roadside equipment generally included in ‘Cabinet issue’. 3. Includes replacement of one or more sensors/loops, up to full re-installation of all in-road equipment, or of whole site including roadside equipment. 4. For the purposes of this table, includes jobs raised to investigate a sensor issue which were resolved without ‘repair to damage’ or replacement, for example by resetting the equipment, or no fault being found. 5. Includes upgrades to classifiers, also additional sensors, particularly cycle sensors. 6. Includes temporary decommissioning of sites expected to be re-instated later, for example after roadworks. 7. Figures do not include approximately 100 site inspection visits carried out as part of the survey process for new sites. |  |  |

Table M2: Jobs raised by month initiated

| Job type | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All faults | 176 | 106 | 103 | 75 | 79 | 77 | 56 | 54 | 63 | 57 | 102 | 127 | 1,075 |
| Battery issue | 62 | 19 | 13 | 1 | 2 | 2 | 6 | 1 | 1 | 9 | 23 | 71 | 210 |
| Cabinet issue | 1 | 3 | 2 | 2 | 5 | 2 |  | 1 | 2 |  | 4 |  | 22 |
| Counter issue | 12 | 1 | 1 | 2 | 3 |  | 6 | 4 | 2 | 5 | 8 | 7 | 51 |
| Data issue | 2 |  | 2 | 2 | 1 | 1 | 2 | 1 | 3 |  | 2 |  | 16 |
| Environment |  | 1 |  |  |  | 3 |  | 3 | 4 |  | 1 |  | 12 |
| No contact/comms issue | 30 | 20 | 13 | 13 | 8 | 16 | 13 | 6 | 8 | 7 | 8 | 10 | 152 |
| Power/mains issue | 1 |  | 6 |  |  | 6 |  | 2 | 1 | 1 |  | 1 | 18 |
| Repair to damage | 9 | 8 | 8 | 6 | 7 | 9 | 2 | 2 | 7 | 6 | 6 | 5 | 75 |
| Replace sensors/loops | 55 | 48 | 49 | 45 | 46 | 36 | 23 | 29 | 31 | 24 | 40 | 28 | 454 |
| Sensor issue | 4 | 6 | 9 | 4 | 7 | 2 | 4 | 5 | 4 | 5 | 10 | 5 | 65 |
| All site amendments | 57 | 25 | 2 | 52 | 3 | 6 | 3 | 2 | 57 | 45 | 29 | 19 | 300 |
| New site | 20 | 10 | 2 | 3 | 1 | 4 | 3 |  |  | 1 | 15 | 11 | 70 |
| Site upgrade | 37 | 15 |  | 49 | 1 |  |  | 2 | 57 | 42 | 14 | 8 | 225 |
| Reconfiguration |  |  |  |  |  | 1 |  |  |  |  |  |  | 1 |
| Decommissioning |  |  |  |  | 1 | 1 |  |  |  | 2 |  |  | 4 |
| All Jobs | 233 | 131 | 105 | 127 | 82 | 83 | 59 | 56 | 120 | 102 | 131 | 146 | 1,375 |

ANNEX N	Minimum Weekly and Monthly reporting requirments

The service provider must provide 3 weekly and one monthly report summarising maintenance activities and the state of the network.

Deliverable 4a - Weekly Job Summary Report

Scope: 	All jobs that remain open at end of week.

Timing: 	Weekly, submitted to Authority by 5pm each Friday.

Format: 	PDF unless otherwise agreed.

Content: 	At least the following:

•	List of all open Jobs, with a brief description of the job, history of all associated job visits with brief description of work carried out and progress details of associated communications/arrangements. Also, a count in days that the job has been open with day one being the day after the job was raised.

•	List of Jobs awaiting Traffic Management, with brief details

•	List of Jobs with Traffic Management bookings, with brief details

•	List of Jobs awaiting DfT clearance, with brief details

Deliverable 4b -  Cumulative weekly Job Status Report

Scope: 	All Jobs raised, and all Job Visits and RSIs carried out from beginning of contract. One data record for each instance of one of these.

Timing: 	Weekly, submitted to Authority by 5pm each Friday.

Format: 	Excel unless otherwise agreed. (Key requirement is for the Authority to be able to readily ingest the file into their database for use in analysis of the count data).

Content:

| Field | Description |
| --- | --- |
| Site_number | ATC site number |
| Job_number | Job number (blank for an RSI record type, unless a job is raised in association with the RSI) |
| Visit_number | Visit number in chronological sequence by job, for Job record types. Blank for Job record types. Blank for RSI record types, unless an RSI and a job visit are carried out together, in which case Visit_number = 1 |
| Data_affected | Whether site data are believed to be affected by the fault (if there is one) - Y/N |
| Lane_1_affected | Whether data for lane 1 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_2_affected | Whether data for lane 2 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_3_affected | Whether data for lane 3 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_4_affected | Whether data for lane 4 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_5_affected | Whether data for lane 5 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_6_affected | Whether data for lane 6 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_7_affected | Whether data for lane 7 are believed to be affected by the fault (if there is one) - Y/N |
| Lane_8_affected | Whether data for lane 8 are believed to be affected by the fault (if there is one) - Y/N |
| Start_date | For Job visit and/or RSI records - Start date of the visit; for Jobs - date the job was raised |
| Start_time | For Job visit and/or RSI records - Start time of the visit; for Jobs - time the job was raised |
| End_date | For Job visit and/or RSI records - End date of the visit; for Jobs - date the job was closed |
| End_time | For Job visit and/or RSI records - End time of the visit; for Jobs - time the job was closed |
| Type | Job, Job visit, or RSI |
| Engineer | For Job visit and/or RSI records -Identity of the engineer carrying out the visit; for Job records - DfT or the supplier project manager who initiated the job |
| Job_Description | Brief description of the job, using standardised terms |
| Work_Undertaken | Brief description of work actually undertaken |

Deliverable 4c -  Cumulative weekly RSI Status Report

Scope: 	All RSIs carried out from beginning of contract, with key details. One data record for each instance of one of these.

Timing: 	Weekly, submitted to Authority by 5pm each Friday.

Format: 	Excel unless otherwise agreed.

Content:

| Field | Description |
| --- | --- |
| Site_number | ATC site number |
| Site name |  |
| Completed | Date visit completed |
| Engineer | Identity of the engineer carrying out the visit |
| Site_damage | Notes of any damage to site |
| Backfilling | Notes of any repairs needed to sensor/loop infill |
| Road_surface | Notes of any repairs needed to road surface |
| Classifier_number |  |
| SIM_Card_(phone_number) |  |
| IMEI_Number | A second reference for SIM card |
| Battery/Mains | Whether mains powered |
| Speed_limit | Speed limit of road at counter location |
| Grid_reference | OS Grid Reference (by convention, of top of roadside cabinet) |
| Minor_works | Note of any minor rworks carried out at inspection |
| Comments |  |

Deliverable 5 - Cumulative Monthly Report

Scope: 	Summary of all activity by site from beginning of contract.

Timing: 	Monthly, submitted to Authority by 5pm each Friday.

Format: 	PDF unless otherwise agreed.

Content: At least the following:

a) KPI analysis (see Annex R of Statement of Requirements)

b) 	Lists of Jobs:

- raised during month

- open at end of month

- visited during month

- cleared during month

- awaiting 3rd Party at end of month

- with ongoing 3rd Party Liaison at end of month

- awaiting Traffic Management at end of month

- with Traffic Management bookings at end of month

- with ongoing Health & Safety issues at end of month

- awaiting DfT clearance at end of month

c) 	List of RSIs Performed during month – including date, attending engineer

d) RSI Histories by site – dates of all RSI visits to date

e) 	Job History by site – full listing of all jobs and associated job visits since beginning of contract

f) List of Sites with no Jobs raised to date

g)	List of Sites decommissioned to date

Sections b and e should include details of job and of all associated job visits to date, with relevant details such as dates opened and closed, brief description of job and work done, associated staff, number of days open

## ANNEX O 	Example of engineer’s equipment

Suggested list of equipment for each site engineer working on the contract.

Measurement equipment requires calibration to the relevant standard.

- Dual Channel Digital Storage Oscilloscope and probe.

- 1000v Megger type insulation test meter that has a 100V range.

- Portable LCR meter including leads.

- Residual current & earth loop tester including leads.

- Mains supply phase tester.

- PAT Tester including leads.

- Laptop computer pre-installed with the relevant software, antivirus software and the appropriate operating system.

- Lightweight portable printer and paper.

- Appropriate mains extension cable (25 or 50m preferably).

- 12V 600W vehicle powered Mains power supply inverter.

- Battery voltage and load tester.

- General and comprehensive tool kit containing general fitting and electrician’s tools.

## ANNEX P 	Example of DfT’s stock held at the DfT’s storage facility

Note: The information in this table is based on the summer 2022 stocklist. The quantities and stock listed are indicative and are subject to change.

| Stocklist | Quantity |
| --- | --- |
| HI-TRAC classifier, EMU3 | 43 |
| HI-TRAC classifier, other | 35 |
| Piezoelectric sensor, various lengths | 342 |
| Loop cable (1000m reels) | 5 |
| PU200 resin (tins) | 257 |
| SIM Card | 38 |
| EMS Amplifier | 8 |
| Antennas (standard, Oscar 1, Oscar 17) | 70 |
| Loop Oscillator | 304 |
| Loop Profiler | 35 |
| Modems, various | 307 |
| Boards, various (primary, secondary, piezo, cycle) | 313 |
| Front Panel | 39 |
| Backplane | 29 |
| IEC Lead | 3 |
| Aux PCB | 20 |
| LCD Unit | 3 |
| TDU Profiler | 4 |
| Battery_12V | 46 |
| Battery_6V | 82 |
| Solar cable kit | 227 |
| Solar panel 10W | 42 |
| Solar panel 20W | 13 |
| Solar panel mounting | 8 |
| Regulator_Solar | 66 |
| Windcharger | 7 |
| Surge Protector | 57 |
| Regulator, 12V | 15 |
| Pole, 6m, rigid | 2 |
| Spiked razor collar | 5 |
| Cabinet, large | 5 |
| Cabinet, small | 13 |
| Cabinet lock | 40 |
| Plus various consumables (adhesive, BNC connectors, fuses, RCDs, cabinet heaters etc.) |  |

## ANNEX Q 	Example of DfT’s spares replenished by the Contractor

A list of the kinds of equipment in the current contract that are sourced by the Contractor. Items listed shall be charged at cost price plus a contract mark up. When billing DfT, proof of the cost price must be supplied with the relevant invoice.

- Classifiers spares & accessories

- GPRS compatible modem

- Wiring loom for classifier cabinet

- 12V battery

- 6V battery

- Solar panels (10 & 20W) & accessories

- Mains distribution panel

- Standard Roadside Cabinet c/w:-Mains Distribution, Surge Protection Devices, Sensor Connection Board and Door Switches

- BL axle sensor 1.50, 2.00, 2.25m (used as cycle sensors)

- BL axle sensor 2.50, 2.75, 3.00, 3.25, 3.50, 4.00, 4.50, 5.00m (class 1 or 2)

- Loop cable per 100m reel

- 16mm oval EGA tube per 3 metre length

- 10mm copper tube per 40m reel

- Epoxy resin

- BNC connectors (50 Ohm Crimp plug)

- Battery and charging system tester

- Pot hole repair kit

- Penetrating oil/maintenance spray

- GPRS enabled SIM cards

- Silicone

- Antenna

- Fuses

- Test equipment for trials

- Locks for cabinets etc.

- Amp fuses

- Labels and signs

## ANNEX R 	Initial KPIs & Performance Measures

| Ref | Service area | KPI/SLA description | Target |
| --- | --- | --- | --- |
| Theme 1: Resourcing |  |  |  |
| 1.1 | Resources are available to deliver the full schedule of work specified by DfT. | Percentage of RSI visits completed within their scheduled month. | 95% |
| 1.2a | Resources are available to deliver the unscheduled maintenance of the network raised by the DfT. | Percentage of visits made within 2 working days of fault being raised. | 90% |
| 1.2b |  | Percentage of visits made within 3 working days of fault being raised. | 100% |
| Theme 2: Health & Safety |  |  |  |
| 2.1 | Contractor adheres to and/or maintains accreditation to the relevant health and safety requirements. | Number of complaints/evidence of non-compliance. | Green=0, Amber=0, Red=1+ |
| 2.2 | Contractors’ measures of ensuring health and safety. | Provide and maintain relevant training for staff. Having robust systems in place for monitoring/assessing that staff working are within the relevant procedures required. | Agenda item in the monthly contract meeting. |
| Theme 3: Quality & Efficiency |  |  |  |
| 3.1 | Data collected meets quality criteria. | Number of rejections or follow ups by DfT due to poor data quality following completion of all works and site setups compared to number works completed. | <1% |
| 3.2 | Efficiency of repair works. | Percentage of jobs completed within expected number of visits, taking into consideration the type of job. | 100% |

The Authority expects to discuss the KPIs with the supplier at the project inception meeting or very shortly after. The above is an initial priority set, but the Authority may wish to specify others, and will invite proposals from the supplier for new KPIs, or reasoned proposals for different measurement of existing ones.

For those KPIs with existing targets, green, amber and red performance thresholds are also generally defined to help in the assessment of performance. Where appropriate, independent thresholds may be set covering the whole of the contract to date and for the latest month, to assist with the assessment of long term and short-term performance.

As described in Section 15, the Authority may also use a range of other sources of information in the assessment of supplier performance, in addition to the specific quantitative measures shown here.