- SPECIFICATION

and

- PRE TENDER HEALTH & SAFETY PLAN

for

- WORKS TO BE EXECUTED AND MATERIALS TO BE USED IN

REFURBISHMENT OF BUILDING No. 219 TO STORE RADIOACTIVE MATERIAL

CONTRACT NO:

IWM/DUX/1574

for

- IMPERIAL WAR MUSEUM

DUXFORD AIRFIELD

DUXFORD

CAMBS

- Client:

- Imperial War Museums

Duxford

Cambs

CB22 4QR						Date September 2016

SECTION 1

Preliminary Matters

1.0 	Introduction

1.1	PRELIMINARY PARTICULARS ETC.

1.2	GENERAL MATTERS

1.3	STATUTORY/GENERAL OBLIGATIONS6

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.6	THE WORKS GENERALLY

1.7	CONTRACTUAL MATTERS

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.9	APPROVED SUB-CONTRACTORS

1.10	PROVISIONAL DAYWORKS

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

1.12	LIAISON

1.13	PHASING OF WORKS

1.14	TEMPORARY SIGNAGE

1.15	SITE RULES

1.16	SECURITY CLEARANCE

APPENDIX 1

SECTION 2

Health & Safety

PRE TENDER HEALTH & SAFETY PLAN

2.0	GENERAL REQUIREMENTS

2.2	Checking/Handling and Site Storage of Materials and Components

2.3	Loading.

2.4	Working at Heights

2.5	Proximity to Public

2.6	Method Statements

2.7	Pre-Tender Health & Safety Plan

2.8	Client’s own Safety Representative

2.9	Health & Safety File

SECTION 3

Trade Preambles

3.01	MATERIALS		WORKMANSHIP

SECTION 4

SCHEDULE OF WORKS

SECTION 5

TENDER SUBMISSION

5.0	TENDER SUBMISSION

5.1	TENDER ACCEPTANCE

5.2	BASIS OF PRICES

5.3	AWARD CRITERIA (other than price)

5.4	PRICING SCHEDULE

SECTION 6

POINTS OF CONTACT

SECTION 7

CONTRACT TIMETABLE

7.0	CONTRACT TIMETABLE

SECTION 1

Preliminary Matters

1.0 	Introduction

  - An Act of Parliament founded the Imperial War Museum in 1920 following the Great War to act as a National War Museum to collect and display material relating to that War. Due to the general interest displayed by the Dominion Governments this led to the title “Imperial War Museum” (IWM).

  - The Imperial War Museum was opened in 1920 at a site in Crystal Palace, but eventually acquired its current home at Lambeth Road in July 1936 on the site of the former Bethlem Royal Hospital.

  - At the commencement of the Second World War the Museum’s terms of reference were extended to cover both the Great War and the Second World War, and again in 1953 to cover all military operations involving British & Commonwealth forces since August 1914.

  - In addition, the IWM provides exhibits of educational and general interest to members of the public.

  - IWM Duxford is a former RAF fighter station originally dating from the First World War. It has a long and distinguished history from 1924 until its closure as an RAF Station in 1961 and is particularly famous for its role in the Battle of Britain in 1940.

  - Today, IWM Duxford is one of the world's leading aviation museums, housing some 200 aircraft ranging from flimsy biplanes to modern day jets. Many of the aircraft at IWM Duxford still fly. The airworthy aircraft at the Museum belong to private collectors and operators and are an important part of IWM Duxford's unique living atmosphere. IWM Duxford is internationally renowned for its annual programme of world-class air shows.

  - The American Air Museum stands as a memorial to the 30,000 Americans who died flying from bases in the UK including IWM Duxford, during the Second World War. This outstanding example of modern British architecture houses IWM Duxford's unique collection of historic American combat aircraft including the SR-71 Blackbird, the only one of its kind on display in Britain and the B-24 Liberator. The arrival of these new stunning exhibits led to the reconfiguration of the American Air Museum, and the Rededication Ceremony on 27 September 2002, with former president of the US, George Bush and HRH the Prince of Wales present.

  - IWM Duxford is not just about aircraft and the Museum houses an outstanding collection of tanks, military vehicles and artillery displayed in walk through battle scenes in the Land Warfare Hall.

  - As of 2016, Imperial War Museums consists of 4 other major public tourist attractions:

IWM London, Lambeth Road

HMS Belfast moored in the Pool of London

Churchill War Rooms

IWM North, Trafford, Manchester

1.1	PRELIMINARY PARTICULARS ETC.

1.1.1	Employer

- Imperial War Museums

Lambeth Road

London

SE1 6HZ

1.1.2	Contract Administrator

- J P Banks

- Capital Projects Manager

Imperial War Museums

Duxford

CB22 4QR

T: 01223 497243

E: jbanks@iwm.org.uk

1.1.3 	Principal Designer

IWM represented by J P Banks as above.

1.1.4	General Description of the Works

The works comprise of the Refurbishment of Building No.219 at IWM Duxford.

1.1.5	Location and Access to Site

The site is located immediately to the east of the M11/A505 junction in South Cambridgeshire.

Access into the site is from the A505 to the south side and via the staff entrance/guardroom.

1.1.6	Tender Drawings

The following drawings form part of this specification and will become contract documents.

: Proposed repair details and alterations for concrete ring beam

1.2	GENERAL MATTERS

1.2.1	Limits of site operations

The contractor shall limit his operations to the immediate area of the works.

The contractor shall be responsible for storing materials, arranging for loading/unloading materials and organizing his works in a manner that does not interfere with other parts of the site and access to same.

1.2.2	Preliminary investigation

The Contractor shall be deemed to have visited and inspected the site and to have examined drawings (if any) and contract documents and to have adequately acquainted himself with local conditions, accessibility of the works and site, the nature of the ground and subsoil, supply of and conditions affecting labour, the availability and supply of materials, water, electricity and telephones, all in relation to the execution of the works as no claim on the ground of want of knowledge in such respect will be entertained.

1.2.3	Abbreviations and references

- The following abbreviations, references or terms are used in the Specification.

CP 	- 	British Standards Code of Practice

BS 	- 	British Standard Specification

The term ‘Contractor’ used throughout this document relates to the Contractor who is contracted to carry out and complete the works. The contractor shall be the ‘Principal Contractor’ under the CDM regulations.

Reference to CA means the Contract Administrator or other consultant, when appointed; with authority under the terms of the contract to act is such a capacity.

The reference to Building Inspector means Building Control Officer, District Surveyor or any other person with authority under statute to require compliance with any regulation or byelaw.

The terms ‘Approved’, ‘Selected’, or ‘Directed’ means the approval, selection, or direction of the client/CA.

The term ‘Provide’ means that the item(s) is to be supplied, delivered and fixed at the expense of the builder.

The term ‘Day Work’ is applicable to the work of an unknown or unforeseen nature that could not be specified and therefore estimated for during the preparation of the schedule of works.  Such work is usually paid for on a time and material basis with the Contractor submitting evidence of hours of work and invoices for materials, plant, etc.

1.3	STATUTORY/GENERAL OBLIGATIONS

1.3.1	Safety/Health and Welfare

Allow for complying with all enactments, regulations and working rules relating to safety, health and welfare of work people.

The Contractor’s attention is drawn to Section 2 of this Specification, which covers Health & Safety matters specifically.

These works are subject to the CDM.

1.3.2	Working Hours

Monday to Friday - 0730 - 1700.  Weekend working will only be allowed with the prior agreement of Employer. The contractor may extend working hours on weekdays subject to approval of Employer.

1.3.3	Statutory Regulations

The works must be undertaken in compliance with Statutory Regulations, Building Regulations, Gas Safety Regulations, Water Bye Laws and IEE Regulations.  Contractor is to allow in his tender for compliance, particularly in respect of service installations, which require contractors design input.

1.3.4	Noise

Fit all compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles.

1.3.5	Public and Private Services

Adequately protect, uphold, maintain and prevent damage to all services.  Do not interfere with their operation without consent of the service authorities or private owners or the CA as appropriate.

1.3.6	Existing Features

Prevent damage to existing buildings, fences, gates, walls, roads, paved areas, other of the site features, which are to remain in position during the execution of the works.

The contractor shall be responsible for properly making good to the satisfaction of the C.A. all areas disturbed during the execution of the works. Contractor is to plan the works and in particular the maneuvering of plant and machinery so as to cause minimal disturbance to adjacent areas.

1.3.7	Structural Fabric

The contractor is advised the hangars are grade 2* listed buildings. Works must be undertaken in a manner that causes the minimum of damage/disturbance to the fabric of the building.

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.4.1	Supervision

Accept responsibility for co-ordination, supervision and administration of the works, including subcontractors.  Arrange and monitor a programme with each subcontractor, supplier, Local Authority and statutory undertaker and obtain and supply information as necessary for co-ordination of the work.

The Contractor is to provide a competent person to constantly supervise the works and shall inform the CA of the name of such person.

The person is to be able to adequately receive directions given to him by the CA and is to remain on site throughout the duration of the contract and shall not be changed without prior approval from the CA.

The person is to implement the requirements of the Construction (Design & Management) Regulations 2015 as they apply to this project, and must assist the Planning Coordinator in maintaining the health and safety file for this project.

1.4.2	Insurances

Before starting work on site submit to the employer (through the CA) documentary evidence and/or policies and receipts for all insurances which are required to be taken out by the Contractor.

1.4.3	Programme

As soon as possible and prior to starting work on site, prepare, in an approved form, a programme for the works that should make allowance for all:

1.Subcontractors works including completion of drawings etc.

2. Work resulting from instructions issued with regard to the expenditure of provisional sums.

3. Other work concurrent with the contract.

4. All operatives are to carry up to date CSCS cards

1.4.4	Commencement of Work

Inform CA as least seven working days before proposed date for commencement of the work on site.

1.4.5	Monitoring

Record progress on site and keep a copy of the programme on site.  Update or re-draft without delay if circumstances arise which affect the progress of the works with copies of all revisions to CA.

1.4.6	Site Meetings

Hold regular site meetings as necessary for the proper management and co-ordination of the contract and as required by the CA.  Meetings will normally be held every four weeks.  Attend all meetings and inform subcontractors and suppliers when their presence is required.  Notify all consultants.  The CA will take the Chair and minutes will be taken and distributed by the CA.

1.4.7	Notice of Completion

Give CA at least one week’s notice of anticipated dates of practical completion of the whole or parts of the works.

1.4.8	Measurements

Give reasonable notice to the CA before covering up work, which the CA requires to  be measured.

1.4.9	Covering Up

Give not less than one working days notice to the CA or Clerk of Works before covering up.

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.5.1	Accommodation

Provide necessary temporary sheds, offices, mess rooms, sanitary accommodation, and other temporary buildings required for Contractor’s own and domestic subcontractor’s use.

Routes from fire escape staircases away from the building must not be obstructed at any time.

Scaffolding/access platforms must not be placed so as to restrict access into/out of the building by any of the users, private, or commercial.

1.5.2	Protection

Provide temporary fencing, hoardings, screens, vans, planked footways, guard rails, gantries and the like as may be necessary for protecting the public and others, for the proper execution of the works and for meeting the requirements of the Local or other Authority.

The contractor will be required to fence the works to prevent unauthorized persons entering into the working area. Plastic mesh fencing held by metal posts, to 1.2m height is considered adequate. Contractor’s materials storage areas will need to be enclosed by Heras fencing, which is to be secured closed at the end of each working day.

1.5.3	Scaffolding

The contractor must allow in his tender for providing a safe means of access for operatives to all areas subject to works. In deciding what access equipment is necessary, the contractor must fully appraise the works, undertake a risk assessment and identify the most suitable method of operation.

Scaffolding must be in accordance with BSEN12811-1 (2003).

Unless the scaffold is a basic configuration described in recognized guidance, for example NASC publication TG20-08 (volume 1), the scaffold should be designed by calculation, by a competent person, to ensure it will have adequate strength and stability.

If proposing a scaffold that is termed a ‘basic’ scaffold under volume 1 of TG20-08, the contractor must demonstrate to the CA that the scaffold meets all of the criteria for a basic scaffold.

Any proposed modifications or alterations outside a generally recognized basic configuration must be designed by a competent person.

Where the scaffold falls outside the scope of a ‘basic’ scaffold, the contractor must supply to the CA detailed information to show compliance with the BSEN12811-1 standard. Further advice on what is required is contained in volume 2 of TG20-08. As a minimum, this must include drawings showing the general scaffold arrangement, a method statement describing the erection and dismantling sequence, the permitted platform loadings, restrictions on use and calculations to show compliance with the standard.

On completion of the scaffold erection, a handover certificate must be issued to the CA, which should refer to the relevant drawings, permitted working loads, and restrictions on use. The handover certificate must be issued by a competent person (by means of experience or assessment, appropriate for the type and complexity of the scaffold erected).

Where the scaffold is the subject of design the person responsible for the design shall make an inspection and certify the scaffold is erected in accordance with the design and drawings.

Regular inspection should be carried out (minimum weekly) also by a competent person.

Erection, alteration and dismantling of all scaffolds must be under the direct supervision of a competent person – an advanced scaffolder or design engineer with the appropriate industry experience.

The contractor is referred to HSE guidance note for a list of scaffold structures that need to be the subject of design.

For the avoidance of doubt, the means of access to any particular working platform must be arranged so that it is outside of the working platforms and does not obstruct or restrict their use. eg a ladder access tower, or platform steps. (The use of trap doors within the platforms will not be permitted).

If ladders are proposed for access to scaffold platforms, these must not be used for bringing materials or tools of any sort to the working platform. A separate arrangement must be made for bringing material to the platform level – hoists etc.

NOTE: The purpose of the scaffold is to facilitate safe access to undertake the various works described in later sections of this specification. Particular care should be taken in the siting and fitting of tubes and boards  – for example by ensuring there is sufficient (but not excessive) gap between the end of a transome and the wall, to enable the wall to be painted.

1.5.4 	Water

Water for the works is available on site free of charge. Contractor is to make temporary arrangements for storing and distributing around the site.

1.5.5	Electricity

The contractor is to make his own arrangements for the supply of power he may require.

1.6	THE WORKS GENERALLY

- GENERALLY

1.6.1	Good Practice

Where and to the extent the materials, products and workmanship are not fully specified they are to be:

1.	Suitable for the purposes of the work stated in or reasonably to be inferred from the contract documents, and

2.	In accordance with good building practice including the relevant provisions or current BSI documents.

1.6.2	Manufacturers Recommendations

Handle, store, prepare and use, or fix each product in accordance with manufacturers printed or written instructions/recommendations.  Inform CA if this conflict with any other specified requirement.

1.6.3	Workmanship

The works to be carried out by, or under the close supervision of, experienced tradesmen, skilled in the particular type of work.

1.6.4	Service Runs

Make adequate provisions for services, including unobstructed routes and fixtures.  Wherever possible, ducts, chases and holes to be formed during construction rather than cut.

- PRODUCTS/MATERIALS GENERALLY

1.6.5	Products to be New

Unless otherwise specified.  Ensure that the whole quantity of each product and material required to complete the work is of consistent kind, size, quality and overall appearance.

Handle, store and fix products with care to ensure that they are not damaged when incorporated into the works.

1.6.6	Or Equivalent Approved

Means that the products of different manufacturers may be substituted if prior approval has been obtained, but the CA reserves the right to insist on a named product(s) the rates or prices will be held to be based on the product(s) specified, unless agreed otherwise.

1.6.7	Proprietary Names

The phrase ‘or equivalent’ is to be deemed included whenever products   are specified by proprietary names.

1.6.8	Single Source

Where a choice of manufacturer or source of suppliers is allowed for any particular product or material, the whole quantity required to complete the works must be the same type manufacturer or source.  Do not change without approval.  Produce written evidence or sources of supple requested by CA.

- ACCURACY/SETTING OUT

1.6.9	Accuracy

- N/A

- PROTECTION/CONDITION OF WORK/DRYING OUT

1.6.10	Site administration and Security

Adequately safeguard the site, products, materials, plant, the works and any existing buildings affected by the works from damage and theft.  Take all responsible precautions to prevent unauthorized access to the site, the works and adjoining property.

1.6.11	Stability

Accept responsibility for stability and structural integrity of the works during the contract, and support as necessary.  Prevent overloading/damage.  Details of design loads may be obtained from the CA.

1.6.12	Inclement Weather

Adequately protect the works from damage by inclement weather and use all reasonable and approved building aids and methods to prevent or minimize delays during cold or inclement weather.

1.6.13	Rubbish

- Remove rubbish and debris as it accrues and keep the site and works clean and tidy.

1.6.14	Drying Out

Control the drying and humidity of the works and the application of heat to prevent:

1.	Blistering and failure of adhesion.

2.	Damage due to trapped moisture.

3.	Excessive movement.

- ALTERATIONS/EXTENSIONS/MAINTENANCE WORK

1.6.19	Make Good

Means carry out local remedial work, including the following as appropriate and necessary to leave the work sound and neat to approval:

1.	Remove defective parts of existing finishes and components and around any stated features.

2.	Fill, dress down, piece in, patch, extending existing finishes, make minor repairs and adjustments.

3.	Refix or re-stick.

4.	Redecorate.

5.	Consequent upon alteration, extension and maintenance work is deemed to be included in such items.

- SAMPLES/APPROVALS/TESTING/INSPECTION

1.6.22	Samples - materials

Where approval of products or materials is specified, submit samples or other evidence of suitability.  Do not confirm orders or use materials until approval has been obtained.  Retain approved samples on site for comparison with products and materials used in the works.  Remove when no longer required.

1.6.23	Samples - workmanship

Where samples of finished work are specified, obtain approval of stated characteristics before proceeding with the works.

Retain approved samples on site for comparison with the works.  Remove samples which are not part of the finished works when no longer required.

1.6.24	Approvals

Inspection of any other action by the CA must not be taken as approval of materials, products or work unless the CA confirms in writing the express terms referring to:

1.	Date of inspection.

2.	Part of the work inspected.

3.	Respect or characteristics which are approved.

4.	Extent and purpose of the approval.

5.	Any associated conditions.

- FIXING/FASTENINGS/ADHESIVES/MORTAR

1.6.25	Fixings Generally

Use fixings and jointing methods and types and spacings of fastenings which are suitable having regard to:

1.	Nature of compatibility with products/materials being fixed and fixed to.

2.	Recommendations of manufacturers of fastenings and manufacturers of products/material being fixed and fixed to.

3.	Materials and loads to be supported.

4.	Conditions expected in use.

5.	Appearance - being subject to approval.

- WORK AT COMPLETION/MAKING GOOD DEFECTS

1.6.26	Cleaning

Cleaning materials and methods are to be as recommended by manufacturer of product being cleaned.

In the absence of such recommendations cleaning materials and methods are to be approved by CA.

1.6.27	Certification

- Not required.

1.7	CONTRACTUAL MATTERS

1.7.1	Form of Contract

The Form of Contract will be the JCT Agreement for Minor Works 2016 Edition, together with amendments to date, including April 2015 amendments incorporating the CDM Regulations 2015 .

The Contractor should include for all costs incurred in compliance with the Terms and Conditions of the Contract.

- The Specification and drawings (if any) will become contract documents.

The Contractor must include for all works shown on the drawings, whether referred to in the Specification or not, and for all works which are not included in the Specification, or on the drawings which could reasonably be expected to be included in the works.

1.7.2	Appendix 1 to these Preliminaries

Appendix 1 at the end of these Preliminaries describes the contractual details that will be included in the Contract Conditions.

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.8.1	Contingency Sum

- Expenditure of such sum shall only be made on express authority of CA.

1.8.2	Provisional Sum

- Refer to section 4 for details of provisional sums allowed for the various sections of work.

1.8.3	Provisional Repairs

- Refer to section 4 as above.

1.8.4	Prime Cost Sums

- Not used.

1.9	APPROVED SUB-CONTRACTORS

Not used

1.10	PROVISIONAL DAYWORKS

Not used.

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

Building services and fabric are maintained on the site by an FM contractor appointed by the IWM. This contractor will be permitted access to the buildings at all times. The normal activities of this contractor are not expected to interfere with these contract works.

1.12	PHASING OF WORKS

N/A

1.13	TEMPORARY SIGNAGE AND FENCING.

The working area is to be enclosed by temporary fencing – ‘Heras’ interlocking fencing or similar, adequately braced to ensure it is robust and will not blow over. Fencing must be arranged so that is does not obstruct pedestrian access/egress from the building.

Contractor is to provide appropriate warning signs and notices in order to warn the public or site users.

- Contractor/subcontractor advertising will not be permitted.

1.14	SITE RULES

The Museum operate specific rules for ‘Safe Working Conditions and Protection of  its Staff, Visitors and Collection’. Please see IWM’s ‘Control of Contractors’ document attached.

These are available as a small booklet from the CA The contractor will be required to sign the form included in the booklet to confirm he has read the rules and will ensure all work is carried out in accordance with same.

The CA will carry out a site induction for all contractors’ operatives on the morning of the first day of starting on site.

1.15	SECURITY CLEARANCE

1.15.1	The Contractor is responsible for the ensuring that all their employees and any sub-contractors, agents etc. who are due to work at the Premises, for longer than four weeks in duration in the delivery of the Contract, whether in a continuous period, or over the duration of the Contract, have been security vetted to Basic level, as defined by Disclosure Scotland1.

1     Please note that Disclosure Scotland is the agency that all security vetting providers will approach to undertake the security vetting.

1.15.2	For Contractor’s with a small employee base, IWM will be prepared to undertake the security clearance on behalf of the Contractor, provided that this is made known at the time of the appointment, and that this agreed by IWM, and that the Contractor agrees to pay the administration charge of £41 per clearance, that IWM is charged for this service.

1.15.3	This requirement will apply to an employee who has not worked at an IWM branch, within the previous 12 months.

1.15.4 A minimum of 48 hours before any individual commences work on the Premises, the Contractor is required to complete and submit the Security Notification Form (see Appendix 2) to the Technical Security Manager at IWM London.

1.15.5	Upon arrival at the Premises, each new employee must report individually to the Control Room with a copy of their certificate of proof of vetting, and some form of ID. IWM will note the details of the certificate of proof of vetting, but will not retain any documents.

1.15.6	IWM will accept a certificate of proof of vetting which is dated within 12 months of the date of their site commencement at IWM, although IWM reserves the right to request they are security vetted, at its discretion, which IWM agrees not to action unreasonably.

1.15.7	Failure to comply with this requirement could result In the employee not being granted access to IWM.

1.16	NEW SUPPLIER

The successful contractor if not on the museum’s approved supplier system, will be required to complete a ‘New Suppliers Form’ as contained within the tender pack and return it to the CA within 5 days of award of contract.

- APPENDIX 1

- Contract:

The form of Contract will be the JCT Agreement for Minor Building Works 2016 together with any amendments to date.

- ARTICLES OF AGREEMENT:

- Recitals:

- First.   The work required is the refurbishment of Building No. 219 IWM Duxford.

- Second. Documents comprise the contract specification.

Third. The contractor shall supply a fully priced copy of the specification to the employer.

Fourth. The employer is not a contractor under the CIS rules.

Fifth. This project is considered to be notifiable under the CDM Regulations 2015.

Sixth. Not applicable.

Seventh. Applies, excepting paragraphs 3,4 and 5.

Articles :

Article 3 	The Contract Administrator shall be IWM Duxford

Article 4	Not applicable.

Article 5	Not applicable.

Article 7 (and Schedule 1) the arbitration provisions of article 7 and schedule 1

apply.

- Contract Particulars:

- Base date					01 October 2016.

- Clause 1.1					2 weeks.

- Beginning on the date of receipt by the contractor of the tender documentation.

Clause 2.2	Commencement		To be agreed

Completion

Clause 2.8	Liquidated Damages	N/A

Clause 2.10	Rectification Period	12 months

  - Percentage is 95%

  - Percentage is 95%

    - 3 months

  - Deleted

5.3.2	Contract Insurance required is £10million minimum

5.4A & 5.4B	Clause 5.4A applies % addition to cover fees – 20%

7.2	Nominator shall be President of the RICS

Schedule 1	Appointment of arbitrator shall be the President of The Royal Institute of Chartered Surveyors

SECTION 2

Health & Safety

PRE CONSTRUCTION INFORMATION.

GENERAL REQUIREMENTS

The works that are the subject of this Specification are subject to the Construction (Design & Management) Regulations 2015. Specifically Part 4 – General Requirements for all construction sites. The principal contractor will be required to comply with the relevant regulations contained in part 4.

The project however is not considered to be notifiable under those regulations.

The Contractor will be required to operate as the ’Principal contractor’ under those regulations and co-ordinate and manage Health & Safety issues during construction work, and, liaise with the Contract administrator during the course of these works.

Prior to commencement the contractor must ensure a construction phase health and safety plan has been produced, and that adequate provision is made on site for provision of welfare.

The Contractor will be responsible for ensuring that subcontractors employed by him operate in a safe manner appropriate to the work they are employed to undertake.

Special attention is to be paid to the scaffold work, not only the design of same, but also the erection and dismantling sequence and methodology.

2.0	INFORMATION ON BUILDING/ENVIRONMENT/RISKS.

The following aspects of the work are considered to represent a risk, either to the person engaged in undertaking that work directly, or to other persons in the area of the works. These are to be addressed in the Principal contractor’s Construction Phase Plan.

2.1	The building fabric.

A single Link trainer building which was constructed under Scheme ‘L’ of RAF expansion. The building was built circa 1930’s and its construction consists of:

Externally laid Flemish bond brickwork with snapped headers with a 50mm cavity and an 225mm inner leaf with a plaster skim. Reinforced concrete slab roof which is covered with asphalt

A series of ‘Crittall’ metal framed and single glazed windows are built within the brickwork. External doors are of a timber material and rainwater goods are original cast iron.

2.2	Proximity to Public and staff

The work will involve working in areas where public have access around, and into buildings. Staff similarly will have access to and be in occupation of buildings during the works.

The principal contractor must consider carefully measures to be employed to prevent injury to these persons.

Contractor is to organize works such that pedestrian access into and egress from buildings is not obstructed.

The museum has 3 major airshows during the year, when the site is particularly busy. The phases referred to above are to be arranged to take place between the airshows, and, where scaffold remains in situ over an airshow weekend, it is to be properly fenced, signed and cleared of all loose material/tools/plant/materials and swept down to leave clean and safe.

Airshows are scheduled for the following weekends:

May airshow	TBC

July airshow	TBC

September airshow	TBC

No work will be permitted during the airshows. Immediately prior to the shows the scaffold is to be cleared of all materials, the site left clean, tidy and secure.

2.3	Checking/Handling and Site Storage of Materials and Components

Contractor is to pay particular attention to how paint and associated materials are handled and stored.

Manufacturer’s COSSH data sheets should be followed and available on site for inspection by CA.

Some products are likely to be flammable - specific provisions for safe storage and handling must be identified by Contractor.

2.4	Lead

Existing joinery is likely to have been painted with lead pigmented paints previously.

Burning off of previously painted surfaces is not permitted.  Contractor to undertake a risk assessment, and produce a Method Statement detailing specifically how preparation work is to be undertaken, and what control measures are to be put in place.

Wet  flatting, or chemical stripping methods are preferred but if dry flatting cannot be avoided then use of appropriate dust respirators should be worn.

Steps should be taken to contain dust created, which must be cleaned up thoroughly.

2.5	Working at Heights

Much of the work to be executed is at high level.

The contractor is to detail the measures to be taken to ensure safe working access and platforms.

Details of scaffolding proposals are to be included in Method Statement.

Handover Certificates are to be issued by the scaffolding contractor stating specifically:

    - It is complete and complies with the Construction (Working Places) Regulations 1966 and relevant British Standards.

    - The distributed loads which are permitted on the working lifts.

- Guarding of working platforms.

- That inspection is required every 7 days, or immediately following adverse weather.

- Inspection findings to be recorded.

- No unauthorized modifications or alterations are to be made.

2.6	Risk Assessments and Method Statements

The Contractor shall provide, with his tender, an outline Method Statement setting out how the work is to be approached, organized and monitored in order to ensure the works are undertaken both efficiently and safely.

2.7	Construction phase plan.

Before works commence on site, the Principal contractor must draw up a Construction Phase Plan. This must set out the health and safety arrangements and site rules, taking account of the nature of the site and use of the building.

Throughout the construction phase the principal contractor is to ensure the plan is reviewed, and updated, or revised so that it continues to be sufficient to ensure construction work is so far as is reasonably practicable carried out without risks to health or safety.

The ‘Principal Designer’ for this project is The IWM, represented by S J Rogers  Capital Project Manager. The Principal contractor shall liaise and cooperate with the Principal designer in preparing the plan and providing information for inclusion into the Health and safety File.

2.9	Health & Safety File

The Principal Designer will prepare the Health and Safety file. The Principal Contractor is to provide information on request to enable the file to be prepared. Such information is likely to include, but not be limited to

a.	The contractor’s risk assessment and method statements.

b.	Copies of manufacturer’s current technical literature and COSHH data sheets for all materials, plant and equipment selected by the contractor.

.

SECTION 3

Trade Preambles

- To be read in conjunction with Preliminaries

and General Conditions

3.01	MATERIALS

0.01	Knotting

Knotting to be in accordance with BS 1336

0.02	Stopping

Stopping for timber should be of proprietary manufacture subject to approval by the CA

0.03	Sources of Supply

Paints are to be obtained from one manufacturer, within the limitations set out below.

0.04	Oil Paint

Oil paint for joinery to be as follows (unless stated otherwise in section 4 schedule of works):

Dulux Trade undercoat and gloss.

Colours from BS range as follows:

Door leaves and panels			BS14C39 Hollybush

0.05	Primer

Oil based wood primer as supplied by the manufacturer of the gloss/undercoat paint described above.

0.06	Masonry/Brickwork

TBC

0.07	Exterior stain

N/A

0.08	Metal Work

N/A

  - Stabilising solution.

Sandtex stabilizing solution for chalking, weathered and porous surfaces.

0.10	Epoxy Wood Stabiliser

Where specified to undertake filling repairs, to be Epoxy System from Nickerson Chemicals (01924 461341)

0.11  Rainwater goods.

Externally – Hopper and  downpipes to be painted Hammerite ‘direct to rust’ smooth Black paint.

3.02	WORKMANSHIP

0.01	Generally

Comply also with the requirements of BS 8000 Part 12 1989  ‘Workmanship on Building Sites’.

0.02	Ordering of Materials

Obtain  liquid materials in containers sized to suit the extent of the work and to prevent deterioration of residual quantities in opened containers.  Maximum generally 5 litres.

0.03	Order of Working

Work in a sequence that ensures the finished work is not spoilt by dust and debris arising from subsequent preparation work.  When planning the work consider the need for cleaning, washing down, heating or drying equipment.

0.04	Initial Preparation of Surfaces

Remove all dirt, and any contaminants before commencing abrasive cleaning, rub down with suitable abrasive paper or cloth, nylon pads and wire wool, all surfaces prior to application of paint system.

Wherever possible use wet abrasives for the reasons described earlier in this Specification.

Masonry paint applied to wall surfaces is generally loose and will require thorough brushing/scraping to remove loose/flaked paint back to a firm edge.

Rub down in a manner that does not cause damage to adjacent surfaces, arrises, mouldings etc.

Remove wet abrasion dust by washing down to remove all surface contaminant.

Stop nails, screws and cracks etc.

Fill and smooth depressions, coarse grained surfaces and like defects and blemishes that cause surfaces to be unsuitable for painting.

Remove any high spots and leave smooth, fill and stop as early as possible before decorating work commences.

Remove defective, flaking, peeling and poorly adhering coatings back to a firm coating or back to the original background.

Note – the preparation of surfaces will require considerable work as the existing paints finishes are in poor condition.

0.05	Defective Backgrounds

Advise CA of defective backgrounds and seek clarification on how to proceed.  Areas of rot, unstable, or incompatible components will usually be subject to replacement or repair on the direction of the CA.

0.06	Preparation of Coatings

Generally follow paint manufacturer’s specific site instructions in respect of preparation of coatings prior to application.

This will usually involve thoroughly stirring liquid paints so that all solids are fully and evenly incorporated.

DO NOT THIN PAINTS UNLESS SO DIRECTED BY THE CA.

Take particular care to observe manufacturer’ pot life, timing and queueing directions when preparing two part coatings.

0.07	Final Preparation of Surfaces

Examine surfaces immediately before painting and where necessary undertake following final preparation.

Wash, wipe or otherwise remove any dirt or contaminant.  (Do not use a brush as this move dust to another surface.)

Remove any moisture or condensation by drying surfaces and/or applying general warmth if necessary.

0.08	Unsuitable Conditions

Do not apply coatings when:

  - Airborne dust and grit is present.

  - Air temperature is below 4 degrees centigrade.

  - Relative humidity exceeds 80%.

        - Temperature of the surface is too low eg. heavy metalwork, plastered external walls etc.

  - Substrates have not properly dried out.

  - Lighting is inadequate.

  - There is inadequate ventilation.

0.09	Application

Subject to the requirements of BS 6150 and BS 8000, apply coatings as follows:

Wipe naturally greasy hardwoods eg teak, or metal, with white spirit.

Apply knotting to any resinous or potentially resinous surfaces for example knots.  Rub down hard knotting with glass paper.

Make good to any existing priming.

Applied specified coatings allowing adequate intervals between coats.

Lightly rub down between coats if necessary by fine wet and dry paper on fully hardened surfacing.

Apply primers by brush unless otherwise approved.

Apply other coatings in accordance with manufacturer=s preferred methods eg brush, roller or spray for the coating and surfaces involved.

When brushing apply full coat, evenly worked.

0.10	Inspections

CA will make frequent site inspections to judge the preparation and quality of the work.

A final visual inspection will be made and the general quality will be judged as follows:

- Satisfactory stopping and filling.

- Uniformity of gloss, sheen or texture.

- Uniformity of colour and obscuration of the substrate.

- Freedom from film defects such as run, snag, shrinking, bulking, thinning, unevenness caused by dust, dirt or paint drips.

- Accuracy of cutting in.

- General cleanliness and an overall appearance of the work.

0.11	Sampling and Testing

Contractor shall supply samples of paints for analysis at the CA=s request during the course of the work.  The samples may be taken from open tins, or from applied work.

0.12	Joinery Surfaces to be Oil Painted

Where preparation of existing joinery results in timber being exposed, timber is to be primed, and painted with undercoat.

All prepared joinery (including those primed and undercoated) to receive one undercoat, and one full coat of gloss paint.

Top edges of doors, leading edges, top/bottom and leadings edges of openable windows to be so painted.

Openable windows/sashes to be eased if found paintbound following redecoration works.

0.13	Metal Surfaces to be Oil Painted

N/A

0.14	Masonry Surfaces

Where the paint finish has lifted from the substrate, additional preparation will be required to remove defective coatings.  Plaster render finishes are to receive a coat of stabilising solution where chalky residue is apparent, followed by an additional coat of masonry paint prior to two further full coats as described above.

0.15	Surfaces to be Varnished, Lacquered or Wax Polished

N/A

  - Surfaces to receive stain finish.

N/A

  - Rainwater goods.

Existing cast iron rainwater goods to be carefully taken down cleaned and re-used

SECTION 4

SCHEDULE OF WORKS

WORKS.

These works are to be commenced on or before 31 October 2016.

BUILDING No. 219-LINK TRAINER

PROVISIONAL REPAIRS

Note:- The repairs described below are an estimate of the repair work likely to be needed. The contractor will be required to liaise with the C.A. for confirmation of specific areas and extent of works required. The C.A. will raise a contract instruction to confirm the actual extent of the work.

Brick repairs and Internal Blockwork

Parapet Walls

Carefully remove and put to one side concrete coping stones for re-use.

Carefully take down existing parapet walls, (approx.58m sq). Allow PC sum of £600 per 1000 for bricks to rebuild parapet walls.

Repair

Chop out 4No. bricks adjacent to main entrance door and allow replacement with full bricks to same bond as existing. Piece in new bricks, bedded in mortar and neatly pointed up.

Allow total 4 No bricks.

Re-pointing

Rake out to a depth of 18mm to 20mm, brush out residue and repoint approximately 23 sq m of mortar. The use of an angle grinder should be avoided as it is easy to slip and damage the face of the brickwork.

Mortar Content

Mortar content to be cement/lime/sharp-sand mix. 2 No. test panels to be constructed. Panel one to be constructed and pointed in using this mix, Panel two constructed using the same mix but pointed in with a 1-3mm aggregate added to the mortar. To be approved by C.A.

Internal Blockwork

Install 4No. Wickes 1165mm Galvanised steel wall starters-Product Code:213643 or similar to 11No. window openings . Block up, using 100mm dense concrete blocks,11No window openings approximately 10.26m sq. Mortar to be 1:6 cement/sand with plasticizer mix as per BS-5628-2. Plaster skim blockwork using British Gypsum Thistle Universal One Coat Plaster minimum 13mm thickness- Product Code-848747

Concrete Ring beam repairs

Carry out concrete ring beam repairs as per Structural Engineers recommendations/specification.

All work to be fully in accordance with ROTAFIX or equivalent  repair mortar manufacturers details.

: Carefully break out all loose and friable concrete with hammer/needle guns or disc cutters as necessary

: Carefully clean all exposed reinforcement of rust etc, use mechanical wire brushing. Advise engineer to inspect if reinforcement is too severely corroded

: Apply HR slurry coat to reinforcement

: Apply P12 Epoxy Primer to concrete and steel to be repaired

: Apply Rendacrete LW to vertical and overhead repair areas. Maximum coat application is 50mm. For thicker coats, provide in successive layers in accordance with instructions

: Final coating to be VINACHEM or decorative masonry paint-CA to advise

Ceiling

Scrape all loose material off existing plasterboard ceiling. Overboard with British Gypsum 12.5mm Plasterboard (Tapered edge) approximately 57m sq. Apply perforated plasterboard tape to all joints with British Gypsum Gyproc Promix Lite Joint Cement-Product Code-148847. Plaster skim plasterboard using British Gypsum Thistle Universal One Coat plaster minimum 13mm thickness- Product Code-848747.

Window repairs.

Replacement windows.

Allow PC Sum of £6,655.00 for METWIN LIMITED quotation for:

To attend on site, remove existing steel windows and clear away. Supply,  hoist into position, and securely fix 11No. purpose made fixed light windows. ( 6No. @ Approximately 900mm x 900mm and

5No. @ Approximately 900mm x 1530mm) coupled with transoms where necessary. (All manufactured using F7 steel sections). Windows to be galvanized and polyester coated RAL 9911 white. To be externally putty glazed with 4mm clear glass with black out glass film applied.

Joinery repairs.

Doors.

Strip out existing 2No pair of timber doors and replace like for like. External doors to be 2.280mm x 900mm x 2 with bottom solid panels 930mm x 550mm and 3No. Glazed panels per door @ 550mm x 315mm. Internal doors to be pair of blank doors with foil-backed insulation attached to the inner face.

Rainwater Downpipes and Hopper repairs

Remove carefully existing Cast Iron rainwater downpipes and hoppers. Carry out any necessary repairs to joints and brackets, rub down all cast iron fixtures and fittings, prep and re-decorate with Hammerite- Direct to Rust Metal Paint-Smooth Finish in Black-Part No. 5092966.

Redecoration

Walls and Ceiling

Prepare walls for repainting. Thoroughly brush down and scrape existing painted finished walls back to a clean and firm edge. Apply Sandtex Quick Dry Stabilising Solution- Part No. 15X5500083, to chalky or loose areas.

Apply two full coats of Dulux Trade Diamond Matt-Colour Almond White-Product Code-AE01225055.

Floor paint application

Any dirt, dust or other loose/ friable materials must be removed. Surface contaminates such as oils, grease and any other past spillages or residues etc to be removed. Allow PC Sum of £20 per sq metre for concrete floor repairs. N.B. The use of ‘pollyfilla’ type materials are unsuitable for pre-floor painting repairs and as such the C.A. will not approve this type of material. Apply, using manufacturers

recommended number of coats, Dulux Trade Floorshield-Product Code-AE015150F-Colour Code-30BB62009-Light Grey.

Joinery

Prepare existing joinery, and effect repairs. All joinery should be rubbed down to a sufficient standard

Prepare and paint all doors, frames and skirting with one undercoat and two gloss coats. Bared areas to be primed, and given an additional undercoat. Any runs must be rubbed down and top coat reapplied. Apply Dulux Trade BS14C39 Colour Holybush.

Warm Deck Roof

Strip existing asphalt roof and all vegitation. Clean concrete roof slab of any loose material and apply sand and cement screed with falls to 2No Hoppers so that sumps can be formed to allow water to flow to these areas. The vapour control layer should consist of a least one layer of bitumen roofing membrane fully bonded to the structural deck and all laps sealed with bitumen. Insulation to be 150mm Rigid Urethane Foam boards such as Kingspan or Celotex or similar. Waterproofing should be a reinforced bitumen membrane to BS 8747 with a perforated layer and a Elastomeric underlay and an Elastomeric capsheet.

N.B. Torching onto insulation boards, except rock/glass or perlite is not acceptable.

N.B. Elastomeric bitumen membranes, with greater extensibility and flexibility are likely to provide longer service life.

Electrical Works.

Allow PC sum of £2,500 for installation of new mains supply. Also allow for:

: Distribution Board

: Single phase metal clad distribution board mounted inside lobby

: Fully populated board with RCBO’s for individual circuits

: Testing and certification

: Small Power

: 5No. surface mounted clad sockets

: Galvanised steel conduit system run surface

: Cabling wired in single runs in steel conduit

: Lighting

: 5No 2x49W T5 5ft weatherproof c/w polycarbonate diffuser

: 1No 28W 2D emergency bulkhead for lobby area

: 1No 28W 2D IP65 emergency bulkhead externally over the door

: Galvanised steel conduit system run surface

: Cabling wired in single runs in steel conduit

: Surface mounted switches including key switch for emergency test

: Fire alarm

IWM Duxford Fire Alarm System is maintained by ECONOGARD SERVICES Ltd Saxon Way, Melbourn, Royston, SG8 6DN Tel-01763 261970.

Fire alarm system to be linked up to the site wide system and be able to be monitored back to the IWM Duxford Guard House.

: Air Handling

: 1No Mitsubishi MSZ-GF71/MuZ-GF71 wall mounted split system

: Power supply to external condenser unit

SECTION 5

Tender Submission

Tenderers are required to comply with the following in order for their tender to be considered. If tenderers do not comply fully, then their tender may be rejected by the Museum.

5.1    Site Visit

As part of the process tenderers shall visit site to acquaint themselves with the site, the extent of the work and the conditions under which it is to be carried out.

The contractor is to contact Mr. S Rogers at the Duxford site on 01223 497220 to make arrangements.

5.2    Tender Acceptance

The ‘Tender Declaration Form’ included in this specification must be submitted as part of the tender. Electronic submissions or e-mails will not be considered.

The tender envelope should be marked TENDER DOCUMENT REF IWM/DUX/1574. Other than this envelopes and/or packages should be plain and bear no reference to the tenderer's name; franking machines that automatically print the company name should not be used.

5.3    Basis of Prices

All prices must be quoted on the basis of lump sum fixed price based on no re-measurement, as indicated in the accompanying documents. Quantities given are for guidance only and prices entered should exclude VAT.

5.4    Supporting Information

The contractor shall as part of tender submission include the following information:

- An outline Method Statement describing how the work are to be undertaken and what arrangements will be put in place to manage health and safety.

- The names & contact details of two references where the company has carried out similar work.

- Copy of the company’s Health & Safety Statement

.

- This tender document complete, with each page initialed to indicate reading.

- The schedule of work items priced up.

- The tender form properly completed and signed.

- Proposed programme of works.

5.5    Award Criteria (other than price)

The employer does not bind himself to accept the lowest or any tender; when awarding the contract in addition to price the Employer will have regard to the following.

- Accuracy of submission.

- Compliance with tender instruction for submission of documentation.

- Suitability of the Method Statement and Risk assessment referred to above.

- Contractor’s ability to demonstrate his competence in carrying out works of this nature.

SECTION 6

CONTRACT TIMETABLE

6.0	CONTRACT TIMETABLE

# The following schedule represents the proposed timetable for the appointment of a consultant and commencement of the contract for the services in relation to various external building projects as detailed within the tender documents on behalf of the Trustees of the Imperial War Museum.

| CT 1.0 | Issue of Tenders: | 27 September 2016 |
| --- | --- | --- |
| CT 1.1 | Tender Period | 3 weeks |
| CT 1.2 | Tender Submission Date: | 17 October 2016 |
| CT 1.3 | Tender Submission Time: | Noon |
| CT 1.4 | Tender Opening Time: | TBA |
| CT 1.6 | Tender Evaluation Period: | One week |
| CT 2.0 | Contract Award: | 24 October 2016 |
| CT 2.1 | Contract Lead-in Time | To be advised |
| CT 2.2 | Contract Start Date: | On, or before 31October 2016 |
| CT 2.3 | Contract Completion Date: | On or before 12 December 2016. |

- Tender Declaration; Contract Reference: IWM/Dux/1574

Project.	Refurbishment of Building No. 219 to store radioactive and other materials

To:		The Trustees of the Imperial War Museum

I/We having read the following documents delivered to me/us in respect of the above Contract:

Do hereby offer to execute and complete in accordance with the Conditions of Contract the whole of the works as described for the sum of: -

£_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ (excluding VAT)

(_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ amount in words)

- Company Name:	___________________________________________________

Address:			___________________________________________________

___________________________________________________

___________________________________________________

Tel. No.		_____________________

Fax. No.		_____________________

Signature:			___________________________________________________

Print Name:		___________________________________________________

In the Capacity of:	___________________________________________________

(The above named person is duly authorised to sign tenders for and on behalf of the Company)

# Dated this . . . . . .day of  . . . . . . . . . . . . . . . . . . . . . . . 2016

TENDER BREAKDOWN.						(£)

Preliminaries (excluding scaffold).					________

Contin	gency sum.

Masonry works						          ________

Concrete ring beam repairs				          ________

Internal works						          ________

Replacement windows				                    ________

Warm deck roof						         ________

Mechanical and Electrical works				         ________

Total Tender figure.					        _________

(Exclusive of VAT)

Appendix 2

STAFF SECURITY CLEARANCE

NOTIFICATION FORM

CONTRACTOR:

|  |  |  | Level of CRB |  |  |
| --- | --- | --- | --- | --- | --- |
| IWM Branch | Start Date | Staff Name | Basic | Standard | Enhanced |

| Signed |  |
| --- | --- |
| Name |  |
| Job Title |  |
| Date |  |

Please complete all sections of the Form and forward to IWM’s Technical Security Manager at jpawley@iwm.org.uk or addressed to Technical Security Manager, CWR, Clive Steps, King Charles Street, London SW1A 2AQ, a minimum of 48 hours prior to the individual(s) commencing work at IWM.