ACCESS CONTROL SPECIFICATION

SITE: PRIESTLEY COLLEGE

MAY 23

Introduction

This specification is to be used to provide a quotation to change the current access control on a selection of external and internal doors.

All prices are to be kept fixed for 2 months. Works to be priced to be carried out during normal working hours.

Detailed plans/designs of the works need to be submitted 3 weeks prior to any works starting.

A full set of risk assessments and method statements to be also sent 3 weeks prior to works starting.

Contact: Lisa Gannon – TCAT Operations Manager.

e-mail	l.gannon@tcat.uk.com

Tel	01925 263919 ext. 317

Closing date for quotations 9th June 2023

Submissions

Section Breakdown

- External Doors

- Internal Doors

- Costing sheet

D) 	Site works

SECTION A – EXTERNAL DOORS

Please use the attached spreadsheet for the Door IDs. Currently the doors have a SMARTAIR system, we would like a price to replace the system with Paxton Net 2.

Some of the doors already have the Paxton Net 2 controllers in position ready to connect.

Any required IP connections will be carried out by the college. Any local cabling that needs to be changed is to be done by the contractor, including reader cable.

Type A

Each door to consist of –

One	Paxton Net 2 Controller with POE (682-284)

Two	Paxton Net 2 Proximity MIFARE reader P50 (353-467)

Removal of the existing access control readers and re-connection of the existing magnetic lock, push to exit button and break glass.

Type B

Each door to consist of –

One	Paxton Net 2 Proximity MIFARE reader P50 (353-467)

Removal of the existing access control readers and re-connection of the existing magnetic lock, push to exit button and break glass. These doors already have the Net 2 Controller installed.

SECTION B – INTERNAL DOORS

There are some internal doors that already have SMARTAIR proximity handle sets installed. The college would like a price to remove the old handles and install new Paxton Pax lock to the doors.

Some of the doors do not have the proximity handle set on at the moment, but the college would like them to be installed.

Please see the attached spreadsheet of the doors, they have been put into WS Bridge Groups. These are the group that the door handles that want to be programmed to the Paxton Net 2 Air Bridges. For example, all the doors in BG 4 are on the same wireless air bridge.

At the location of each Net2 Air Bridge the college will provide a POE (802.3af) connection. The bridges are to be installed flat (if possible) on the ceiling.

Type A

Each door to consist of –

One	Paxton PaxLock Pro (900-600-BL)

One 	Paxton Cover Plate (900-051) or similar to hide old fixing holes left when existing 	equipment is removed.

Type B

Each door to consist of –

One	Paxton PaxLock Pro (900-600-BL)

Air Bridges

To supply and install 18 No. Paxton Net2Air Bridges (477-600).

Includes setup and commissioning onto the Net 2 software.

SECTION C – COSTING SHEET

Section A

1 No. Type A

4 No. Type B

Total Cost £ + vat

Section B

53 No. Type A

6 No. Type B

18 No. Net 2 Air Bridges

Total Cost £ + vat

Company	-

Responsible Person	-

Signature	-

Date		-

SECTION D – SITE WORKS

The following works has been included by the college –

a) Any required IP connections for the controllers and Net 2 Air bridges.

b) Any fire connections to the controllers.

You do not need to include for –

a) Cards/tokens, the existing are to be re-used.

b) Software, the required software will be installed prior to installation.

c) Replacing any magnetic locks, push to exit or break glasses. All the existing are to be re-used.

d) Replacing any closers or automatic closers, all are to be re-used.

Works you will need to include for –

a) Full installation and commissioning of the equipment.

b) Removal of the existing access control equipment and disposal from site.

c) Replacing any reader cable from the reader to the controllers, if not the correct type.

d) Two days onsite training when the system is complete.