2 : PREAMBLES

2.1 : GENERALLY

Application of Descriptions etc

| 2.1.1 | Descriptions, notes, directions, etc., having once appeared herein, are to, insofar as the context permits, apply throughout this Specification whether expressly repeated or not. In case of ambiguity, that which has express application will prevail over that which is general. |
| --- | --- |
| Description of Terms |  |
| 2.1.2 | The term ‘allow’, where used, is to mean the inclusion, at the Contractor’s expense, of everything necessary to ensure the full and proper discharge of that responsibility. |
| 2.1.3 | The term ‘approved’ where used is to mean approved by the Contract Administrator and any approval will be confirmed in writing. |
| Protection |  |
| 2.1.4 | The Contractor is to allow for providing all necessary temporary casings and protection of work likely to be damaged during the works. He is to allow for all necessary tarpaulins, temporary covers, etc., to protect the existing and new work from damage. The whole of the above is to be to the entire satisfaction of the Contract Administrator. |
| British Standards and Codes of Practice |  |
| 2.1.5 | All references to British Standards (B.S.) and Code of Practice (C.P.) are deemed to be to the current standards and codes available at the date of tendering. |
| 2.1.6 | If required by the Contract Administrator, a copy of the relevant Code of Practice is to be provided by the Contractor and kept on site. |

| Workmanship |  |
| --- | --- |
| 2.1.7 | The whole of the works is to be executed in the best and most workmanlike manner and with materials of the best description in accordance with the directions and to the satisfaction of the Contract Administrator. |
| 2.1.8 | The appropriate sections of the work are to be inspected and measured from time to time before being covered up and the Contractor is to give due notice and is to afford every facility to the Contract Administrator for inspecting and checking the work, an to the Quantity Surveyor for valuing the work. |
| 2.1.9 | All work to be executed under the Contract is to be carried out in accordance with the requirements of the appropriate current British Standard Code of Practice unless otherwise specified. |
| 2.1.10 | Inspection or any other action by the Contract Administrator must not be taken as approval of samples, materials, products or work unless the Contract Administrator so confirms in writing. |
| Making Good |  |
| 2.1.11 | The Contractor is to include for making good to match the surrounding work and for bringing forward affected work for redecoration. |
| 2.1.12 | Unless otherwise described all making good shall be carried out in materials and finishes to match existing, original or adjoining work. Edges of new work shall be undercut, lined up or levelled off to the old surfaces and new angles, mitres, etc., formed where appropriate. |
| Removing Fittings |  |
| 2.1.13 | The Contractor shall include for removing fittings so described as appropriate unless otherwise stated and making out and bringing forward all work disturbed and for all other items of making good. |
| Redundant Services |  |
| 2.1.14 | Any Services, drains and like made redundant by reason of the works are to be properly removed, cut off and sealed to the satisfaction of the Contract Administrator. |

| Scaffolding |  |
| --- | --- |
| 2.1.15 | The Contractor shall include for all necessary scaffolding and for making good all work disturbed in connection therewith. |

| 2.2 : ROOFING |  |
| --- | --- |
| Generally |  |
| 2.2.1 | Lay the full roof covering in a single operation to provide a secure, free draining and completely weathertight roof. |
| 2.2.2 | Ancillary products and accessories, where not specified, to be types recommended for the purpose by the felt manufacturer. |
| 2.2.3 | Store rolls of felt indoors in reasonably warm conditions until immediately before use. |
| 2.2.4 | Provide temporary covers and drainage as required to keep unfinished areas of the roof dry. |
| 2.2.5 | Protect daywork joints in warm deck roofs with a lapped and fully bonded strip of top layer felt. |
| 2.2.6 | Suspend work in severe or continuously wet weather unless an effective temporary roof is provided over the working area. |
| 2.2.7 | If unavoidable wetting of the construction does occur, take prompt action to minimise and make good any damage. |
| 2.2.8 | Primers are to be of the type recommended for the purpose by the felt manufacturer. Apply by mopping, brushing or spraying to achieve an even and full cover of the surface. Allow to dry thoroughly before covering. |

| 2.3 : WOODWORK |  |
| --- | --- |
| Plugged Timber |  |
| 2.3.1 | Timber described as ‘plugged’ shall mean timber fixed to brickwork, concrete, or other similar background by means of timber or plastic plugs, shot firing may be used at the Contractor’s option providing guns and pins of an appropriate size to suit the timber section to be fixed are used and the Contract Administrator’s permission is obtained. |
| Quality of Timber in Joinery |  |
| 2.3.2 | Softwood and hardwood shall be to BS 1186-3 : 1990 and be of the best quality well seasoned, of good grain and free from all defects and blemishes. In particular, timber described as kept clean for staining or clear finish shall be to Class IS of BS 1186- 3 : 1990. |
| Ironmongery |  |
| 2.3.3 | The Contractor shall allow for removing fittings before painting and refixing as may be required and for oiling, easing and leaving clean and in perfect order on completion. |
| 2.3.4 | The Contractor is to note that all flush doors are to have all blockings as required for ironmongery. |
| 2.3.5 | All lock furniture and special fittings are to be protected until completion of the works. |
| 2.3.6 | All ironmongery is to have screws to match. |
| 2.3.7 | Locks, hinges, fastenings, etc., shall be oiled, adjusted and left in perfect working order at completion and all keys properly labelled and delivered to the Contract Administrator. |

| 2.4 : PAINTING AND DECORATING Generally |  |
| --- | --- |
| 2.4.1 | The Contractor must provide all necessary guards and wet paint notices, and will be held responsible for damage which may be caused by or through wet paint. |
| 2.4.2 | The Contractor will only be allowed to use materials delivered to the site in sealed cans or drums bearing the name of the manufacturer and properly labelled as to quality and date of despatch. |
| 2.4.3 | Any defective or unsatisfactory materials delivered to the site must be immediately removed. |
| 2.4.4 | All painting and proprietary systems are to be carried out strictly in accordance with the manufacturer’s recommendations. |
| 2.4.5. | The paints shall be thoroughly mixed of stirred before use. They shall be so stored as to minimise exposure to extreme of temperature. |
| 2.4.6 | Thinning of paint materials, where necessary, shall be carried out with the type of thinner and in the proportions recommended by the manufacturer of the paint. |
| 2.4.7 | All primers shall be applied by brush. Subsequent coats may be applied by brush, spray or roller, unless otherwise directed. |
| 2.4.8 | Each coat of paint shall be allowed to harden before the next is applied. |
| 2.4.9 | Paint shall not be applied to surfaces structurally or superficially damp and all surfaces must be ascertained to be free from condensation, dirt, etc., before application of each coat. |
| 2.4.10 | Primed or undercoated woodwork and metalwork should not be left in an exposed or unsuitable situation for an undue period before completing the painting process. |

| 2.4.11 | All areas to be painted are to be cleared of rubbish prior to the commencement of the work and adequate precautions taken to ensure that areas are reasonably dust free until all paintwork is completely dry. |
| --- | --- |
| 2.4.12 | No consecutive coats of paint are to be of the same shade. |
| 2.4.13 | Plasterboard and chipboard shall be dry and thoroughly cleaned down and dusted off before application of the decoration. |
| 2.4.14 | All woodwork shall be cleaned to remove dirt, grease, etc., rubbed down or scraped smooth and dusted off. A thin coat of knotting shall be applied to all knots. After priming, all cracks, nail holes, etc., shall be made good with hard stopping. |
| 2.4.15 | The knotting is to be best quality shellac knotting. |
| 2.4.16 | The stopping shall consist of one part white lead and two parts gold size putty. |
| 2.4.17 | The whole of the work shall be finished to tints and colours selected by the Contact Administrator and the Contractor shall include for painting samples in-situ as required for the selection and approval of the Contract Administrator. The work shall be executed and finished in such multi-colours as are selected and the prices shall include for such multi-colours and for cutting in at angles and around openings. |
| 2.4.18 | All locks and fastenings are to be removed before the painting processes are commenced and are to be refixed in position on completion. |
| 2.4.19 | Provide all necessary dust sheets or other protective materials to protect floors, fittings and decorations, during the course of the work. |
| 2.4.20 | Do all necessary patching up on completion, clear and cart away all rubbish and surplus materials, clean off all paint spots, stains, etc., clean off paint and other marks from glass both sides and leave the whole of the work in this section to the complete satisfaction of the Contract Administrator. |