Invitation to Tender

ADAPTED LORRY TO INCLUDE CRANE FOR BLOCK LIFTING AND TRANSPORTATION

BIG2173

# Background/Introduction

St Mary's Leadworks supply an extensive range of building materials and supplies to commercial and domestic clients across Devon and Cornwall.  In particular, we specialise in the delivery of ready mixed concrete, concrete blocks and decorative gravel.  We deliver in assorted formats; loose, in dumpy bags or in 25kg pre-packs.

We are now looking to grow the business by purchasing a new bespoke lorry that will maximum our ability to deliver, specifically, concrete blocks.

The purchase of this system is part of a grant funded application process and therefore procurement will be subject to grant approval of the project as a whole. We will compare tenders received on a compliance basis only.

# Project Specifications

# We wish to procure a lorry that maximises the number of 4” inch blocks that we can carry. As standard we stack the 4'' blocks, 3 or 4 bands high.  Height with 3 bands is 25.6 inches,

height with 4 bands 33.7 inches, Width 34.6 inches and Length 41.5 inches.  Weight of the bricks wet for 4 band height is 1.7 tonnes and for the 3 band height is 1.275 tonnes.

The tenderer is provide the maximum configuration of the 3 and 4 band loads to maximise the overall transportable tonnage that can be achieved complying to the following specification:.

# 2.1	Chassis adaptation: Rigid (B)

# 2.2	Wheel configuration: 8x4*4

# 2.3	Chassis height: normal

# 2.4	Suspension front/rear: parabolic/air

# 2.5	Front axle design capacity: Minimum  9000 kg

# 2.6	Rear suspension technical weights: Minimum 24000 kg

# 2.7	Axle lift mid and tag: Axle lift operated by a three position switch in the cab

# 2.8	Legal weight control:

# 2.9	WVTA, Whole Vehicle Type Approval according to EC, incomplete

# Incomplete Vehicle: Scania will issue a European Whole Vehicle Type Approval Certificate of Conformity for an ‘Incomplete Vehicle’ for the ex-factory specification. Subsequent multi stage manufacturers must issue a Certificate of Conformity for their part of the vehicle specification with the final stage being for the ‘Completed Vehicle’ supplied by the tenderer

# 2.10	Steering

# Steering wheel position, right hand

# Steering system, 2-circuit

# Steering force EU Directive 70/311/EEC (or equivalent)

# Steering maneuverability according to 97/27/EC (or equivalent)

# 2.11	Front suspension type,  Parabolic Spring bushing material front, steel

# 2.12	Rear Suspension system  air,

# 2.13	  Anti-roll bar, front

# 2.14	Engine volume, Minimum 13 litres

# 2.15	Gearbox, Automatic

# 2.16	Clutch system, automatic 2 pedal

# 2.17	Power Take Off PTO

# 2.18	Fuel system

# Fuel volume right side, 400 dm3

# Fuel tank material, aluminium

# Fuel filler cap lockable,

# 2.19	SCR system: SCR reductant tank volume side RHS, minimum 47 dm3

# 2.20	Exhaust system: Exhaust outlet, direction, right

# 2.21	Brake system

# Brake adaptation, Rigid (B)

# Brake type, disc

# Brake control, electronic including ABS and traction-control

# Brake control combination, electronic with disc brake

# Hill hold

# 2.22	Auxiliary brakes: Exhaust brake control (auxiliary), manual + automatic

# 2.23	Frame type heavy duty

# 2.24	Electrics

# Batteries, minimum 180 Ah

# Battery position, left hand side

# Alternator charge, minimum 100 A

2.25	Wheels Bridgestone or equivalent

2.26   Cab exterior

Roof rails

Horn on cab roof (Black),

External sun visor

Front bumper: 	FUP legal demand, high approach

Bumper protrusion, 40 mm

Bumper beam vertical position, low

Headlamps, H7

Automatic headlamps

Daytime running light function, LED and position lamps

Spotlamp in roof, with

Spotlamps in the grille

Fog lamps mounted in the front bumper

Fog lamp front type, LED 2 diodes

Beacon, prepared

Working lamp rear wall cab top, prepared

Windscreen, untinted

Rain sensing wipers

Door window glass type, single

Side window, left

Rear cab window in toughened glass

Rear view mirror casing shape, smooth

Heated rear view mirrors

Rear view mirror electrical adjustable, passenger and driver side

Wide angle mirror, both sides

Close-proximity mirror, electrical adjusted + heating

Front view mirror, electrical adjusted + heating

Locking type cab, remote central locking

Number of keys/remotes, two Immobiliser

## 2.27  	Cab interior

Sun visor driver door, blind

Curtain windscreen/door, preparation

Centre floor mat, rubber

Internal lighting, normal

Passenger seat, folding

Seat upholstery driver side, velour black

Seat upholstery passenger side, velour black

Adjustable shock absorber for driver's seat

Adjustable backrest for drivers seat

Seat heating for the driver's seat

Armrest for the driver's seat

Steering wheel upholstery, exclusive leather

Adjustable steering wheel, tilt and slide Instrument panel finish, hard with chrome details Instrument panel lower part colour, dark sand Instrument cluster, 7 inch screen mph

Tachograph, Digital Stoneridge

Speed limitation, 89 km/h

Infotainment system, 2 DIN with 7 inch screen

Loudspeaker, 4 x 20W

Map data, Europe

Topographic data, Europe

Bluetooth

USB IP driver side, USB

USB IP centre position, USB + mobile phone preparation

USB port for charging on the passenger side

Rear wall storage, drawer

External storage compartment, driver side

Air conditioning

Climate system control, manual

# 2.28	Light Bar 1200mm long (with beacon prep and roof rails) Basic not heavy duty use

# 2.29	Stainless Steel Mirror Guards [pair]

# 2.30	Rear camera

# 2.31	“BRICK & BLOCK” BODYWORK

# 2.32	Fassi F155A 2.22 hydraulic crane rear mounted on full length chassis or equivalent

# 2.33	Basic Sign Writing

# 2.34	Repair and Maintenance

# Tenderer’s standard  Repair and maintenance but must cover minimum  1 year with MOT and out of hours for 220,000 Km within the UK

# ITT Timetable

The anticipated timetable for submission of the tender and commission milestones are set out below:

| Activity | Date |
| --- | --- |
| Date ITT available on Contracts Finder | 26 January 2018 |
| Last date for raising queries | 2 February 2018 |
| Last date for clarifications to queries | 5 February 2018 |
| Deadline to return ITT | 9 February 2018 |
| Evaluation of ITT | 12 February 2018 |
| Award of Contract | This is subject to successfully obtaining grant funding and will normally be no later than 90 days from contract evaluation |

# Conflicts of Interest

Please provide a statement with regards to a conflict of interest for this procurement through the provision of either:-

A Declaration that to your knowledge there is no conflict of interest between your company and the St Marys Leadworks that is likely to influence the outcome of this procurement either directly or indirectly through financial, economic or other personal interest which might be perceived to compromise their impartiality and independence in the contexts of this procurement procedure.

Or

A Declaration that there is a likely conflict of interest between your company and St Marys Leadworks that is likely to influence the outcome of this procurement either directly or indirectly through financial, economic or other personal interest which might be perceived to compromise their impartiality and independence in the contexts of this procurement procedure, please provide details of this connection.

This will permit St Marys Leadworks , that in the event of a conflict of interest, appropriate steps are taken to ensure that the evaluation of any submission will be undertaken by an independent and impartial party.

Exclusion

St Marys Leadworks shall exclude applicants from participation in this procurement procedure where they have established or are otherwise aware that the applicant, to include administrative, management or supervisory staff that have powers of representation, decision or control of the applicants company, has been the subject of a conviction by final judgment of one of the following reasons:-

Participation in a criminal organisation

Corruption

Fraud

Terrorist offences or offences linked to terrorist activities

Money laundering or terrorist financing

Child labour and other forms of trafficking in human beings

# Consortium or sub-contracting

Where a consortium or sub-contracting approach is proposed, all information requested should be given in respect of the proposed prime contractor or consortium leader. Relevant information should also be provided in respect of consortium members or sub-contractors who will play a significant ( greater than 25%) role in the delivery of the services under any ensuing Contract.

# Tender Application Requirements

Please provide paper copies of your application which should include:

- Confirmation that you the supplier are able to meet the requirements outlined in the brief above.

- Dated your response, used our company’s full postal address (albeit you submission might be by email) and included the Reference:  ADAPTED LORRY TO INCLUDE CRANE FOR BLOCK LIFTING AND TRANSPORTATION BIG2173

- Details of who to contact in your company in relation to this tender

- Company registration Number and VAT number (if appropriate)

- Total cost of providing the goods/services requested in section 2

# Tender Scoring Criteria

The tender will be scored only on their compliance to the specification set out in section 2 and awarded to the lowest compliant tender.

# Tender Returns

Tenders may be returned by email or post, or by delivery in person.

Tenders are to be returned by:-

Latest date to be returned:	09/02/2018

Latest time to be returned:	1700

If submitting by email, tenders should be sent electronically to simon.stmarys@yahoo.co.uk with the following message clearly noted in the Subject box; ‘ADAPTED LORRY TO INCLUDE CRANE FOR BLOCK LIFTING AND TRANSPORTATION BIG2173@

Tenderers are advised to request an acknowledgement of receipt when submitting by email.

If submitting by post or in person, the Tender must be enclosed in a sealed envelope, only marked as follows:-

Tender - Strictly Confidential –  ‘ADAPTED LORRY TO INCLUDE CRANE FOR BLOCK LIFTING AND TRANSPORTATION‘

Contract Reference Number: BIG2173

Addressed to:

Lee Crocker

Unit 6/7, St Marys business Park

Moorswater Industrial Estate

Liskeard

Cornwall

PL14 4LN

The envelope should not give any indication to the Tenderer’s identity.  Marking by the carrier will not disqualify the tender.

If delivery by hand please obtain an official Receipt at point of delivery

# Clarification

There will not be any negotiations of any of the substantive terms of the Tender Documents.  Only clarification queries will be answered. Any clarification queries arising from the Tender Documents which may have a bearing on the offer should be raised as soon as possible in writing. The deadline for clarification questions is 5 February 2018. All e-mailed queries should be sent to:-

Name: Lee Crocker

E-mail: simon.stmarys@yahoo.co.uk

No representation by way of explanation or otherwise to persons or corporations tendering or desirous of tendering as to the meaning of the tender, Contract or other Tender Documents or as to any other matter or thing to be done under the proposed contract shall bind us unless such representation is in writing and duly signed by Lee Crocker of St Marys Leadworks. All such correspondence shall be returned with the Tender Documents and shall form part of the Contract.

Tenderers must provide a single point of contact in their organisation for all contact between the Tenderer and St Marys Leadworks

Responses to any queries will be shared  through Contracts Finder website

# Disclaimer

The issue of this documentation does not commit St Marys Leadworks to award any contract pursuant to the bid process or enter into a contractual relationship with any provider of the service.  Nothing in the documentation or in any other communications made between {St Marys Leadworks or its agents and any other party, or any part thereof, shall be taken as constituting a contract, agreement or representation between St Marys Leadworks  and any other party (save for a formal award of contract made in writing by or on behalf of St Marys Leadworks.

Bidders must obtain for themselves, at their own responsibility and expense, all information necessary for the preparation of their tender responses.  Information supplied to bidders by St Marys Leadworks or any information contained in St Marys Leadworks’ publications are supplied only for general guidance in the preparation of the tender response.  Bidders must satisfy themselves by their own investigations as to the accuracy of any such information and no responsibility is accepted by St Marys Leadworks for any loss or damage of whatever kind and howsoever caused arising from the use by bidders of such information.

Bidders shall be responsible for their own costs and expenses in connection with or arising out of their response. St Marys Leadworks  reserves the right to vary or change all or any part of the basis of the procedures for the procurement process at any time or not to proceed with the proposed procurement at all.

Cancellation of the procurement process (at any time) under any circumstances will not render  St Marys Leadworks liable for any costs or expenses incurred by bidders during the procurement process.