Industrial Dwellings Society

Contract for:

Internal Works Programme – Kitchens & Bathrooms Replacement

Ref: IDS 001				Date: July 2023

Contract Administrator:	Alvin Sum

# Preliminaries

# Document 7

1st Floor, Anna House,

214-218 High Street

London, N15 4NP

| A10 PROJECT PARTICULARS 110 The Works Name: Kitchen and bathroom renewals 2023-23 Nature: Strip out of existing and supply and installation of new kitchens, bathrooms and WC’s. Contract Area: Various addresses within properties and estates owned by Industrial Dwellings Society (see Property List in Document 5) 120 Employer (Client) Name: Industrial Dwellings Society Address: 1st Floor, Anna House, 214-218 High Street, London, N15 4NP Contact: Alvin Sum Telephone: E-mail: asum@ids.org.uk 130 Principal contractor (CDM) Name: TBC Address: TBC. Contact: TBC. Telephone: TBC. E-mail: TBC. 140 Employer’s Agent (herein referred to as 'EA') Name: TBC Address: TBC Contact: TBC Telephone:TBC 150 Principal designer Name: TBC Address: TBC Contact: TBC Telephone: E-mail: 190 Clerk of Works Name: TBC Address: TBC Contact: TBC Telephone: A11 TENDER AND CONTRACT DOCUMENTS 160 Preconstruction information Format: The Preconstruction information is described in these preliminaries in Section A34. It refers to information given elsewhere in the preliminaries and other tender documents. 180 Other documents Inspection: Drawings and other documents relating to the Contract but not included in the tender documents may be seen by appointment during normal office hours at the office of the employer The documents include: Not Applicable A12 THE SITE/ EXISTING BUILDINGS 110 The sites Locations: Various addresses within the properties owned by Industrial Dwellings Society 120 Contract area Location: Residential properties 140 Existing utilities and services Drawings: (Information shown is indicative only): To be requested from Employer's contact if available Other information: Not known at present 180 Access to the site Description: Via local road networks. It is important to note that some parking restrictions may apply, 220 Use of the site General: Do not use the site for any purpose other than carrying out the Works. Do not obstruct emergency access routes. 230 Surrounding land/ building uses General: Adjacent or nearby uses or activities are as follows: Predominantly residential, but with schools in the vicinity of some blocks. 240 Health and safety hazards General: The nature and condition of the site/ building cannot be fully and certainly ascertained before it is opened up. However, the following hazards are or may be present: Asbestos Containing Materials (ACM's) Information: The accuracy and sufficiency of this information is not guaranteed. Ascertain if any additional information is required to ensure the safety of all persons and the Works. Site staff: Draw to the attention of all personnel working on the site the nature of any possible contamination and the need to take appropriate precautionary measures. 250 Site visits Assessment: Visit buildings, land or other places within the Contract Area to assess local conditions and restrictions likely to affect the execution of the Works. Arrangements for visits may be made by appointment with the person identified in the letter of invitation to tender who may be contacted Monday to Friday between the hours of 9:30am and 4:30pm. A13 DESCRIPTION OF THE WORK 120 Types of work Scope: Strip out of existing and supply and installation of new kitchens, bathrooms and WC’s A20 JCT MEASURED TERM CONTRACT The Contract: JCT Measured Term Contract 2016 Edition. THE CONDITIONS A21 EXECUTION • The Contract: Will be executed as a Deed. A22 CONTRACT GUARANTEE BOND Required A30 TENDERING/ SUBLETTING/ SUPPLY MAIN CONTRACT TENDERING 110 Scope General: These conditions are supplementary to those stated in the Invitation to Tender and on the form of tender. 145 Tendering procedure General: In accordance with NBS Guide to Tendering for Construction Projects. Errors: Alternative 2 is to apply. 160 Exclusions Inability to tender: Immediately inform if any parts of the work as defined in the tender documents cannot be tendered. Relevant parts of the work: Define those parts, stating reasons for the inability to tender. 170 Acceptance of tender Acceptance: No guarantee is offered that any tender will be recommended for acceptance or be accepted, or that reasons for non-acceptance will be given. Costs: No liability is accepted for any cost incurred in the preparation of any tender. 190 Period of validity Period: After submission or lodgement, keep tender open for consideration (unless previously withdrawn) for not less than 4 months Date for possession/ commencement: See section A20. PRICING/ SUBMISSION OF DOCUMENTS 310 Tender General: Tenders must include for all work shown or described in the tender documents as a whole or clearly apparent as being necessary for the complete and proper execution of the Works. Alterations: Do not alter or qualify the priced schedules of work without written consent. Tenders containing unauthorised alterations or qualifications may be rejected. 510 Alternative method tenders General: In addition to and at the same time as tendering for the Works as defined in the tender documents, alternative methods of construction/ installation may be submitted for consideration. Alternatives, which would involve significant changes to other work, may not be considered. Alternative tenders: Such alternatives will be deemed to be alternative tenders and each must include a complete and precise statement of the effects on cost and programme. Safety method statement: Carry out a health and safety risk assessment for each alternative and where appropriate provide a safety method statement suitable for incorporation in the Health and Safety Plan. Full technical data: Submit for each alternative together with details of any consequential amendments to the design and/ or construction of other parts of the Works. Submit: With tender 530 Substitute products Details: If products of different manufacture to those specified are proposed, submit details with the tender giving reasons for each proposed substitution. Substitutions, which have not been notified at tender stage, may not be considered. Compliance: Substitutions accepted will be subject to the verification requirements of clause A31/200. 550 Health and safety information Content: Describe the organization and resources to safeguard the health and safety of operatives, including those of subcontractors, and of any person whom the Works may affect. Include: - A copy of the health and safety policy document, including risk assessment procedures. - Accident and sickness records for the past five years. - Records of previous Health and Safety Executive enforcement action. - Records of training and training policy. - The number and type of staff responsible for health and safety on this project with details of their qualifications and duties. Submit: with the Construction Stage Health and Safety Plan to the Principal Designer for comment prior to work commencing. 570 Outline construction phase health and safety plan Content: Submit the following information within one week of request: - Method statements on how risks from hazards identified in the pre- construction information and other hazards identified by the contractor will be addressed. - Details of the management structure and responsibilities. - Arrangements for issuing health and safety directions. - Procedures for informing other contractors and employees of health and safety hazards. - Selection procedures for ensuring competency of other contractors, the self- employed and designers. - Procedures for communications between the project team, other contractors and site operatives. - Arrangements for cooperation and coordination between contractors. - Procedures for carrying out risk assessment and for managing and controlling the risk. - Emergency procedures including those for fire prevention and escape. - Arrangements for ensuring that all accidents, illness and dangerous occurrences are recorded. - Arrangements for welfare facilities. - Procedures for ensuring that all persons on site have received relevant health and safety information and training. - Arrangements for consulting with and taking the views of people on site. - Arrangements for preparing site rules and drawing them to the attention of those affected and ensuring their compliance. - Monitoring procedures to ensure compliance with site rules, selection and management procedures, health and safety standards and statutory requirements. - Review procedures to obtain feedback. 599 Freedom of Information Records: Retain, make available for inspection and supply on request information reasonably required to allow response to requests made under the provisions of the Freedom of Information Act. Determination: Submit requests received. Do not supply information to anyone other than the project participants without express written permission. Confidentiality: Maintain at all times. 599 FREEDOM OF INFORMATION • Records: Retain, make available for inspection and supply on request information reasonably required to allow response to requests made under the provisions of the Freedom of Information Act. • Determination: Submit requests received. Do not supply information outside the project participants without express written permission. • Confidentiality: Maintain at all times. SUBLETTING/SUPPLY 630 DOMESTIC SUBCONTRACTS • General: Comply with the Construction Industry Board 'Code of Practice for the selection of subcontractors'. • List: Provide details of all subcontractors and the work for which they will be responsible. • Submit: with Tender. A31 PROVISION, CONTENT AND USE OF DOCUMENTS DEFINITIONS AND INTERPRETATIONS 110 Definitions Meaning: Terms, derived terms and synonyms used in the preliminaries/ general conditions and specification are as stated therein or in the appropriate British Standard or British Standard glossary. 120 Communication Definition: Includes advise, inform, submit, give notice, instruct, agree, confirm, seek or obtain information, consent or instructions, or make arrangements. Format: In writing to the person named in clause A10/140 unless specified otherwise. Response: Do not proceed until response has been received. 130 PRODUCTS • Definition: Materials, both manufactured and naturally occurring, and goods, including components, equipment and accessories, intended for the permanent incorporation in the Works. • Includes: Goods, plant, materials, site materials and things for incorporation into the Works. 135 SITE EQUIPMENT • Definition: All appliances or things of whatsoever nature required in or about the construction for completion of the Works but not materials or other things intended to form or forming part of the Permanent Works. • Includes: Construction appliances, vehicles, consumables, tools, temporary works, scaffolding, cabins and other site facilities. 150 Contractor's Design Meaning: Design to be carried out or completed by the Contractor and supported by appropriate contractual arrangements, to correspond with specified requirements. 155 Submit proposals Meaning: Submit information in response to specified requirements. 160 Terms used in specification Remove: Disconnect, dismantle as necessary and take out the designated products or work and associated accessories, fixings, supports, linings and bedding materials. Dispose of unwanted materials. Excludes taking out and disposing of associated pipework, wiring, ductwork or other services. Fix: Receive, unload, handle, store, protect, place and fasten in position and disposal of waste and surplus packaging including all labour, materials and site equipment for that purpose. Supply and fix: As above but including supply of products to be fixed. All products to be supplied and fixed unless stated otherwise. Keep for reuse: Do not damage designated products or work. Clean off bedding and jointing materials. Stack neatly, adequately protect and store until required by the Employer/ Purchaser or for use in the Works as instructed. Make good: Execute local remedial work to designated work. Make secure, sound and neat. Excludes redecoration and/ or replacement. Replace: Supply and fix new products matching those removed. Execute work to match original new state of that removed. Repair: Execute remedial work to designated products. Make secure, sound and neat. Excludes redecoration and/ or replacement. Refix: Fix removed products. Ease: Adjust moving parts of designated products or work to achieve free movement and good fit in open and closed positions. Match existing: Provide products and work of the same appearance and features as the original, excluding ageing and weathering. Make joints between existing and new work as inconspicuous as possible. System: Equipment, accessories, controls, supports and ancillary items, including installation, necessary for that section of the work to function. 170 Manufacturer and product reference Definition: When used in this combination: - Manufacturer: the person or legal entity under whose name or trademark the particular product, component or system is marketed - Product reference: the proprietary brand name and/ or identifier by which the particular product, component or system is described. Currency: References are to the particular product as specified in the manufacturer's technical literature current on the date of the invitation to tender. 200 Substitution of products Products: If an alternative product to that specified is proposed, obtain approval before ordering the product. Reasons: Submit reasons for the proposed substitution. Documentation: Submit relevant information, including: - manufacturer and product reference; - cost; - availability; - relevant standards; - performance; - function; - compatibility of accessories; - proposed revisions to drawings and specification; - compatibility with adjacent work; - appearance; - copy of warranty/ guarantee. Alterations to adjacent work: If needed, advise scope, nature and cost. Manufacturers' guarantees: If substitution is accepted, submit before ordering products. 210 Cross references Accuracy: Check remainder of the annotation or item description against the terminology used in the section or clause referred to. Related terminology: Where a numerical cross-reference is not given the relevant sections and clauses of the specification will apply. Relevant clauses: Clauses in the referred to specification section dealing with general matters, ancillary products and execution also apply. Discrepancy or ambiguity: Before proceeding, obtain clarification or instructions. 230 Equivalent products Inadvertent omission: Wherever products are specified by proprietary name the phrase 'or equivalent' is to be deemed included. 240 Substitution of standards Specification to British Standard or European Standard: Substitution may be proposed complying with a grade or category within a national standard of another Member State of the European Community or an international standard recognised in the UK. Before ordering: Submit notification of all such substitutions. Documentary evidence: Submit for verification when requested as detailed in clause A31/200. Any submitted foreign language documents must be accompanied by certified translations into English. 250 Currency of documents and information Currency: References to published documents are to the editions, including amendments and revisions, current on the date of the Invitation to Tender. 260 Sizes General dimensions: Products are specified by their co-ordinating sizes. Timber: Cross section dimensions shown on drawings are: - Target sizes as defined in BS EN 336 for structural softwood and hardwood sections. - Finished sizes for non-structural softwood or hardwood sawn and further processed sections. A32 MANAGEMENT OF THE WORKS 120 Insurance Documentary evidence: Before starting work on site submit details, and/ or policies and receipts for the insurances required by the Conditions of Contract. 130 Insurance claims Notice: If any event occurs which may give rise to any claim or proceeding in respect of loss or damage to the Works or injury or damage to persons or property arising out of the Works, immediately give notice to the Employer, the person named in clause A10/140 and the Insurers. Failure to notify: Indemnify the Employer against any loss, which may be caused by failure to give such notice. 150 Ownership Alteration/ clearance work: Materials arising become the property of the Service Provider except where otherwise stated. Remove from site as work proceeds. 420 Removal/ replacement of existing work Extent and location: Agree before commencement. Execution: Carry out in ways that minimize the extent of work. 430 Proposed instructions Estimates: If a proposed instruction requests an estimate of cost, submit without delay and in any case within seven days. Include: - A detailed breakdown of the cost, including any allowance for direct loss and expense. - Details of any additional resources required. - Details of any adjustments to be made to the programme for the Works. - Any other information as is reasonably necessary to fully assess the implications of issuing such an instruction. Inability to comply: Inform immediately if it is not possible to comply with any of the above requirements. 440 Measurement Covered work: Give notice before covering work required to be measured. A33 QUALITY STANDARDS/ CONTROL 110 Incomplete documentation General: Where and to the extent that products or work are not fully documented, they are to be: - Of a kind and standard appropriate to the nature and character of that part of the Works where they will be used. - Suitable for the purposes stated or reasonably to be inferred from the project documents. Contract documents: Omissions or errors in description and/ or quantity shall not vitiate the Contract nor release the Contractor from any obligations or liabilities under the Contract. 120 Workmanship skills Operatives: Appropriately skilled and experienced for the type and quality of work. Registration: With Construction Skills Certification Scheme. Evidence: Operatives must produce evidence of skills/ qualifications when requested. 130 Quality of products Generally: New. (Proposals for recycled products may be considered). Supply of each product: From the same source or manufacturer. Whole quantity of each product required to complete the Works: Consistent in kind, size, quality and overall appearance. Tolerances: Where critical, measure a sufficient quantity to determine compliance. Deterioration: Prevent. Order in suitable quantities to a programme and use in appropriate sequence. 135 Quality of execution Generally: Fix, apply, install or lay products securely, accurately, plumb, neatly and in alignment. Colour batching: Do not use different colour batches where they can be seen together. Dimensions: Check on-site dimensions. Finished work: Not defective, e.g. not damaged, disfigured, dirty, faulty, or out of tolerance. Location and fixing of products: Adjust joints open to view so they are even and regular. 150 Inspections Products and executions: Inspection or any other action must not be taken as approval unless confirmed in writing referring to: - Date of inspection. - Part of the work inspected. - Respects or characteristics which are approved. - Extent and purpose of the approval. - Any associated conditions. 170 Manufacturer's recommendations/ instructions General: Comply with manufacturer's printed recommendations and instructions current on the date of the Invitation to tender. Changes to recommendations or instructions: Submit details. Ancillary products and accessories: Use those supplied or recommended by main product manufacturer. Agrément certified products: Comply with limitations, recommendations and requirements of relevant valid certificates. 330 Appearance and fit Tolerances and dimensions: If likely to be critical to execution or difficult to achieve, as early as possible either: - Submit proposals; or - Arrange for inspection of appearance of relevant aspects of partially finished work. General tolerances (maximum): To BS 5606, tables 1 and 2. 410 Services regulations New or existing services: Comply with the Byelaws or Regulations of the relevant Statutory Authority. 420 Water regulations/ byelaws notification Requirements: Notify Water Undertaker of any work carried out to or which affects new or existing services and submit any required plans, diagrams and details. Consent: Allow adequate time to receive Undertaker's consent before starting work. Inform immediately if consent is withheld or is granted subject to significant conditions. 430 Water regulations/ byelaws contractor's certificate On completion of the work: Submit (copy where also required to the Water Undertaker) a certificate including: - The address of the premises. - A brief description of the new installation and/ or work carried out to an existing installation. - The Contractor's name and address. - A statement that the installation complies with the relevant Water Regulations or Byelaws. - The name and signature of the individual responsible for checking compliance. - The date on which the installation was checked. 435 Electrical installation certificate Submit: When relevant electrical work is completed. Original certificate: To be lodged in the Building Manual. 445 Service runs General: Provide adequate space and support for services, including unobstructed routes and fixings. Ducts, chases and holes: Form during construction rather than cut. Coordination with other works: Submit details of locations, types/ methods of fixing of services to fabric and identification of runs and fittings. 510 Supervision Replacement: Give maximum possible notice before changing person in charge or site agent. 540 Defects in existing work Undocumented defects: When discovered, immediately give notice. Do not proceed with affected related work until response has been received. Documented remedial work: Do not execute work which may: - Hinder access to defective products or work; or - Be rendered abortive by remedial work. 710 Work before completion General: Make good all damage consequent upon the Works. Temporary markings, coverings and protective wrappings: Remove unless otherwise instructed. Cleaning: Clean the Works thoroughly inside and out, including all accessible ducts and voids. Remove all splashes, deposits, efflorescence, rubbish and surplus materials. Cleaning materials and methods: As recommended by manufacturers of products being cleaned, and must not damage or disfigure other materials or construction. COSHH dated data sheets: Obtain for all materials used for cleaning and ensure they are used only as recommended by their manufacturers. Minor faults: Touch up in newly painted work, carefully matching colour and brushing out edges. Repaint badly marked areas back to suitable breaks or junctions. Moving parts of new work: Adjust, ease and lubricate as necessary to ensure easy and efficient operation, including doors, windows, drawers, ironmongery, appliances, valves and controls. 720 Security at completion General: Leave the Works secure with, where appropriate, all accesses closed and locked. Keys: Account for and adequately label all keys and hand over together with an itemized schedule, retaining duplicate schedule signed as a receipt. 730 Making good defects Remedial work: Arrange access with Employer Rectification: Give reasonable notice for access to the various parts of the Works. Completion: Notify when remedial works have been completed. A34 SECURITY/ SAFETY/ PROTECTION 110 Preconstruction information Location: Integral with the project Preliminaries, including but not restricted to the following sections: - Description of project: Sections A10 and A11. - Client's consideration and management requirements: Sections A12, A13 and A36. - Environmental restrictions and on-site risks: Section A12, A35 and A34. - Significant design and construction hazards: Section A34. - The Health and Safety File: Section A37. - Tender Document 6 & Document 11 140 Construction phase health and safety plan Submission: Present to the Employer/ Client no later than seven (7) days from request. Confirmation: Do not start construction work until the Employer has confirmed in writing that the Construction Phase Health and Safety Plan includes the procedures and arrangements required by the CDM Regulations. Content: Develop the plan from and draw on the Outline Construction Phase Health and Safety Plan, clause A30/570, and the Pre-tender Health and Safety Plan/ Preconstruction information. 150 Security Protection: Safeguard the site, the Works, products, materials, and any existing buildings affected by the Works from damage and theft. Access: Take all reasonable precautions to prevent unauthorized access to the site, the Works and adjoining property. Special requirements: [To be confirmed by Employer] 160 Stability Responsibility: Maintain the stability and structural integrity of the Works and adjacent structures during the Contract. Design loads: Obtain details, support as necessary and prevent overloading. 170 Occupied premises Extent: Existing buildings will be occupied and/ or used during the Contract as follows: All residential accommodation. Works: Carry out without undue inconvenience and nuisance and without danger to occupants and users. Overtime: If compliance with this clause requires certain operations to be carried out during overtime, and such overtime is not required for any other reason, the extra cost will be allowed, provided that such overtime is authorized in advance. 200 Mobile telephones and portable electronic equipment Restrictions on use: Radios or other audio equipment are not to be used at any time. 210 Safety provisions for site visits Safety: Submit details in advance of safety provisions and procedures (including those relating to materials, which may be deleterious), which will require their compliance when visiting the site. Protective clothing and/ or equipment: Provide and maintain on site for visitors to the site. 220 Working precautions/ restrictions Hazardous areas: Operatives must take precautions as follows: - Work area: All residential accommodation - Precautions: As detailed in Contractor's Construction Phase Health and Safety Plan Permit to work: Operatives must comply with procedures in the following areas: - Work area: All residential accommodation - Procedures: As detailed in Contractor's Construction Phase Health and Safety Plan 330 Noise and vibration Standard: Comply with the recommendations of BS 5228-1, in particular clause 7.3, to minimize noise levels during the execution of the Works. Noise levels from the Works: Maximum level: 70 dB(A) when measured from outside the nearest window of the occupied room closest to the site boundary. Equipment: Fit compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles. Restrictions: Do not use: - Percussion tools and other noisy appliances without consent during the hours of 9:30am to 4:00pm - Radios or other audio equipment or permit employees to use in ways or at times that may cause nuisance. 340 Pollution Prevention: Protect the site, the Works and the general environment including the atmosphere, land, streams and waterways against pollution. Contamination: If pollution occurs inform immediately, including to the appropriate Authorities and provide relevant information. 350 Pesticides Use: Only where specified or approved, and then only suitable products listed on www.pesticides.gov.uk. Restrictions: Work near water, drainage ditches or land drains must comply with the 'Guidelines for the use of herbicides on weeds in or near watercourses and lakes'. Containers: Comply with manufacturer's disposal recommendations. Remove from site immediately empty or no longer required. Competence: Operatives must hold a BASIS Certificate of Competence, or work under supervision of a Certificate holder. 360 Nuisance Duty: Prevent nuisance from smoke, dust, rubbish, vermin and other causes. Surface water: Prevent hazardous build-up on site, in excavations and to surrounding areas and roads. 370 Asbestos containing materials Duty: Report immediately any suspected materials discovered during execution of the Works. - Do not disturb. - Agree methods for safe removal or encapsulation. 371 Dangerous or hazardous substances Duty: Report immediately suspected materials discovered during execution of the Works. - Do not disturb. - Agree methods for safe removal or remediation. 380 Fire prevention Duty: Prevent personal injury or death, and damage to the Works or other property from fire. Standard: Comply with Joint Code of Practice 'Fire Prevention on Construction Sites', published by Construction Industry Publications and The Fire Protection Association (The 'Joint Fire Code'). 390 Smoking on site Smoking on site: Not permitted. 400 Burning on site Burning on site: Not permitted. 420 Infected timber/ Contaminated materials Removal: Where instructed to remove material affected by fungal/ insect attack from the building, minimize the risk of infecting other parts of the building. Testing: carry out and keep records of appropriate tests to demonstrate that hazards presented by concentrations of airborne particles, toxins and other micro-organisms are within acceptable levels. 430 Waste Includes: Rubbish, debris, spoil, surplus material, containers and packaging. General: Minimize production. Prevent accumulations. Keep the site and Works clean and tidy. Handling: Collect and store in suitable containers. Remove frequently and dispose off site in a safe and competent manner: - Non-hazardous material: In a manner approved by the Waste Regulation Authority. - Hazardous material: As directed by the Waste Regulation Authority and in accordance with relevant regulations. Recyclable material: Sort and dispose at a Materials Recycling Facility approved by the Waste Regulation Authority. Voids and cavities in the construction: Remove rubbish, dirt and residues before closing in. Waste transfer documentation: Retain on site. 440 Electromagnetic interference Duty: Prevent excessive electromagnetic disturbance to apparatus outside the site. 460 Powder actuated fixing systems Use: Not permitted. 510 Existing services Confirmation: Notify all service authorities, statutory undertakers and/ or adjacent owners of proposed works not less than one week before commencing site operations. Identification: Before starting work, check and mark positions of utilities/ services. Where positions are not shown on drawings obtain relevant details from service authorities, statutory undertakers or other owners. Work adjacent to services: - Comply with service authority's/ statutory undertaker's recommendations. - Adequately protect, and prevent damage to services: Do not interfere with their operation without consent of service authorities/ statutory undertakers or other owners. Identifying services: - Below ground: Use signboards, giving type and depth; - Overhead: Use headroom markers. Damage to services: If any results from execution of the Works: - Immediately give notice and notify appropriate service authority/ statutory undertaker. - Make arrangements for the work to be made good without delay to the satisfaction of service authority/ statutory undertaker or other owner as appropriate. - Any measures taken to deal with an emergency will not affect the extent of the Contractor's liability. Marker tapes or protective covers: Replace, if disturbed during site operations, to service authority's/ statutory undertakers recommendations. 520 Roads and footpaths Duty: Maintain roads and footpaths within and adjacent to the site and keep clear of mud and debris. Damage caused by site traffic or otherwise consequent upon the Works: Make good to the satisfaction of the Employer, Local Authority or other owner. 560 Existing features Protection: Prevent damage to existing buildings, fences, gates, walls, roads, paved areas and other site features, which are to remain in position during execution of the Works. Special requirements: Not Applicable. 570 Existing work Protection: Prevent damage to existing work, structures or other property during the course of the work. Removal: Minimum amount necessary. Replacement work: To match existing. 580 Building interiors Protection: Prevent damage from exposure to the environment, including weather, flora, fauna, and other causes of material degradation during the course of the work. 600 Existing furniture, fittings and equipment Protection: Prevent damage or move as necessary to enable the Works to be executed. Reinstate in original positions. Extent: Before work in each room starts, the following will be removed: Assessed on a case by case basis and agreed with the Contract Administrator. 630 Existing structures Duty: Check proposed methods of work for effects on adjacent structures inside and outside the site boundary. Supports: During execution of the Works: - Provide and maintain all incidental shoring, strutting, needling and other supports as may be necessary to preserve stability of existing structures on the site or adjoining, that may be endangered or affected by the Works. - Do not remove until new work is strong enough to support existing structure. - Prevent overstressing of completed work when removing supports. Adjacent structures: Monitor and immediately report excessive movement. Standard: Comply with BS 5975 and BS EN 12812. 640 Materials for recycling/ reuse Duty: Sort and prevent damage to stated products or materials, clean off bedding and jointing materials and other contaminants. Storage: Stack neatly and protect until required by the Employer or for use in the Works as instructed. A35 SPECIFIC LIMITATIONS ON METHOD/ SEQUENCE/ TIMING 160 Use or disposal of materials Specific limitations: Assessed on a case by case basis and agreed with the Contract Administrator. A36 FACILITIES/ TEMPORARY WORK/ SERVICES 280 Accommodation Use/ Location Restrictions: - Location The Contractor is to allow for the necessary site accommodation and storage facilities within their tender submission and locate suitable locations throughout the tender period and be within the IDS estate areas. It will be the responsibility of the Contractor to source suitable site and storage accommodation for the complete contract duration. - Timing: Throughout the contract duration. 410 Lighting Finishing work and inspection: Provide temporary lighting, the intensity and direction of which closely resembles that delivered by the permanent installation. 420 Lighting and power Supply: Electricity from the Employer's mains may be used for the Works as follows: - Metering: Metered by the Contractor and charged to the Contractor - Point of supply: To be determined by Contractor on a case by case basis and agreed with the Contract Administrator. - Available capacity: To be determined by Contractor - Frequency: 50 Hz. - Phase: To be determined by Contractor - Current: Alternating. Continuity: No responsibility will be accepted for the consequences of failure or restriction in supply. 430 Water Supply: The Employer's mains may be used for the Works as follows: - Metering: Metered by the Contractor and charged to the Contractor - Source: To be determined by Contractor on a case by case basis and agreed with the Contract Administrator. - Location of supply point: To be determined by Contractor - Conditions/ Restrictions: To be determined by Contractor Continuity: No responsibility will be accepted for the consequences of failure or restriction in supply. 440 Telephones Direct communication: As soon as practicable after the Date of Possession provide the Contractor's person in charge with a mobile telephone. 530 Beneficial use of installed systems Permanent systems: Do not use for the Works. A37 OPERATION/ MAINTENANCE OF THE FINISHED WORKS 115 Operation/ maintenance information General: For each works order supply information that is required for inclusion in either the building manual or the health and safety file. Compilation: Prepare information for Contractor designed or performance specified work including as built drawings. - Include adequate information about the structure or materials used which might affect the health or safety of anyone carrying out construction or cleaning work or of anyone who may be affected by such work. - Obtain or prepare other information to be included in the Manual. Format: To match existing manual or file. 220 Training Objective: Where required, explain and demonstrate to the Employer's maintenance staff or the end users the purpose, function and operation of the installation including items and procedures listed in the Building Manual. |  |  |  |  |
| --- | --- | --- | --- | --- |