- SPECIFICATION

and

- PRE TENDER HEALTH & SAFETY PLAN

for

- WORKS TO BE EXECUTED AND MATERIALS TO BE USED IN

APPLICATION OF SPECIALIST COATING TO THE METAL CLADDING AT ROOF LEVEL AT IMPERIAL WAR MUSEUM

CONTRACT No:

IWM/FM/1569

for

- IMPERIAL WAR MUSEUMS

LAMBETH ROAD

LONDON

SE1 6HZ

- Client:

- Imperial War Museums

Lambeth Road

London

SE1 6HZ

Date: 31 August 2016

SECTION 1

PRELIMINERY MATTERS

1.0 	INTRODUCTION

1.1	PRELIMINARY PARTICULARS ETC.

1.2	GENERAL MATTERS

1.3	STATUTORY/GENERAL OBLIGATIONS

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.6	THE WORKS GENERALLY

1.7	CONTRACTUAL MATTERS

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.9	APPROVED SUB-CONTRACTORS

1.10	PROVISIONAL DAYWORKS

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

1.12	LIAISON

1.13	PHASING OF WORKS

1.14	TEMPORARY SIGNAGE

1.15	SITE RULES

1.16	SECURITY CLEARANCE

APPENDIX 1

SECTION 2

Health & Safety

PRE TENDER HEALTH & SAFETY PLAN

2.0	GENERAL REQUIREMENTS

2.2	CHECKING/HANDLING AND SITE STORAGE OF MATERIALS + COMPONENTS

2.3	LOADING

2.4	WORKING AT HEIGHTS

2.5	PROXIMITY TO PUBLIC

2.6	METHOD STATEMENTS

2.7	PRE-TENDER HEALTH + SAFETY PLAN

2.8	CLIENTS OWN SAFETY REPRESENTATIVE

2.9	HEALTH + SAFETY FILE

SECTION 3

TRADE PREAMBLES

3.01	MATERIALS		WORKMANSHIP

SECTION 4

SCHEDULE OF WORKS

SECTION 5

TENDER SUBMISSION

5.0	TENDER SUBMISSION

5.1	TENDER ACCEPTANCE

5.2	BASIS OF PRICES

5.3	AWARD CRITERIA (other than price)

5.4	PRICING SCHEDULE

SECTION 6

POINTS OF CONTACT

SECTION 7

CONTRACT TIMETABLE

7.0	CONTRACT TIMETABLE

SECTION 1

Preliminary Matters

1.0 	Introduction

- An Act of Parliament founded the Imperial War Museum in 1920 following the Great War to act as a National War Museum to collect and display material relating to that War. Due to the general interest displayed by the Dominion Governments this led to the title “Imperial War Museum” (IWM).

- Imperial War Museum was opened in 1920 at a site in Crystal Palace, but eventually acquired its current home at Lambeth Road in July 1936 on the site of the former Bethlem Royal Hospital.

- At the commencement of the Second World War the Museum’s terms of reference were extended to cover both the Great War and the Second World War, and again in 1953 to cover all military operations involving British & Commonwealth forces since August 1914.

- In addition,  IWM provides exhibits of educational and general interest to members of the public

- As of 2016, Imperial War Museums consists of 4 other major public tourist attractions:

- HMS Belfast moored in the Pool of London

- Churchill War Rooms

- IWM North, Trafford, Manchester

- IWM Duxford

1.1	PRELIMINARY PARTICULARS ETC.

1.1.1	Employer

- Imperial War Museums

Lambeth Road

London

SE1 6HZ

1.1.2	Contract Administrator

- M J Boyle

- Projects Manager

Imperial War Museums

Lambeth Road

SE1 6HZ

Tel 020 7091 3107 e-mail mboyle@iwm.org.uk

1.1.3 	Principal Designer.

IWM represented by M J Boyle as above.

1.1.4	General Description of the Works

The works comprise the external repairs and redecorations to corrugated metal roof cladding panels. The Plastisol system is beginning to break down but the roof is not presently leaking. We are seeking a suitable product to be applied to the panels to extend the life of the cladding by a further 15 years.

1.1.5	Location and Access to Site

The site is located near to Waterloo station/Elephant and Castle. There is limited on-site contractor parking facility by prior agreement with IWM

1.1.6	Tender Drawings

The following drawings form part of this specification and will become contract documents.

IWML-CSD-ROOF-001

1.2	GENERAL MATTERS

1.2.1	Limits of site operations

The Contractor shall limit his operations to the immediate area of the works.

The Contractor shall be responsible for storing materials, arranging for loading/unloading materials and organizing his works in a manner that does not interfere with other parts of the site and access to same.

1.2.2	Preliminary investigation

The Contractor shall be deemed to have visited and inspected the site and to have examined drawings (if any) and contract documents and to have adequately acquainted himself with local conditions, accessibility of the works and site, the nature of the ground and subsoil, supply of and conditions affecting labour, the availability and supply of materials, water, electricity and telephones, all in relation to the execution of the works as no claim on the ground of want of knowledge in such respect will be entertained.

1.2.3	Abbreviations and references

- The following abbreviations, references or terms are used in the Specification.

CP 	- 	British Standards Code of Practice

BS 	- 	British Standard Specification

The term ‘Contractor’ used throughout this document relates to the Contractor who is contracted to carry out and complete the works. The contractor shall be the ‘Principal Contractor’ under the CDM regulations.

Reference to CA means the Contract Administrator or other consultant, when appointed; with authority under the terms of the contract to act is such a capacity.

The reference to Building Inspector means Building Control Officer, District Surveyor or any other person with authority under statute to require compliance with any regulation or byelaw.

The terms ‘Approved’, ‘Selected’, or ‘Directed’ means the approval, selection, or direction of IWM/CA.

The term ‘Provide’ means that the item(s) is to be supplied, delivered and fixed at the expense of the builder.

The term ‘Day Work’ is applicable to the work of an unknown or unforeseen nature that could not be specified and therefore estimated for during the preparation of the schedule of works.  Such work is usually paid for on a time and material basis with the Contractor submitting evidence of hours of work and invoices for materials, plant, etc.

1.3	STATUTORY/GENERAL OBLIGATIONS

1.3.1	Safety/Health and Welfare

Allow for complying with all enactments, regulations and working rules relating to safety, health and welfare of work people.

The Contractor’s attention is drawn to Section 2 of this Specification, which covers Health & Safety matters specifically.

1.3.2	Working Hours

Monday to Friday - 0730 - 1700.  Weekend working will only be allowed with the prior agreement of Employer. The contractor may extend working hours on weekdays subject to approval of Employer.

1.3.3	Statutory Regulations

The works must be undertaken in compliance with Statutory Regulations, Building Regulations, Gas Safety Regulations, Water Bye Laws and IEE Regulations.  Contractor is to allow in his tender for compliance, particularly in respect of service installations, which require contractors design input.

1.3.4	Noise

Fit all compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles.

1.3.5	Public and Private Services

Adequately protect, uphold, maintain and prevent damage to all services.  Do not interfere with their operation without consent of the service authorities or private owners or the CA as appropriate.

1.3.6	Existing Features

Prevent damage to existing buildings, fences, gates, walls, roads, paved areas, other of the site features, which are to remain in position during the execution of the works.

The contractor shall be responsible for properly making good to the satisfaction of the C.A. all areas disturbed during the execution of the works. Contractor is to plan the works and in particular the maneuvering of plant and machinery so as to cause minimal disturbance to adjacent areas.

1.3.7	Structural Fabric

The contractor is advised that Lambeth Road site is a grade 2* listed building. Works must be undertaken in a manner that causes the minimum of damage/disturbance to the fabric of the building.

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.4.1	Supervision

Accept responsibility for co-ordination, supervision and administration of the works, including subcontractors.  Arrange and monitor a programme with each subcontractor, supplier, Local Authority and statutory undertaker and obtain and supply information as necessary for co-ordination of the work.

The Contractor is to provide a competent person to constantly supervise the works and shall inform the CA of the name of such person.

The person is to be able to adequately receive directions given to him by the CA and is to remain on site throughout the duration of the contract and shall not be changed without prior approval from the CA.

The person is to implement the requirements of the Construction (Design & Management) Regulations 2007 as they apply to this project, and must assist the Planning Coordinator in maintaining the health and safety file for this project.

1.4.2	Insurances

Before starting work on site submit to the employer (through the CA) documentary evidence and/or policies and receipts for all insurances which are required to be taken out by the Contractor.

1.4.3	Programme

As soon as possible and prior to starting work on site, prepare, in an approved form, a programme for the works that should make allowance for all:

1.Sub-contractors works including completion of drawings etc.

2. Work resulting from instructions issued with regard to the expenditure of provisional sums.

3. Other work concurrent with the contract.

1.4.4	Commencement of Work

Inform CA as least seven working days before proposed date for commencement of the work on site.

1.4.5	Monitoring

Record progress on site and keep a copy of the programme on site.  Update or re-draft without delay if circumstances arise which affect the progress of the works with copies of all revisions to CA.

1.4.6	Site Meetings

Hold weekly site meetings as necessary for the proper management and co-ordination of the contract and as required by the CA.  Meetings will normally be held every four weeks.  Attend all meetings and inform subcontractors and suppliers when their presence is required.  Notify all consultants.  The CA will take the Chair and minutes will be taken and distributed by the CA.

1.4.7	Notice of Completion

Give CA at least one week’s notice of anticipated dates of practical completion of the whole or parts of the works.

1.4.8	Measurements

Give reasonable notice to the CA before covering up work, which the CA requires to  be measured.

1.4.9	Covering Up

Give not less than one working days notice to the CA or Clerk of Works before covering up.

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.5.1	Accommodation

Provide any deemed necessary temporary office, mess room and sanitary accommodation required. This can be sited at All Saints rear yard, an adjacent building to the museum.

1.5.2	Protection

Provide temporary fencing, hoardings, screens, vans, planked footways, guard rails, gantries and the like as may be necessary for protecting the public and others, for the proper execution of the works and for meeting the requirements of the Local or other Authority.

1.5.3	Working at height

The Contractor must allow in his tender for providing a safe means of access for operatives to all areas subject to works. In deciding what access equipment is necessary, the Contractor must fully appraise the works, undertake a risk assessment and identify the most suitable method of operation. There is a Mansafe  fall arrest system installed throughout the roof which contractors would be authorised to use provided that they can provide relevant training certificates.. If scaffolding is required it must be in accordance with BSEN12811-1 (2003).

Unless the scaffold is a basic configuration described in recognized guidance, for example NASC publication TG20-08 (volume 1), the scaffold should be designed by calculation, by a competent person, to ensure it will have adequate strength and stability.

If proposing a scaffold that is termed a ‘basic’ scaffold under volume 1 of TG20-08, the contractor must demonstrate to the CA that the scaffold meets all of the criteria for a basic scaffold.

Any proposed modifications or alterations outside a generally recognized basic configuration must be designed by a competent person.

Where the scaffold falls outside the scope of a ‘basic’ scaffold, the contractor must supply to the CA detailed information to show compliance with the BSEN12811-1 standard. Further advice on what is required is contained in volume 2 of TG20-08.

As a minimum, this must include drawings showing the general scaffold arrangement, a method statement describing the erection and dismantling sequence, the permitted platform loadings, restrictions on use and calculations to show compliance with the standard.

On completion of the scaffold erection, a handover certificate must be issued to the CA, which should refer to the relevant drawings, permitted working loads, and restrictions on use. The handover certificate must be issued by a competent person (by means of experience or assessment, appropriate for the type and complexity of the scaffold erected).

Where the scaffold is the subject of design the person responsible for the design shall make an inspection and certify the scaffold is erected in accordance with the design and drawings.

Regular inspection should be carried out (minimum weekly) also by a competent person.

Erection, alteration and dismantling of all scaffolds must be under the direct supervision of a competent person – an advanced scaffolder or design engineer with the appropriate industry experience.

The contractor is referred to HSE guidance note for a list of scaffold structures that need to be the subject of design.

For the avoidance of doubt, the means of access to any particular working platform must be arranged so that it is outside of the working platforms and does not obstruct or restrict their use. eg a ladder access tower, or platform steps. (The use of trap doors within the platforms will not be permitted).

If ladders are proposed for access to scaffold platforms, these must not be used for bringing materials or tools of any sort to the working platform. A separate arrangement must be made for bringing material to the platform level – hoists etc.

1.5.4 	Water

Water for the works is available on site free of charge. Contractor is to make temporary arrangements for storing and distributing around the site.

1.5.5	Electricity

The contractor is to make his own arrangements for the supply of power he may require.

1.6	THE WORKS GENERALLY

- GENERALLY

1.6.1	Good Practice

Where and to the extent the materials, products and workmanship are not fully specified they are to be:

1.	Suitable for the purposes of the work stated in or reasonably to be inferred from the contract documents, and

2.	In accordance with good building practice including the relevant provisions or current BSI documents.

1.6.2	Manufacturers Recommendations

Handle, store, prepare and use, or fix each product in accordance with manufacturers printed or written instructions/recommendations.  Inform CA if this conflict with any other specified requirement.

1.6.3	Workmanship

The works to be carried out by, or under the close supervision of, experienced tradesmen, skilled in the particular type of work.

1.6.4	Service Runs

Make adequate provisions for services, including unobstructed routes and fixtures.  Wherever possible, ducts, chases and holes to be formed during construction rather than cut.

- PRODUCTS/MATERIALS GENERALLY

1.6.5	Products to be New

Unless otherwise specified.  Ensure that the whole quantity of each product and material required to complete the work is of consistent kind, size, quality and overall appearance.

Handle, store and fix products with care to ensure that they are not damaged when incorporated into the works.

1.6.6	Or Equivalent Approved

Means that the products of different manufacturers may be substituted if prior approval has been obtained, but the CA reserves the right to insist on a named product(s) the rates or prices will be held to be based on the product(s) specified, unless agreed otherwise.

1.6.7	Proprietary Names

The phrase ‘or equivalent’ is to be deemed included whenever products   are specified by proprietary names.

1.6.8	Single Source

Where a choice of manufacturer or source of suppliers is allowed for any particular product or material, the whole quantity required to complete the works must be the same type manufacturer or source.  Do not change without approval.  Produce written evidence or sources of supple requested by CA.

- PROTECTION/CONDITION OF WORK/DRYING OUT

1.6.10	Site administration and Security

Adequately safeguard the site, products, materials, plant, the works and any existing buildings affected by the works from damage and theft.  Take all responsible precautions to prevent unauthorized access to the site, the works and adjoining property.

1.6.11	Stability

Accept responsibility for stability and structural integrity of the works during the contract, and support as necessary.  Prevent overloading/damage.  Details of design loads may be obtained from the CA. The contractor must not overload the roofs which are designed to accommodate for traffic for maintenance purposes only. Do not store materials on the roof without prior agreement form the CA.

1.6.12	Inclement Weather

Adequately protect the works from damage by inclement weather and use all reasonable and approved building aids and methods to prevent or minimize delays during cold or inclement weather.

1.6.13	Rubbish

- Remove rubbish and debris as it accrues and keep the site and works clean and tidy.

1.6.14	Drying Out

Control the drying and humidity of the works and the application of heat to prevent:

1.	Blistering and failure of adhesion.

2.	Damage due to trapped moisture.

3.	Excessive movement.

- ALTERATIONS/EXTENSIONS/MAINTENANCE WORK

1.6.19	Make Good

Means carry out local remedial work, including the following as appropriate and necessary to leave the work sound and neat to approval:

1.	Remove defective parts of existing finishes and components and around any stated features.

2.	Fill, dress down, piece in, patch, extending existing finishes, make minor repairs and adjustments.

3.	Re-fix or re-stick.

4.	Redecorate.

5.	Consequent upon alteration, extension and maintenance work is deemed to be included in such items.

- SAMPLES/APPROVALS/TESTING/INSPECTION

1.6.22	Samples - materials

Where approval of products or materials is specified, submit samples or other evidence of suitability.  Do not confirm orders or use materials until approval has been obtained.  Retain approved samples on site for comparison with products and materials used in the works.  Remove when no longer required.

1.6.23	Samples - workmanship

Where samples of finished work are specified, obtain approval of stated characteristics before proceeding with the works.

Retain approved samples on site for comparison with the works.  Remove samples which are not part of the finished works when no longer required.

1.6.24	Approvals

Inspection of any other action by the CA must not be taken as approval of materials, products or work unless the CA confirms in writing the express terms referring to:

1.	Date of inspection.

2.	Part of the work inspected.

3.	Respect or characteristics which are approved.

4.	Extent and purpose of the approval.

5.	Any associated conditions.

- FIXING/FASTENINGS/ADHESIVES/MORTAR

1.6.25	Fixings Generally

Use fixings and jointing methods and types and spacing of fastenings which are suitable having regard to:

1.	Nature of compatibility with products/materials being fixed and fixed to.

2.	Recommendations of manufacturers of fastenings and manufacturers of products/material being fixed and fixed to.

3.	Materials and loads to be supported.

4.	Conditions expected in use.

5.	Appearance - being subject to approval.

- WORK AT COMPLETION/MAKING GOOD DEFECTS

1.6.26	Cleaning

Cleaning materials and methods are to be as recommended by manufacturer of product being cleaned.

In the absence of such recommendations cleaning materials and methods are to be approved by CA.

1.6.27	Certification

- Not required.

1.7	CONTRACTUAL MATTERS

1.7.1	Form of Contract

The Form of Contract will be the JCT Agreement for Minor Works 2011 Edition, together with amendments to date, including April 2015 amendments incorporating the CDM Regulations 2015 .

The Contractor should include for all costs incurred in compliance with the Terms and Conditions of the Contract.

- The Specification and drawings (if any) will become contract documents.

The Contractor must include for all works shown on the drawings, whether referred to in the Specification or not, and for all works which are not included in the Specification, or on the drawings which could reasonably be expected to be included in the works.

1.7.2	Appendix 1 to these Preliminaries

Appendix 1 at the end of these Preliminaries describes the contractual details that will be included in the Contract Conditions.

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.8.1	Provisional Sum

- Not Used

1.8.2	Provisional Repairs

- Refer to section 4

1.8.3	Prime Cost Sums

- Not used.

1.9	APPROVED SUB-CONTRACTORS

Not used

1.10	PROVISIONAL DAYWORKS

Not used.

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

Building services and fabric are maintained on the site by Engie, the FM contractor appointed by IWM.  This contractor will be permitted access to the buildings at all times. The normal activities of this contractor are not expected to interfere with these contract works.

1.12	PHASING OF WORKS

To be agreed.

1.13	TEMPORARY SIGNAGE AND FENCING.

The working area is to be cordoned off using Barrier tape until the application has had time to dry.

- Contractor/subcontractor advertising will not be permitted.

1.14	SITE RULES

The Museum operate specific rules for ‘Safe working Conditions and Protection of  its Staff, Visitors and Collection’. Please see IWM’s ‘Control of Contractors’ document attached.

These are available as a small booklet from the C.A. The Contractor will be required to sign the form included in the booklet to confirm he has read the rules and will ensure all work is carried out in accordance with same.

The C.A. will carry out a site induction for all contractors’ operatives on the morning of the first day of starting on site.

1.15	SECURITY CLEARANCE

All of the contractor’s staff will need to be security cleared prior to undertaking any work on site. Security forms are available from the guardroom and must be competed and returned at least 3 weeks prior to commencement.

1.15.1	The Contractor is responsible for the ensuring that all their employees and any sub-contractors, agents etc. who are due to work at the Premises, for longer than four weeks in duration in the delivery of the Contract, whether in a continuous period, or over the duration of the Contract, have been security vetted to Basic level, as defined by Disclosure Scotland1.

1     Please note that Disclosure Scotland is the agency that all security vetting providers will approach to undertake the security vetting.

1.15.2	For Contractor’s with a small employee base, IWM will be prepared to undertake the security clearance on behalf of the Contractor, provided that this is made known at the time of the appointment, and that this agreed by IWM, and that the Contractor agrees to pay the administration charge of £41 per clearance, that IWM is charged for this service.

1.15.3	This requirement will apply to an employee who has not worked at an IWM branch, within the previous 12 months.

1.15.4  A minimum of 48 hours before any individual commences work on the Premises, the Contractor is required to complete and submit the Security Notification Form (see Appendix X) to the Technical Security Manager at IWM London.

1.15.5	Upon arrival at the Premises, each new employee must report individually to the Control Room with a copy of their certificate of proof of vetting, and some form of ID. IWM will note the details of the certificate of proof of vetting, but will not retain any documents.

1.15.6	IWM will accept a certificate of proof of vetting which is dated within 12 months of the date of their site commencement at IWM, although IWM reserves the right to request they are security vetted, at its discretion, which IWM agrees not to action unreasonably.

1.15.7	Failure to comply with this requirement could result In the employee not being granted access to IWM.

APPENDIX 1

- Contract:

The form of contract will be the JCT Agreement for Minor Building Works 2011 together with any amendments to date.

- ARTICES OF AGREEMENT:

- Recitals:

- First.   The work required is external repairs and decorations to the Plastisol coated roof cladding panels at IWM Lambeth Road site. The factory applied plastisol coating has begun to break down on the cladding and requires an overlay treatment to fully protect and extend the life of the panels by a 15 year period.

- Second. Documents comprise the contract specification and drawing number IWM-CSD-ROOF-001

Third. The Contractor shall supply a fully priced copy of the specification to the employer.

Fourth. IWM is not a contractor under the CIS rules.

Fifth. This project is considered not to be notifiable under the CDM Regulations 2015.

Sixth. Not applicable.

Seventh. Applies, excepting paragraphs 3,4 and 5.

Articles :

Article 3 	The Contract Administrator shall  be Mr M Boyle, FM manager IWM.

Article 4	Not applicable.

Article 5	Not applicable.

Article 7 (and Schedule 1) the arbitration provisions of article 7 and schedule 1

apply.

- Contract Particulars:

- Base date					 N/A.

- Clause 1.1					3 weeks.

- Beginning on the date of receipt by the contractor of the tender documentation.

Clause 2.2	Commencement		To be agreed

Completion

Clause 2.8	Liquidated Damages	£  N/A.

Clause 2.10	Rectification Period	12 months

  - Percentage is 95%

  - Percentage is 95%

    - 3 months

  - Deleted

5.3.2	Contract Insurance required is £10million minimum

5.4A & 5.4B	Clause 5.4A applies % addition to cover fees – 20%

7.2	Nominator shall be President of the RICS

Schedule 1	Appointment of arbitrator shall be the President of The Royal Institute of Chartered Surveyors

SECTION 2

Health & Safety

PRE CONSTRUCTION INFORMATION.

GENERAL REQUIREMENTS

The works that are the subject of this Specification are subject to the Construction (Design & Management) Regulations 2015. Specifically Part 4 – General Requirements for all construction sites. The principal contractor will be required to comply with the relevant regulations contained in part 4.

The project however is not considered to be notifiable under those regulations.

The Contractor will be required to operate as the ’Principal contractor’ under those regulations and co-ordinate and manage Health & Safety issues during construction work, and, liaise with the Contract administrator during the course of these works.

Prior to commencement the contractor must ensure a construction phase health and safety plan has been produced, and that adequate provision is made on site for provision of welfare.

The Contractor will be responsible for ensuring that subcontractors employed by him operate in a safe manner appropriate to the work they are employed to undertake.

Special attention is to be paid to the  Means of providing safe working access to the areas subject to repair – whether this be by use of scaffold or use of existing latch way systems.

2.0	INFORMATION ON BUILDING/ENVIRNMENT/RISKS.

2.1	Proximity to Public and staff

The work will involve working in areas above public pathways at ground level.

The principal contractor must consider carefully measures to be employed to prevent injury to these persons.

Contractor is to organize works such that pedestrian access into and egress from buildings is not obstructed.

The roof areas worked on to be properly fenced, signed and cleared of all loose material/tools/plant/materials and swept down to leave clean and safe.

2.2	Checking/Handling and Site Storage of Materials and Components

Contractor is to pay particular attention to how paint and associated materials are handled and stored.

Manufacturer’s COSSH data sheets should be followed and available on site for inspection by CA.

Some products are likely to be flammable - specific provisions for safe storage and handling must be identified by Contractor.

2.3	Working at Heights

Much of the work to be executed is at high level.

The contractor is to detail the measures to be taken to ensure safe working access and platforms.

Details of  access proposals are to be included in Method Statement.

If any scaffold is proposed then handover certificates are to be issued by the scaffolding contractor stating specifically:

    - It is complete and complies with the Construction (Working Places) Regulations 1966 and relevant British Standards.

    - The distributed loads which are permitted on the working lifts.

- Guarding of working platforms.

- That inspection is required every 7 days, or immediately following adverse weather.

- Inspection findings to be recorded.

- No unauthorized modifications or alterations are to be made.

2.4	Risk Assessments and Method Statements

The Contractor shall provide, with his tender, an outline Method Statement setting out how the work is to be approached, organized and monitored in order to ensure the works are undertaken both efficiently and safely.

2.7	Construction phase plan.

Before works commence on site, the Principal contractor must draw up a Construction Phase Plan. This must set out the health and safety arrangements and site rules, taking account of the nature of the site and use of the building.

Throughout the construction phase the principal contractor is to ensure the plan is reviewed, and updated, or revised so that it continues to be sufficient to ensure construction work is so far as is reasonably practicable carried out without risks to health or safety.

The ‘Principal Designer’ for this project is M J Boyle IWM Project Manager. The Principal contractor shall liaise and cooperate with the Principal Designer in preparing the plan and providing information for inclusion into the Health and safety File.

2.8	Health & Safety File

The Principal Designer will prepare the Health and Safety file. The Principal Contractor is to provide information on request to enable the file to be prepared. Such information is likely to include, but not be limited to

a.	The contractor’s risk assessment and method statements.

b.	Copies of manufacturer’s current technical literature and COSHH data sheets for all materials, plant and equipment selected by the contractor.

.

SECTION 3 – MATERIALS AND WORMANSHIP

3.02	WORKMANSHIP

0.01	Generally

Comply with the requirements of BS 6150 ‘Painting of Buildings’ subject to any qualifications given below.

Comply also with the requirements of BS 8000 Part 12 1989  ‘Workmanship on Building Sites’.

0.02	Ordering of Materials

Obtain liquid materials in containers sized to suit the extent of the work and to prevent deterioration of residual quantities in opened containers. Maximum generally 5 litres.

0.03	Order of Working

Work in a sequence that ensures the finished work is not spoilt by dust and debris arising from subsequent preparation work.  When planning the work consider the need for cleaning, washing down, heating or drying equipment.

Wash, wipe or otherwise remove any dirt or contaminant.  (Do not use a brush as this move dust to another surface.)

Remove any moisture or condensation by drying surfaces and/or applying general warmth if necessary.

0.08	Unsuitable Conditions

Do not apply coatings when:

  - Airborne dust and grit is present.

  - Air temperature is below 4 degrees centigrade.

  - Relative humidity exceeds 80%.

        - Temperature of the surface is too low eg. heavy metalwork, plastered external walls etc.

  - Substrates have not properly dried out.

  - Lighting is inadequate.

SECTION 4

SCHEDULE OF WORKS

Specification Objectives

The object is to extend the life of the metal roof as well as maintain some of the ancillary aspects to the roof I.E replace any missing fillers/ replace the mastic at the base of the dome roof and to cap the end laps of the metal

roof.

The following is a summary of the issues, defects and vulnerable areas our survey detected. Provision must be made within the specification for tackling each of them.

- Flaking paint/ bare metal

- Previously coated areas

- Replace fillers

- Mastic replacement

- New cover for end laps

- Additional scaffolding requirements.

Flaking paint/bare metal

Any flaking paint will require scraping back to a firm

edge and any bare metal will require spot priming.

Previously Coated Areas

There are areas on the roof that have been previously

Coated which will require solvent wiping before the

System is applied

Replace fillers

Any fillers missing from the roof are to be replaced. Allow the provisional sum of £2000 for supply and fitting of fillers to CA instructions.

Mastic sealer

Where the dome glazing meets the frame at the base it will require the existing mastic raked out to a depth of 10mm and repointed to a neat watertight finish.

Cut edge corrosion

.Where cut edge corrosion is present any lifting/peeling plastisol coating will need to be scraped back to a firm edge. Bared metal will need to be cleaned down and primed followed by application of the coating system described below.

.

End laps

The endlaps are open to the elements and the insulation is saturated or has been disturbed by birds. Allow provisional sum of £2000 for providing and fitting of end caps to insulated panels where these are missing

General Guidance Clauses

Inspect roofing substrate to ensure integrity and suitability for coating.

Previously coated areas must be carefully checked to ensure integrity and adhesion to the substrate. Pay particular attention to solar reflective (especially aluminum pigmented) coatings.

Ensure by adhesion tests that the proposed coating system is fully compatible with existing surfaces. Testing should include determining the necessity/selection of a primer.

All roof surfaces to be treated should be prepared to a clean, sound, dry, contaminant-free condition.

Inspect all roof fittings, i.e. trims, flashings, capping’s, etc. Make good all defective items (renew if necessary) prior to coating. Note if defects are found – seek approval from CA on how to proceed and confirmation of repairs needed.

Inspect all old remedial repairs, patches, cracks, etc. Make good where necessary (remove unsound repairs) prior to coating.

Where air intake units of air conditioning/ventilation systems are present, the contractor should consult the client (and if necessary) the coatings manufacturer to agree on steps necessary to prevent entry of solvent vapors (where applicable) into the equipment.

Inspect all upstands, vents and any other protrusions. Make good or replace prior to coating.

Do not remove, or suspend, cables, lightning conductors, wires, etc., without due authorization. Lightning conductors should not be coated under any circumstances. Other items should not be coated unless agreed.

Before work commences ensure all health and safety data sheets are read and understood.

Coverage rates of all materials will vary according to weather conditions, dimensions and nature/condition/surface profile etc. Make appropriate allowances where applicable.

On completion of all coating activity, dispose of all empty material cans, material contaminated tools, overalls and site waste in accordance with the relevant environmental regulations.

Completely clear site including scaffolding, etc.

All work should be carried out in a safe and tidy manner. Relevant Health & Safety/COSHH regulations should be observed at all times.

Inspection and Preparation

It is important to thoroughly inspect all plastic coated metal profile surfaces to establish degree of corrosion, substrate and inter-coat adhesion, and overall condition. Test areas should be prepared and a St Andrews Cross Cut adhesion test should be carried out on a south facing orientation to establish suitability for coating and whether a

primer is required. Any existing factory coated or re-painted surfaces should also be checked for soundness and suitability for treatment. Note. If no adhesion test is carried out, it will be assumed the substrate is adequate for over coating.

Scrape away any heavy or loose deposits of surface contamination, including suspect oil, grease or dirt. Thoroughly clean down all areas to be treated by high pressure water jetting or vigorously scrubbing with a suitably diluted detergent cleaner. Thoroughly rinse down with clean water until all residues are removed and allow to dry. “Plastisol” (™ Corus) coatings should be thoroughly wiped with solvent. (Non “Corus” plasticized PVC coatings will be subject to compatibility test verification).

Surface chalking (if present) should be removed vigorous rubbing etc. with a fine ‘scouring pad (eg ‘Scotchbrite’) followed by removal of debris Note: Abrasion of PVF2 coatings (other than around loose or corroded areas) with crushable or polishable abrasives (eg emery, carborundum, sandpaper etc) should be avoided as this may impair adhesion of subsequent coatings. Coarse white aluminum oxide paper/cloth (P120 or rougher) may be used where necessary to “score” the coating to help promote adhesion.

Slick surfaces or hard coatings should be abraded and/or solvent wiped (adhesion test).

Inspect all roof fittings, i.e. trims, flashings, ridge details (capping’s, filler pads etc.) etc. Make good any damage, distortion, gaps etc. or replace prior to coating.

Remove all loose, suspect and friable materials, cutting back to a sound firm edge. Prepare all surfaces to a clean, dry and sound condition to promote satisfactory adhesion, if necessary removing existing coatings and test patches.

Localized corrosion should be removed by suitable mechanical means (grinding, abrasion, etc.) and the adjacent areas prepared to a firm feathered edge. Spot prime with suitable primer as specified in the Priming/Pre-treatment section. Particular attention should be paid to vulnerable areas such as bolt holes, edges and ends.

Remove any unsound remedial repairs and make good. Replace broken and defective sheets as required.

The original factory applied coating should be checked for good integrity and adhesion to the metal profiled sheets. Remove if found to be suspect.

Mechanically abrade any ferrous metal surfaces included in the coating schedule. Remove debris and apply suitable primer as specified in the Priming/Pre-treatment section. Non-ferrous metals should also be abraded

Coating Specification and application

Priming:

Apply one coat of Tor TP Epoxy Surface tolerant primer (EP118) (or equivalent) by brush at a coverage rate of 4 to 5 metre per litre to prepared bare metal areas of metal cladding tin roof. Allow to dry for 16 hours. Overcoat within 7 days ensuring surfaces are clean.

Waterproofing System:

Torclad Colour Dynamic Semi-Gloss Finish (RC402) – 15 year (or equivalent) – to be applied by roller or spray system.

To weathered surfaces/previous systems apply two overall coats of Torclad colour Dynamic Semi-Gloss Finish (RC402) – 15 year at an approximate rate of 8-10m2/litre per coat. Allow tin roof cladding panels to dry for at least 8 hours (maximum 48 hours) between coats. Note: Rough, uneven or chilled surfaces may significantly reduce the above coverage rates – make further material allowances where necessary. Colour to match existing

Ensure good application of materials, paying particular attention to profile angles, edges, bolt heads and protrusions.

Allow finishing coat to dry overnight, then carefully inspect the painted cladding panels to identify any near misses or low thickness areas and touch in.

SECTION 5

TENDER SUBMISSION

Tenderers will be required to comply with the following in order for their tender to be considered.

If the tenderer does not comply fully, then the tender may be rejected by the Museum.

5.1    Site Visit

As part of the process tenderers shall visit site to acquaint themselves with the site, the extent of the work and the conditions under which it is to be carried out, by contacting M Boyle at the Lambeth Road Site on 020 7091 3107, or mboyle@iwm.org.uk to make arrangements for site visits.

5.2    Tender Acceptance

The ‘Tender Declaration Form’ included in this specification must be submitted, in an envelope, by post. Electronic submissions or e-mails will not be considered.

The tender envelope should be marked TENDER DOCUMENT CONTRACT NO IWM/FM/1569. Other than this envelopes and/or packages should be plain and bear no reference to the tenderer's name; franking machines that automatically print the company name should not be used.

5.3    Basis of Prices

All prices must be quoted on the basis of lump sum fixed price based on no re-measurement, as indicated in the accompanying documents. Quantities given are for guidance only and prices entered should exclude VAT.

5.4   Basis for tender submission

Please view drawing IWML-CSD-ROOF-001.

The drawing shows the roof divided up into 7 sections, numbered 1 to 7. What roofs will be done this financial year will depend on costings. We may decide to phase the work over two financial years in order to budget to complete all areas 1 to 7.. Total area to be coated is approximately 2180sqm.

Please provide separate quotes against:

- Roof Area 1. 260.2 m²

- Roof Area 2  493.8 m²

- Roof Area 3  470.3 m²

- Roof Area 4  493.5 m²

- Roof Area 5    39.6 m²

- Roof Area 6  210.3 m²

- Roof Area 7  212.0 m²

5.5    Supporting Information

Tenderers shall as part of tender submission include the following information:

- An outline Method Statement describing how the work are to be undertaken and what arrangements will be put in place to manage health and safety.

- The names & contact details of two references where the company has carried out similar work.

- Copy of the company’s Health & Safety Statement

.

- This tender document complete, with each page initialed to indicate reading.

- The schedule of work items priced up.

- The tender form properly completed and signed.

- Proposed programme of works.

5.4    Award Criteria (other than price)

IWM does not bind himself to accept the lowest or any tender; when awarding the Contract in addition to price IWM will have regard to the following.

- Accuracy of submission.

- Compliance with tender instruction for submission of documentation.

- Suitability of the Method Statement and Risk assessment referred to above.

- Tenderer’s ability to demonstrate his competence in carrying out works of this nature.

SECTION 6

CONTRACT TIMETABLE

7.0	CONTRACT TIMETABLE

# The following schedule represents the proposed timetable for the appointment of the Principal Contractor and commencement of the contract for the external repairs and decorations of the Plastisol coated roof cladding panels as detailed within the tender documents on behalf of the Trustees of the Imperial War Museum.

| Issue of Tenders | 31 August 2016 |
| --- | --- |
| Tender Period | 3 weeks |
| Tender Submission Date/Time | 2pm on 21 September 2016 |
| Tender Evaluation Period | 22-28 September 2016 |
| Contract Award | 30 September 2016 |
| Contract Start Date | To be advised |
| Contract Completion Date | To be advised |

- Tender Declaration; Contract Reference: IWM/FM/1569

Project.	 Lambeth Road Tin roof Repairs and redecorations

To:		The Trustees of the Imperial War Museum

I/We having read the following documents delivered to me/us in respect of the above Contract:

Do hereby offer to execute and complete in accordance with the Conditions of Contract the whole of the works as described for the sum of: -

£_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ (excluding VAT)

(_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ amount in words)

- Company Name:	___________________________________________________

Address:			___________________________________________________

___________________________________________________

___________________________________________________

Tel. No.		_____________________

Fax. No.		_____________________

Signature:			___________________________________________________

Print Name:		___________________________________________________

In the Capacity of:	___________________________________________________

(The above named person is duly authorised to sign tenders for and on behalf of the Company)

# Dated this . . . . . .day of  . . . . . . . . . . . . . . . . . . . . . . . 2016

TENDER BREAKDOWN.						(£)

Preliminaries   including access     				                                 ________

•	Roof Area 1. 260.2 m²                                                   ________

•	Roof Area 2  493.8 m²                                                   ________

•	Roof Area 3  470.3 m²                                                   ________

•	Roof Area 4  493.5 m²                                                   ________

•	Roof Area 5    39.6 m²                                                   ________

•	Roof Area 6  210.3 m²                                                   ________

•	Roof Area 7  212.0 m²                                                   ________

Total Tender figure.						_________

(Exclusive of VAT)

Appendix

STAFF SECURITY CLEARANCE

NOTIFICATION FORM

CONTRACTOR:

|  |  |  | Level of CRB |  |  |
| --- | --- | --- | --- | --- | --- |
| IWM Branch | Start Date | Staff Name | Basic | Standard | Enhanced |

| Signed |  |
| --- | --- |
| Name |  |
| Job Title |  |
| Date |  |

Please complete all sections of the Form and forward to IWM’s Technical Security Manager at jpawley@iwm.org.uk or addressed to Technical Security Manager, CWR, Clive Steps, King Charles Street, London SW1A 2AQ, a minimum of 48 hours prior to the individual(s) commencing work at IWM.