[image: image4.emf]  TEMPORARY TRAFFIC REGULATIONS ORDER                  TRF4B (TfL) 200 8 (2)                 (formerly TR514) (4.6.96)     PLEASE COMPLETE  EVERY   SECTION OF THIS FORM     AGENCY:____________________________     AGENT: ________ _____________________ ______WBS Code: _________ ______ _______     Request for an Order under Section 14 - 16 of the ROAD TRAFFIC REGULATION  ACT 1984     On the  TLRN Road  ______________ ____________________________________                                    (Full statutory name of GLA road)                Type of Order:   Tempo rary (Section 14 - 16)     Council in whose area the   Le ngth(s) of road is/are situated:  LB  of    ______       Any other Council concerned:  _______________ ___________________________________     If a road bridge, give local name of *road/railway/canal/river it crosses:  N/A       1.   Reason for restriction  (Note : It must be within scope of  s ection 14(1) of Road Traffic     Regulation Act 1984    ie  WORKS   ( Note:  state NATURE of works)   or  LIKELIHOOD OF  DANGER TO THE PUBLIC  or  SERIOUS DAMAGE TO THE HIGHWAY   ( Note:  state  NATURE of dange r/damage)   or  LITTER CLEARING AND CLEANING ) :               2.   Nature and extent of restriction required on GLA road . Be specific  –   DO NOT  simply state  ‘see TM plans.’   (Notes :  A plan  must   be enclosed. State   exact  terminal points, directions   (particularly for speed l imit changes,)  and times of operation . Use a separate sheet if  necessary).           3.   Access requirements.     Is access required:   (a)   f or construction traffic?   *Yes/ No   ( b)   to property?   * Yes /No   (c)   for any other purpose?   * Yes /No     *Delete as appropriate        


Tunnels Outstation Maintenance 
Services Agreement

Schedule 18

Permitting, Traffic Management and Lane Rental

SCHEDULE 18
PERMITTING, TRAFFIC MANAGEMENT AND LANE RENTAL
Scope

This Schedule outlines the Service Provider’s responsibilities in relation to permitting, Traffic Management and Lane Rental and how the Service Provider should comply with the requirements of the NRSWA and the TMA. 

The Service Provider shall comply with TfL’s statutory code of practice and interact with the appropriate bodies as set out in the following documents: 
	Document
	Location

	TfL Lane Rental Scheme (“TLRS”);
	http://www.oneroadnetwork.org/ 



	London Permit Scheme (“LoPS”); and 
	http://www.londoncouncils.gov.uk/policylobbying/transport/publications/roadworksconsultation.htm

	The Mayor’s Code of Conduct for Road Works 
	Available for download from the TfL website

Current location: http://www.tfl.gov.uk/assets/downloads/2012-13-mayor-code-of-conduct-roadworks.pdf


A small number of London's Street Authorities have not transferred to the LoPS, and retain street works noticing practices under the NRSWA. As such, any reference to permitting should be construed as applying to street works notices under the NRSWA as well as the LoPS.

Service Provider’s Responsibilities

The Service Provider shall be responsible for permitting, Traffic Management and Lane Rental for schemes on the TLRN (including roads managed under the A13 DBFO contract which means the section of the A13 and other adjoining routes East of Butcher Row (in the London Borough of Tower Hamlets) that is managed under contract by Road Management Services PLC), SRN and Borough Road Network and any other Street Authority’s road when: 

the Services are not covered by TfL’s Permits; and
if there is a delay between the completion of activities by Other Service Providers or Third Parties as described in paragraph 2.2 and Service Provider’s provision of the Services such that new Permits and Traffic Management arrangements are required. 
The Service Provider shall ensure that prior to undertaking any Services-related activities it liaises with Other Service Providers and Third Parties that might be carrying out activities in the area in order to ascertain the possibility of sharing their Permits, consents or licences.
Permitting

Where required by paragraph 2.1 (Service Provider’s Responsibilities), the Service Provider shall be responsible for obtaining all Permits, consents, licences, and agreements necessary in order to carry out Services at the Sites and for obtaining the permission of the police and the local Highway Authority and LoHAC, for working on any highway. 
Pursuant to paragraph 3.1 above, the Service Provider shall comply with the LoPS when applying for Permits, consents, licences and agreements. The Service Provider shall ensure that necessary applications for Permits are submitted correctly, and consent, if required, has been granted, before undertaking any opening of the highway or other Site activities.
Where TfL has requested an emergency response, the Service Provider shall carry out the Services in accordance with section 52 of NRSWA.
The Service Provider shall make sure to minimise impact on traffic flow at all times. However, the Service Provider shall ensure that any planned activity that impacts on traffic flow on TLRN and SRN shall be Approved by TfL in advance with the appropriate forms being logged and approved as required by the TMA and/or NRSWA. 

Although TfL’s works are not subject to charges under section 74 of the NRSWA, any other costs of arrangements for Services and permitting shall be borne by the Service Provider. The Service Provider shall be responsible for re-submission, including, without limitation, where work times are directed by the Street Authority under section 56 of the NRSWA, any administrative failures including, but not limited to, submitting notifications within required timescales, or failure to include accurate information within such notifications.

The Service Provider shall be liable for the standard of reinstatement for the prescribed period following reinstatement in accordance with the NRSWA. Any Street Authority inspection fees levied against TfL for initial and subsequent failed inspection as a result of the Service Provider's provision of the Services shall be the responsibility of the Service Provider.
The Service Provider shall ensure that application for Permits are submitted electronically using an Electronic Transfer of Notices (“EToN”) system that is compliant with TfL’s EToN system and the Department for Transport Technical Specification for Electronic Transfer of Notices.
The Service Provider shall comply with and be bound by the TLRS and LoPS.
The electronic register of all current and planned road works undertaken in London (called “Londonworks”) can be accessed by the Service Provider to facilitate planning and coordination of the Services. Londonworks can be accessed at: www.londonworks.gov.uk/
If required by the Service Provider, TfL can arrange free-of-charge workshops to explain the permitting arrangements.
Traffic Management
Where the Service Provider is required to design and produce Traffic Management Drawing Plans under their responsibilities outlined in paragraph 2.1 (Service Provider’s Responsibilities) above, the Traffic Management Drawing Plans shall:

comply with all requirements of the TMA and NRSWA and associated regulations and statutory codes of practice, including without limitation the noticing requirements made under Part III of the NRSWA; and 

conform to Chapter 8 (Traffic Safety Measures and Signs for Road Works and Temporary Situations) of the Traffic Signs Manual published by Her Majesty’s Stationery Office or any amendment thereof; and

be subject to the approval of the Street Authority responsible for issuing the associated Permit(s); and

be subject to the approval of the Engineer unless specified in the Works Instruction.

Where portable signals are to be deployed by the Service Provider the Traffic Management Drawing Plans shall comply with the Engineer's requirements in line with TfL's current Guidance Note for the deployment of portable traffic signals.
The Service Provider shall be responsible for organising any Traffic Management Meetings as required to agree appropriate methods of working. The Service Provider shall invite the following representatives and any other relevant bodies to the initial Traffic Management Meeting as appropriate:
London Street Tunnels Operation Centre (LSTOC) representative;
LoHAC representative;
relevant London borough representative;
London Ambulance Service representative;
London Fire Brigade representative;
London Buses representative;
Police liaison representative; 
TfL Forward Planning representative; 
the Engineer; 
TfL’ clerk of works;
developer representative;
relevant Highway Authorities;
any other relevant body working for the Service Provider; and
any other relevant stakeholder as agreed with TfL.
The Service Provider shall submit full details of each of the Service Provider’s proposed Traffic Management Drawing Plans electronically to TfL.
Some Sites may require advanced planning for Traffic Management in order to undertake the Services. For the avoidance of doubt, the Service Provider shall ensure that sufficient time is given for the planning to enable the Services activities to be completed within the specified timeframe.

The Service Provider shall ensure that Traffic Management Drawing Plans required for Preventative Maintenance (where these do not already exist) are produced within the three (3) months immediately following the Operational Commencement Date and are submitted to TfL for Approval and stored for future use in respect to Traffic Management.

The Service Provider shall ensure the Traffic Management Drawing Plans required for Preventative Maintenance are Site specific for each location where Preventative Maintenance is required.
For any activity under paragraph 2.1 (Service Provider’s Responsibilities) which requires the implementation of a TTRO the Service Provider shall submit a request in writing to the relevant Traffic Authority and the Engineer, in sufficient time to ensure that the permitting dates are met, providing full details of the activity and the TTRO requested by the Service Provider. The TTRO form is Annex 1 to this Schedule.
The Service Provider shall submit the request for the TTRO in accordance with Section 14(1) or 14(2) as applicable of the Road Traffic Regulation Act 1984 and the relevant Traffic Authority’s procedures in relation to the type of TTRO requested.
The Service Provider shall ensure that after carrying out any Traffic Management activities, the Site is left in a clean and sound condition. This includes, without limitation, clearing away all rubbish and surplus material at the Service Provider’s cost.

The Service Provider shall be responsible for traffic safety and Traffic Management and associated work described in clause 117 of the Specification for Highway Works published by Her Majesty’s Stationary Office. The Service Provider shall (as appropriate and in accordance with such clause) erect, alter, cover, uncover and take down signs on the existing highways to be compatible with the state of the work underway.

The Service Provider's methods for implementing, changing and removing Traffic Management schemes shall be subject to the approval of the Engineer and LSTOC, which shall not be unreasonably withheld or delayed.
The Service Provider shall submit full details of its proposed methods for each such operation to the Engineer at least one (1) week before it is intended to be undertaken to allow time for the approval process including for any amendment required by the Engineer and/or LSTOC as necessary. 
The Service Provider shall provide and erect information boards at the locations and to the specification given below. The boards shall be to the approval of TfL and shall be placed in positions which do not impede or obstruct the passage or progress of vehicular or pedestrian traffic. 
General Information Boards: The following general information boards shall be displayed as directed by TfL or as required by scheme-specific drawings:

Traffic Signs Regulations diagram 7002;
Traffic Signs Regulations diagram 7003; and
Traffic Signs Regulations diagram 7007.

Other than as provided for above, the Service Provider shall not display its name. No photograph, advertisement or publicity of any kind relating to the works may be used without the express approval of TfL. If such photographs, advertisements or publicity are approved by TfL, they shall contain acknowledgement as shall be prescribed by TfL in writing.

The Service Provider shall erect, clean, maintain and remove such boards upon completion. All signs shall be removed upon substantial completion of the work or as agreed with the Engineer.
Lane Rental
The Service Provider shall at all times during the Term adhere to the TLRS, and seek to avoid impact for planned activities at Lane Rental charging times.
The Service Provider shall pay the Lane Rental charges:

as stated in the TLRS; and 

at the rate stated in the TLRS for the locations listed in the TLRS Schedule of Locations, but only during the times specified.

Subject to paragraph 5.4, TfL shall reimburse the Service Provider the applicable Lane Rental charges if such occupation is solely due to:

the Service Provider fulfilling its obligations to provide emergency response services to a Third Party incident; or

the Service Provider is instructed by TfL to carry out any Services during the chargeable period.

The Service Provider shall be liable for any Lane Rental charges in accordance with paragraph 2.1 (Service Provider’s Responsibilities), unless expressly agreed in advance with TfL. For the avoidance of doubt, where a Lane Rental charge has been incurred outside of terms stated in paragraph 5.3, or as a result of a failure to meet Commissioning, then the Lane Rental charges shall not be reimbursable and the Service Provider shall remain liable to pay such Lane Rental charges.
Annex 1
Temporary Traffic Regulations Order (TTRO) and Road Closure Application

[image: image5.png]2 Transport for London - Transport for London

| Hle Edt Vew Favoies Ioos Heb

MEIE

[ Adees €1 /g b crpdsignogosces i

BT

-

Foa]

]

Stop

2

Hame.

Back Refresh

J -

Q B

Search Favores

-

i

History

Piint

Transport for

Tube | DLR | Buses | River | Streets | Ta;

ondon

Private Hire | Coach Station | Trams | Rail | Dial-a-Ride |

> Logas

™

Buses

Underground

DLR

Streets

Trarms

Coach Station

River

Rail

Dial-A-Ride

Taxi Private Hire

Others

Reguest Logos
[~Search T —————
[—
Advanced search | Site map

MAYOR OF LOND

Logos

Transport
for London

©

-

©
e
©

fexdPoudel o)

Live travel news

LUL SERVICE
DISRUPTION
The Piccaily Line has

delays oceuing n bath
directions. This wil

o ©
©

©
S

The corporate identity of TL and the modes is based on the world
farmous roundel logo. Developed since the tum of the last century the
raundel has had many and varied maniestations. The recently
completed iderntity has given TIL and the modes a distinct roundel
utilising colours available in common applications, such as CMYK,
Pantone and BS.

The controlled and consistent application of the corporate idertity
relies upon control of it's distribution and use. T owns all design and
copyrights for all raundels and application for copies is only avalable

via the Tollowing eforrm

[ About Transport for London | Help using this site | Tetrns and conditions | Privacy policy |

|

FERECEM ]

e e e I S = e =

K

&1 hpplt sated
g Start | 2Folder Mesting -Docume...| B Microsaft Ward - CCS000..

=
|[&3Transport for Londo... & witer Sales - Transpart

(5 Evploring - LALS CLoCC... | 5 Microsoft Excel

| oo

Internet

AL 0758




[image: image1.emf]11.   Does the highway authority for the alternative roads consent to their use and to any  restrictions, and/or the suspension of existing restrictions?       *   (a)    N/A   *  (b)    Copy of consent enclosed       *  Delete as appropriat e     12.   Local Newsp aper/s in which Statutory Notices may be published.     Newspaper  Publication day              13.   Any other comments:     Enclosures:     *(a)   a plan showing the length of the road to be restricted and the alternative route/s.       *(b)   a copy of any section 1 4(2) Notice.     *(c)   a plan showing restrictions to be imposed on alternative roads.     *(d)   a copy of the consent given by other traffic authorities whose roads are to be    restricted     *(e)   copies of any existing restrictions  (Order/s)  on the GLA road and/or a lternative roads  to be suspended                   *Delete as appropriate         Signed:   ................................ ................................   Date:   ................................ ...............     Print Name………………………………………….       Designation /Title ………………… ………………..     Organization/Company...................     Fax:     ………………………………………   Telephone :…………………………..       PLEASE STAT E NAME, TELEPHONE, FAX , COMPANY  O F  THE  PERSON  R ESPONSIBLE FOR POSTING THE NOTICE ON SITE  (at number 13 above under  ‘Any other comments’) if this individual is different fr o m  the person signing this form .       PLEASE  D O NOT  SUBMIT THIS APPLICATION UNLESS  ALL   Q UESTIONS HAVE BEEN ANSWERED   INCLUDING  NAME , DATE ETC.  


[image: image2.emf]  ROAD CLOSURE APPLICATION     New Roads & S treet Works Act 1991     StreetCare  –  Public Realm       NRSWA@havering.gov.uk       Applicants   name:      Applicants a ddress                   Organisation   (if a pplicable):                     Telephone numbers  Day:                          Evening:                       I hereby request the London Borough of Havering to give permission for a road closure  notice, details of which are giv en below, and  understand  that this application must be  received by the council at least eight weeks before the proposed date of the road closure.       A 1/200 scale plan showing the area must be included wi th this application.  It should  clearly  show  the boundary of the proposed work area, and the locations of all temporary barriers and  road signs.  Also, a suitable scale plan should be included showing the proposed diversion  route for vehicles and pedestrians.  Adherence to this scale plan will form  part of the licence  agreement and any required changes by the applicant may result in the need for a new  application being made along with the associated fees.     Date of proposed road closure :                     Duration of proposed road closure :                       Estimated start date :                        Estimated End Date:                        24 hour Emergency contact for  duration of works :    Name:                       Telephone No:                       Location:                        


[image: image3.emf]  Reason for  road closure     and proposed  diversion route                           Payment of fees   –  please include payment of fees with application to the sum of  £10 35 ,  made payable to London Borough of Havering.  This will be refunded if your  application is  refused.     Please note: Failure to provide payment will prevent this application from being  processed, please attach payment or an invoice address.       Signed by applicant: _____________________________________________       (Print Name)   _______ _____________________________________________       Date:  __________________________________________________________         Please ensure that all sections have been completed  as f ailure to provide this  information will result in a delay in processing your applic ation .           London Borough of Havering   6 th  Floor   Mercury House   Mercury Gardens   Romford   Essex   RM1 3SL     Tel:    01708 43 4645   Fax: 01708 432799   NRSWA@havering.gov.uk      


11/11100545_1
3
TfL_scp_000996

Page 10 of 10

