COMMUNAL ENTRANCE DOORSETS AND ASSOCIATED PRODUCTS – CED1

SCOPE OF THE FRAMEWORK

This LHC framework is for the specialist design, manufacture, supply and installation of enhanced security communal entrance doorsets operated by a compatible door entry electronic access control system.

It has been developed in association with Secured by Design. Access to the organization’s design criteria and advice reflecting crime and environmental statistics forms a significant part of the design brief. Therefore, SBD accreditation is considered a requirement essential to this framework.

Products to be offered under this arrangement would mainly comprise communal entrance doorsets providing secure access to and from:

- The main entrance to buildings containing multiple dwellings – e.g. flats or apartments

- A secondary entrance area – e.g. entrance lobby

- An internal access floor space – e.g. protected corridor

The performance and enhanced security accreditation specification requirements shall be as follows:

- Third party certification to PAS 24

- Certification to STS 202 Burglary Rating 2 (minimum) or LPS 1175 Security Rating 2 (minimum)

Associated products may include:

- Doorsets providing enhanced security and access to facilities such as meter cupboards, storage areas, bin stores, cycle stores etc.

- Composite fanlights and side screens.

A comprehensive list of product requirements within this framework is set out under Clause E1.2 of this Invitation to Tender.

Note:                                                                                                                                                                         Entrance doorsets providing access directly to and from dwellings (e.g. front doors) and FD30 / FD60 fire doors not operated by an access control system, do not fall within the scope of this framework. These are specifically provided for within the LHC Entrance Doorsets Framework Arrangement – currently C7.

Included within the services will be the ability to offer a cyclical maintenance package and emergency call-out service. Products and installation shall be supported by a minimum 10-year guarantee.

A comprehensive list of services expected to be offered under this framework is set out under Clause E1.3 of this Invitation to Tender.

The applications for which this framework will be used shall be projects funded by public finance undertaken by local authorities, tenant management organizations, housing associations and other bodies throughout the United Kingdom (excluding NI).

A comprehensive list of applications this framework will be used for is set out under Clause E1.5 of this Invitation to Tender.

This framework will be operated under four (4) main workstreams, designated by the doorleaf construction, namely –

- Workstream 1 = Aluminium

- Workstream 2 = Mild Steel

- Workstream 3 = Stainless steel

- Workstream 4 = Timber (timber / timber core)

On completion of the tender evaluation, a further workstream – Workstream 5 – will then be established comprising those companies selected to service Workstreams 1, 2, 3 and 4 from this tender, thereby offering a choice of materials to the specifier under one workstream

The framework will be operated over three (3) Regional Lots (England, Wales & Scotland). An illustration of the Areas contained within each Lot is set out in Clause D1.3 of this Invitation to Tender.

You are invited to tender to service one, or more, of the Lots – offering products from one, or more workstreams – to the specification and service requirements set out in Section E1.

ENDS