LOCKING PARISH COUNCIL

OLD BANWELL ROAD CAR PARK

TENDER SPECIFICATION

# Please complete this form for the contact details

| Name |  |
| --- | --- |
| Address |  |
| Telephone number |  |
| Email address |  |
| Contact name |  |
| Position in company of named person |  |
| If the business is a company |  |
| Is it a subsidiary of another Company |  |
| If yes please give details |  |
| Date of business formation |  |
| Number of grounds employees |  |
| Branch project will be serviced from |  |

## Background

The car park at Old Banwell Road Playing Field is primarily used as a “Park & Stride” facility for Locking Primary School , parking for sports teams, although the car park is open to the public vehicle access is limited. It is currently a gravel surface which has been susceptible to pot holing, despite frequent redressing.

LPC has planning permission to do works to the above car park.

LPC has had all drainage works completed to enable surface water to be discharged to a swale system that has already been constructed.

The swale system is fed by 3 x 100mm pipes that are laid (below ground) and are ready for surface water for the car park to be connected to and thus for the surface water to discharge to the swale.

## Site Description

Location: Old Banwell Road Playing Field BS24 8BT

The area is accessible via a pedestrian gate or through a locked vehicle gate by prior arrangement.

## Sketch plans

These have been provided as visual concepts and should not be used to scale from. Layout surveys by contractors should be used to confirm the viability of the project.

## Site Visits

Please contact the Parish Clerk at clerk@lockingparishcouncil.co.uk to arrange a site visit.

## TENDER REQUIREMENTS

Fixed Fee Quote

Please provide a fixed fee quote to complete all works in accordance with the attached plans.

The approved scheme is detailed on the documents attached and includes the following with their file names:

- Car park make up with bull nosed edging

- CS1 concrete channel detail

- Knee rail fencing detail

- Locking car park plan existing plan with existing levels Dwg1 A1

- Locking car park plan with existing and proposed levels A1

- OBR car park drainage scheme plan Locking

- OBR car park plan Dwg2 A1  proposed

- OBR car park red line location plan

LPC request a price to implement all of the work as detailed on these plans and to include:

Construction of car park through provision of stone and tarmac as referenced, dished channels, bull nosed kerbs, removal of bunds where annotated, provision of knee rail fencing as indicated and connection of the surface water run off to the 3 x drainage pipes in situ.  The price should allow for any excess soil or spoil to be removed from site.

LPC does not expect any existing stone to be removed, and the levels shown on the existing plan are as they were measured in 2022.  Due to issues with flooding, some bunds (that are not referenced) have been removed and replaced with stone to a similar level of those quoted – but there may be some small discrepancies.

For bunds to be removed, it is anticipated that the soil will be utilised on the playing field and this work should also be included within your tender.

Contingency plan

LPC is conscious that in the past few years, construction costs (including materials) has become volatile and prices have, in certain instances, grown substantially.

For this reason, LPC needs to consider that the works may need to be reduced and to that effect, a lesser specification may be proposed based on the attached plan that is entitled:

Locking car park options plan

The current condition of the car park has resulted in high costs to provide top up stone and regrettably, those temporary works are not working and have to be regularly topped up.

In that regard, LPC is considering whether to provide a lesser area of tarmac with a lesser spec and to partially and temporarily ‘stone’ the rest of the car park until funds allow for the completion of the project to LPC’s requirement as detailed on the main plans.

IMPORTANT - ALL LAID STONE MUST BE ‘ROLLERED’ and not left loose and LPC assumes that this would be done at the same time as the tarmac is laid.

The lesser requirement show 2 x options – 1000 sqm of tarmac with the remainder in stone or a 520 sqm tarmac ‘track’ with the remainder in stone.

For those works, LPC would not provide the bull nosed edgings or the CSI channels as referenced, more so, they would seek the requirement that the stone and the tarmac would be contoured so that surface water follows the route of drainage as shown on the plan entitled ‘OBR car park drainage scheme plan Locking’ with connections to the 3 x 100mm pipes already in situ.

The stone proposed would be sub base type 1 as detailed on the plan entitled ‘Car Park make up with timber edging’.  The stone should be kept inside of the current bunds and only where stone or tarmac cannot be retained by the existing bunds, timber edging should be provided to retain the stone/tarmac.

The tarmac proposed would only be the 100mm binder course as referenced on the same plan of ‘Car Park make up with timber edging’ but should not include the 30mm surface course as referenced.

LPC requests that additional prices are provided for the 520 sqm ‘track’ with associated stone and the 1000 sqm tarmac with associated stone.

It would also be useful if the tender could advise on the cost of tarmac for the contingency plan per 200 sqm as well as the cost of stone per 200 sqm to bring the levels up to those detailed on the proposed plan as this will enable LPC to consider possible alternative amounts of tarmac and stone.  LPC appreciate that there may be economies of scale for a larger area but may need to consider what can be done based on budget constraints.

## Environment

Locking Parish Council is committed to safeguarding our environment, tender applications should give consideration to this and provide appropriate information and environmental policy documents for your company, including details of sustainably sourced materials and waste disposal.

## Equality

Consideration should be given to the equality impacts of the project design. The successful submission will have inclusivity designed into the scheme from the outset. All potential barriers to use should be identified and considered in the submission.

## Materials

Accessibility, durability and longevity are key criteria for the equipment. This may mean a mix of material types but all materials will need to be the best of their respective kinds and shall conform to appropriate standards.

## Existing Services (manholes, rodding eyes etc.)

The Contractor should allow for adjusting any utilities and service access points, such as water meter and drainage to the required new surface levels. The Contractor is also responsible for

locating any services prior to any excavation work and the repair or reinstatement if damaged in anyway during the undertaking of the works.

## CONDITIONS OF CONTRACT

Description of Works

The works comprise the provision, delivery and installation as per detailed above for the location at Old Banwell Road Playing Field.

Detailed unmarked plans show the entire scheme that is proposed but for the purposes of this quote, each plan has been marked as separate documents.

Any excess soil left as a result of those works is to be retained on site and utilised on the playing field.

Start and Completion Dates

At present the hope is to start and complete the works during school holidays. As this is very much weather dependent, we would appreciate a start date and completion date early September 2023.

## Submission of Price

Tenderers - (hereinafter referred to as "The Contractor") - are required to submit a price for the works detailed in the accompanying documents.

All materials specified should be included within the quote as well as all works required to complete and undertake the said works.

## Payment

The successful contractor will invoice the Council for full payment on successful completion of the works.

## Retainer

Locking Parish Council will either request a guarantee or a 10% retainer for the proposed works.  For a guarantee, LPC would request details of how long you have traded and examples of similar jobs in the past 3 years.  For a retainer, this will be for 12 months.  Please make it clear in your tender if your tender proposes a guarantee (with details requested) or a 10% retainer.

## Variations

Should the Council require more or less work to be carried out, an increase or decrease in price will be agreed on the basis of the rates included in the original tender, as reviewed in 3 above.

LPC accepts that due to the volatility of costs of materials, there may be increases in the prices of such so if any materials required for the project are deemed to fit this criteria, please specify this within the quote and why there may be variances in those elements.

## Council's Indemnity

The Contractor shall be held responsible for any damage caused to sites during the performance of the works. Any such damage will be made good at the Contractor's expense. If the Contractor, on receiving an instruction to repair such damage does not affect repairs within one calendar month, the Council reserves the right to have the work carried out by others and deduct the cost from the Contractor's invoices. Any damage to property caused in the performance of this contract must be informed by the Contractor to the Council within 2 working days.

The Contractor will indemnify the Council from any claims for damage to property or persons

arising from the performance of the contract and will be required to produce evidence of Public Liability Insurance to a minimum value of £10 million per claim.

## Format of submission

Submissions should be clearly marked TENDER – CONFIDENTIAL and should be sent by post or hand delivered to:

Mrs D Tremlett

Parish Clerk/RFO

Locking Parish Council Locking Parish Hall Grenville Avenue Locking

WSM BS24 8AR

To arrive no later than midday on Friday 4th August 2023

information required If you wish to quote for this project:

- Your price fully itemised (with VAT itemised separately, if applicable) for undertaking all the

works on the enclosed ‘Specification of works’ this should be split down as follows:

- Cost of materials to undertake the job

- Cost of plant machinery required during the works

- Number of man days required to undertake the works

- Any other works you feel are required or any amendments to the above items, please

- explain why these are proposed

- itemise the additional costs for these separately

- Please advise if you visited the site in person

- Availability based on the proposed completion date, together with the proposed start and completion date

- Examples of previous relevant experience and two references from previous clients]

## Acceptance of Tenders

The Council does not bind itself to accept the lowest, or any, tender.

## Working Practices

The Contractor will ensure that all staff wear clearly identifiable uniforms. The Contractor’s employees shall, at all times, act in a reasonable manner, particularly in their dealing with the general public. All work is to be carried out between Monday and Friday inclusive (Saturday/Sunday work is not permitted unless in an extreme emergency). No sub- contracting is permissible without the prior written consent of the Clerk to the Council.

## Termination of Contract

The Council reserves the right to terminate the contract where the Contractor fails to meet any of the Conditions of Contract as identified in Part B of this document.

## Contract Manager

The Contractor shall ensure that during the installation period a member of the Contractor's management staff is empowered to act on behalf of the Contractor and be available to be contacted by the Clerk of the Council at all "reasonable" times, i.e. during office hours (generally 9.00 a.m. - 3.00 p.m. Monday to Friday). The Contractor should be contactable, or an answering facility made available outside office hours (generally 5.00 p.m. - 8.00 a.m.

Mondays to Friday, 24 hours Saturday and Sunday). The Contractor will provide an emergency contacts list. Both the Contractor and the Council will make known to each other the persons responsible for daily matters affecting the contract.

## Health and Safety

The Contractor shall comply with all relevant sections of the Health and Safety at Work Act:

`Electricity at Work Act: Control of Substances Hazardous to Health Regulations. The Contractor will adopt safe methods of work in order to protect the health and safety of its own employees, the employers of the service users and all other persons including members of the public. The Contractor will review his health and safety policies and safe working procedures as often as may be necessary in the light of changing legislation or work practices. The Contractor will ensure its staff are aware of basic fire safety regulations and are trained accordingly. N.B. - No burning allowed on sites.

## Sufficiency of Information

The Contractor shall be deemed to have satisfied itself before submitting its tender as to the accuracy and sufficiency of the rates and prices stated by the Contractor in its tender which shall (except in so far as is otherwise provided in the Contract) cover all the Contractor's obligations under the Contract and shall be deemed to have obtained for itself all necessary information as low risks contingencies and any other circumstances which might influence or affect the Tender or provision of the Service.

## Site Works

Confirmation of work hours, storage of materials, access, location of skips, final Programme of Works and Risk Assessment Methods Statements (RAMS) will need to be finalised and agreed with the Parish Council before works commence.

During the works, the supplier should provide a secure work compound for the build, this compound should ensure public cannot gain access to the work site, we suggest the supplier erect suitable fencing and safety notices around the construction site. The supplier must take all reasonable action to ensure that newly installed surfaces and lighting are not used until practical completion is confirmed.

Any machinery left on site overnight must, where possible, be immobilised. The supplier will be held responsible for a) any damage caused to machinery/materials left on site by the supplier; b) any damage caused by machinery/materials left on site by the supplier; c) any vandalism caused to machinery/materials left on site by the supplier. The supplier will be held responsible for and must make good any damage caused to existing buildings, roads, paths, grassed areas, car parks, fences, drains, sewers, service mains, landscaping etc. and for completion of the project, any area that has been damaged during the installation must be made good and returned to new or as a minimum returned to the original condition at the supplier’s expense.

On completion of the works all equipment, including portaloos will be removed from site in a timely manner and the Council notified of the completion/removal date in advance.

## Waste

All waste shall be stored safely while on site and removed from site by the contractor.

## Inspection

All works and installations shall be inspected upon completion. This shall be arranged by and at the expense of the contractor.