Area 2 – Footpath 2419 (415M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Raise the height of the path and grade for to establish a crossfall to the west side of path

- Excavate the path to required depths, levelling all arisings on site.

- Try to achieve a minimum width of 1.2m, maximum width 1.5m.

- Grade and strip 415 M

- Heavy infill of 200MM of course crush concreate 300 M starting from the South point of path to be grated to create cross fall to the West, with all surfacing materials to be compacted in layers of no more than 100MM using a minimum of a Bowmag 90 roller or equivalent.

- 330M of 100MM deep MOT to be laid from the South point of the path.

- At the south point, where the path turns east, the pat is to be raised to direct and flow of water during extreme weather conditions to the West on to the open path and track ways

- Install two large mobility plain hoop style gates to limit cyclists, similar to the picture below. One is to be installed where the existing boundary style is currently located separating the two properties, the other where footpath 2419 joins footpath 2421 towards the Mill. Locations shown on plans in Appendix 3