Footpath 2423 (252M in total path length – 179M of works)

(Includes small part of footpath 2421 which doglegs off the Sothern end of 2423)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Clear area adjacent to low side of path and ensure slope of grip access to adjacent drainage trench or water way.

- Raise the height of the path and grade to establish a crossfall to the west side of path at a minimum of 1:40 fall.

- Heavy infill of 200MM of course crush concreate any deep areas with Type 2 crushed concrete laid on a geotextile mat and compact.

- MOT type 1 surface laid to 100mm depth to a crossfall to the west at a minimum of 1:40 fall.

- At southern end of path repair to the West continue on footpath 2421 to fill holes in surface to the dogleg point,

- Install a large mobility plain hoop style kissing gate to limit cyclists, similar to the picture below. This is to be installed to the northern entry point to the path section from Church Street. Locations shown on plans in Appendix 3