Footpath 2424 part 1 (approximate length 295M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Scrape off detritus from the existing surface and level on site grading into existing

areas taking care not to damage tree roots.

- Clear area adjacent to low side of path and ensure slope of grip access to adjacent drainage trench or water way.

- Work together with the landowners staff to clear access to the drainage ditch and clean out the ditch.

- Supply and lay geotextile membrane in any deep areas along the path.

- Supply, level and compact Type 2 crushed concrete in any deep areas to form a firm sub-base (these are isolated areas along the path, not the full length).

- Supply, level and compact 150mm of Type 1 limestone along the entire length of this path achieving a maximum width of 2m where possible.

- Install some timber edging for approximately 20m the eastern end of the path.to stop any of the new aggregate eroding down the bank in line with Appendix 2.

- Supply, level and compact 25mm of limestone dust to provide a finished surface.

- At the western end of the path take away the existing footbridge and clear drainage pipes.

NOTE: The farm track crosses the path between part1 and part 2. The landowners staff will clear the drainage ditch and set a fall from the track to drain the farm track and footpath via grips into the drainage ditch.