Footpath 2424 part 2 (175M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Very heavy clearance, remove all small trees on North fence line at 120 M from the East (approx. 14) Cut back all overhead vegetation.

- Remove the obsolete kissing gate and associated structures at 155 M from the East.

- Remove unused concreate post by disused kissing gate,

- Install a large mobility plain hoop style kissing gate to limit cyclists, similar to the picture below. This is to be installed to the northern entry point to the path section from Church Street. Locations shown on plans in Appendix 3

- Coppice Hazel at the corner of the fence at the West end of the path at the top of the slope.

- Cut back Western end as hard as practical without opening up driveway to the South.

- Remove existing rotten 3 plank crossing at Western end of path.

- Clear out ditch underneath the old plank crossing.

- Install a new 4.2 M double handrail 4 plank crossing with 1200 MM length legs at one end and 900 MM legs at the other end, so it can be levelled to be used by wheelchairs and pushchairs.

- Heavy infill along the whole path length by 150 MM of course crushed concrete no fines, 155 M from the Eastern end to the old gate.

- Very heavy infill at Western end 20 M from the new bridge 300 MM deep of course crushed concrete to lessen the slope.

- 175 M of top surfacing 100 MM of MOT type 1 centre high to shed water to the North side of the path towards the field track. NOTE: this track is to be flattened and graded with a fall away from the path.