WEST CHILTINGTON PARISH COUNCIL

REPAIR OF FOOTPATHS

ref: 2419, 2423, 2424 parts 1 & 2

INVITATION TO TENDER

Scope of Works

Mark Foss

3rd March 2025

| SCOPE OF WORK Contents Page |  |  |
| --- | --- | --- |
| 1. | Preliminaries | 2 |
| 2. | Obligations imposed by the Employer | 3 |
| 3. | General | 7 |
| 4. | Environmental Considerations | 12 |
| 5. | Site Set Up | 12 |
| 6 | Information & Tender Requirements | 12 |

Appendices								 13

Appendix 1 – Conditions of Contract

Appendix 2 – Resurface Specification

Appendix 3 – Footpath Maps

| 1.0 | Preliminaries |
| --- | --- |
| 1.1 | Employer: West Chiltington Parish Council, The Parish Office, Church Street, West Chiltington, West Sussex, RH20 2JW |
| 1.2 | Project Contacts Contract Administrator: Parish Clerk, 01798 817434, clerk@wcpc.org.uk Project Manager: Councillor Mark Foss BSc CEng, FInstMC MIET MISPE, 07768 578144 mark.foss@wcpc.org.uk |
| 1.3 | Tenders: Tenders are to be delivered to the above address |
| 1.4 | Inspection of Site: The Contractor is assumed to have visited the site to assess the nature of the work, access, topography etc and carried out relevant checks. Should site visits be required with the Project Manager then this should take place well in advance of the tender close date. |
| 1.5 | Contract: Tenders to be received: Monday 28th April 2025 Contractor Insurance minimum: £5 million Experience Contractor to give information in regard to similar works they have completed. |
| 1.6 | Contractor’s Proposals: The contractor is required to submit the following information with its tender: - Suitable drawings, specifications, details, etc., to clearly define the extent of the proposed works and quality of materials and workmanship to be provided. - Preliminary programme as defined in Section 1. This programme should take account of appropriate design periods in addition to construction. - Completed Form of Tender. - The Contractor is to note that it will be required to incorporate these Employer’s Requirements into its proposal document. In addition, the Contractor will be required to identify within its documents its proposals in respect of Design Documents that it intends to submit for the Employers approval in accordance with Section 2 of the Conditions of Contract. - The breakdown of cost on the form of tender must be used to describe the listed sections. |

| 2.0 | Obligations and Restrictions Imposed by the Employer: |
| --- | --- |
| 2.1 | General: The Employer is not bound to accept any tender in its entirety and shall accept whole or part of the submitted tender as required. Contractors tendering do so at their own cost and their tender shall remain open for acceptance for a period of a minimum of 10 weeks after the due date for submission. 2.1.2. All works rendered necessary in consequence of the development, hereby agreed upon, shall be deemed to be in, and form part of this contract whether or not the same are mentioned in the drawings and specification, provided always that the Contractor shall not deviate from the works shown, or reasonably implied in the said drawings and specification, without obtaining the previous agreement of the Employer, which agreement should not be unreasonably withheld. 2.1.3. The Contractor shall be deemed to have satisfied himself before tendering as to the correctness and sufficiency of his tender for the works specified, or reasonably inferred from the contract documents, and that his tender covers all his obligations under the contract and all matters and things necessary for the proper completion and maintenance of the works. 2.1.4. The works will be designed and executed in accordance with all local authority by-laws, Building Regulations, statutory obligations, fire officer’s requirements and recommendations and the like. NOTE: The Contractor shall note the general access difficulties present on the different sites. No allowance for want of such knowledge will be made. The Contractor is to consider the works as scheduled and make allowance for executing the completed project in a safe and expeditious manner. |
| 2.2 | Labour / Materials / Plant: The Contractor will provide all labour, materials, fuel, carriage, plant, tools, and machinery of all descriptions to execute the works. 2.2.1. Where and to the extent the materials are not fully specified, they are to be in order of priority suitable for the purposes of the works stated in or reasonably to be inferred from the contract documents, in accordance with good practice, and to comply strictly with British Standards or the ISO equivalent, or in general with British Standards or the ISO equivalent. 2.2.2. Any materials in the following three categories should be excluded: a) Materials that could pose a danger to health. b) Materials used in such a way as to have a likelihood of failure or cause defect. c) Materials, the use of which would endanger the longevity of the project. 2.2.3. Branded materials are to be handled stored and used and processes are to be carried out strictly in accordance with manufacturer’s instructions and recommendations. Such materials are to be obtained direct from the manufacturers or through their accredited distributors. 2.2.4. All materials to be incorporated in the works shall at all times be carefully handled, stored in accordance with the manufacturer/supplier’s instructions where appropriate, and protected from the weather. 2.2.5 Ideally contractors are asked to provide costs for the removal and disposal of all foliage and ground clearance. If this is not possible for ground clearance, a schedule of rates must be provided. 2.2.6 Parish Council will work with the landowners to provide an area that is at a reasonable distance from the working area that is safe to tip and store materials, reasonable dump spoil and to be removed at the council's expense. 2.2.7 The Parish council are in discussions with the local land owners close to each pathway to arrange for a safe storage area for Machinery and materials. In addition to this there will need to be an area for all the spoil to be tipped/left or removal of this spoil will be at the council’s expense. 2.2.8 Materials and deliveries must be made to agreed areas and must be in quantities that do not obstruct adjacent areas or properties. Materials and waste must be collected in these agreed areas and must be in quantities that do not obstruct adjacent areas or properties. Agreed main material storage and waste areas are: - Area of grass on Churchfield Farm north of the footpath 2423 - Area on Churchfield Farm for path 2419 and 2424 - Area on Stables farm (access from Mill road) for store area to upgrade 2424 Shown on sketch below: 2.2.10 All work shall be completed in line with BS 3998:2010 “Recommendations for Tree Work” where appropriate. All recyclable waste material, such as foliage and wood shall be removed from site for recycling unless otherwise stated at the cost of the contractor 2.2.11. Workmanship is to be of a high standard throughout and where applicable is to comply with the relevant British Standards specifications or the ISO equivalent, and British Standard 2.2.12 Where and to the extent that workmanship is not fully specified, it is to be, in order of priority, suitable for the purposes of the works stated, or reasonably to be inferred from the contract documents, in accordance with British Standard Codes of Practice. 2.2.13 Equipment and plant to comply with and be maintained to all relevant safety Regulations and guidelines at all times. |
| 2.3 | Trespass: The Contractor shall prevent trespass onto other areas by his / her own employees or those employed by a subcontractor. |  |
| 2.4 | Protection of Site: The Contractor must ensure that all reasonable efforts are made to close off the site during the works. The Contractor will provide, install, and maintain adequate fencing around individual areas. At no time will the public have access to construction areas, storage areas, site vehicles, delivery areas etc. Warning signs will be supplied and maintained by the Contractor. Details of protection requirements given in Section 5. The Contractor will be held responsible for: 1. Any damage caused to machinery / materials left on site by the Contractor; 2. Any damage caused by machinery / materials left on site by the Contractor, and; 3. Any vandalism caused to machinery / materials left on site by the Contractor. |

2.5	Safety, Health, Welfare and CDM Regulations:

The Contractor shall allow for providing and maintaining all welfare and safety measures to a standard not inferior to that laid out in statute regulations, rules, and orders and subsequent amendments thereto for all employees working on the site, to include employees of any approved subcontractor.

The Contractor’s attention is particularly drawn to his / her obligations under the Health and Safety at work act 1974 and Construction Design and Management Regulations 2015. The Contractor will provide, with his / her tender submission, a copy of his / her Health and Safety policy covering all aspects of the work to be carried out in this project.

| 2.6 | Control of Substances Hazardous to Health Regulations 2002: Assessments carried out by the Contractor of all work which is liable to expose his / her employees and any other persons in the vicinity of the site to hazardous solids, liquids, dust, fumes, vapours, gases, etc shall be made available to the Project Manager ten working days prior to the commencement of works. |
| --- | --- |
| 2.7 | Risk Assessments: The Contractor will ensure that Risk Assessments are carried out for all activities to be carried out during the works, and for the use, storage and disposal of any hazardous materials highlighted in the COSHH Assessments. |
| 2.8 | Impact and Damage: The Contractor will be held responsible for, and must make good, any damage caused to the existing buildings, roads, paths, grassed areas, car parks, fences, drains, sewers, service mains, landscaping etc. The Contractor must take all the necessary steps to prevent roads becoming fouled with materials from vehicles entering and leaving the site and allow for cleaning if the roads / paths do become dirty on immediate exit of said vehicles. |
| 2.9 | Inclement Weather: The Contractor will allow for protecting the works against inclement weather and shall include taking all reasonable precautions to ensure the regular progress of works during adverse weather conditions. |

3. GENERAL

3.1 	Overview

To repair the denoted sections of three footpaths, one in two parts. within the parish of West Chiltington, creating an all-weather walkable surface suitable for wheelchair and pram use.

3.2 	Preliminaries

Provide risk assessment and method statement (RAMS) for the works

3.3 	Work Schedule

1] To erect safety fencing and warning signage where required.

2] To resurface footpaths (2419, 2423, 2424 parts 1 and 2) providing an all-weather surface and appropriate drainage. The works may be carried out continuously or as separate projects.

(West Sussex County Council Specification is shown in Appendix 2. This is the specification for a path in a clear area where it is possible to achieve a 1:40 cross fall. Areas where Timber edging will be required are highlighted in individual sections)

3) To remove all material, equipment, fencing and signage from site

4) To ensure materials are delivered and removed from site and appropriately disposed of.

3.4	Nesting Birds

It will be necessary to schedule the cutting back of hedges and trees along the paths to avoid nesting periods

3.4 	Area 1 – Footpath 2423 (252M in total path length – 179M of works)

(Includes small part of footpath 2421 which doglegs off the Sothern end of 2423)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Clear area adjacent to low side of path and ensure slope of grip access to adjacent drainage trench or water way.

- Raise the height of the path and grade to establish a crossfall to the west side of path at a minimum of 1:40 fall.

- Heavy infill of 200MM of course crush concreate any deep areas with Type 2 crushed concrete laid on a geotextile mat and compact.

- MOT type 1 surface laid to 100mm depth to a crossfall to the west at a minimum of 1:40 fall.

- At southern end of path repair to the West continue on footpath 2421 to fill holes in surface to the dogleg point,

- Install a large mobility plain hoop style kissing gate to limit cyclists, similar to the picture below. This is to be installed to the northern entry point to the path section from Church Street. Locations shown on plans in Appendix 3

3.5 	Area 2 – Footpath 2419 (415M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Raise the height of the path and grade for to establish a crossfall to the west side of path

- Excavate the path to required depths, levelling all arisings on site.

- Try to achieve a minimum width of 1.2m, maximum width 1.5m.

- Grade and strip 415 M

- Heavy infill of 200MM of course crush concreate 300 M starting from the South point of path to be grated to create cross fall to the West, with all surfacing materials to be compacted in layers of no more than 100MM using a minimum of a Bowmag 90 roller or equivalent.

- 330M of 100MM deep MOT to be laid from the South point of the path.

- At the south point, where the path turns east, the pat is to be raised to direct and flow of water during extreme weather conditions to the West on to the open path and track ways

- Install two large mobility plain hoop style gates to limit cyclists, similar to the picture below. One is to be installed where the existing boundary style is currently located separating the two properties, the other where footpath 2419 joins footpath 2421 towards the Mill. Locations shown on plans in Appendix 3

3.6 	Area 3 – Footpath 2424 part 1 (approximate length 295M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Scrape off detritus from the existing surface and level on site grading into existing

areas taking care not to damage tree roots.

- Clear area adjacent to low side of path and ensure slope of grip access to adjacent drainage trench or water way.

- Work together with the landowners staff to clear access to the drainage ditch and clean out the ditch.

- Supply and lay geotextile membrane in any deep areas along the path.

- Supply, level and compact Type 2 crushed concrete in any deep areas to form a firm sub-base (these are isolated areas along the path, not the full length).

- Supply, level and compact 150mm of Type 1 limestone along the entire length of this path achieving a maximum width of 2m where possible.

- Install some timber edging for approximately 20m the eastern end of the path.to stop any of the new aggregate eroding down the bank in line with Appendix 2.

- Supply, level and compact 25mm of limestone dust to provide a finished surface.

- At the western end of the path take away the existing footbridge and clear drainage pipes.

NOTE: The farm track crosses the path between part1 and part 2. The landowners staff will clear the drainage ditch and set a fall from the track to drain the farm track and footpath via grips into the drainage ditch.

3.7 	Area 4 – Footpath 2424 part 2 (175M)

- Cut back vegetation sympathetically to allow plant access and to achieve working width and access to drainage.

- Very heavy clearance, remove all small trees on North fence line at 120 M from the East (approx. 14) Cut back all overhead vegetation.

- Remove the obsolete kissing gate and associated structures at 155 M from the East.

- Remove unused concreate post by disused kissing gate,

- Install a large mobility plain hoop style kissing gate to limit cyclists, similar to the picture below. This is to be installed to the northern entry point to the path section from Church Street. Locations shown on plans in Appendix 3

- Coppice Hazel at the corner of the fence at the West end of the path at the top of the slope.

- Cut back Western end as hard as practical without opening up driveway to the South.

- Remove existing rotten 3 plank crossing at Western end of path.

- Clear out ditch underneath the old plank crossing.

- Install a new 4.2 M double handrail 4 plank crossing with 1200 MM length legs at one end and 900 MM legs at the other end, so it can be levelled to be used by wheelchairs and pushchairs.

- Heavy infill along the whole path length by 150 MM of course crushed concrete no fines, 155 M from the Eastern end to the old gate.

- Very heavy infill at Western end 20 M from the new bridge 300 MM deep of course crushed concrete to lessen the slope.

- 175 M of top surfacing 100 MM of MOT type 1 centre high to shed water to the North side of the path towards the field track. NOTE: this track is to be flattened and graded with a fall away from the path.

4. ENVIRONMENTAL CONSIDERATIONS

4.1. Resurfacing material must comply with West Sussex County Council’s specification contained herein.

4.2. All necessary steps and precautions must be taken to limit the environmental impact (noise, airborne particles, damage to vegetation etc) of the works being carried out.

5. SITE SETUP

5.1. Save for the fenced areas, the footpath network will be used by the general public throughout the contract. The Contractor is to agree strategies for separating works from the general public and maintaining access for users as far as is practically possible.

5.2. The Contractor’s site set up, including temporary welfare facilities, must be contained within at a location to be agreed in advance with the client. All allowances should be made and costs provided with tender return.

5.3. Site security: The Contractor is to ensure the site is secure.

5.4. The Contractor shall make all arrangements for safe site storage for the duration of the works.

5.5. The Contractor shall be responsible for providing detailed information to all sub-contractors.

6.  Information and Tender Requirements

Contractor’s Proposals

6.1. The contractor is required to submit the following information with its tender:

- Suitable detail to clearly define the extent of the proposed works and RAMS for all aspects of the work.

- Details of qualifications and/or registration necessary.

- Completed Form of Tender.

- The Contractor is to note that it will be required to incorporate these Employer’s Requirements into its proposal document.

- Please return your quotations for options you wish to put forward in the form of the document ‘Footpath Repair Project Form of Tender date Feb 2025’ backed up by any supporting details of your operational methods, health and safety considerations, design, selection of materials and any other information that may support your proposal.

- The Contractor shall be required to provide a suitable breakdown of the full

cost of the works. This must include separate priced options, where indicated

- The contract must provide a program of works giving the details of each part of the works and the approximate time for the stages of the work on each of the footpaths

Appendices

Appendix 1 - Conditions of Contract

Appendix 2 – Resurface Specification. Note: - this is guidance and all designs below will not apply to all areas. The final solution will need to take account of the needs and opportunities that the location topography offers.

Appendix 3 – Footpath Maps

Part 2

Part 1

Footpath 2419

Kissing Gate Location  -

Footpaths 2423 & 2424

Part 2

Part 1

Kissing Gate Location  -