RFQ049 Appendix B, Lot 2 Bio Science Laboratory

Please see below the specifications for the following

- Digital microscopes

- Laminar Flow Cavinet

Digital microscopes

- Total magnification: 40x, 100x, 400x

- Eyepiece: Wide Field 10x/18mm

- Head: Monocular digital observation head, 360° rotating, with 45° inclined eyepiece tube

- Nosepiece: Quadruple revolving nosepiece, rotation on ball bearings

- Objectives: Anti-fungal treated achromatic DIN

- 4x A.N. 0.10, W.D. 18mm

- 10x A.N. 0.25, W.D. 7mm

- 40x A.N. 0.65, W.D. 0.53mm

- Stage: : Double layer with mechanical sliding stage with specimen holder, 125 x 116mm. X-Y movement range 76 x 30mm. Vernier scale on the two axes, accuracy 0.1mm

- Focussing: Coaxial coarse and fine focusing mechanism (graduated 0.002mm) with upper stop. Adjustable tension of coarse focussing knob

- Condenser: Abbe condenser, N.A. 1.2 pre-centred

- Illumination: 1W LED with adjustable intensity

- LED Details: 1W, 6300K colour temperature, average lifetime approx. 50000h

- Dimensions: H350 x W170 x D210mm

- Weight: 4kg

- Voltage: External power supply 100/240V, 50/60Hz, output 6V D.C. 2.5A. Internal NiMH rechargeable battery (up to 15 hours of use from a single charge, LED at medium intensity)

- Accessories included: Power supply, OPTIKA Vision Lite software disc, 1.8m USB cable, dust cover and instructions

Digital Camera Specification:

- Resolution: 1280 x 1024 pixels, 5:4 aspect ratio

- Pixel size: 2.8μm x 2.8μm

- Sensor: 1/3.2”CMOS

- Output: USB 2.0 port

- Framerate: 15fps (1280 x 1024) or 30 fps (640 x 480)

- S/N Ratio: 42.3dB

- Dynamic Pro Range: 71dB

- Exposure Time: 20s - 1/1000s

- Effective Digits: 10bit - on chip

- Sensitivity: 1.0 V/lux-sec (550nm)

- Software: OPTIKA Vision Lite for Windows XP, Vista, 7, 8 (32/64bit)

Laminar flow cabinet

- Hood Width (in)39

- Exterior Width (in)39.5

- Exterior Height (in)53

- Exterior Depth (in)24.75

- Interior Width (in)38

- Interior Height (in)34

- Interior Depth (in)21.5

- Power (VAC)115

- Power (Hz)60

- Exhaust Volume (CFM)135

- Description39" Laminar Ductless Fume Hood, 115V

Microcentrifuge

This compact mini-centrifuge is ideal for quick spin downs from tube walls and caps.

- Use to microfilter cell separations and HPLC samples.

- Most spins last around 20 seconds,

- Operation is extremely simple load tubes, close lid and press on/off button. For repeated spins, simply open or close the lid to operate.

- Internal safety switch ensures unit will not operate with lid open.

- The rubber footplate includes soft feet to increase stability and deaden vibrations. Remove cord for compact storage. Unit is CE marked and complies with RoHS directive.

Specifications:

Maximum speed 6000 rpm

Maximum RCF 2000 x g

Dimensions 153 x 128 x104 mm

Power:220/230 VAC, 50/60 Hz