CHOBHAM AND WEST END BURIAL COMMITTEE

Specifications for works to be carried out at Chobham Burial Ground. Please see attached plan of the cemetery.

- New paths to be installed as shown on attached plan:

All new footpaths to be installed between section J and B, I and J, H and I, and the hedge on western boundary and section H.

Specification for these footpaths:

To excavate for the footpath to a width of 1.5m and depth of 150mm and to tip all surplus excavated materials on site.

To supply and lay bullnose concrete path edging set in concrete.

To supply and lay Terram membrane and cover with type 1 limestone and roll.

To top off with 20mm of 10mm pea shingle.

- New footpath along southern boundary of the new burial ground:

To install a new path south of sections H, I, J and B.

To excavate for the footpath to a width of 1.5m and depth of 150mm and to tip all surplus excavated materials on site.

To supply and lay bullnose concrete path edging set in concrete.

To supply and lay Terram membrane and cover with type 1 limestone and roll.

To top off with 20mm of 10mm pea shingle.

- Widen existing footpath between sections K and A:

To widen existing footpath by 1.5 m

To excavate area to a depth of 200mm and to tip all surplus excavated materials on site.

To supply and lay bullnose concrete path edging set in concrete.

To supply and lay Terram membrane and cover with type 1 limestone and roll.

To top off with 20mm of 10mm pea shingle.

- To move water supply:

To excavate a trench approx. 50m x 750mm deep and pick up existing water supply. To supply and lay 25mm water pipe with a stand pipe on end of run in new bin area.

- To build a small bin area, similar to one in existing cemetery and provide two bins.

- To extend the car park

To remove grass area and border shrubs to increase the size of the car park:

To excavate area approximately 33m x 3m x 200mm deep and tip excavated material on site.

To supply and lay bullnose concrete path edging set in concrete.

To supply and lay Terram membrane and cover with type 1 limestone and roll.

To top off with 20mm of 10mm pea shingle.